3GPP TSG-RAN WG4 Meeting # 114bis	R4-2503876

Wuhan, China, 07 - 11, April, 2025

Agenda item:	7.22.3.2

Source: 	Huawei, HiSilicon

Title: 	Discussion on RRM impact for Rel-19 MIMO on other aspect

Document for:	Discussion

- Introduction

The Rel-19 MIMO WI is approved in [1], In this contribution, we further provide our views regarding the RAN4 impacts for the WI.

- Discussion

UE reporting enhancement for CJT calibration

In the last meeting, some simulation assumptions have been updated. The updated info captured from WF[2]:

Agreement: - RAN4 to decide on the number of samples based on its impact on the reported interval (granularity of the report) for the given report resolution, while using the following parameters - Report resolution (MD = 128, MFO = 128). - Offset range (AD and AFO) - AD = CP - AFO = 0.2 ppm - Time offset between the two TRPs: 0.5 CP - Frequency offset between the two TRPs: 0.1 ppm - Note that report resolution and offset range are network configuration via RRC signalling. - Simulation results can be provided based on the CDF accuracy at 5%, 50% and 95%.

In this part, we want to share our simulation result about CJT calibration. The simulation results can be found in the R4-2503877.

Enhancement for asymmetric DL sTRP/UL mTRP scenarios

For the objective for asymmetric DL sTRP/UL mTRP, the open issues are summarized as follows [2]:

Issue 2-1: Whether RAN4 need to consider the scenario when UE is/isn’t configured with pathloss offset? Agreement: For Scenario#1: RAN4 define the timing requirements for FR1, FFS on FR2. For Scenario#2, FFS on whether to reuse the legacy timing requirements (Rel-18 m-DCI two TA with updated or not). RAN4 common understanding: the RAN1 agreements in RAN1#118bis “One downlink reference timing is supported and applied to both TAGs.” are currently only applied for Scenario #1. - Scenario#1: if UE is configured with PL offset in joint/UL TCI state(s), UE does not expect to receive SSB from UL TRP(s) - Scenario#2: if UE is not configured with PL offset in joint/UL TCI state(s), UE expect to receive SSB from UL TRP(s). Note 1: the scenario 1 and 2 are based on RAN1 LS. Note 2: The above agreements apply when 2TA are configured. Issue 2-2: How to update the RRM requirements for 2TA enhancement for asymmetric DL sTRP/ UL mTRP? Agreement: - For scenario 1, for both TAGs, the uplink transmission(s) timing for PUCCH/PUSCH/SRS take place (NTA + NTA offset) x Tc before the reception of the first detected path in time of one of the corresponding downlink reference signal(s) of the reference cell associated with DL or Joint TCI state. - The above agreement can be revisited if not aligned with RAN1 further agreement. Issue 2-3: Whether to define UL TCI state activation requirement based on SRS? Way forward: Further discuss the two proposals and down-select one proposal in the next meeting: - Proposal 1 (Qualcomm, Huawei, MediaTek, Samsung, Apple, QC): No RRM impacts on TCI state switching requirement. - Proposal 3 (Xiaomi, Nokia): - Legacy DCI based UL TCI state activation requirement can be re-used for UL TRP. - RAN4 only to define MAC CE based UL TCI state activation requirement based on known TCI. The legacy MAC CE based delay requirement can be used as baseline. - For DL-TRP, legacy UL TCI state activation requirement based on DL-RS can be re-used. no need to define SRS based ULTCI state activation delay. - Further refine the wording for P3 in the WF, if needed

In the last meeting, RAN4 sent an LS[3] to RAN1 to check the effect of the agreement from the RAN 1 #118bis meeting on the scenario that UE is not configured with PL offset in joint/UL TCI state(s). RAN 4 waits for RAN1's conclusion or clarification regarding the sent LS.

Proposal 1: Wait for RAN1's conclusion or clarification regarding the sent LS.

Another issue is about whether there is TCI state requirements impacts for TCI state switching. The controversial part is whether to whether to define UL TCI state switching requirements based on SRS as for some cases only SRS can be the QCL source.

As commented in last meeting, RAN4 do not define requirements when QCL source is the SRS. The similar discussion happened in Rel-16 RRM enhancement [4] and Rel-17 feMIMO [5].

R4-2005340 - No requirement is needed for spatial relation info switching associated with UL SRS. - No requirement is needed for MAC CE based spatial relation info switching associated with SRS for PUCCH. - No requirement is needed for RRC based spatial relation info switching associated with SRS for P-SRS. R4-2120320 - Define the following requirements - DCI and MAC-CE based TCI switching delay requirements - DL/UL TCI switching requirements for both joint TCI and separate TCI. - TCI switching delay requirements for the case when target TCI is associated with one of serving cell or non-serving cell (i.e., cell with different PCI) - Define requirements at least for Rel-17 TCI state switching framework - UL TCI state switching delay requirements when target TCI is associated with DL-RS - Pathloss RS switch delay requirement for beam alignment case

If we look at the discussion of these WIs, the considerations are the same, for instance, there is no time tracking/measurement needed for TCI state switching associated with SRS, and no need to verify the behaviour of Tx sweeping. There is also a proposal to define the requirements as MAC CE decoding time. But it seems redundant which is already covered in RAN1/2 spec. Besides, we may need to consider the known/unknown conditions.

In summary, this is exactly the same discussion that happened in Rel-16/Rel-17. If companies really want to consider the UL TCI state switching requirements associated with SRS, it should be overall considered as a whole. Otherwise, it will result in only defining requirements for UL TCI state switching associated with SRS only for this particular case.

Observation 1: The same discussion happened in Rel-16 RRM and Rel-17 feMIMO about whether to define requirements when the TCI state is associated with SRS. The conclusion is not to have requirements.

Proposal 2: No RRM impacts on TCI state switching requirement.

- Conclusions

Proposal 1: Wait for RAN1's conclusion or clarification regarding the sent LS.

Observation 1: The same discussion happened in Rel-16 RRM and Rel-17 feMIMO about whether to define requirements when the TCI state is associated with SRS. The conclusion is not to have requirements.

Proposal 2: No RRM impacts on TCI state switching requirement.

References

[1] RP-242394 WID: NR MIMO Phase 5

[2] R4-25 02596 WF on RRM requirements for NR_MIMO_Ph5_Part2

[3] R4-2502667 LS on DL timing reference to support two TA enhancement for asymmetric DL sTRP/ UL mTRP

[4] R4-2005340 WF on R16 NR RRM enhancements - UL Spatial Relation Info Switching

[5] R4-2120320 WF on RRM impact for unified TCI in FeMIMO