3GPP TSG-RAN WG4 Meeting #114-bis	R4-2503905

Wuhan, China, 7 – 11 April, 2025

Title: 	Discussion on other RRM requirements for Rel-19 NTN

Source: 	Huawei, HiSilicon

Agenda item:	7.29.4.2

Document for:	Discussion

Introduction

RRM requirements for other requirements in NTN are discussed in RAN4#114, and the outcomes are captured in WF [1]. Based on the discussions, RAN4 needs to further discuss the following issues for DL coverage enhancement.

- SSB periodicity extension

In this paper, we will provide our views on RRM requirements for DL coverage enhancement in NTN.

Discussion

SSB periodicity extension

RRC Re-establishment

In RAN1#120 the following is agreed related to SSB periodicity extension. It can be seen that besides the existing default SSB periodicity of 20ms, another default SSB periodicity of 160ms is supported.

Agreement For NR NTN, support extended periodicity of the half frames with SS/PBCH blocks assumed by UE during initial access. - The additional default value assumed by UE during initial access (apart from the existing 20ms value) is 160 ms.

In RAN4#114 following agreements and issues are captured in [1].

## Issue 1-2-A: RRC Re-establishment (default TSMTC,i) Agreement (online): - In RRC re-establishment requirements, update is needed to the TSMTC,i value for case where the SMTC for the inter-frequency carrier is not provided. Further discussion: - In RRC re-establishment requirements, update the value of TSMTC,i to 160ms for case where the SMTC for the inter-frequency carrier is not provided. - FFS: whether the above will apply only for UEs capable of coverage enhancement feature, if such a UE capability is defined. - FFS: whether the above is for only inter-frequency or for both intra-frequency and inter-frequency ## Issue 1-2-B: RRC Re-establishment (SSB Ês/Iot < -8dB and SSB periodicity >20 ms) Further discussion: - In RRC re-establishment requirements, discuss whether any update is needed to the TSMTC,i value for case where the SMTC for the inter-frequency carrier is not provided: - Option 1: Yes, an update is needed - Note: With the understanding that the by-default SSB periodicity for initial access is 160ms for Rel-19, from RRM requirement perspective. - Companies are encouraged to provide details on how to update the specification. - Other option is not precluded.

We copied the relevant requirements for RRC Re-establishment from 38.133 as below. There are two issues discussed in RAN4#114.

- Issue 1-2-A is for the default SMTC periodicity related to the yellow highlighted texts, and

- Issue 1-2-B is for requirements for the case with serving cell SSB Ês/Iot < -8dB related to the cyan highlighted texts.

TSMTC: It is the periodicity of the SMTC occasion configured for the intra-frequency carrier. If the UE has been provided with higher layer in TS 38.331 [2] signaling of smtc2, Tsmtc follows smtc1 or smtc2 according to the physical cell ID of the target cell. TSMTC,i: It is the periodicity of the SMTC occasion configured for the inter-frequency carrier i. If it is not configured, the UE may assume that the target SSB periodicity is no larger than 20 ms. Table 6.2C.1.2.1-1: Time to identify target NR cell for RRC connection re-establishment to NR intra-frequency cell Serving cell; FR of target NR; Tidentify_intra_NR [ms]. SSB Ês/Iot (dB); cell; Known NR cell; Unknown NR cell. ≥ -8; FR1; MAX (200 ms, 5 x TSMTC); Kmulti_SMTC * MAX (800 ms, 10 x TSMTC). < -8; FR1; N/A; k * 800 ms Note1, 3. Note 1: The UE is not required to successfully identify a cell on any NR frequency layer when TSMTC > 20 ms and serving cell SSB Ês/Iot < -8 dB. Note 2: Kmulti_SMTC is defined in clause 9.2C.5.1. Note 3: k = 1 if the cells on the target frequency are served by GEO. k = (N+1) if the cells on the target frequency are served by LEO, where N is the number of different satellites associated to the list of configured neighbor cells in ntn-NeighCellConfigList and ntn-NeighCellConfigListExt.. Table 6.2C.1.2.1-2: Time to identify target NR cell for RRC connection re-establishment to NR inter-frequency cell Serving cell SSB Ês/Iot (dB); FR of target NR cell; Tidentify_inter_NR, i [ms]. Known NR cell; Unknown NR cell. ≥ -8; FR1; MAX (200 ms, 6 x TSMTC, i); K_satellite * MAX (800 ms, 13 x TSMTC, i). < -8; FR1; N/A; k * 800 ms Note1, 3. NOTE 1: The UE is not required to successfully identify a cell on any NR frequency layer when TSMTC,i > 20 ms and serving cell SSB Ês/Iot < -8 dB. NOTE 2: K_satellite is defined in clause 9.3C.4. NOTE 3: k = 1 if the cells on the target frequency are served by GEO. k = (N+1) if the cells on the target frequency are served by LEO, where N is the number of different satellites associated to the list of configured neighbour cells in ntn-NeighCellConfigList and ntn-NeighCellConfigListExt..

For Issue 1-2-A, we support to update the value of TSMTC,i to 160ms for case where the SMTC for the inter-frequency carrier is not provided. The reason is that there is no dedicated configuration for RRC Re-establishment, and UE may search an inter-frequency carrier for RRC Re-establishment which has not been configured for mobility measurement, i.e. the carrier is without SMTC.

The first FFS from RAN4#114 is related to the applicability of 160ms default SSB periodicity. In our view, this issue is under discussion in RAN1. From UE perspective, the question is whether all R19 UEs will support 160ms default SSB periodicity or it depends on UE capability (declaration). From NW perspective, the question is whether 160ms default SSB periodicity should be assumed by applicable UE in all NWs or it depends on e.g. which bands the NW belongs to. We suggest to leave the discussion to RAN1 to avoid duplicated discussion in multiple WGs.

The second FFS from RAN4#114 is for intra-frequency. The question is whether it is possible that the intra-frequency carrier is not configured for mobility measurement thus SMTC is not provided. In our view, this is in theory possible but should be a corner case. As the case was not considered in TN (default SMTC period is only defined for inter-frequency), we suggest to also not consider it for NTN.

Proposal 1: For the case where SMTC is not provided for the inter-frequency carrier, update the default value of TSMTC,i to 160ms.

- Exact applicability condition for the update from UE and NW perspective is pending on RAN1

- Do not define default value of TSMTC for the inter-frequency carrier

For Issue 1-2-B, we support to remove Note 1 in Table 6.2C.1.2.1-1 and Table 6.2C.1.2.1-2

In current requirements, the assumption is that when serving cell SSB Ês/Iot is < -8dB, serving cell timing may be unreliable, and UE is supposed to do blind search. However, UE is only required to do blind search based on 20ms SSB periodicity, and there is no requirement when configured SMTC periodicity is > 20ms. In R19 NTN, the restriction in Note 1 should be removed because some NW may transmit SSB with 160ms periodicity, and some UE is supposed to do blind search based on 160ms SSB periodicity in those NW.

With Note 1 removed, the value of TSMTC and TSMTC,i would be

- the actually configured SMTC periodicity (up to 160ms) if SMTC is provided for the carrier, and

- the default SMTC periodicity of 160ms if SMTC is not provided for the carrier

It is noted that in existing requirements, when SMTC is not provided for the carrier, the SMTC is assumed to be no larger than 20ms, so the requirements (k * 800 ms) apply for such carriers when serving cell SSB Ês/Iot is < -8dB. Following same principle, we will also have requirements for carriers without SMTC when serving cell SSB Ês/Iot is < -8dB, based on 160ms SSB periodicity.

Proposal 2: For the case where serving cell SSB Ês/Iot is < -8dB, remove Note 1 in Table 6.2C.1.2.1-1 and Table 6.2C.1.2.1-2, and the value of TSMTC and TSMTC,i is

- the actually configured SMTC periodicity (up to 160ms) if SMTC is provided for the carrier,

- the default SMTC periodicity of 160ms if SMTC is not provided for the carrier

Next, RAN4 needs to discuss the exact delay requirements for when serving cell SSB Ês/Iot is < -8dB and TSMTC or TSMTC,i is > 20ms. In this case, UE is supposed to do blind search based on SSB periodicity >20ms.

RAN4 has defined requirements (800ms) for blind search based on SSB periodicity of 20ms. The rationale behind the existing values are provided in [2], and the related part are copied below.

The simulation results have shown that when SMTC offset is known then the UE can detect the target cell within 3 attempts at SCH Es/Iot ≥ -6 dB [3]. But without using the timing information, the NR cell detection will be similar for inter-RAT SFTD measurement, and would require 5 SMTC periods for each attempt to capture PSS/SSS [1-2]. We assume that the UE after losing the serving cell detects the target NR cell assuming 20 ms SMTC periodicity. The cell search shall be done once every 20 ms by assuming 5 ms radio time. Then one additional SMTC is needed between each attempt to take into account the fact that due to resource restrictions (cell search is done over 5ms radio time/20ms) there is restriction on how the offset can be changed. Therefore assuming 3 attempts to detect the PSS/SSS of an unknown intra-frequency NR cell, total number SMTC will be: 3*5 SMTC periods + 2 SMTC periods = 17 SMTC periods. Then 5 additional SMTC periods are needed for SS-RSRP measurement leading to total cell search time of up to 22 SMTC periods for FR1. For FR2, the intra-frequency cell search time needs to be scaled (multiplied) with a scaling factor (K3). …… - Proposal # 3: When the UE has lost serving cell timing (i.e. SCH Ês/Iot < X) the UE shall be able to identify a known intra-frequency FR1 cell within MAX (800 ms, [22] x TSMTC) and an unknown intra-frequency FR2 cell within MAX (1000 ms, (K3 x [22]) x TSMTC)) for RRC re-establishment to that cell. - Proposal # 4: When the UE has lost serving cell timing (i.e. SCH Ês/Iot < X) the UE shall be able to identify a known inter-frequency FR1 cell within MAX (800 ms, [25] x TSMTC, i) and an unknown inter-frequency FR2 cell within MAX (1000 ms, (K5 x [25]) x TSMTC, i)) for RRC re-establishment to that cell.

It is straightforward to follow the same derivation for other default SSB periodicities, i.e. Tidentify_intra_NR: MAX (800 ms, 22 x TSMTC) for intra-frequency carrier and Tidentify_inter_NR,i: MAX (800 ms, 25 x TSMTC,i) for intra-frequency carrier.

One issue RAN4 needs to discuss is the definition of TSMTC. In our view, it should be the periodicity with which NW transmits SSB, instead of the SMTC periodicity as in Proposal 2 or the periodicity UE assumes for initial access. The reason is that even UE assumes 160ms SSB periodicity for blind search, it should be able to find a suitable cell when it searches for the first 40ms in an attempt, if NW transmits SSB with 40ms periodicity. According to the following UE behaviour in 38304 for cell selection, UE is only required to select to a suitable cell but is not required to find the best cell. Therefore, once UE finds a suitable cell based on blind search for the first 40ms, it can start the RRC re-establishment procedure towards that cell.

### 5.2.3 Cell Selection process #### 5.2.3.1 Description Cell selection is performed by one of the following two procedures: a) Initial cell selection (no prior knowledge of which RF channels are NR frequencies): 1. The UE shall scan all RF channels in the NR bands according to its capabilities to find a suitable cell. 2. On each frequency, the UE need only search for the strongest cell, except for operation with shared spectrum channel access where the UE may search for the next strongest cell(s). 3. Once a suitable cell is found, this cell shall be selected.

Proposal 3: Update the delay requirements for RRC re-establishment for the case of Serving cell SSB Ês/Iot is -8dB as

- Tidentify_intra_NR: MAX (800 ms, 22 x TSMTC) for intra-frequency carrier

- Tidentify_inter_NR,i: MAX (800 ms, 25 x TSMTC,i) for inter-frequency carrier

where TSMTC is the periodicity with which NW transmits SSB.

Cell Reselection

## Issue 1-1: Cell Reselection Further discussion: - In cell reselection requirements, discuss whether any update is needed for case where SSB periodicity is larger than 20 ms: - Option 1 (Samsung): FFS - Option 2 (vivo): No update is needed

In last meeting, some companies suggested to discuss the cell reselection requirements. The concern is that “Observe that for enhanced measurements, when the DRX Cycle is equal to 0.32 s, only two SSB periods (of 160 ms) can fit one DRX Cycle, and the UE shall perform measurements across both of these opportunities necessarily in order to achieve the requirements for Tmeasure,NR_xx_enh/2”. We do not see this is an issue because 1) SMTC of 160ms is already possible based on existing spec and 2) the samples for filtering does not need to be within same DRX cycle, e.g. UE should be allowed to take one sample per DRX cycle.

Proposal 4: Do not update the requirements for cell reselection for default SSB periodicity extension.

Paging

## Issue 1-5: Paging Further discussion: - Further discuss whether any update is needed to paging interruption requirements for NGSO scenarios when the SSB periods is equal to 160 ms: - Option 1 (Huawei, vivo): no update is needed - Option 2 (Nokia): reduce the multiplying factor K in 4.2C.2.5

In last meeting, some companies also suggested to update the paging interruption requirements. In our understanding, the existing interruption requirements, which are based on configured SMTC periodicity, should apply for all configurable SMTC periodicities including 160ms. We can see the point that the paging interruption time becomes longer with larger SMTC periodicity, but this is the consequence from the trade-off for better coverage, and we do not think RAN4 should tighten the requirements for this trade-off.

Proposal 5: Do not update the requirements for paging interruption for default SSB periodicity extension.

Conclusions

In this paper we provided our views on RRM requirements for DL coverage enhancement in NTN.

Proposal 1: For the case where SMTC is not provided for the inter-frequency carrier, update the default value of TSMTC,i to 160ms.

- Exact applicability condition for the update from UE and NW perspective is pending on RAN1

- Do not define default value of TSMTC for the inter-frequency carrier

Proposal 2: For the case where serving cell SSB Ês/Iot is < -8dB, remove Note 1 in Table 6.2C.1.2.1-1 and Table 6.2C.1.2.1-2, and the value of TSMTC and TSMTC,i is

- the actually configured SMTC periodicity (up to 160ms) if SMTC is provided for the carrier,

- the default SMTC periodicity of 160ms if SMTC is not provided for the carrier

Proposal 3: Update the delay requirements for RRC re-establishment for the case of Serving cell SSB Ês/Iot is -8dB as

- Tidentify_intra_NR: MAX (800 ms, 22 x TSMTC) for intra-frequency carrier

- Tidentify_inter_NR,i: MAX (800 ms, 25 x TSMTC,i) for inter-frequency carrier

where TSMTC is the periodicity with which NW transmits SSB.

Proposal 4: Do not update the requirements for cell reselection for default SSB periodicity extension.

Proposal 5: Do not update the requirements for paging interruption for default SSB periodicity extension.

Reference

- R4-2502605, WF on RRM requirements for NR_NTN_Ph3_Part2, Moderator (Qualcomm Incorporated)

- R4-1810777, Analysis of RRC Re-establishment without Serving Cell Timing, Ericsson