3GPP TSG-RAN WG4 Meeting #114-bis	R4-2503909

Wuhan, China, 7th – 11th April 2025

Source:	Ericsson

Title:	On general issues for Rel-19 MMSE-IRC receiver under interference

Agenda Item:	7.18.1

Document for:	Discussion

# 1	Introduction

During the RAN#104 meeting, the revised WID on NR Demodulation Performance: Phase 5 was approved [1]. The WID outlines the following objectives for the UE and BS components:

- Define the following UE performance requirements for 8Rx CPE/FWA/vehicle/industrial devices: - PDSCH demodulation and CQI reporting requirements with inter-cell interference with MMSE-IRC receiver - Reuse the Rel-17 interference model for 2/4Rx UE as a baseline - PDSCH demodulation requirements with intra-cell inter-user interference with MMSE-IRC receiver - Reuse the existing interference model for 2/4Rx UE as a baseline - Define FR1 PUSCH performance requirements with inter-cell interference in homogeneous and heterogeneous deployment based on the following assumptions: - Receiver type: MMSE-IRC receiver - Scenarios: - FDD synchronous deployments - TDD synchronous deployments with aligned UL:DL configurations among cells - Slot-based transmission and aligned SCS among cells - Reuse LTE based interference profile in TR 36.884 as a starting point. Other interference profiles are not precluded.

In this contribution, we provided our view on general issues for UE and BS demodulation requirement with interference.

# 2	Discussion

In this section, we address the general issues related to the UE performance requirements for 8Rx with interference and BS performance requirements with IRC.

Manufacturer declaration on MMSE-IRC receiver

- Candidate options:

  - Proposal 1: Introduce new manufactory declaration for BS MMSE-IRC receiver

| Declaration identifier | Declaration | Description | Applicability |  |
| --- | --- | --- | --- | --- |
|  |  |  | BS type 1-C | BS type 1-H |
| D.xxx | MMSE-IRC receiver | Declaration of BS supporting MMSE-IRC receiver. | x | x |

  - Proposal 2: Do not need to introduce a new Manufacturer Declaration on MMSE-IRC

Applicability

  - Candidate options on test requirement applicability for different BS classes and test scenarios:

- Proposal 1: HomNet requirements are not applicable for Local Area BS and HetNet requirements are not applicable for Wide and Medium Area BS

- Other options are not precluded.

  - Based on test requirement applicability, further discuss whether to consider additional applicability rule on how the test is performed:

- Proposal 1: If a BS declared to support Wide Area BS, Medium Range BS and pass the tests for HomNet (DIP Set1), the test for HetNet (DIP Set 2) can be skipped

- Proposal 2: If a BS declared to support HomNet or HetNet and pass the tests for HomNet, can also consider the test for HetNet as passed

- Proposal 3: Do not define additional applicability rule for homogenous and heterogenous network scenarios

2.1	BS performance requirements

According to [2], both homogeneous and heterogeneous network scenarios have been agreed upon for defining BS requirements for Rel-19 MMSE-IRC. Since this not a mandatory feature, but an optional one, consequently, a manufacturer declaration should be introduced, followed by the appropriate applicability in Table 4.6 of TS 38.141-1 for BS types 1-C and 1-H.

Proposal 1: Introduce new manufactory declarations and applicability rules.

Table 4.6-1 Manufacturer declarations for BS type 1-C and BS type 1-H conducted test requirements

| Declaration identifier | Declaration | Description | Applicability |  |
| --- | --- | --- | --- | --- |
|  |  |  | BS type 1-C | BS type 1-H |
| D.xxx | MMSE-IRC receiver | Declaration of BS supporting MMSE-IRC receiver. | X | X |

Proposal 2:

8.1.2.1.x.x	Applicability of PUSCH with synchronous interference

Unless otherwise stated, PUSCH with synchronous interference requirements shall apply only for a BS declare to support MMSE-IRC receiver.

For applicability rules in different test scenarios, in LTE (TS 36.141 Sec. 8.2.6.5), a note was added to limit enhanced PUSCH performance requirements to Wide Area BS under synchronous interference. However, a similar approach would require adding a note in multiple test requirement tables for PUSCH MMSE-IRC in TS 38.141-1 Sec. 8 for NR. A more efficient way would be to declare the applicability rule once in the beginning of the Sec. 8.2.x as given in proposal 1 under applicability rule open issues:

Proposal 3:

8.2.x Performance requirements for PUSCH with synchronous interference

8.2.x.1 Definition and applicability

The performance requirements for PUSCH with synchronous interference only apply to Wide and Medium Area Base Stations for INR set 1, and Local Area Base Station for INR set 2 based on manufactory declaration D.2 in Table 4.6-1 (refer to Table 8.2.x.x.x-x for INR set 1 and 2). The requirements apply to the BS receiving the synchronous interference i.e. the interference is time-synchronous with the tested signal.

Further, mentioning both INR sets corresponding to homogeneous and heterogeneous network scenarios explicitly regarding the applicability seems more feasible to prevent any implicit assumptions.

Also, in LTE (TS 36.141 Table 8.2.6.4.2-2, Note 2), a note in the test parameters to define the DIP value and corresponding DIP set 1 and set 2 is added, for homogenous and heterogenous network scenarios, respectively. However, INR-based interference modeling is agreed in the previous meeting [3]. Therefore, for NR TS 38.141-1 and TS 38.104, a similar approach like LTE can be followed adding a note in the test parameters.

Proposal 4: Adding a note to PUSCH with synchronous interference test parameters to define INR value and corresponding set 1 and set 2 for respective homogenous and heterogenous scenarios.

Table 8.2.x.x.x-x: Test parameters for testing PUSCH with synchronous interference

| Parameter |  |  |  | Value |
| --- | --- | --- | --- | --- |
| Transform precoding |  |  |  | Disabled |
| Default TDD UL-DL pattern (Note 1) |  |  |  | 15 kHz SCS: 3D1S1U, S=10D:2G:2U 30 kHz SCS: 7D1S2U, S=6D:4G:4U |
| HARQ |  | Maximum number of HARQ transmissions |  | 4 |
|  |  | RV sequence |  | 0, 2, 3, 1 |
| DM-RS |  | DM-RS configuration type |  | 1 |
|  |  | DM-RS duration |  | single-symbol DM-RS |
|  |  | Additional DM-RS position |  | pos1 |
|  |  | Number of DM-RS CDM group(s) without data |  | 2 |
|  |  | Ratio of PUSCH EPRE to DM-RS EPRE |  | -3 dB |
|  |  | DM-RS port(s) |  | {0} |
|  |  | DM-RS sequence generation |  | NID0=0, nSCID =0 |
| Time domain resource assignment |  | PUSCH mapping type |  | A |
|  |  | Start symbol |  | 0 |
|  |  | Allocation length |  | 14 |
| Frequency domain resource assignment |  | RB assignment |  | Full applicable test bandwidth |
|  |  | Frequency hopping |  | Disabled |
| TPMI index for 2Tx two layer or 4Tx four layer spatial multiplexing transmission |  |  |  | 0 |
| Code block group based PUSCH transmission |  |  |  | Disabled |
| Interference (Note 2) | Set 1 (HomNet) | 1 interferer | INR1 (dB) | 5.36 |
|  |  | 2 interferers | INR1 (dB) | 7.29 |
|  |  |  | INR2 (dB) | -2.5 |
|  | Set 2 (HetNet) | 1 interferer | INR1 (dB) | 9.83 |
|  |  | 2 interferers | INR1 (dB) | 12.38 |
|  |  |  | INR2 (dB) | -0.97 |
| NOTE 1: The same requirements are applicable to FDD and TDD with different UL-DL patterns. NOTE 2: The respective received average power spectral density of each interferer cell relative to Noc(j) is defined by its associated INR value as specified in clause B.x.x. INR set 1 and set 2 are derived respectively in homogeneous and heterogeneous network scenarios. |  |  |  |  |

Since the applicability rules for different test scenarios are already defined independently in proposal 3, there is no need for an additional applicability rule. If a BS is declared to support Wide Area BS or Medium Range BS, as per the proposal 3, it needs to pass the test for INR set 1 only. Similarly, if the BS is declared to support Local Area BS, it is required to pass INR set 2 only.

Proposal 5: Do not define additional applicability rule for homogenous and heterogenous network scenarios. For a homogeneous network, both Wide Area BS and Medium Range BS, but not Local area BS are required to pass the test. In contrast, for a heterogeneous network, only Local Area BS needs to pass the test.

## 2.2	UE performance requirements

As for the MMSE- IRC receiver capability for 8Rx UE, following agreements have been made:

Issue 1-1-1: Definition of MMSE-IRC receiver capability for 8Rx UE Agreement: - If agreed to change the receiver definition, it also needs to be changed in Rel-18 maintenance. - If receiver definition is agreed to be changed, an LS will be sent to RAN2 for updating TS38.306.

Besides, there are still several options proposing the way of either revising the existing UE capability or define a new Note to make further clarification.

Proposals: - Option 1: Redefine the simplified and baseline receiver definitions in 38.101-4 section 3.1 to the following: - Baseline 8Rx Receiver: 8Rx receivers for MIMO transmissions with support of up to 8 layers with joint 8Rx MIMO detector. - Simplified 8Rx Receiver: 8Rx receivers for MIMO transmissions with support of only up to 4 layers with two joint 4Rx MIMO detectors. - Option 2: Redefine the simplified and baseline receiver definitions in 38.101-4 section 3.1 to the following: - Baseline 8Rx MMSE-IRC Receiver: 8Rx MMSE-IRC receivers for MIMO transmissions with support of up to 8 layers with joint 8Rx MIMO detector. - Simplified 8Rx MMSE-IRC Receiver: 8Rx MMSE-IRC receivers for MIMO transmissions with support of only up to 4 layers with two joint 4Rx MIMO detectors. - Option 3: Not to introduce new UE capability in Rel-19 or to update the existing Rel-18 UE capability definition. Instead, clearly state in the applicability rule that the new Rel-19 requirements under MU-MIMO are applicable for UE declares supporting SU-MIMO 8Rx receiver capability

In our view, it is more straightforward to simply redefine the existing UE capability to remove the description of ‘SU-MIMO’ in order to better fit the need of applying the MU-MIMO requirements. Even it is known that IRC receiver is a SU-MIMO receiver, it is still confusing with the phrase ‘SU-MIMO’ in the 8Rx receiver definition as if SU-MIMO is the only scenario considered for 8Rx receiver. Thus, we propose to redefine the definition in TS38.101-4 and update the UE capability.

Proposal 6: Option 1 for redefine the definition in TS38.101-4.

Regarding the receiver coverage, companies agreed to set a criteria used for deciding whether it is needed to define two sets of requirement. The actual value is still FFS

Issue 1-1-2: 8Rx UE MMSE-IRC receiver coverage Agreement: - Criteria for decision if one or two sets of PDSCH requirements are to be introduced: - Secure the span is <=2.5 dB for each receiver type by removal of outliers - If difference in the average of each receiver type for a specific configuration is > [TBD:limit] then define two sets of requirements for the specific configuration. - Further discuss the actual value for the [limit]. - Proposals: - Option 1: Set [limit] to 0.5dB - Option 2: Set [limit] to 1.0dB - Option 3: Set [limit] to 1.2dB - Other options not precluded.

According to our observation, the performance gap between the baseline receiver and simplified receiver for Rel-18 8Rx cases is around 2dB for rank 2 and 3dB for rank 4. We believe similar approach can be assume so that to set the criteria to 2dB.

Proposal 7: Set [limit] to 2dB.

# 3	Summary

In this contribution, we shared our views on the general issues related to the IRC receiver under interference scenario. Our proposals are summarized as follows:

Proposal 1: Introduce new manufactory declarations and applicability rules.

Proposal 2:

8.1.2.1.x.x	Applicability of PUSCH with synchronous interference

Unless otherwise stated, PUSCH with synchronous interference requirements shall apply only for a BS declare to support MMSE-IRC receiver.

Proposal 3:

8.2.x Performance requirements for PUSCH with synchronous interference

8.2.x.1 Definition and applicability

The performance requirements for PUSCH with synchronous interference only apply to Wide and Medium Area Base Stations for INR set 1, and Local Area Base Station for INR set 2 based on manufactory declaration D.2 in Table 4.6-1 (refer to Table 8.2.x.x.x-x for INR set 1 and 2). The requirements apply to the BS receiving the synchronous interference i.e. the interference is time-synchronous with the tested signal.

Proposal 4: Adding a note to PUSCH with synchronous interference test parameters to define INR value and corresponding set 1 and set 2 for respective homogenous and heterogenous scenarios.

Proposal 5: Do not define additional applicability rule for homogenous and heterogenous network scenarios. For a homogeneous network, both Wide Area BS and Medium Range BS, but not Local area BS are required to pass the test. In contrast, for a heterogeneous network, only Local Area BS needs to pass the test.

Proposal 6: Option 1 for redefine the definition in TS38.101-4.

Proposal 7: Set [limit] to 2dB.

# References

[1]	RP-241297: Revised WID on NR demodulation performance: Phase 5, China Telecom, NTT DOCOMO, RAN#104, Shanghai, China

[2] 	R4-2502272, “Way Forward for [114][323] NR_demod_Ph5_Part1_General_BS”, China Telecom, RAN4#114, Athens, Greece.

[3] 	R4-2416548, “WF on [112bis][319] NR_demod_Ph5_Part1_General_BS, RAN4#112-bis, Hefei, China.