# 3GPP TSG-RAN WG4 Meeting #114bis	R4-2503919

# Wuhan, Hubei, China, 07th – 11th April, 2025

Agenda item:	7.26.4.2

Source:	China Telecom

Title:	Discussion on RRM core requirements for LP-WUS/WUR

Document for:	Discussion

- Introduction

In RAN4 #114 meeting, RRM core requirements for LP-WUS/WUR was discussed and the WF was approved [1].

In this paper, we present our views on RRM core requirements for LP-WUS/WUR issues captured in the WF [1] and summary [2].

- Discussion

General aspects

Status in the summary [2]:

Issue 1-1-2: Threshold for switch between different cases - Proposals - P1: Up to RAN2 decision (CMCC CT) - P2: New LP-based threshold(s) based on LP-RSRP or LP-RSRQ used for switch into case 1 from other cases or exit from case 1 to other cases are needed (Apple ZTE vivo) - P2-1: LR measurement result shall be comparable to MR measurement result or shall be equivalent to MR measurement result with certain offset/margin (e.g., LR threshold is MR threshold + offset/margin). (Apple ZTE) - P2-2: Details on threshold are up to RAN2 (vivo) - P3: Discuss first what has been agreed in RAN2 (Nokia) - P4: RAN4 to study switching between Case#3 and Case#1 based on a timer. Since UE need to be confident enough to enter Case#1 before deactivating MR, it is better for the UE to enter Case#3 first for a period of time (confidence period) before UE can enter Case#1. (MTK) Recommendations: Check RAN2’s progress on this issue. Discuss whether new LP-based threshold(s) used for switch into case 1 from other cases or exit from case 1 to other cases are needed. Whether other cases include case 2 or not depends on issue 1-1-1, i.e., whether case 2 will be considered or not.

In our understanding, the threshold for switch between different cases is up to RAN2 decision.

Proposal 1: Threshold for switch between different cases is up to RAN2 decision.

Status in the summary [2]:

Issue 1-1-10: Considerations on higher priority frequency layer - Proposals - P1: Support relaxed higher priority frequency layer search at case 1 and case 3 (xiaomi CT ZTE Huawei vivo Ericsson CT) - P1-1: For case 1 and case 3, when the serving cell is above threshold for inter-frequency neighbour cell measurement, existing relaxed requirements K2*Thigher_priority_search are re-used for higher priority frequency search. (Ericsson vivo) - P1-2: If serving cell is above threshold for inter-frequency neighbor cell measurement (Case #4), FFS whether existing requirements (60 * Nlayers) are re-used or further relaxed. If serving cell is below threshold for inter-frequency neighbor cell measurement (Case #3), UE meet the relaxed measurement requirements. (Huawei) - P2: Higher priority layer neighbou cell measurement shall also be OFF for case #1. (Apple vivo) - P3: When LR and MR operates at different band, whether to turn on higher priority measurement depends on the solution for issue 1-1-8. If Solution 1 for issue 1-1-8 is adopted, then the higher priority measurement can be OFF. If Solution 2 or 3 for issue 1-1-8 is adopted, the higher priority measurement should be ON (CMCC) Recommendations: Study on the case when LR and MR at the same carrier frequency firstly, i.e., P1 and P2 firstly. Check P1 is agreeable.

In our views, higher priority frequency layer search can be performed in relaxed mode for case #1 and case #3.

Proposal 2: Higher priority frequency layer search can be performed in relaxed mode for case #1 and case #3.

Detail LP-WUR requirements at RRC_IDLE/INACTIVE state

Status in the summary [2]:

Issue 1-2-1: LR Accuracy requirements - Proposals - P1: No dedicated accuracy requirement is defined in the performance section for LR-WUR based RRM measurement in Idle/inactive states, and reflect the accuracy performance as a margin in the core requirement. (ZTE Apple xiaomi Ericsson Huawei CT vivo MTK) - P2: Option 1: define dedicated accuracy requirement in the performance section but no test case; Option 2: don’t define the accuracy requirement for LR-WUR based RRM measurement in Idle/inactive states, but use it as the simulation target when evaluate the measurement delay requirement simulation. (CMCC) - P3: To enable reliable return to MR from LR in a timely manner, it is necessary to have absolute measurement accuracy requirements defined for LR; RAN4 should develop LP-SS based and SSB based measurement accuracy requirements (Nokia) Recommendations: Check P1 is agreeable.

For accuracy requirements, since there is no measurement reporting to network for LP-WUR measurement at RRC_IDLE/INACTIVE state, it’s proposed to reflect the accuracy performance as a margin in the core requirement, no dedicated accuracy requirement is defined in the performance section for LP-WUR based RRM measurement in Idle/inactive states.

Proposal 3: No dedicated accuracy requirement is defined in the performance section for LR-WUR based RRM measurement in Idle/inactive states, and reflect the accuracy performance as a margin in the core requirement.

Status in the summary [2]:

Issue 1-2-2: Periodicity for LP-WUR RRM requirements - Proposals - P1: Regardless of whether LP-SS is configured or not, periodicity for SSB based LP-WUR measurement delay requirements is based on LO periodicity. (LG Apple CMCC oppo) - P2: For periodicity for SSB based LP-WUR measurement delay requirements, when LP-SS is configured, the periodicity for SSB based LP-WUR measurement delay requirements can be max { LO periodicity, LP-SS periodicity}. (CATT) - P3: For periodicity for SSB based LP-WUR measurement delay requirements, postpone the discussion for the case where LP-SS periodicity > LO periodicity until RAN1 has more agreements for LP-SS periodicity. (Huawei) - P4: For periodicity for SSB based LP-WUR measurement delay requirements, no requirement is defined if LO periodicity is smaller than LP-SS periodicity.(Ericsson) Background: RAN4 113 agreement: If LP-SS is not configured, periodicity for SSB based LP-WUR measurement delay requirements - based on LO periodicity - FFS whether to define an upper bound - Note: DRX cycle and LO periodicity are the same based on RAN1 agreement. When LP-SS is configured, apply the same agreement as for when LP-SS is not configured - The above agreement applies if LO periodicity is equal to or larger than LP-SS periodicity, FFS if LO periodicity is smaller than LP-SS periodicity. Issue 1-2-2-1: Lower bound on LP-SS measurement periodicity - Proposals - P1: In case the periodicity of LP-SS can be configured to small value like 80ms or 160ms (xiaomi) - Option 1: define the lower limit for measurement delay requirements; - Option 2: define the measurement delay requirements based on max(LP-SS periodicity, LO periodicity). - P2: Periodicity for measurement and evaluation delay requirements for LP-WUR based on LP-SS in IDLE/Inactive mode is 320ms if LP-SS periodicity 80ms and 160ms are determined to be included. (Huawei vivo) - P3: No need for lower bound, before RAN1 agrees other periodicities. (Nokia) Background: RAN4 112bis meeting agreement: Measurement delay requirements for LP-WUR based on LP-SS in IDLE/Inactive mode shall be defined based on LP-SS periodicity. Issue 1-2-2-2: Upper bound on SSB-based LP-WUR measurement periodicity - Proposals - P1: Do not define upper bound for SSB-based LP-WUR measurement periodicity. (xiaomi CMCC oppo Huawei) - P2: Prefer the periodicity for measurement and evaluation delay requirements for LP-WUR based on SSB in IDLE/Inactive mode is 1.28s if the LO periodicity is 2.56s. (vivo)

For periodicity for LP-WUR RRM requirements, when LP-SS is configured, if LO periodicity is equal to or larger than LP-SS periodicity, periodicity for SSB based LP-WUR measurement delay requirements is based on LO periodicity, and if LO periodicity is smaller than LP-SS periodicity, in our views, LP-SS periodicity is not very important for SSB based LP-WUR measurement, therefore, regardless of whether LP-SS is configured or not, periodicity for SSB based LP-WUR measurement delay requirements is based on LO periodicity.

Proposal 4: Regardless of whether LP-SS is configured or not, periodicity for SSB based LP-WUR measurement delay requirements is based on LO periodicity.

For upper bound on SSB-based LP-WUR measurement periodicity, since DRX cycle and LO periodicity are the same based on RAN1 agreement, and periodicity for SSB based LP-WUR measurement delay requirements is based on LO periodicity, in our views, it’s preferred not to define upper bound for SSB-based LP-WUR measurement periodicity.

Proposal 5: Do not define upper bound for SSB-based LP-WUR measurement periodicity.

Status in the summary [2]:

Issue 1-2-4: On LR measurement/evaluation requirements Issue 1-2-4-1: On LR measurement requirement - Proposals - P1: For LR measurement requirements, the following options are available (vivo) - Option 1: Define LR measurement requirement as the minimum duration where one LR measurement shall be executed, for example like “ The UE shall measure the LP -RSRP and LP -RSRQ level of the serving cell at least once every LP-SS periodicity for LP-SS based LR or [LO periodicity] for SSB based LR” - Option 2: Define LR measurement requirement as the minimum duration to meet accuracy requirement as [x] * LP-SS periodicity for LP-SS based LR or [y] * LO periodicity for SSB based LR, assuming [x] samples and [y] samples are number of samples to meet LP-SS based LR or SSB based LR accuracy requirement. Background: Recommendations: Issue 1-2-4-1: On LR measurement filtering requirement - Proposals - P1: The UE shall filter the LP-RSRP and LP-RSRQ and [SS-RSRP and SS-RSRQ] measurements of the serving cell using at least 2 measurements. Within the set of measurements used for the filtering, at least two measurements shall be spaced by either LP-SS periodicity/2 for LP-SS based LR or [LO periodicity /2] for SSB based LR. (Huawei Ericsson vivo) Background: Recommendations: Check whether P1 is agreeable

On LR measurement filtering requirement, it’s preferred to follow existing DRX case that each measurement should be based on filtering of at least 2 samples, i.e., the UE shall filter the LP-RSRP and LP-RSRQ and SS-RSRP and SS-RSRQ measurements of the serving cell using at least 2 measurements, within the set of measurements used for the filtering, at least two measurements shall be spaced by either LP-SS periodicity/2 for LP-SS based LR or LO periodicity /2 for SSB based LR.

Proposal 6: The UE shall filter the LP-RSRP and LP-RSRQ and SS-RSRP and SS-RSRQ measurements of the serving cell using at least 2 measurements. Within the set of measurements used for the filtering, at least two measurements shall be spaced by either LP-SS periodicity/2 for LP-SS based LR or LO periodicity /2 for SSB based LR.

Status in the summary [2]:

Issue 1-2-7: On LP-WUR cell selection evaluation requirements for case 1 - Proposals - P1: No need to define the LR-WUR based cell selection criterion evaluation in case 1. (Apple CMCC vivo Huawei MTK) - P2: Define LP-WUR based serving cell selection criterion S evaluation requirement in case 1. (CATT Ericsson) - P2-1：The existing MR based evaluation requirements can be the baseline (CATT)

In case 1, LP-WUR cell selection is used to trigger MR measurement, the exit conditions on LP-WUR achieve the same purpose as cell selection evaluation, therefore, no need to define the LR-WUR based cell selection criterion evaluation in case 1.

Proposal 7: No need to define the LR-WUR based cell selection criterion evaluation in case 1.

Status in the summary [2]:

Issue 1-2-8-1: Accuracy for normal or relaxed MR serving cell measurement - Proposals - P1: No new accuracy requirements will be introduced for MR either normal or relaxed serving cell measurement, - no margin will be introduced for MR serving cell measurement in specs. (vivo) - same margins as existing MR neighbour cell measurement apply (Huawei) - P2: To enable reliable offloading from MR it is necessary to have absolute measurement accuracy requirements defined for MR. The absolute measurement accuracy already defined for idle mode can be re-used when defining idle mode measurements for LR offloading purposes (Nokia) Recommendations: Check P1 is agreeable Issue 1-2-8-2: Accuracy for relaxed MR neighbour cell measurement - Proposals - P1: For relaxed MR measurement, the legacy accuracy requirements for MR are re-used. (CMCC Huawei vivo) Recommendations: Check P1 is agreeable

For Accuracy for relaxed MR serving cell measurement and neighbour cell measurement, no new accuracy requirements will be introduced for relaxed MR measurement, the legacy accuracy requirements for MR are re-used.

Proposal 8: No new accuracy requirements will be introduced for relaxed MR measurement, the legacy accuracy requirements for MR are re-used.

LP-WUR CONNECTED mode

Status in the summary [2]:

Issue 1-4-1: LP-WUR at CONNECTED mode - Proposals - P1: No RRM objectives is expected for connected mode in this WI. (CATT xiaomi oppo ZTE Apple CT MTK) - P3: Postponed until more progress in other groups (Ericsson Huawei vivo) - P4: RAN4 to introduce interruption requirements on the WUS monitoring in Connected mode, e.g., when the WUS MO collides with a MG. The interruption on WUS monitoring follows the current interruption requirements on DL data during MGs. (Qualcomm) Recommendations:

In our views, no RRM objective is expected for connected mode in this WI.

Proposal 9: No RRM objective is expected for connected mode in this WI.

- Conclusion

The contribution presents our views on Rel-19 RRM core requirements for LP-WUS/WUR, with the following proposals:

Proposal 1: Threshold for switch between different cases is up to RAN2 decision.

Proposal 2: Higher priority frequency layer search can be performed in relaxed mode for case #1 and case #3.

Proposal 3: No dedicated accuracy requirement is defined in the performance section for LR-WUR based RRM measurement in Idle/inactive states, and reflect the accuracy performance as a margin in the core requirement.

Proposal 4: Regardless of whether LP-SS is configured or not, periodicity for SSB based LP-WUR measurement delay requirements is based on LO periodicity.

Proposal 5: Do not define upper bound for SSB-based LP-WUR measurement periodicity.

Proposal 6: The UE shall filter the LP-RSRP and LP-RSRQ and SS-RSRP and SS-RSRQ measurements of the serving cell using at least 2 measurements. Within the set of measurements used for the filtering, at least two measurements shall be spaced by either LP-SS periodicity/2 for LP-SS based LR or LO periodicity /2 for SSB based LR.

Proposal 7: No need to define the LR-WUR based cell selection criterion evaluation in case 1.

Proposal 8: No new accuracy requirements will be introduced for relaxed MR measurement, the legacy accuracy requirements for MR are re-used.

Proposal 9: No RRM objective is expected for connected mode in this WI.

- References

- R4-2502600, WF on RRM requirements for NR_LPWUS, vivo, RAN WG4 Meeting #114, 17 – 21 February, 2025.

- R4-2500539, Topic summary for [114][225] NR_LPWUS, vivo, RAN WG4 Meeting #114, 17 – 21 February, 2025.