3GPP TSG-RAN WG4 Meeting #114bis	    R4-2503931

Wuhan, China, 07  – 11 April 2025

Agenda Item:	7.21.3

Source: 	Ericsson

Title:	On testing of AI/ML based mobility

Document for:	Approval

# Introduction

In this contribution testing issues related to AI/ML based mobility are discussed. Issues identified FFS in previous meetings and extraction of ground truth information in the tests are specifically discussed.

# Discussion

## Testing setup

In RAN4#113 meeting, setup for AI/ML based mobility testing was discussed and it was agreed to further discuss if it feasible to reuse the legacy testing setup for L3 measurements for testing AI/ML based mobility.

Agreement from RAN4#113 [1]

- RAN4 to further discuss:

  - Whether and how legacy testing setup for L3 measurements can be reused for FR1 and FR2

For AI/ML based mobility, two different types of scenarios are being considered. One of the scenarios refer to intra-cell scenario where the AI/ML model at UE takes the measurement from a cell to predict the RRM measurement for the same cell. This is a typical scenario for temporal prediction of RRM measurements. For example, scenarios 2 and 4 agreed in RAN4#112bis meeting. The other type of scenario is inter-cell scenario, where the AI/ML model at UE takes measurement from a cell in one carrier frequency to predict the RRM measurements for other cell(s) in another carrier frequency(ies). For example, scenario 3 agreed in RAN4#112bis meeting as one of the prioritized scenarios for RAN4 discussions.

Agreement from RAN4#112bis [2]

RAN4 to start the discussion for scenario 2, 3, 4. Further clarification can be discussed for the scenarios.

| 2 | High | FR1 to FR1 intra-frequency temporal domain case B | 1st goal | Intra-cell |
| --- | --- | --- | --- | --- |
| 3 | High | FR1 to FR1 inter-frequency (frequency domain) | 1st goal | Inter-cell |
| 4 | High | FR2 to FR2 intra-frequency temporal domain case A | 2nd goal | Intra-cell |

### Testing for intra-cell scenarios (temporal prediction)

Intra-cell scenarios characterize deployment of AI/ML model at UE to predict the RRM measurements in temporal domain, where the measurements performed within observation window (OW) are used by the UE to perdict measurements valid for time instances inside the prediction window (PW) for the same cell as shown in Figure 1. The UE may report the measurements performed inside the OW and/or the measurements predicted to be valid for the instances inside the PW depending on the measurement prediction condition/configuration received from the network. The aim of testing for intra-cell scenarios should therefore be to validate that the measurements reported by the UE for a cell meet:

- requirement for the measurements performed inside the OW,

- requirement for the measurements predicted for the time instances inside the PW.

From testing setup point of view, two-cell setup used to validate UE capability to meet requirements for the legacy L3-based mobility can be used to validate whether requirements for intra-cell predictions can be achieved when AI/ML model is deployed at the UE for the prediction of RRM measurements in temporal domain for mobility purposes. The two-cell setup is needed to also consider the impact of interference coming from the cell other than the measured cell on temporal predictions during testing.

Intra-cell temporal prediction case-A.

Intra-cell temporal prediction case-B.

Figure 1. Measurement timeline for intra-cell temporal prediction scenarios.

Proposal 1: Test setup for testing the AI/ML model deployed at UE to support intra-cell temporal prediction scenario is based on two-cell setup, similar to the test setup for legacy L3-based mobility.

Proposal 2: The aim of testing for intra-cell prediction should be to validate that the measurements reported by the UE for each cell in the test setup meet:

- requirement for the measurements performed inside the OW,

- requirement for the measurements predicted for the time instances inside the PW.

### Testing for inter-cell scenario

Inter-cell scenario characterizes deployment of AI/ML model to predict RRM measurement of a cell in one carrier frequency by exploiting the measurements performed by UE on another cell in another carrier frequency. The carrier frequency of measurement cell and the predicted cell belong to the same frequency region. The aim of testing should therefore be to validate that the measurements reported by the UE

- meet the requirements for measured cell, and

- meet the requirements for the target cell.

This scenario is similar to the inter-frequency measurement scenario supported for non-AI/ML mobility, where the UE is configured to perform and report measurement for two different cells that are in two different carrier frequencies. Test setup defined to validate requirements for inter-frequency measurements can therefore be re-used for testing AI/ML based mobility for inter-frequency scenario. While testing for AI/ML based mobility, cell 1 in the test setup can be considered as the cell for measurement and cell 2 in the test setup can be considered as the target cell for which the UE predicts the measurement for.

Proposal 3: The aim of testing for inter-cell scenario should be to validate that the measurements reported by the UE:

- meet the requirements for measured cell, and

- meet the requirements for the target cell.

Proposal 4: Existing test setup defined to validate requirements for inter-frequency measurements is re-used or used as a baseline for discussion on testing AI/ML based mobility for scenario 3.

- Cell 1 in the test can be considered as the cell for measurement.

- Cell 2 in the test can be considered as the target cell for measurement prediction.

## Relative RSRP Prediction Accuracy

In the last meeting definition for predicted relative RSRP accuracy was discussed and the agreement below was reached.

Agreement from RAN4#114 [3]

- Predicted relative RSRP accuracy = (reported predicted RSRP of cell 1/beam1 – reported RSRP of cell 2/beam2) – (ground truth of RSRP of cell 1/beam1 – ground truth of RSRP of cell 2/beam2)

  - FFS: Beam 1 and Beam 2 can be from same cell or different cells.

  - FFS: whether ground truth can be reported or ideal

  - FFS: whether the reported RSRP of cell2/beam2 can be predicted or measured or a combination of both predicted and measured samples.

One of the FFS items in the agreement from RAN4#114 meeting is whether beam 1 and beam 2 in the agreed formula can be from the same cell or different cells. For the legacy measurement, relative accuracy requirements are defined for the cases when the measurements are from the same cell and different cells on the same frequency and different frequencies. Similar approach can also be considered for the predicted relative RSRP accuracy definition. Meaning, beam 1 and beam 2 in the agreed formula for predicted relative RSRP accuracy can be from the same cell or different cells.

The other FFS item in the agreement above is related to whether the reported RSRP of cell 2/beam 2 can be predicted value or measured measured value or a combination of both predicted and measured values. When a UE is configured to report the measurements, the UE can report both measured and predicted measurements. From the requirement point of view, relative accuracy requirement should apply regardless of whether the reported measurements are measured or predicted. Therefore, the reported RSRP of cell2/beam2 in the predicted relative RSRP accuracy formula agreed in RAN4#114 can be predicted or measured RSRP values. Whether the reported RSRP of cell2/beam2 in test is predicted or measured RSRP values can be further discussed. Potentially when we are in the work item phase.

Proposal 5: Beam 1 and beam 2 in the predicted relative RSRP accuracy formula agreed in RAN4#114 can be from the same cell or different cells.

Proposal 6:  Reported RSRP of cell2/beam2 in the predicted relative RSRP accuracy formula agreed in RAN4#114 can be predicted or measured RSRP values.

The agreement from RAN4#114 meeting only concerns the cases where the reported measurements (measured or predicted) are for two different cells or beams. The agreed formula does not address the scenario where the reported measurements are for the same cell or beams, which is a typical scenario for intra-cell temportal prediction identified by scenarios 2 and 4 agreed to be studied in RAN4 in RAN4#112bis meeting [2]. The relative RSRP accuracy definition for the temporal prediction cases should also be defined.

Proposal 7: Predicted relative RSRP accuracy for temporal prediction is defined as:

- Predicted relative RSRP accuracy for temporal prediction = (reported predicted RSRP1 of a cell/beam – reported RSRP2 of the same cell/beam) – (ground truth of RSRP1 of a cell/beam – ground truth of RSRP2 of the same cell/beam).

- Reported RSRP2 can be both predicted or measured.

## Ground Truth Definition

In the last meeting ground truth for the predicted measurements in the testing was discussed. It was agreed that the ground truth extraction in the testing may differentiate between FR1 and FR2. For testing in FR2 it has been agreed that the ground truth of L3 RSRP is the reported L3 RSRP measurement under sufficient high SNR condition. Definition of ground truth for testing in FR1 is pending agreement in RAN4.

Agreements related to ground truth definition for RSRP accuracy from RAN4#114 [3]:

- The way the ground truth is extracted in the testing may differentiate between FR1 and FR2.

- Take Option 1-1-FR2 as baseline, and further discuss any additional aspects need to be considered.

  - Option 1-1-FR2: The ground truth of L3 RSRP is the reported L3 RSRP measurement under sufficient high SNR in FR2.

- Alt 1: Ground truth for FR1 will be based on the transmitted or reception power

- Alt 2: Ground truth for FR1 will be based on the reported measurement value under certain conditions

- Further discuss the advantages and disadvantages for Alt 1 and Alt 2 considering the aspects such as:

  - The information at the TE side, on the timing UE performs and/or finishes the measurement and/or prediction

  - Impact of L3 filtering

  - Number of samples for L1 filtering

  - Timing window, if it exists, where UE can’t measure the ground truth in Alt2

  - channel condition

  - Avoid UE cheat in the test

  - S(I)NR variations in the test at different time instances (for Alt2)

  - Other aspects are not precluded

- Companies are encouraged to provide analysis on all or some of the aspects in the list

In RAN4#112bis meeting it was agreed to consider conducted testing as a baseline option for testing in FR1. This agreement was motivated by the fact that the conducted testing provides a controlled testing environment where the DUT and the TE are physically connected to each other with a well calibrated cable. In such a setup the RSRP values to be measured by the UE at a given time instance can be calculated and used as ground truth in the FR1 testing. Calculation of the ground truth values can at least be based on the transmitted power and the channel condition that can be known to the TE.

The agreement from RAN4#114 has a list of aspects that are considered to have an impact on testing in FR1. Some of which does not necessarily have an impact on the ground truth extraction in FR1 testing but are more relevant for the test configuration discussions. Aspects such as availability of the timing information relating to the measurements and predictions performed and reported by the UE, impact of L3 filtering, and number of samples for L1 filtering in the test are more relevant for the test configuration discussion. Noting that these aspects are also relevant for testing in FR2, discussion on how to capture these aspects in the test configuration shall potentially be done during the work item phase.

Agreement from RAN4#112bis [2]:

- RAN4 to consider Option 2 as a baseline option for testing in FR1.

  - Option 2: Conducted testing.

Observation 1: Aspects such as availability of the timing information relating to the measurements and predictions performed and reported by the UE, impact of L3 filtering, and number of samples for L1 filtering in the test are more relevant for the test configuration discussion.

Proposal 8: Ground truth in FR1 testing is the calculated value based on transmitted power known at the TE. Other parameters that may have an impact on ground truth calculation are not precluded.

# Summary

The observations and proposals below summarize the discussion presented in this paper.

Proposal 1: Test setup for testing the AI/ML model deployed at UE to support intra-cell temporal prediction scenario is based on two-cell setup, similar to the test setup for legacy L3-based mobility.

Proposal 2: The aim of testing for intra-cell prediction should be to validate that the measurements reported by the UE for each cell in the test setup meet:

- requirement for the measurements performed inside the OW,

- requirement for the measurements predicted for the time instances inside the PW.

Proposal 3: The aim of testing for inter-cell scenario should be to validate that the measurements reported by the UE:

- meet the requirements for measured cell, and

- meet the requirements for the target cell.

Proposal 4: Existing test setup defined to validate requirements for inter-frequency measurements is re-used or used as a baseline for discussion on testing AI/ML based mobility for scenario 3.

- Cell 1 in the test can be considered as the cell for measurement.

- Cell 2 in the test can be considered as the target cell for measurement prediction.

Proposal 5: Beam 1 and beam 2 in the predicted relative RSRP accuracy formula agreed in RAN4#114 can be from the same cell or different cells.

Proposal 6:  Reported RSRP of cell2/beam2 in the predicted relative RSRP accuracy formula agreed in RAN4#114 can be predicted or measured RSRP values.

Proposal 7: Predicted relative RSRP accuracy for temporal prediction is defined as:

- Predicted relative RSRP accuracy for temporal prediction = (reported predicted RSRP1 of a cell/beam – reported RSRP2 of the same cell/beam) – (ground truth of RSRP1 of a cell/beam – ground truth of RSRP2 of the same cell/beam).

- Reported RSRP2 can be both predicted or measured.

Observation 1: Aspects such as availability of the timing information relating to the measurements and predictions performed and reported by the UE, impact of L3 filtering, and number of samples for L1 filtering in the test are more relevant for the test configuration discussion.

Proposal 8: Ground truth in FR1 testing is the calculated value based on transmitted power known at the TE. Other parameters that may have an impact on ground truth calculation are not precluded.

# References

- R4-2420095, WF on AI/ML Mobility, Moderator (Nokia).

- R4-2416855, WF on AI/ML Mobility, Moderator (Nokia).

- R4-2502594, WF on AI/ML Mobility, Moderator (Nokia).