3GPP TSG RAN WG4 Meeting #114bis	R4-2503937

Wuhan, China, 07 Feb. 2025 – 11 Feb. 2025

Agenda Item:	7.27.2.2

Source: 	Ericsson

Title:	Discussion on CSI-RS based L1 measurement requirements

Document for:	Discussion

# Introduction

In this contribution we discuss CSI-RS based L1 measurement requirements.

# Discussion

## CSI-RS based L1 measurement requirements

### Known cell conditions

For CSI-RS based L1-RSRP measurement, known cell conditions were discussed in last meeting and the following WF is agreed.

Issue 4-1-1: Whether to use detectable or measurable? Tentative agreement: - Use measurable and specify the side condition for RS being measurable. Issue 4-1-2: Side condition for CSI-RS based L1 measurement for neighbor cell Candidate options: - Option 1: SNR of the CSI-RS from the target cell configured for L1 measurement is above -6dB (Nokia) - Option 2: It is proposed to use legacy side condition of SINR=-3dB. (Apple, Xiaomi, MTK, OPPO, Huawei, Ericsson)

When the same SSB is configured for both L3 and L1 measurements, the detectability conditions are as follows:

- For L3 measurements, the SSB_RP must be at least -6dB.

- For L1 measurements, the SSB_RP must be at least -3dB.

For CSI-RS-based L1 measurements, the UE requires coarse timing and QCL information from the network. Since these are already obtained by SSB-based L3 measurements, the L3 measurement detectability condition should be sufficient for initial CSI-RS reception. SSB-based L1 measurements are used for providing finer timing and QCL relations for improved accuracy and performance. We believe that SSB L1 measurement detectability is not necessary, as there is an additional condition regarding CSI-RS measurability or detectability.

Other open issue is whether to use the wording measurable or detectable. We believe "detectable" is used for RS that can be detected without any assistance information from the NW. Since CSI-RS requires SSB as a QCL relation, it cannot be detected on its own. However, there are instances in the specs where "measurable" is also used for SSB. Therefore, we could use either "measurable" or "detectable" and define what each term means. We consider CSI-RS to be measurable if the side condition for the SNR is satisfied, and the SNR side condition (-3dB) for serving cell CSI-RS measurements can be reused.

We think CSI-RS based L3 measurements needs SSB as a pre-requisite. Hence, we suggest considering SSB-based L3 measurement only.

- For CSI-RS based L1-RSRP measurements on neighbour cells, an LTM candidate cell is known if the following conditions are met

  - The UE has performed SSB based L3 measurement on the target cell during the last 5 seconds, and

  - The SSB QCL’ed with the CSI-RS remains detectable (Es/Io ≥ -6 dB) according to the cell identification requirements in clause 9.2.

  - The CSI-RS from the target cell configured for L1 measurement remains measurable.

    - CSI-RS is considered measurable if the CSI_RS Es/Io is ≥ -3dB.

### Intra-frequency and Inter-frequency definition

Intra-frequency and inter-frequency measurement is pending for many meetings and the following candidate options are shortlisted after last meeting.

- Agreement: - Option 1: (Apple, ZTE, MTK, CMCC, Huawei, Ericsson) A measurement is defined as a CSI-RS based intra-frequency measurement provided that: - the SCS of the CSI-RS resource of the neighbor cell configured for L1 measurement is the same as the SCS of active DL BWP, and - the CP type of the CSI-RS resource of neighbor cell configured for L1 measurement is the same as the CP type of active DL BWP, and - It is applied for SCS = 60KHz - the center frequency of the CSI-RS resource of the neighbour cell configured for L1 measurement is the same as the centre frequency of any of the CSI-RS resources within DL BWP (if configured). - Option 2: (Qualcomm, MTK, Nokia, E///, vivo, China Telecom, HW, ZTE, CATT) A measurement is defined as a CSI-RS based intra-frequency L1 measurement provided that: - the SCS of the CSI-RS resource of the neighbour cell configured for L1 measurement is the same as the SCS of active DL BWP, and - the CP type of the CSI-RS resource of neighbour cell configured for L1 measurement is the same as the CP type of active DL BWP, and - the CSI-RS resource of the neighbour cell configured for L1 measurement is included within the active DL BWP. - at least 48 RBs of the CSI-RS resource, needs to be confined within the active DL BWP.

We think both options work from defining RAN4 requirements. However, we have a slight preference for Option 2, as it eliminates the need to further define requirements for intra-frequency measurements with gaps and requirements for inter-frequency measurements without gaps. Option 1 is driven by the definition of CSI-RS-based L3 measurements, and as some companies have noted, we are uncertain whether it has been implemented in the field. Therefore, we think it should not be a limiting factor.

- A measurement is defined as a CSI-RS based intra-frequency L1 measurement provided that:
- the SCS of the CSI-RS resource of the neighbour cell configured for L1 measurement is the same as the SCS of active DL BWP, and

- the CP type of the CSI-RS resource of neighbour cell configured for L1 measurement is the same as the CP type of active DL BWP, and

- the CSI-RS resource of the neighbour cell configured for L1 measurement is included within the active DL BWP.

- at least 48 RBs of the CSI-RS resource, needs to be confined within the active DL BWP.

### Measurement procedure

- Agree the following measurement procedure for CSI-RS based LTM candidate cell measurements:

  - Step 1: SSB based L3 measurements and reports at least before LTM configuration.

  - Step 2: For FR2-1, SSB-based L1 measurements and reports on the same candidate cell

    - Further discuss whether Step 2 can be skipped under certain conditions or based on UE capability or based on network RRC configuration. FFS the details.

    - Note: Step 2 is removed for FR1.

  - Step 3: CSI-RS based L1 measurements on the same candidate cell

    - Depending on whether step 2 is skipped, the requirements in step 3 may be different.

  - Note: The CSI-RS in Step 3 should be QCL’ed with the SSB in Step 1 or 2.

    - [Note: For FR1, the CSI-RS in Step 3 should be QCL’ed with the SSB in Step 1.]

The following is concluded in RAN4#113 meeting.

In the last meeting, there was further discussion about whether the UE should always perform SSB-based L1 RSRP measurement before conducting a CSI-RS-based L1-RSRP measurement and the status looks as shown below.

- Option A: Whether step 2 can be skipped or not can be based on UE capability reporting. (Apple, CATT, Nokia, Ericsson, Qualcomm, CMCC - compromise, ZTE, China Telecom, vivo, Xiaomi, OPPO)

- Option B: Always keep step 2 for FR2-1 (MTK, HW)

- Option C: Remove L1 measurement for FR2-1 in the 2nd bullet. (CMCC, OPPO)

The key issue is whether the UE should derive the receive (Rx) beam from the SSB or the CSI-RS. We discussed the advantages of CSI-RS based RX beam sweeping for CSI-RS based L1-RSRP measurement in the last meeting, and the details can be found in the annex for ease of reference. However, there were some concerns raised during the last meeting and we address them below.

One concern is that the using CSI-RS for beam sweeping results in larger measurement period for CSI-RS based L1-RSRP. We would like to acknowledge that the concern about using CSI-RS for beam sweeping is valid, as it could lead to longer delays in obtaining CSI-RS-based L1-RSRP measurements. Specifically, if both the SSB and CSI-RS periods are 20 ms, the Rx beam determined by SSB can be assumed for all subsequent CSI-RS-based L1-RSRP measurements until the Rx beam from the SSB changes, resulting in a measurement period of 20 ms. However, if CSI-RS-based Rx beam sweeping is used and the CSI-RS period is 20 ms, the worst-case scenario could lead to a delay of up to 8 times the CSI-RS period, or 160 ms (8 * 20 ms).

We acknowledge this concern and would like to highlight that, for the initial CSI-RS-based L1-RSRP measurement following configuration, the delay associated with CSI-RS-based Rx beam sweeping may actually be lower than that of SSB-based Rx beam sweeping. One potential implementation could involve the network enabling CSI-RS-based L1-RSRP measurements before a cell switch, along with early CSI acquisition. In this context, the delay for subsequent L1-RSRP measurements might be less critical, and the Rx beam derived from CSI-RS could significantly enhance CSI acquisition.

Ultimately, this approach could lead to improved network performance by ensuring that the Rx beam is optimally aligned with CSI-RS signals, thereby facilitating better CSI information and improved throughput after the cell switch command. We believe that the network may not gain much by activating CSI-RS-based L1-RSRP in conjunction with SSB-based L1-RSRP. The real advantage of CSI-RS-based L1-RSRP stems from the finer Rx beam formed using CSI-RS, along with enhanced CSI acquisition benefits due to the precise Rx beam alignment provided by CSI-RS.

Considering the advantages of the CSI-RS based RX beam sweeping, we suggest defining Rx beam sweeping based on CSI-RS.

Another concern raised involves the configuration of a SCell as a candidate cell. In legacy, for serving cell measurements, the UE would perform SSB-based Rx beam sweeping for CSI-RS-based L1-RSRP measurements. However, when the same cell is configured as a candidate cell, the UE is expected to use CSI-RS-based Rx beam sweeping for the candidate cell measurements. This dual approach could introduce ambiguity and confusion in handling measurements for SCells. To address this potential issue and maintain clarity, we are supportive of excluding this requirement for SCells.

Based on the above analysis, we suggest the following proposals:

- RAN4 to define CSI-RS based RX beam sweeping as an optional UE capability for CSI-RS based L1-RSRP measurements.

- For a UE supporting CSI-RS based RX beam sweeping, RAN4 to agree on the following measurement procedure for CSI-RS based LTM candidate cell measurements:

  - Step 1: SSB based L3 measurements and reports at least before LTM configuration

  - Step 3: CSI-RS based L1 measurements on the same candidate cell

- For a UE not supporting CSI-RS based RX beam sweeping, RAN4 to agree on the following measurement procedure for CSI-RS based LTM candidate cell measurements:

- Step 1: SSB based L3 measurements and reports at least before LTM configuration.

- Step 2: For FR2-1, SSB-based L1 measurements and reports on the same candidate cell

  - Note: Step 2 is removed for FR1.

- Step 3: CSI-RS based L1 measurements on the same candidate cell

- Note: The CSI-RS in Step 3 should be QCL’ed with the SSB in Step 1 or 2.

- Note: For FR1, the CSI-RS in Step 3 should be QCL’ed with the SSB in Step 1.

### Defintion of the frequency layer

If two CSI-RS resources belong to the same frequency layer, the UE is expected to measure them at the same time using the same FFT. As per our understanding, the UE can measure the two CSI-RS resources simultaneously using the same FFT if their SCS, CP type, BW, and center frequency are the same, and they have overlapping periodicity (i.e., they are fully or partially overlapping in the time domain).

- Two CSI Resource are of same frequency layer if they have same SCS, CP type, BW, center frequency, and the CSI-RS occasions are fully/partially overlapping in time domain.

### Measurement gap

When only a single gap configuration is supported, the UE should share the measurement gap (MG) between the SSB and CSI-RS. Sharing the measurement gap with the SSB may further extend the measurement delay of each measurement objects on the RRM measurements. If the CSI-RS does not overlap with the SMTC, the UE may perform both the CSI-RS based and SSB based measurements in different gap occasions.

- RAN4 to define the gap-based CSI-RS L1-RSRP measurement which do not impact the legacy SSB based L1-/L3-RSRP measurement within gap.

In Rel-17, concurrent gaps were introduced which may be useful for CSI-RS based L1-RSRP measurement; however, it may bring additional workloads in RAN4. We think RAN4 can further discuss which type of measurement gap can be used for CSI-RS based L1-RSRP once all the essential issues for defining CSI-RS based L1-RSRP requirements are settled.

- RAN4 to define single gap-based CSI-RS L1-RSRP requirement first. RAN4 can further discuss concurrent gaps-based CSI-RS L1-RSRP requirement if time allows.

### CSI-RS resource type

In the last meeting, the resource types for CSI-RS based L1-RSRP were discussed. Currently, RAN1 has agreed to support both periodic and semi-persistent reporting. RAN4 requirements are supported for periodic reporting anyway. There was a discussion in the last meeting on whether to support the requirements applicability for semi-persistent CSI-RS. Since the RAN4 core requirements should be accompanied with performance requirements to make sure the performance of the core requirements, RAN4 requirements are defined for the features that are widely used. This procedure is followed mainly to reduce the test cost for the UE.

We think the semi-persistent reporting for L1-RSRP can be widely used in the NWs as it brings the flexibility of enabling the reporting when certain events are met (This can be used in conjunction with other L3/L1 events to enable or disable the measurement reporting) and disabling it for saving the UL resource overhead. Hence, it should be supported by RAN4 requirements for guaranteeing the measurement performance.

- Semi-persistent CSI-RS should be considered for defining CSI-RS based L1-RSRP requirements

### Restriction on CSI-RS resource configuration

In the last meeting, restriction on CSI-RS resource configuration was discussed and following WF was agreed.

- In FR1, when CSI-RS based L1 measurement does not cause scheduling restriction, no need to consider restriction on measurement timing configuration. - In FR2, or in FR1 when CSI-RS based L1 measurement would cause scheduling restriction: - All CSI-RS resources on [one intra-frequency layer] are configured within up to two separate windows where each window is up to [5 ms]

CSI-RS based L3-RSRP uses the CSI-RS type, CSI-RS-Resource-Mobility. The periodicity and offset of CSI-RS-Resource-Mobility are {4ms, 5ms, 10ms, 20ms, 40ms}, whereas for CSI-RS for L1-RSRP, they are {4, 5, 8, 10, 16, 20, 32, 40, 64, 80, 160, 320, 640} slots.

The maximum periodicity for L3-RSRP is 40 ms, during which one intra-frequency layer can have two separate 5 ms windows where the CSI-RS is allowed. Given that the CSI-RS for L1-RSRP can have a periodicity of up to 640 slots, we would like to clarify the configuration restriction. Therefore, we propose that all CSI-RS resources within a 40 ms window on one intra-frequency layer should be configured within up to two separate windows, each lasting up to 5ms.

- In FR2, or in FR1 when CSI-RS based L1 measurement would cause scheduling restriction:

  - all CSI-RS resources within a 40 ms window on one intra-frequency layer should be configured within up to two separate windows, each lasting up to 5 ms

# Summary and Conclusion

In this contribution we analyzed potential RAN4 RRM requirements impact and made following observations and proposals.

- For CSI-RS based L1-RSRP measurements on neighbour cells, an LTM candidate cell is known if the following conditions are met

  - The UE has performed SSB based L3 measurement on the target cell during the last 5 seconds, and

  - The SSB QCL’ed with the CSI-RS remains detectable (Es/Io ≥ -6 dB) according to the cell identification requirements in clause 9.2.

  - The CSI-RS from the target cell configured for L1 measurement remains measurable.

    - CSI-RS is considered measurable if the CSI_RS Es/Io is ≥ -3dB.

- A measurement is defined as a CSI-RS based intra-frequency L1 measurement provided that:
- the SCS of the CSI-RS resource of the neighbour cell configured for L1 measurement is the same as the SCS of active DL BWP, and

- the CP type of the CSI-RS resource of neighbour cell configured for L1 measurement is the same as the CP type of active DL BWP, and

- the CSI-RS resource of the neighbour cell configured for L1 measurement is included within the active DL BWP.

- at least 48 RBs of the CSI-RS resource, needs to be confined within the active DL BWP.

- RAN4 to define CSI-RS based RX beam sweeping as an optional UE capability for CSI-RS based L1-RSRP measurements.

- For a UE supporting CSI-RS based RX beam sweeping, RAN4 to agree on the following measurement procedure for CSI-RS based LTM candidate cell measurements:

  - Step 1: SSB based L3 measurements and reports at least before LTM configuration

  - Step 3: CSI-RS based L1 measurements on the same candidate cell

- For a UE not supporting CSI-RS based RX beam sweeping, RAN4 to agree on the following measurement procedure for CSI-RS based LTM candidate cell measurements:

- Step 1: SSB based L3 measurements and reports at least before LTM configuration.

- Step 2: For FR2-1, SSB-based L1 measurements and reports on the same candidate cell

  - Note: Step 2 is removed for FR1.

- Step 3: CSI-RS based L1 measurements on the same candidate cell

- Note: The CSI-RS in Step 3 should be QCL’ed with the SSB in Step 1 or 2.

- Note: For FR1, the CSI-RS in Step 3 should be QCL’ed with the SSB in Step 1.

- RAN4 to define the gap-based CSI-RS L1-RSRP measurement which do not impact the legacy SSB based L1-/L3-RSRP measurement within gap.

- RAN4 to define the gap-based CSI-RS L1-RSRP measurement which do not impact the legacy SSB based L1-/L3-RSRP measurement within gap.

- RAN4 to define single gap-based CSI-RS L1-RSRP requirement first. RAN4 can further discuss concurrent gaps-based CSI-RS L1-RSRP requirement if time allows.

- Semi-persistent CSI-RS should be considered for defining CSI-RS based L1-RSRP requirements

- In FR2, or in FR1 when CSI-RS based L1 measurement would cause scheduling restriction:

  - all CSI-RS resources within a 40 ms window on one intra-frequency layer should be configured within up to two separate windows, each lasting up to 5 ms.

# References

[1] 	RP-241515, New WID: NR mobility enhancements Phase 4

[2] 	R4-2414060, WF on NR mobility enhancements Phase 4

# Annex

Figure 1: Measurement procedure using Alternative 1

Alternative 1:

Step 1: UE performs SSB based L3 measurements with RX beam sweeping in FR2 and determine the RX beam for L3 measurements.

Step 2: If the same SSB is configured for L1 measurements, UE performs RX beam sweeping to find the best Rx beam for the SSB based L1-RSRP measurements. In this step UE may find a finer RX beam for receiving the SSB. This step is useful for finding the best Tx and Rx beam combination for SSB based L1-RSRP measurement. Due to the Rx beam gain and able to receive the SSB in Tx beam peak direction, UE may get higher L1-RSRP compared to L3-RSRP. In this step UE determines the best RX beam for a given SSB and uses the same for the receiving CSI-RS which was QCLed with SSB. In the figure 1, red solid beam may be the best Rx beam.

Step 3: the Rx beam determined in step 2 may be used to receive the CSI-RS which was configured with same SSB as QCL source RS. In the figure 1, UE uses the red solid beam to receive all the three CSI-RS (as three CSI-RS has same SSB as QCL source).

Why Alternative 1 may not work well: Based on the Rx beam width selected by UE, it may happen that the RX beam may be good for receiving one CSI-RS but may not be good for receiving the other CSI-RS. In the figure 1, the red RX beam (best Rx beam for SSB based L1-RSRP) may be good for receiving the green CSI-RS beam but may be not so good for receiving other two CSI-RS. We think wide beam used for SSB based L3 measurement may work well for receiving three CSI-RS resources than the SSB based fine beam for L1 measurement.

Figure 2: Measurement procedure using alternative 2

Alternative 2:

In this approach, UE performs the RX beam sweeping using the periodic or semi-persistent CSI-RS. In step 1, UE performs SSB based L3 measurements and determine the wide beam to be used for SSB based L3 measurements. In step 2, instead of doing Rx beam sweeping on SSB to determine the fine beam, UE uses the periodic or semi-persistent CSI-RS for the Rx beam sweeping. We show the periodic CSI-RS based Rx beam sweeping in figure 3.

Figure 3: Rx beam sweeping using CSI-RS

From the figure 3, we can observe that UE can get the best Tx and Rx beam pair if the UE has performed the RX beam sweeping using CSI-RS. As shown in figure 3, orange CSI-RS beam can be best received with black Rx beam, green CSI-RS beam can be best received with green Rx beam, and blue CSI-RS beam can be received with red Rx beam.

If a UE supports early CSI acquisition and reporting, best Tx and Rx beam pair can be useful in determining the CQI more accurately. When NW schedules data in the candidate cell using fine beam UE will be able to support better throughput using alternative 2.