# 3GPP TSG-RAN WG4 Meeting #114bis	R4-2503946

# Wuhan, Hubei, China, 07th – 11th April, 2025

Source:			Ericsson

Title:			mmWave in NR: EESS protection – Remaining UE RF issues

Agenda item:		6.17.2

Document for:		Discussion

# Introduction

In RAN#105 meeting, a WI ([1]) was adopted to consider China and EU/CEPT Regulation related to the 24GHz bands updates and the EESS protection.

This contribution is discussing the remaining issues on UE RF aspects based on RAN4#114 meeting agreements.

# Discussion

## Background

All BS aspects have been clarified in previous meetings and the relevant CRs were agreed. Based on Rel-18 version, TS 38.104 Rel-19 is already aligned with RAN4 agreements and clarification on EESS protection in the 24GHz frequency range.

On the UE side, CR to TS 38.101-2 Rel-19 ([2]) was agreed in last meeting capturing RAN4 agreed update to correctly consider existing worldwide regulation related to EESS protection in the 24GHz frequency range.

But, as RAN4 agreed those updates shall also be reflected in earlier Releases, the main remaining open issue is how to capture those in the other TS 38.101-2 Releases (down to Rel-15), keeping trace of the “old” limit as requested by some companies. This issue was discussed but no agreement was made, and discussion was postponed to this RAN4#114bis meeting.

## Earlier Releases update - TS 38.101-2

Regarding the further update of the previous Release of TS 38.101-2, the starting point should be the agreed CR to TS 38.101-2 Rel-19 ([2]).

As discussed in last RAN4#114 meeting, a note could be added to the table capturing the additional spurious requirement for NS_202. This note would indicate that previously the limit was less stringent and set to +1dBm/200MHz. We would propose the following wording for this note.

Proposal1: Add a note2 in Table 6.5.3.2.3-1 with the following wording:

Note 2: For versions older than v15.26.0, a limit of +1 dBm was specified and the corresponding bit of modifiedMPR-Behavior was set to 0 for bands n257 and n258 as defined in Annex H.

Our companion draft CRs to TS 38.101-2 Rel-15 ([3]) are capturing this proposal.

# Conclusion

In this contribution, we discussed the remaining open related to the WI on EESS protection in the 23.6-24.0 GHz frequency range. We made the following proposals:

Proposal1: Add a note2 in Table 6.5.3.2.3-1 with the following wording:

Note 2: For versions older than v15.26.0, a limit of +1 dBm was specified and the corresponding bit of modifiedMPR-Behavior was set to 0 for bands n257 and n258 as defined in Annex H.

# References

- RP-242371, WID on mmWave UE spurious emssion, China Unicom, Deutsche Telekom, Vodafone, CATT, Huawei, HiSilicon, OPPO, Qualcomm, Xiaomi, ZTE

- R4-2501408, Big CR on UE RF requirements for WI NR_mmWave_protect, China Unicom

- R4-2503947, Draft CR to TS 38.101-2 Rel-15 – EESS protection, Ericsson