# 3GPP TSG-RAN WG4 Meeting #114bis	R4-2503949

# Wuhan, Hubei, China, 07th – 11th April, 2025

Source:			Ericsson

Title:			EESS protection - TP to TR 38749 - UE aspects

Agenda item:		6.17.2

Document for:		Approval

# Introduction

This contribution is a text proposal to the UE section in TR 38.749 on Protection of Earth Exploration Satellite Service (EESS).

# Conclusion

Proposal: Approve the TP as proposed in Annex.

<Start of the change>

# 8	UE RF requirements

## 8.1	Background

The Earth Exploration Satellite Service (EESS) protection requirements in the frequency range 23.6-24.0 GHz (as per WRC-19 Radio Regulations Resolution 750 [2] and EU Decision 2020/590 [4]) were specified using NS_202/CA_NS_202 and NS_203/CA_NS_203 signaling.

NS_202 should be signaled for operation in band 257 or n258 in CEPT countries, while NS_203 would be signaled for operation in those bands in other countries.

Nevertheless, only the +1dBm/200MHz limit was captured, which is applicable until 1 September 2027 (ITU-R) and until 1 January 2024 (CEPT countries).

The other limit of -5dBm/200MHz was not considered, not anticipating 1 September 2027 when this limit should apply according to ITU-R. Also, this is causing a major issue for CEPT countries where this limit is applicable for any device brought to use from 1 January 2024.

## 8.2	UE RF impacts

Within this WI, to address the Regulation issues mentioned above, the following updates were made to TS 38.101-2:

- The limit specified in NS_202 has been changed to -5dBm/200MHz considering this limit is already applicable from 1 January 2024 in CEPT countries. The corresponding A-MPR values were adjusted based on this new more stringent limit.

- A new NS_205 (and its corresponding CA_NS_205) was specified with a -5dBm/200MHz limit to anticipate 1 September 2027, when this limit will be applicable according to ITU-R. The corresponding A-MPR values were also specified.

- A modifiedMPRbehavior bit was specified to indicate the UE supports the updated NS_202 (with the more stringent -5dBm/200MHz limit) in bands n257 and n258. The UE shall set this bit to 1. This bit could be used to distinguish between UE supporting the less stringent limit and brought to use before 1 January 2024 (this bit shall be equal to 0) from the UE supporting the more stringent limit and brought to use from 1 January 2024 (this bit shall be equal to 1).

- Another modifiedMPRbehavior bit was specified to indicate the UE supports the new NS_205 in bands n257 and n258. The UE shall set this bit to 1.

In CEPT countries where the -5dBm/200MHz limit already applies, only NS_202 shall be broadcasted by the network.

While in other ITU-R countries, from 1 September 2027, the network will have to broadcast NS_205 followed by NS_203 to allow any UE brought to use after or before 1 September 2027 to connect to the network.

As those updates have been considered as critical to comply with the Regulations, they have been reported back to Rel-15.

<End of the change>