# 3GPP TSG-RAN WG4 Meeting #114bis	R4-2503953

# Wuhan, Hubei, China, 07th – 11th April, 2025

Source:			Ericsson

Title:			LTE based 5G BCT – bands definition

Agenda item:		6.16.2

Document for:		Discussion

# Introduction

In RAN#106 meeting, a WI ([1]) was adopted to consider new bands for LTE based 5G terrestrial broadcast.

This contribution is discussing the definition of the new SDO bands based on the RAN4#114 meeting agreements ([2]).

# Discussion

In last RAN4#114 meeting, RAN4 agreed ([2]) to specify two new separate bands (i.e., 470-608 MHz, and 606-698 MHz), in a release independent manner from Rel-17.

During this meeting, considering those 2 new bands are actually sub-bands of the SDO band 108, it was also proposed ([3]) to use “108a” and “108b” for their band number. It was highlighted that such “sub-bands definition” was already used for band 46 where sub-bands 46a, 46b, 46c and 46d have been specified in TS 36.104.

If we understand the motivation for such proposal, reducing the band numbering and keeping a logical link with the “big” band 108, we don’t think we should define the 2 new sub-bands using such suffix notation.

If band 46 sub-bands have been specified in TS 36.104, this seems to be a local definition, only used to specify coexistence requirements within band 46. This sub-bands definition was not made in TS 36.101, nor in TS 38.104/TS 38.101-1 when the band 46 was ported to NR band n46.

Also, looking at the RRC specification (TS 36.331), the IE FreqBandIndicator is used to indicate “the E-UTRA operating band as defined in TS 36.101 [42], table 5.5-1 and TS 36.102 [113], table 5.2-1 for NTN capable UE”. Also, this IE FreqBandIndicator is specified as an integer. If RAN4 specifies then the new SDO bands using prefix “a” and “b”, this would cause some problems with the RRC signalling.

Proposal: Specify the two new SDO bands 470-608 MHz and 606-698 MHz using the next available band numbers, i.e. 112 and 113.

# Conclusion

In this contribution, we discussed the definition of the new SDO bands. We made the following proposal:

Proposal: Specify the two new SDO bands 470-608 MHz and 606-698 MHz using the next available band numbers, i.e. 112 and 113.

# References

- RP-243268, New bands for LTE based 5G terrestrial broadcast for early deployments, EBU

- R4-2502376, WF on 5G Broadcast bands, EBU

- R4-2502136, Handling of band 108 sub-bands introduction, Huawei.