3GPP TSG-RAN4 Meeting # 114bis	R4-2503958

Wuhan, China, Apr.7th – 11th, 2025

# Source: 	ZTE Corporation, Sanechips

# Title: 	Clarification on the multi-band requirements for ATG BS CA

# Agenda item:	7.12.2

# Document for:	Approval

# 1 Introduction

In this contribution, we give some discussion on the clarification on the multi-band requirements for ATG BS CA after further checking the contents in the specification and the history information.

# 2	Discussion

## 2.1 History information

We try to provide some history information related to the LTE BS capable of multi-band operation. Actually, multi-band operation requirements was introduced in MB-MSR WID, and the technical discussions were captured in the TR37.812. Since MSR also includes the single-RAT like LTE requirements, then the requirements are also applied for LTE. For MB-MSR, the agreements for multiple bands with both FDD and TDD bands can be found in WF[3], which are:

The discussion on feasibility of multi-band BS configuration supporting both FDD and TDD bands shall be decoupled from the completion of this WI. - Multi-band BS configuration supporting both FDD and TDD bands is not supported in Release 12 BS specification

Indeed, multi-band BS configuration supporting both FDD and TDD bands was discussed in RAN4. However, there were no consensus achieved and meanwhile these exist some technical issues and specification impacts/ambiguity for multi-band BS configuration supporting both FDD and TDD bands, such as how to interpret the transmit ON/OFF requirements due to it is only for TDD, and how to interpret the some Rx requirements for FDD due to it shall be met with the transmitter(s) on, and so on.

Moreover, In principle, separate active RF components for TDD and FDD are assumed to support TDD-FDD CA. The RF requirements are applied separately for TDD and FDD. If a common active RF components for TDD and FDD to support TDD-FDD CA (which is multi-band operation), then it would require large isolation between RX and TX (on the order of 100 dB) which makes it impractical for multi-band operation. It is the same for a common active RF components for both TDD bands when non-simultaneous Rx/Tx is allowed between the TDD bands.. Therefore, in TR37.812[3], the applicable scenarios for multi-band are:

The MB-MSR application scenarios also cover both FDD and TDD, but are limited to MB-MSR for FDD only scenarios and TDD only scenarios respectively.

For the reasons above, the general clause explaining "Applicability of MB-MSR requirements" also clarifies that for multi-band capable BS, operation in the multiple bands must be synchronized.

Observation 1: When multi-band operation is introduced in RAN4, it is limited to FDD only scenarios and TDD only scenarios respectively. For TDD only scenarios, operation in the multiple bands must be synchronized.

Consequently, requirements for BS capable of multi-band operation are derived for the above applicable scenarios for MB-MSR in TS37.104, and for LTE in TS36.104.

More importantly, the above conclusions are still valid today considering there are no further WID/SID to re-discuss the multi-band operation requirements for NR BS.

## 2.2 Current situation

For ATG BS, no matter only single carrier operation or intra/inter band CA operation, the RF frameworks are based on the NR BS.

For ATG enhancement, in terms of the WID [3], the band combination have been included clearly.

- Specify the RF and RRM core requirements for intra-band co-located and inter-band co-located DL CA [RAN4]: - FR1 intra-band contiguous CA - Example band combination: CA_n79C - FR1+FR1 inter-band CA - Example band combination: CA_n3A-n39A

For FR1+FR1 inter-band CA band combination, CA_n3A-n39A is the only one example band combination in the WID, and also there are no other inter-band CA band combination proposed by companies as of today. It means the discussion for ATG enhancement only focus on the example band combinations captured in the WID, this is also true for ATG UE discussion where only the MSD requirements of CA_n3A-n39A are discussed.

Observation 2: Except the example band combination in the WID, there are no other band combinations proposed by company in Rel-19.

The discussion for ATG BS CA supporting multi-band operation happened in RAN4 #113 meeting, and the conclusion were achieved quickly and it was captured in the WF[1] as below:

- Issue 1-2: Multi-band operation requirement for ATG BS CA - Way Forward： - For the CR to TS38.104 to introduce ATG BS CA, remove the sentence ‘There is no multi-band operation requirement for ATG BS.’ in clause 4.8.

Later, the draft CR [2] was endorsed where the above WF was reflected.

However, after further checking the specification, we found there are two sentence under sub-clause 4.8 in TS38.104 to say:

For multi-band connectors and multi-band RIBs supporting the bands for TDD, the RF requirements in the present specification assume no simultaneous uplink and downlink occur between the bands. The RF requirements for multi-band connectors and multi-band RIBs supporting bands for both FDD and TDD are not covered by the present release of this specification.

In TS38.141-1, the corresponding texts can be found under clause 4.11:

For multi-band connectors supporting the bands for TDD, the RF requirements in the present specification assume no simultaneous uplink and downlink occur between the bands. NOTE 1: The case of an operating band being supported by both multi-band connectors and single-band connectors in a TAB connector TX min cell group or a TAB connector RX min cell group is not covered by the present release of this specification. NOTE 2: The case of an operating band being supported by multi-band connectors which are not all supporting the same operating band combination in a TAB connector TX min cell group or a TAB connector RX min cell group is not covered by the present release of this specification. NOTE 3: The conducted test requirements for multi-band connectors supporting bands for both FDD and TDD are not covered by the present release of this specification.

In TS38.141-2, the corresponding texts can be found under clause 4.11:

For multi-band RIBs supporting the bands for TDD, the radiated test requirements in the present specification assume no simultaneous uplink and downlink occur between the bands. NOTE: The radiated test requirements for multi-band RIBs supporting bands for both FDD and TDD are not covered by the present release of this specification.

It can be seen that there are no RF requirements and test requirement defined for multi-band connectors and multi-band RIBs supporting bands for both FDD and TDD are not covered in the present release of TS38.104 and TS38.141-1/2 specification. In other words, only the multiple bands with both FDD and FDD bands, or both TDD and TDD bands with no simultaneous Rx/Tx can be supported for a multi-band connector/RIBs.

The above texts have been existed in the specifications of TS38.104/TS38.141-1/2 from Rel-15.

Actually in the beginning of Rel-15, when the specifications of TS38.104/TS38.141-1/2 was generated, the NR BS RF framework/requirements were largely based on the LTE BS RF requirement. For multi-band requirements, although the terminology was changed to ‘multi-band connector’ or ‘multi-band RIBs’, the similar principle of LTE multi-band operation was adopted.

Observation 3: The requirements for NR BS capable of multi-band operation adopt the similar principle as LTE BS.

Observation 4: There are no RF requirements and test requirement defined for multi-band connectors and multi-band RIBs supporting bands for both FDD and TDD are not covered in the present release of TS38.104 and TS38.141-1/2 specification. In other words, only the multiple bands with both FDD and FDD bands, or both TDD and TDD bands with no simultaneous Rx/Tx can be supported for a multi-band connector/RIBs.

When only consider the CA_n3A-n39A FDD-TDD band combination for ATG BS, then removing the sentence ‘There is no multi-band operation requirement for ATG BS.’ in clause 4.8 in TS38.104 would cause misunderstanding and discrepancy in the context.

Observation 5: Removing the sentence ‘There is no multi-band operation requirement for ATG BS.’ in clause 4.8 in TS38.104 would cause misunderstanding and discrepancy in the context.

## 2.3 Clarification on ATG BS CA

Since CA_n3A-n39A is the only band combination for ATG BS CA in release 19, and it belongs to FDD-TDD band combination, to solve the misunderstanding and discrepancy in the specification, we propose to revisit the previous agreements.

Proposal 1. Multi-band operation requirements are not applied for ATG BS supporting CA_n3A-n39A.

But of course, if there are other new TDD-TDD CA band combination introduced for ATG in future release, we think the multi-band operation requirements can be applied, which is also compliant to the current situation of the TS38.104 specification.

With the above consideration, it seems there are two options on how to handle the sentence in Rel-19.

Option 1. Keep the previous agreement, i.e. removing ‘There is no multi-band operation requirement for ATG BS.’

Option 2. Revisit the previous agreement, i.e. keep ‘There is no multi-band operation requirement for ATG BS.’

Option 1 can be seen as a future-proofing solution, which can accommodate the new TDD-TDD and FDD-FDD band combinations introduced in future if any. Nevertheless, we slight prefer option 2 since it is clear in Rel-19 and can avoid unnecessary misunderstanding in current release of specification. Hence, we propose a revised draft CR in [6]

Proposal 2. To endorse the revised draft CR [5].

# 3 Conclusion

The proposals and conclusions are:

Observation 1: When multi-band operation is introduced in RAN4, it is limited to FDD only scenarios and TDD only scenarios respectively. For TDD only scenarios, operation in the multiple bands must be synchronized.

Observation 2: Except the example band combination in the WID, there are no other band combinations proposed by company in Rel-19.

Observation 3: The requirements for NR BS capable of multi-band operation adopt the similar principle as LTE BS.

Observation 4: There are no RF requirements and test requirement defined for multi-band connectors and multi-band RIBs supporting bands for both FDD and TDD are not covered in the present release of TS38.104 and TS38.141-1/2 specification. In other words, only the multiple bands with both FDD and FDD bands, or both TDD and TDD bands with no simultaneous Rx/Tx can be supported for a multi-band connector/RIBs.

Observation 5: Removing the sentence ‘There is no multi-band operation requirement for ATG BS.’ in clause 4.8 in TS38.104 would cause misunderstanding and discrepancy in the context.

Proposal 1. Multi-band operation requirements are not applied for ATG BS supporting CA_n3A-n39A.

Proposal 2. To endorse the revised draft CR [5].

# 4	Reference

[1] R4-2419796, Way Forward for [113][305] NR_ATG_enh, ZTE Corporation

[2] R4-2502280, draft CR to TS 38.104: the introduction of Rel-19 ATG BS supporting CA, ZTE Corporation, CMCC, CATT, Huawei, Ericsson.

[3] TR 37.812 Multi-band and Multi-standard radio (MB-MSR) Base Station (BS)

[4] R4-142486, Way forward on Multi-band BS for TDD-FDD joint operation including Carrier Aggregation,NTT DOCOMO, NSN, Ericsson.

[5] R4-25xxxx, draft CR to TS 38.104: the introduction of Rel-19 ATG BS supporting CA, ZTE Corporation