3GPP TSG-RAN WG4 Meeting # 114bis	R4-2504016

# Wuhan, China, 7th – 11th April 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | <CR#> | rev | <Rev#> | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Test cases requirements for handover with PSCell delay and deactivated PSCell measurements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | WG4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_FR1_lessthan_5MHz_BW_Ph2-Perf |  |  |  |  |  | Date: |  |  | 2025-03-25 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Test cases for handover with PSCell delay and deactivated PSCell measurements are missing when the target PSCell is with 12PRB SSB BW. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Specify the test cases requirements - Handover with PSCell delay requirements when PSCell is with 12PRB SSB BW - Deactivated PSCell measurements delay requirements when PSCell is configured with 12PRB SSB BW |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Requirement is missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.6.3.1.X, A.6.6.1.X |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### <<Start of Change1>>

#### A.6.3.1.X	Handover with PSCell change delay where target PSCell is with 12PRB SSB bandwidth

##### A.6.3.1.X.1	Test Purpose and Environment

The purpose of this test is to verify the handover delay requirements and PSCell change delay requirements in HO with PSCell from NR-DC (FR1-FR1) to NR-DC (FR1-FR1) defined in clauses 6.1.5.4. The requirements are applicable to NR FR1-FR1 intra-frequency PCell handover and NR FR1-FR1 intra-frequency PSCell change. The requirements are only applicable when the target PSCell is configured with 12PRB SSB bandwidth.

##### A.6.6.1.X.2	Test Parameters

The supported test configurations are given in table A.6.3.1.X.2-1. Only 15kHz FDD cases are considered. The test scenario comprises 3 NR cells, source PCell(Cell 1), target PCell(Cell 2), target PSCell(Cell 3).

Cell 1 and Cell 2 are on radio channel 1 in FR1. Cell 3 is on radio channel 2 in FR1. Test parameters are given in Tables A.6.3.1.X.2-2, and A.6.3.1.X.2-3 below. Note that for Cell 3 the SSB configuration refers to SSB pattern 13 in FR1: SSB allocation for SSB SCS=15kHz in 3 MHz. In the test, the SSB is configured with 12PRB bandwidth.

The test consists of two successive time periods, with time durations of T1, T2 respectively. At the start of T1, the UE shall be connected to Cell 1 on radio channel 1. UE is not aware of Cell 2 and Cell 3. Starting T2, Cell 2 and Cell 3 becomes detectable to the UE and the UE receives a RRC handover command from the network. The start of T2 is the instant when the last TTI containing the RRC message implying handover is sent to the UE.

The UE is required to successfully handover from Cell 1 to Cell 2 and at the same time in time period T2 add Cell 3 as PSCell attached to Cell 2. This test case verifies the delay starting from the start of T2 to time points where the UE finishes handover to Cell 2 and where the UE finishes addition of Cell 3. Test cases delay requirements are specified.

Table A.6.3.1.X.2-1: Supported test configurations for HO with PSCell with 12PRB SSB bandwidth

| Config | Description |
| --- | --- |
| 1 | Source PCell: FR1 NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode Target PCell: FR1 NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode Target PSCell: FR1 NR 15 kHz SSB SCS, 3 MHz bandwidth, FDD duplex mode |

Table A.6.3.1.X.2-2: Cell specific test parameters for Intra-frequency handover

| Parameter |  |  | Unit | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T1 | T2 |
| NR RF Channel Number |  |  |  | 1 |  |  |  |
| Duplex mode |  | Config 1 |  | FDD |  |  |  |
| TDD configuration |  | Config 1 |  | Not Applicable |  |  |  |
| BWchannel |  | Config 1 | MHz | 10: NPRB,c = 52 |  |  |  |
| BWP BW |  | Config 1 | MHz | 10: NPRB,c = 52 |  |  |  |
| TRS configuration |  | Config 1 |  | TRS.1.1 FDD |  |  |  |
| DRX Cycle |  |  | ms | Not Applicable |  |  |  |
| PDSCH Reference measurement channel |  | Config 1 |  | SR.1.1 FDD |  |  |  |
| CORESET Reference Channel |  | Config 1 |  | CR.1.1 FDD |  |  |  |
| OCNG Patterns |  |  |  | OP.1 |  |  |  |
| SMTC Configuration |  |  |  | SMTC.1 |  |  |  |
| SSB Configuration |  | Config 1 |  | SSB.1 FR1 |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1 | kHz | 15 kHz |  |  |  |
| PUCCH/PUSCH subcarrier spacing |  | Config 1 | kHz | 15 kHz |  |  |  |
| PRACH configuration |  |  |  | FR1 PRACH configuration 1 |  |  |  |
| BWP |  | Initial DL BWP |  | DLBWP.0.1 |  |  |  |
|  |  | Dedicated DL BWP |  | DLBWP.1.1 |  |  |  |
|  |  | Initial UL BWP |  | ULBWP.0.1 |  |  |  |
|  |  | Dedicated UL BWP |  | ULBWP.1.1 |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |
| Note2 |  |  | dBm/15kHz | -98 |  |  |  |
| Note2 | Config 1 |  | dBm/SCS | -98 |  |  |  |
|  |  |  | dB | 8 | 8 | -Infinity | 12 |
|  |  |  | dB | 8 | 8 | -Infinity | 12 |
| SSB_RP | Config 1 |  | dBm/SCS | -90 | -90 | - Infinity | -93 |
| IoNote3 | Config 1 |  | dBm/ 9.36 MHz | -61.41 | -61.41 | - Infinity | -57.78 |
| Propagation condition |  |  | - | AWGN |  | AWGN |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |

Table A.6.3.1.X.2-3: Cell specific test parameters for Cell 3

| Parameter |  |  | Unit | Cell 3 |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 |
| NR RF Channel Number |  |  |  | 2 |  |
| Duplex mode |  | Config 1 |  | FDD |  |
| TDD configuration |  | Config 1 |  | Not Applicable |  |
| BWchannel |  | Config 1 | MHz | 3: NPRB,c = 15 |  |
| BWP BW |  | Config 1 | MHz | 3: NPRB,c = 15 |  |
| TRS configuration |  | Config 1 |  | TRS.1.1 FDD |  |
| DRX Cycle |  |  | ms | Not Applicable |  |
| PDSCH Reference measurement channel |  | Config 1 |  | SR.1.2 FDD |  |
| CORESET Reference Channel |  | Config 1 |  | CR.1.2 FDD |  |
| OCNG Patterns |  |  |  | OP.1 |  |
| SMTC Configuration |  |  |  | SMTC.1 |  |
| SSB Configuration |  | Config 1 |  | SSB.13 FR1 |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1 | kHz | 15 kHz |  |
| PUCCH/PUSCH subcarrier spacing |  | Config 1 | kHz | 15 kHz |  |
| PRACH configuration |  |  |  | FR1 PRACH configuration 1 |  |
| BWP |  | Initial DL BWP |  | DLBWP.0.1 |  |
|  |  | Dedicated DL BWP |  | DLBWP.1.1 |  |
|  |  | Initial UL BWP |  | ULBWP.0.1 |  |
|  |  | Dedicated UL BWP |  | ULBWP.1.1 |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |
| Note2 |  |  | dBm/15kHz | -98 |  |
| Note2 | Config 1 |  | dBm/SCS | -98 |  |
|  |  |  | dB | -Infinity | 12 |
|  |  |  | dB | -Infinity | 12 |
| SSB_RP | Config 1 |  | dBm/SCS | -Infinity | -86 |
| IoNote3 | Config 1 |  | dBm/ SCS MHz | -Infinity | -85.7 |
| Propagation condition |  |  | - | AWGN | AWGN |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |

##### A.6.3.1.X.3	Test Requirements

Handover to PCell delay requirements

In this test, the UE shall start to transmit the PRACH to target PCell (Cell 2) less than [78] ms from the beginning of time period T2.

The handover delay is defined as

DHOwithPSCell_PCell = RRC procedure delay + Tinterrupt, where

Tinterrupt = Tsearch + TIU + Tprocessing + T∆ + Tmargin

The UE is required to follow the below parameters according to derivation from the test setup specified:

- RRC procedure delay = 16 ms,

- Tsearch = 20 ms,

- TIU = 20 ms where PRACH association period should be configured as 1,

- Tprocessing = 20 ms,

- Tmargin = 2ms.

That sums up to 78ms exactly.

PSCell change/addition delay requirements

The UE shall transmit the PRACH to target PSCell (Cell 3) no later than [178] ms from the beginning of time period T2.

PSCell change delay during handover is defined as

DHOwithPSCell_PSCell = TRRC_delay + Tprocessing + Tsearch_PCell + Tsearch_PSCell + T∆ + TPSCell_ DU + 2 ms.

In this test the UE is required to follow the below parameters according to derivation from the test setup specified:

- TRRC_delay = 16ms,

- Tprocessing = 20 ms,

- Tsearch_PCell = 0,

- Tsearch_PSCell = 60 ms,

- T∆ = 60 ms,

- TPSCell_ DU = 1*10+10 = 20 ms where PRACH association period should be configured as 1.

That sums up to 178ms exactly.

The test is considered successful only when both requirements are correctly verified, and the success rate during repeated tests shall be at least 90 %.

<<End of Change1>>

### <<Start of Change2>>

#### A.6.6.1.X	Deactivated PSCell measurement test with 12 PRB SSB bandwidth in FR1

##### A.6.6.1.X.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of deactivated PSCell measurement results when the PSCell is configured with 12 PRB SSB bandwidth. This test verifies the deactivated PSCell measurement period requirements specified in clause 9.2.5.1.

##### A.6.6.1.X.2	Test Parameters

The supported test configurations are given in table A.6.6.1.X.2-1. Only 15kHz FDD cases are considered. The test scenario comprises 2 NR cells – PCell (Cell 1) and target PSCell (Cell 2).

Cell 1 is on radio channel 1 in FR1. Cell 2 is on radio channel 2 in FR1. Test parameters are given in Tables A.6.6.1.X.2-2, A.6.6.1.X.2-3 below. Note that for Cell 2 the SSB configuration refers to SSB pattern 13 in FR1: SSB allocation for SSB SCS=15kHz in 3 MHz. In the test, the SSB is configured with 12PRB bandwidth.

The test consists of two successive time periods, with time duration of T1, and T2 respectively.

At the start of T1, the UE is in connected mode to Cell 1. During time duration T1, the UE is configured to add Cell 2 as PSCell so that Cell 1 and Cell 2 serve UE under NR-DC operation. Meanwhile the UE is configured to measure on both Cell 1 and Cell 2 serving carrier frequencies. The UE is configured to read the SSB index of the target deactivated PSCell.

At the end of T1, the UE deactivates Cell 2. T2 starts exactly when the UE starts carrying out deactivated PSCell measurements on Cell 2. During T2, Cell 2 remains detectable to the UE.

Measurement gap or DRX is not configured.

The UE is required to make correct and timely reporting during T2. This test case verifies that the UE reporting is within the delay requirement and measurement accuracy is tested in other test cases.

Table A.6.6.1.X.2-1: Supported test configuration

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, PCell with 10 MHz bandwidth and PSCell with 3 MHz bandwidth, both FDD duplex mode |

Table A.6.6.1.X.2-2: General test parameters for deactivated PSCell measurements when PSCell is configured with 12 PRB SSB bandwidth

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| NR RF Channel Number |  | Config 1 | 1, 2 | Two FR1 NR carrier frequencies are |
| PCell |  | Config 1 | NR Cell 1 (PCell) | NR Cell 1 is on NR RF channel number 1. |
| Deactivated PSCell |  | Config 1 | NR Cell 2 | NR Cell 2 is on NR RF channel number 2. |
| measCyclePSCell | ms | Config 1 | 20 | Same as SMTC and SSB periodicity |
| Gap Pattern Id |  | Config 1 | Not configured |  |
| Measurement gap offset |  | Config 1 | Not configured |  |
| SMTC-SSB parameters |  | Config 1 | SSB.1 FR1 for Cell 1 SSB.13 FR1 for Cell 2 | As specified in clause A.3.10.1 |
| CP length |  | Config 1 | Normal |  |
| TimeToTrigger | s | Config 1 | 0 |  |
| Filter coefficient |  | Config 1 | 0 | L3 filtering is not used |
| DRX |  | Config 1 | OFF | DRX is not used |
| Time offset between cells |  | Config 1 | 3 ms | Asynchronous cells. The timing of Cell 2 is 3 ms later than the timing of Cell 1. |
| T1 | s | Config 1 | 5 |  |
| T2 | s | Config 1 | 1 |  |

Table A.6.6.1.X.2-3: Cell specific test parameters for deactivated PSCell measurement with 12 PRB SSB bandwidth

| Parameter |  | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T1 | T2 |
| NR RF Channel Number |  |  | Config 1 | 1 |  | 2 |  |
| Duplex mode |  |  | Config 1 | FDD |  |  |  |
| TDD configuration |  |  | Config 1 | Not Applicable |  |  |  |
| BWchannel |  | MHz | Config 1 | 10: NPRB,c = 52 |  | 3: NPRB,c = 15 |  |
| BWP BW |  | MHz | Config 1 | 10: NPRB,c = 52 |  | 3: NPRB,c = 15 |  |
| BWP configuration | Initial DL BWP |  | Config 1 | DLBWP.0.1 |  | DLBWP.0.1 |  |
|  | Initial UL BWP |  |  | ULBWP.0.1 |  | ULBWP.0.1 |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  | DLBWP.1.1 |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  | ULBWP.1.1 |  |
| TRS configuration |  |  | Config 1 | TRS.1.1 FDD |  | NA |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  |  | Config 1 | OP.1 |  | OP.1 |  |
| PDSCH Reference measurement channel |  |  | Config 1 | SR.1.1 FDD |  | SR.1.2FDD |  |
| RMSI CORESET Reference Channel |  |  | Config 1 | CR.1.1 FDD |  | CR.1.2 FDD |  |
| Dedicated CORESET Reference Channel |  |  | Config 1 | CCR.1.1 FDD |  | CCR1.6 FDD |  |
| SSB parameters |  |  | Config 1 | SSB.1 FR1 |  | SSB.13 FR1 |  |
| SMTC configuration defined in A.3.11 |  |  | Config 1 | SMTC.2 |  | SMTC.1 |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | Config 1 | 15 |  |  |  |
| EPRE ratio of PSS to SSS |  |  | Config 1 | 0 |  | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |
| Note2 |  | dBm/15 kHz |  | -98 |  | -98 |  |
| Note2 |  | dBm/SCS | Config 1 | -98 |  | -98 |  |
| SS-RSRP Note 3 |  | dBm/SCS | Config 1 | -94 |  | -91 |  |
|  |  | dB | Config 1 | 4 |  | 7 |  |
|  |  | dB | Config 1 | 4 |  | 7 |  |
| IoNote3 |  | dBm/SCS MHz | Config 1 | -92.6 |  | -90.2 |  |
| Propagation Condition |  |  | Config 1 | AWGN |  | AWGN |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |  |  |  |

##### A.6.6.1.X.3	Test Requirements

The UE shall send one measurement report for the deactivated PSCell, with a measurement reporting delay less than 680 ms from the beginning of time period T2.

The UE is required to successfully detect PSS/SSS for Cell 2 within

Ceil(5 x Kp) x measCyclePSCell x CSSFintra;

The UE is required successfully read the SSB index for Cell 2 within

Ceil(7 x Kp) x measCyclePSCell x CSSFintra;

The UE is required to successfully measure on Cell 2 within

Ceil(5 x Kp) x measCyclePSCell x CSSFintra;

Where

- measCyclePSCell = 20 ms,

- Kp = 1,

- CSSFintra = 2.

This sums up as 680 ms in total, consisting of 200 ms of PSS/SSS detection, 280 ms of SSB index detection and 200 ms of measurement time.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

<<End of Change2>>