3GPP TSG-RAN WG4 Meeting #114bis		R4-2504020

Wuhan, China, 7th – 11th April 2025

Agenda item:	7.17.2.1

Source:	Intel Corporation

Title:	Discussion on delay reduction by optimizing Rx beam sweeping factors

Document for:	Discussion

# Introduction

The Rel-19 NR RRM enhancements work item was approved in March 2024 and an updated WID was approved in RANP #105 in [1]. The work item descriptions of objectives of the item are shown in the below box.

- FR2-1 SSB based L3 measurement delay reduction for connected mode - For UE supporting multiple-Rx simultaneous reception on single carrier: - Study suitable scenarios and conditions and, if feasible, introduce methods to reduce FR2-1 L3 measurement delay by optimizing: - Rx beam sweeping factor - The CSSF requirement optimization is defined assuming a UE not operating with multiple-Rx based simultaneous L3 measurement: - Study suitable scenarios and conditions and, if feasible, introduce methods to reduce FR2-1 L3 measurement delay by optimizing: - CSSF outside gap in CA/DC scenarios - Baseline assumption on number of searchers is 2 - Note: Rel-19 CSSF optimization applies for CA/DC scenario independently of the UE support of multi-Rx capabilities. - Fast SCell activation for UE supporting Rel-18 EMR - Study and, if feasible, to reduce the SCell activation delay with valid EMR reporting - Apply fast scell activation in FR1 and FR2-1 - Note: RAN4 to start this work from Q3’2024 and aim for completion in Dec’2024. Workplan for this bullet can be discussed in May’2024

In the last WG meeting, a WF was approved in [2] to capture all the discussions and agreements.

In this paper we provide our analysis on delay reduction by optimizing Rx beam sweeping factors.

# Discussion

## Multi-Rx simultaneous reception enabling delay reduction of FR2-1 SSB based L3 measurements

Faster beam sweeping during handover and RRC re-establishment

In the previous WG meeting RAN4 agreed that for handover and re-establishment, UE supporting Rel-19 faster beams sweeping should always apply reduced beam sweeping factor during handover and RRC re-establishment procedures.

Agreement: - For Scenario 5/7: - For UE supporting Rel-19 BSF reduction capability, faster beam sweeping always applies during handover (Tsearch). - For UE supporting Rel-19 BSF reduction capability, faster beam sweeping always applies during RRC re-establishment.

Regarding handover delay requirements, this agreement means that Tsearch = N * Trs or 3N * Trs when the target cell is an unknown FR2 cell where N should be 4 (compared to 8 in the legacy spec) for FR2-1 and 6 (compared to 12) for FR2-2.

In the last WG meeting the discussion about handover on the additional processing time is as below.

[Wayforward] - For scenario 5 - FFS: - Option 1: to consider an additional processing delay (2ms) in the handover delay equation - Option 2: In the handover delay equation, SSB processing time (2ms) can be scaled by the number of RX beams that UE employs to simultaneously receive signals from multiple directions. - Option 3: no need to add additional processing time in the delay requirement.

We propose the below TP for Rel-19 specification for FR2 handover delay requirements.

When intra-frequency or inter-frequency handover is commanded, the interruption time shall be less than Tinterrupt Tinterrupt = Tsearch + TIU + Tprocessing + T∆ + Tmargin ms Where: Tsearch is the time required to search the target cell when the handover command is received by the UE. Regardless of whether DRX is in use by the UE, Tsearch shall still be based on non-DRX target cell search times. - If the target cell is a known intra-frequency cell, then Tsearch = 0ms. - If the target cell is an unknown intra-frequency cell and the target cell Es/Iot≥-2 dB, then Tsearch = N* Trs ms. - Otherwise, the target cell is an unknown inter-frequency cell and Tsearch = N*3* Trs ms if the target cell Es/Iot≥-2 dB. Where N = 8 when the target cell is in FR2-1, and N = 12 when the target cell is in FR2-2. If the UE supports [Rel-19 BSF reduction capability] and the network configures handover with reduced beam sweeping factor to the UE, N = 4 when the target cell is in FR2-1, and N = 6 when the target cell is in FR2-2.

Similarly for the re-establishment delay requirements, time to identify target NR FR2 cells for RRC connection re-establishment to NR intra- or inter- frequency cell is reduced in half.

Plus in the last WG meeting, SSB processing delay is agreed for RRC re-establishing.

Agreement: - For scenario 7 - to consider an additional processing delay (2ms) after the last SMTC periodicity

So, we propose the below TP:

TUE_re-establish_delay=50 ms+Tidentify_intra_NR+i=1Nfreq-1Tidentify_inter_NR,i+TSI-NR+TPRACH+TSSB_processing TSSB_processing: It is the additional processing delay of 2ms when the UE is configured with reduced beam sweeping factor and the UE supports [reducedBSFcapabilities]. It is 0ms otherwise.

New Table in 6.2.1.2.1: Time to identify target NR cell for RRC connection re-establishment to NR FR2 intra-frequency cell for UE supporting [Rel-19 BSF reduction] Serving cell; FR of target NR; Tidentify_intra_NR [ms]. SSB Ês/Iot (dB); cell; Known NR cell; Unknown NR cell. ≥ -8; FR2-1; N/A; MAX (1000 ms, [40] x TSMTC)). ≥ -8; FR2-2; N/A; MAX (1000 ms, [60] x TSMTC)). < -8; FR2-1; N/A; [1760]Note1. < -8; FR2-2; N/A; [2640]Note1. Note 1: The UE is not required to successfully identify a cell on any NR frequency layer when TSMTC > 20 ms and serving cell SSB Ês/Iot < -8 dB..

New Table in 6.2.1.2.1: Time to identify target NR cell for RRC connection re-establishment to NR FR2 inter-frequency cell for UE supporting [Rel-19 BSF reduction] Serving cell SSB Ês/Iot (dB); FR of target NR cell; Tidentify_inter_NR, i [ms]. Known NR cell; Unknown NR cell. ≥ -8; FR2-1; N/A; MAX (1000 ms, [52] x TSMTC, i)). ≥ -8; FR2-2; N/A; MAX (1000 ms, [78] x TSMTC, i)). < -8; FR2-1; N/A; [2000]Note1. < -8; FR2-2; N/A; [3000] Note1. Note 1: The UE is not required to successfully identify a cell on any NR frequency layer when TSMTC,i > 20 ms and serving cell SSB Ês/Iot < -8 dB..

Proposal 1: Beam sweeping factor/total sample number of measurements during handover and re-establishment can be cut half as a baseline while a margin can be added to the requirements.

Conditions to trigger reduced beam sweeping factor in UE L3 measurements

In the last WG meeting it is agreed that

Agreement: - RAN4 to introduce serving cell’s signal quality (i.e., based on RSRP and/or RSRQ threshold(s)) based triggering condition to activate/deactivate multi-Rx for L3 measurement. Agreement: - Not support UE mobility status based triggering condition in Rel-19.

Regarding serving cell quality triggering, we can look up to the existing specifications for references. In the context of CONNECTED mode measurements, serving cell quality is a critical factor in determining whether to trigger measurement relaxation. The serving cell quality is typically assessed based on certain criteria such as RSRP and RSRQ.

Regarding reduced sweeping factor in L3 measurements, specification for criteria over good cell quality should describe the conditions under which the UE can enhance its measurement activities based on the quality of the serving cell. This include the specific thresholds and criteria that the network can configure to determine when measurement relaxation is appropriate.

- The network configures the UE with specific thresholds for RSRP (Reference Signal Received Power) and/or RSRQ (Reference Signal Received Quality) to assess the quality of the serving cell.

- The configuration for reduced BSF and measurement enhancements is provided through specific information elements which define the quality thresholds for the serving cell. This is specified in TS 38.331.

- To avoid frequent toggling between measurement states, hysteresis and time-to-trigger parameters can be configured. Hysteresis adds a margin to the threshold to prevent rapid changes in measurement reporting. Time-to-trigger specifies the duration for which the condition must be met before triggering the event.

- In case serving cell quality criterion is configured, the UE is required to perform L3 measurements with reduced beam sweeping factor (basically cut in half), when the serving cell quality criterion is met, and the measurements enhancement is specified in TS 38.133.

- When the criterion is met, the Tidentify_intra_without_index = (TPSS/SSS_sync_intra + TSSB_measurement_period_intra +TSSB_processing) ms, and Tidentify_inter_without_index = (TPSS/SSS_sync_inter + T SSB_measurement_pe riod_inter +TSSB_processing) ms; where TSSB_processing = 2 ms, where TPSS/SSS_sync + TSSB_measurement_period is cut in half.

Proposal 2: To avoid frequent toggling between measurement states, hysteresis and time-to-trigger parameters can be configured. Hysteresis adds a margin to the threshold to prevent rapid changes in measurement reporting. Time-to-trigger specifies the duration for which the condition must be met before triggering the event.

Proposal 3: When the criterion is met, the Tidentify_intra_without_index = (TPSS/SSS_sync_intra + TSSB_measurement_period_intra +TSSB_processing) ms, and Tidentify_inter_without_index = (TPSS/SSS_sync_inter + T SSB_measurement_pe riod_inter +TSSB_processing) ms; where TSSB_processing = 2 ms, where TPSS/SSS_sync + TSSB_measurement_period is cut in half.

Exiting reduced beam sweeping factor state

The reduced beam sweeping factor state continues to apply until the UE reports the measurement results to the network. And the UE makes another exam on the criteria that are configured for the serving cell quality. If the criteria are met, the UE continues to use reduced beam sweeping factor while the UE exits the state when the quality criteria are no longer met.

At the same time, UE is allowed to use UE assistance information to indicate to the network its status in the context of reduced BSF.

Proposal 4: The reduced beam sweeping factor state continues to apply until the UE reports the measurement results to the network. And the UE makes another exam on the criteria that are configured for the serving cell quality. If the criteria are met, the UE continues to use reduced beam sweeping factor while the UE exits the state when the quality criteria are no longer met.

# Conclusion

In this paper we provide our views on the scope of FR2-1 SSB based L3 measurement delay reduction for connected mode.

Proposal 1: Beam sweeping factor/total sample number of measurements during handover and re-establishment can be cut half as a baseline while a margin can be added to the requirements.

Proposal 2: To avoid frequent toggling between measurement states, hysteresis and time-to-trigger parameters can be configured. Hysteresis adds a margin to the threshold to prevent rapid changes in measurement reporting. Time-to-trigger specifies the duration for which the condition must be met before triggering the event.

Proposal 3: When the criterion is met, the Tidentify_intra_without_index = (TPSS/SSS_sync_intra + TSSB_measurement_period_intra +TSSB_processing) ms, and Tidentify_inter_without_index = (TPSS/SSS_sync_inter + T SSB_measurement_pe riod_inter +TSSB_processing) ms; where TSSB_processing = 2 ms, where TPSS/SSS_sync + TSSB_measurement_period is cut in half.

Proposal 4: The reduced beam sweeping factor state continues to apply until the UE reports the measurement results to the network. And the UE makes another exam on the criteria that are configured for the serving cell quality. If the criteria are met, the UE continues to use reduced beam sweeping factor while the UE exits the state when the quality criteria are no longer met.

# References

- RP-242380, Revised WID: NR Radio Resource Management (RRM) Phase 5, Apple, CATT

- R4-2502593, WF on RRM requirements for NR_RRM_Ph5_Part2, CATT