3GPP TSG RAN WG4 Meeting #114bis	R4-2504047

Wuhan, Hubei, China, 07th – 11th April, 2025

Agenda item:			7.21.2.1

Source:	vivo

Title:	Discussion on RAN4 impacts for RRM measurement prediction

Document for:	Discussion

- Introduction

In RAN#105 meeting, the revised SID [1] was approved and RAN4 work is copied as

- Evaluate testability, interoperability, and impacts on RRM requirements and performance [RAN4] - Study the impacts on requirements based on RAN2 assumptions, and coordinate with RAN2 if needed - Study the testability and interoperability based on RAN2 framework (e.g., number of cells to measure, beams etc.) - NOTE 4: Leverage the work from “AI/ML for NR air interface” led by RAN1 and avoid the duplicate study for testability and interoperability. - NOTE 5: Avoid the overlaps with RAN2 work for evaluation

In this contribution, we further provide our views on the RAN4 impacts for RRM measurement prediction based on the latest WF[2].

- Discussion

<The definition of relative RSRP prediction accuracy>

In the RAN4#114 meeting, the issues on KPI definition on relative RSRP prediction accuracy was discussed. The outcomes are shown as follows.

Issue 2-1-1a: Relative RSRP Prediction Accuracy Agreement: - Predicted relative RSRP accuracy = (reported predicted RSRP of cell 1/beam1 – reported RSRP of cell 2/beam2) – (ground truth of RSRP of cell 1/beam1 – ground truth of RSRP of cell 2/beam2) - FFS: Beam 1 and Beam 2 can be from same cell or different cells. - FFS: whether ground truth can be reported or ideal - FFS: whether the reported RSRP of cell2/beam2 can be predicted or measured or a combination of both predicted and measured samples.

For the FFS aspects in the definition, we have the following analysis:

- Aspect 1:

Regarding reported RSRP of Cell2 in FR2 Scenarios, based on previous meeting conclusion on ground truth for testing purpose-"For FR2, the ground truth of L3 RSRP is defined as the reported L3 RSRP measurement under sufficiently high SNR.", if the reported RSRP of Cell2 is treated as a measured value, then in the predicted relative RSRP accuracy equation, the terms "reported RSRP of Cell2" and "ground truth RSRP of Cell2/Beam2" would cancel each other out. The equation would reduce to the difference between "reported predicted RSRP of Cell1" and "ground truth RSRP of Cell1"—which is exactly the definition of Absolute RSRP Accuracy. For cases involving a combination of both predicted and measured samples, we do not recommend using this as the reported RSRP of Cell2, considering such a hybrid value (partly predicted, partly measured) is not a suitable benchmark for comparison with the purely predicted value of Cell1, the reported RSRP of Cell 2 should refer to the predicted RSRP of Cell 2.

For FR1 Scenarios, while the ground truth definition for FR1 remains under discussion, from the perspective of the practical intent of Relative RSRP Accuracy, defining the reported RSRP of Cell2 as a predicted result (consistent with reported predicted RSRP of Cell1) is more logically justified. This approach ensures consistency in evaluating the model’s prediction performance for both predicted cells.

- Aspect 2:

Regarding the inclusion of RSRP comparisons between beams in the definition, we believe the first priority is to clarify which specific scenario this refers to. For measurement prediction, beam-related scenarios include:

- Scenario 6: FR2 to FR2 intra-frequency spatial domain

| Scenario number | Priority | Evaluation scenario | Target study goal | Methodology |
| --- | --- | --- | --- | --- |
| 6 | Middle | FR2 to FR2 intra-frequency spatial domain | 1st goal | Intra-cell |

This scenario is currently classified as medium priority in RAN2 discussions, but has not yet been agreed upon in RAN4 as one of the study items. Furthermore, while the definition of relative RSRP for spatial domain prediction has been discussed in AI PHY working groups, neither its necessity nor specific definition has been sufficiently justified through technical analysis.

- L3 beam-level prediction (Point E)

Based on RAN2 conclusion, L3 filtered beam level prediction cases are lower priority. Therefore, when defining relative RSRP accuracy, this scenario can be excluded from consideration.

Based on analysis on the possible scenarios for beam level, it can be observed that at least in the discussion of SI, there is no beam-level scenarios to be considered. For this reason, we recommend that the current relative RSRP accuracy should exclude beam-level RSRP prediction, and propose removing the term "beam" from the above definition.

- Aspect 3:

Regarding the definition of ground truth, we have discussed the ground truth for testing in several previous meetings. Assuming the discussion here focuses on Predicted Relative RSRP Accuracy from testing perspective, the definition of ground truth would depend on the resolution of Issue 2-1-2.

At least for FR2, it has already been agreed that the reported L3 RSRP measurement under sufficiently high SNR in FR2 will serve as the ground truth. As for FR1, we may wait for a clearer analysis and further conclusions regarding the two proposed approaches before finalizing whether to use reported or ideal measurements as the ground truth.

Additionally, since we understand this conclusion is intended to apply to the following three high-priority scenarios (Scenario 2, 3, and 4), we should specifically note that for Scenario 2 (FR1 to FR1 intra-frequency temporal domain Case B) and Scenario 4 (FR2 to FR2 intra-frequency temporal domain Case A), the prediction occurs in the temporal domain. Therefore, when comparing Relative RSRP Accuracy, the RSRP difference should be calculated between the predicted values and the corresponding ground truth values of Cell 1 and Cell 2 at a given same time instance.

| Scenario number | Priority | Evaluation scenario | Target study goal | Methodology |
| --- | --- | --- | --- | --- |
| 1 | Low | FR1 to FR1 intra-frequency temporal domain case A | 2nd goal | TBD |
| 2 | High | FR1 to FR1 intra-frequency temporal domain case B | 1st goal | Intra-cell |
| 3 | High | FR1 to FR1 inter-frequency (frequency domain) | 1st goal | Inter-cell |
| 4 | High | FR2 to FR2 intra-frequency temporal domain case A | 2nd goal | Intra-cell |
| 5 | Low | FR2 to FR2 intra-frequency temporal domain case B | 1st goal | TBD |
| 6 | Middle | FR2 to FR2 intra-frequency spatial domain | 1st goal | Intra-cell |

Since this is only SI phase, the specific KPI definitions and corresponding requirements will be further discussed in RAN4 Work Item phase. To knowledge, RAN2’s current discussions and simulations remain limited to scenarios where single/multiple cells are used as inputs and single cells are used as outputs. Therefore, the present discussion should be treated primarily as an alignment effort for the definition of Relative RSRP Accuracy. Whether Relative RSRP Accuracy will ultimately be adopted as a formal KPI in RAN4 requirements depends on whether scenarios with multiple cells as output will be supported in future standardization.

Proposal 1: The Predicted relative RSRP accuracy refers to

- Predicted relative RSRP accuracy = (reported predicted RSRP of cell 1/beam1 – reported predicted RSRP of cell 2/beam2) – (ground truth of RSRP of cell 1/beam1 – ground truth of RSRP of cell 2/beam2)

- The ground truth:

  - For FR1, the definition depends on the outcome of issue 2-1-2

  - For FR12, it refers to the reported L3 RSRP measurement

Observation 1: For the prediction case on frequency domain, the discussion and simulation of RAN2 is still limited to the scenario of single/multiple cells as input and a single cell as output.

Proposal 2: Whether Relative RSRP Accuracy will ultimately be included as a formal KPI in the final RAN4 requirements can be further evaluated upon confirmation of scenario support - particularly for cases involving single/multiple cells as input and multiple cells as output.

< Ground Truth Definition for RSRP accuracy >

Issue 2-1-2: Ground Truth Definition for RSRP accuracy Agreement: - The way the ground truth is extracted in the testing may differentiate between FR1 and FR2. Agreement: Take Option 1-1-FR2 as baseline, and further discuss any additional aspects need to be considered. - Option 1-1-FR2: The ground truth of L3 RPRP is the reported L3 RSRP measurement under sufficient high SNR in FR2. Agreement: - Alt 1: Ground truth for FR1 will be based on the transmitted or reception power - Alt 2: Ground truth for FR1 will be based on the reported measurement value under certain conditions - Further discuss the advantages and disadvantages for Alt 1 and Alt 2 considering the aspects such as: - The information at the TE side, on the timing UE performs and/or finishes the measurement and/or prediction - Impact of L3 filtering - Number of samples for L1 filtering - Timing window, if it exists, where UE can’t measure the ground truth in Alt2 - channel condition - Avoid UE cheat in the test - S(I)NR variations in the test at different time instances (for Alt2) - Other aspects are not precluded - Companies are encouraged to provide analysis on all or some of the aspects in the list

During the previous meeting, RAN4 reached preliminary conclusions regarding the Ground Truth definition for RSRP accuracy. For FR1, we will proceed to conduct separate analyses of Alt 1 and Alt 2 across all the proposed aspects mentioned above.

- The information at the TE side, on the timing UE performs and/or finishes the measurement and/or prediction

For Alt 2, to some company’s understanding, the primary challenge lies in potential misalignment between: The predicted time instance X, and the actual UE measurement time instance. Thereforem, it may need to define additional criteria for determining the ground truth value corresponding to time instance X, such as: Using the nearest measurement start/completion time (Y), or the UE reporting timestamp closest to X. To avoid defning such compalicated criteria, we propose to firstly consider the case that the the interval duration between time instances in both observation and prediction windows are aligned, in which the corresponding starting points are aligned as well. The feasibility lies in which both measurement and prediction configuration can be configured. The timing information can be known to TE by controling over both measurement configuration and prediction timing.

For Alt 1, considering the reference signal reception power value at the UE for any given time instance can be directly derived, from TE perspective, the reception power values at prediction time instance can be known.

- Impact of L3 filtering

For Alt 2, it can be discussed in case-by-case manner:

For the case where L1 predicts L1 (point A1 predicts point A1), the L1 measured value and predicted value are the values before L3 filtering, and the subsequent L3 filtering and legacy are the same.

If it is for the case that L1 predicts L3 (point A1 predicts point C), the input is the L1 measured value and the output is the L3 predicted value. The L3 filter is inserted in the model. The consolidation process and filtering is treated as the black box. Similarly, for the case where L3 predicts L3 (point C predicts point C), if L3 filtering is considered in both measurement and prediction, the L3 measured value is derived with conducting the specific consolidation process and filtering. The filtering coefficient is specific value configured by NW. And for the predicted value, the L3 result is directly output (the prediction is completely dependent on the model implementation, and there is no process from L1 to L3), so if the measured value is used as the ground truth (a specific implementation), there may be inaccurate due to the different filting processing.

RAN2 may also conduct further study on the effect of filter coefficient on the model inference performance. Considering that some models have poor generalization performance and are more sensitive to the filtering coefficient, possible ways include: 1)	output the predicted value at point B, and then complete the filtering process according to the coefficient configured by the network, and then get the result at point C; 2)	the filter coefficient is also used as part of the model input, and the explicit visible filtering process is executed in the model. However, from RAN4 perspective, if the reported value is considered as the ground truth, in order to avoid the inconsistency between the filtering coefficient and the specific process corresponding to the obtained measured value and the model implementation, at this stage, we can start from a simple assumption and consider that the filtering is configured optionally by the network, so we can first not consider the filtering process in the test, including measurement and prediction

For Alt 1, the measured value is the actual value, there is no such problem.

- Number of samples for L1 filtering

For L1 filtering, legacy is entirely dependent on the UE implementation, not an AI-specific issue, so it doesn't need to be considered here.

- Timing window, if it exists, where UE can’t measure the ground truth in Alt2

For L3-RSRP prediction, we believe that the analysis should combine with temporal domain case A and case B:

In the temporal domain case A, continuous measurement results in the prediction window are predicted by continuous historical measurement result(s) in the observation window. Given that the RRM measurement is performed continuously, the ground truth of each time instance can be obtained from the UE measurement.

For temporal domain case B, considering the measurement in the prediction window may be skipped directly, the RSRP measurement value corresponding to the time instance cannot be obtained. If the ground truth is defined as reported RSRP measurement results, then the UE is required to keep measuring, and the UE behavior is similar to case A. Therefore, we believe that a further study on temporal domain case B is needed before the reported RSRP measurement results are regarded as the ground truth.

- channel condition

- S(I)NR variations in the test at different time instances (for Alt2)

For the channel environment, it is considered that there will be some power changes between transmission and reception. From the perspective of UE, reception power should prevail. And for the channel and the SNR changes during the test, according to our understanding, as long as the interval length between every two time instances in the observation window is set to the same as the interval length between every two time instances in the prediction window and the starting point of the corresponding interval is aligned, then the signal environment corresponding to the measured value is the signal environment at the moment of the predicted time instance, and there is no influence caused by changes in channel conditions or excessively low channel conditions

The analysis of the above aspects is summarized in the following table:

Table 1 Analysis on Alt 1 vs. Alt 2 from potential aspects

| Aspect | Alt 1 Ground truth for FR1 will be based on the reception power | Alt 2 Ground truth for FR1 will be based on the reported measurement value under certain conditions |
| --- | --- | --- |
| TE-side information & timing | The RSRP value at any time instance can be directly derived by TE without alignment issues. | Misalignment risk between predicted (X) and actual measurement time (Y). Proposal: Suggestion to firstly consider the case that the the interval duration and starting point between time instances in both observation and prediction windows are aligned |
| Impact of L3 filtering | Measured values are actual; no filtering-related problems. | Complexity varies (e.g., L1→L1, L1→L3, L3→L3). Proposal: Filtering can be omitted in tests to avoid filtering coefficient inconsistencies. |
| L1 filtering samples | Legacy UE implementation; not AI-specific. | Legacy UE implementation; not AI-specific. |
| Timing window constraints | Ground truth is always available in both Temporal Case A and B. | In Temporal Case B, ground truth may be missing if measurements are skipped. It Needs further analysis. |
| Channel conditions & SNR | Channel variations irrelevant | Synchronized observation/prediction windows mitigate channel/SNR variability. |

Proposal 3: For the ground truth definition of L3-RSRP for FR1, the analysis for Alt 1 and Alt 2 considering relevant aspects listed as follows:

| Aspect | Alt 1 Ground truth for FR1 will be based on the reception power | Alt 2 Ground truth for FR1 will be based on the reported measurement value under certain conditions |
| --- | --- | --- |
| TE-side information & timing | The RSRP value at any time instance can be directly derived by TE without alignment issues. | Misalignment risk between predicted (X) and actual measurement time (Y). Proposal: Suggestion to firstly consider the case that the the interval duration and starting point between time instances in both observation and prediction windows are aligned |
| Impact of L3 filtering | Measured values are actual; no filtering-related problems. | Complexity varies (e.g., L1→L1, L1→L3, L3→L3). Proposal: Filtering can be omitted in tests to avoid filtering coefficient inconsistencies. |
| L1 filtering samples | Legacy UE implementation; not AI-specific. | Legacy UE implementation; not AI-specific. |
| Timing window constraints | Ground truth is always available in both Temporal Case A and B. | In Temporal Case B, ground truth may be missing if measurements are skipped. It Needs further analysis. |
| Channel conditions & SNR | Channel variations irrelevant | Synchronized observation/prediction windows mitigate channel/SNR variability. |

- Summary

Proposal 1: The Predicted relative RSRP accuracy refers to

- Predicted relative RSRP accuracy = (reported predicted RSRP of cell 1/beam1 – reported predicted RSRP of cell 2/beam2) – (ground truth of RSRP of cell 1/beam1 – ground truth of RSRP of cell 2/beam2)

- The ground truth:

  - For FR1, the definition depends on the outcome of issue 2-1-2

  - For FR12, it refers to the reported L3 RSRP measurement

Observation 1: For the prediction case on frequency domain, the discussion and simulation of RAN2 is still limited to the scenario of single/multiple cells as input and a single cell as output.

Proposal 2: Whether Relative RSRP Accuracy will ultimately be included as a formal KPI in the final RAN4 requirements can be further evaluated upon confirmation of scenario support - particularly for cases involving single/multiple cells as input and multiple cells as output."

Proposal 3: For the ground truth definition of L3-RSRP for FR1, the analysis for Alt 1 and Alt 2 considering relevant aspects listed as follows:

| Aspect | Alt 1 Ground truth for FR1 will be based on the reception power | Alt 2 Ground truth for FR1 will be based on the reported measurement value under certain conditions |
| --- | --- | --- |
| TE-side information & timing | The RSRP value at any time instance can be directly derived by TE without alignment issues. | Misalignment risk between predicted (X) and actual measurement time (Y). Proposal: Suggestion to firstly consider the case that the the interval duration and starting point between time instances in both observation and prediction windows are aligned |
| Impact of L3 filtering | Measured values are actual; no filtering-related problems. | Complexity varies (e.g., L1→L1, L1→L3, L3→L3). Proposal: Filtering can be omitted in tests to avoid filtering coefficient inconsistencies. |
| L1 filtering samples | Legacy UE implementation; not AI-specific. | Legacy UE implementation; not AI-specific. |
| Timing window constraints | Ground truth is always available in both Temporal Case A and B. | In Temporal Case B, ground truth may be missing if measurements are skipped. It Needs further analysis. |
| Channel conditions & SNR | Channel variations irrelevant | Synchronized observation/prediction windows mitigate channel/SNR variability. |

- References

- RP-242393 Revised SID on AIML for mobility in NR, OPPO

- R4-2502594 WF on AI/ML Mobility, Nokia