3GPP TSG-WG RAN4 Meeting #114bis	R4-2504053

# Wuhan, Hubei, China, 07th – 11th April, 2025

Document for:	Approval

Agenda item:	7.9.2

Source: 	Qualcomm Incorporated, Astrum Mobile

Title: 	Discussion on band definition and system parameters

Document for:	Approval

# 1. Introduction

In RAN#107, the new WID for new band for 5G Broadcast Utilizing Geosynchronous satellite was approved in [1] with the following objectives:

- Specify a new SDO band for LTE-based 5G Broadcast operating in 1467 to 1492 MHz in Region 3 for GSO: - Specify related UE RF core requirements for 10MHz PMCH - UE requirements specified in TS 36.101 for LTE-based 5G Broadcast shall be considered as baseline (reuse assumption of 10 MHz UE Rx filter) - Specify related SAN RF core requirements for 10 MHz PMCH considering also applicable regulatory requirements: - Take existing SAN RF requirements specified in TS 38.108 for 10MHz as baseline - Requirements based on Resolution 761 in WRC-19 - Only consider QPSK and 16QAM modulation Note: UE only needs to support 5G broadcast for this band Note: Release independence applicability will be further discussed

In this paper, we provide our views on band definition and system parameters.

# 2. Discussion

As [1], the new SDO band for LTE based 5G utilizing GSO shall be specified in NTN specifications including TS 36.102 and TS 36.108. Following the same specification structure as LTE 5G terrestrial broadcast, it is proposed to create a new clause with suffix C in TS 36.102 and TS 38.108 to specify the corresponding SDO band, system parameters, UE and SAN RF requirements for LTE based 5G broadcast over GSO/GEO satellite.

Proposal 1: To create a new clause with suffix C in TS 36.102 and TS 38.108 to specify the corresponding SDO band, system parameters, UE and SAN RF requirements (if needed) for LTE based 5G broadcast over GSO/GEO satellite.

The spectrum of 1467-1492MHz has been using more than 20 years for satellite broadcasting. The LTE based 5G broadcast over GSO/GEO is designed to operate in 1467 to 1492 MHz in Region 3. Therefore, we have the following proposal:

Proposal 2: To specify the new SDO band operating in 1467 to 1492 MHz, i.e., Band 252, in Table 5.2C-1 of TS 36.102 and Table 5.2C-1 of TS 36.108.

## 5.2C	Operating bands for 5G broadcast

…

Table 5.2C-1 5G broadcast operating bands for satellite access in TS 36.102

| E-UTRA Operating Band | Uplink (UL) operating band BS receive UE transmit |  |  | Downlink (DL) operating band BS transmit UE receive |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| 252 |  | N/A |  | 1467 MHz | – | 1492 MHz | SDO |
| NOTE: Band 252 only applies for GSO in region 3. |  |  |  |  |  |  |  |

Table 5.2C-1 5G broadcast operating bands for satellite access in TS 36.108

| E-UTRA Operating Band | Uplink (UL) operating band BS receive UE transmit |  |  | Downlink (DL) operating band BS transmit UE receive |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| 252 |  | N/A |  | 1467 MHz | – | 1492 MHz | SDO |
| NOTE: Band 252 only applies for GEO in region 3. |  |  |  |  |  |  |  |

PMCH bandwidth for the LTE based 5G terrestrial broadcast with 6/7/8 MHz bandwidth are defined in TS36.101. As [1], this WI is to specify 10MHz PMCH for new SDO band for LTE based 5G broadcast over satellite.

Proposal 3: For the new band of band 252, it is proposed to specify 10 MHz PMCH bandwidth in TS 36.102 and TS 36.108.

## 5.3C	Channel bandwidth for LTE based 5G broadcast

…

Table 5.3C-1: Transmission bandwidth configuration NRB for 5G broadcast in TS 36.101 and TS 36.108

| PMCH bandwidth [MHz] | 10 |
| --- | --- |
| Transmission bandwidth configuration NRBPMCH | 50 |

Table 5.3C-2: 5G broadcast PMCH bandwidths per operating band in TS 36.101 and TS 36.108

| Operating Band | PMCH bandwidth |
| --- | --- |
|  | 10MHz |
| 252 | Yes |

The channel spacing, channel raster and EARFCN for applicable bands designated for LTE based 5G broadcast utilizing satellite shall leverage from requirements defined in TS36.101 and TS 36.108 for LTE based 5G terrestrial broadcast.

Proposal 4: The channel spacing, channel raster and EARFCN for LTE based 5G broadcast utilizing satellite should be defined in TS 36.102 and TS 36.108 as following.

## 5.4C	Channel arrangement for LTE based 5G broadcast

### 5.4C.1	Channel spacing

Nominal channel spacing between adjacent broadcast channels is defined as follows

Nominal Channel spacing = PMCH bandwidth

where PMCH bandwidth is the broadcast bandwidth indicated by upper layer signaling pmch-Bandwidth in the MBSFN area (see TS 36.331 [7]).

### 5.4C.2	Channel raster, carrier frequency and EARFCN

The channel raster is 100 kHz for all bands, which means that the carrier centre frequency must be an integer multiple of 100 kHz.

The EARFCN for applicable bands designated for LTE based 5G broadcast according to Table 5.2C-1 are specified in Table 5.4C.2-1.

Table 5.4C.2-1: E-UTRA channel numbers for LTE based 5G broadcast in TS 36.102 and TS 36.108

| Operating Band | Downlink |  |  | Uplink |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | FDL_low (MHz) | NOffs-DL | Range of NDL | FUL_low (MHz) | NOffs-UL | Range of NUL |
| 252 | 1467 | 228521 | 228501 – 228500 | N/A |  |  |
| NOTE 1: The channel numbers that designate carrier frequencies so close to the operating band edges that the broadcast carrier with PMCH bandwidth extends beyond the operating band edge shall not be used. |  |  |  |  |  |  |

# 3. Conclusion

In this paper, we have the following proposals as below.

Proposal 1: To create a new clause with suffix C in TS 36.102 and TS 38.108 to specify the corresponding SDO band, system parameters, UE and SAN RF requirements (if needed) for LTE based 5G broadcast over GSO/GEO satellite.

Proposal 2: To specify the new SDO band operating in 1467 to 1492 MHz, i.e., Band 252, in Table 5.2C-1 of TS 36.102 and Table 5.2C-1 of TS 36.108.

## 5.2C	Operating bands for 5G broadcast

…

Table 5.2C-1 5G broadcast operating bands for satellite access in TS 36.102

| E-UTRA Operating Band | Uplink (UL) operating band BS receive UE transmit |  |  | Downlink (DL) operating band BS transmit UE receive |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| 252 |  | N/A |  | 1467 MHz | – | 1492 MHz | SDO |
| NOTE: Band 252 only applies for GSO in region 3. |  |  |  |  |  |  |  |

Table 5.2C-1 5G broadcast operating bands for satellite access in TS 36.108

| E-UTRA Operating Band | Uplink (UL) operating band BS receive UE transmit |  |  | Downlink (DL) operating band BS transmit UE receive |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| 252 |  | N/A |  | 1467 MHz | – | 1492 MHz | SDO |
| NOTE: Band 252 only applies for GEO in region 3. |  |  |  |  |  |  |  |

Proposal 3: For the new band of band 252, it is proposed to specify 10 MHz PMCH bandwidth in TS 36.102 and TS 36.108.

## 5.3C	Channel bandwidth for LTE based 5G broadcast

…

Table 5.3C-1: Transmission bandwidth configuration NRB for 5G broadcast in TS 36.101 and TS 36.108

| PMCH bandwidth [MHz] | 10 |
| --- | --- |
| Transmission bandwidth configuration NRBPMCH | 50 |

Table 5.3C-2: 5G broadcast PMCH bandwidths per operating band in TS 36.101 and TS 36.108

| Operating Band | PMCH bandwidth |
| --- | --- |
|  | 10MHz |
| 252 | Yes |

Proposal 4: The channel spacing, channel raster and EARFCN for LTE based 5G broadcast utilizing satellite should be defined in TS 36.102 and TS 36.108 as following.

## 5.4C	Channel arrangement for LTE based 5G broadcast

### 5.4C.1	Channel spacing

Nominal channel spacing between adjacent broadcast channels is defined as follows

Nominal Channel spacing = PMCH bandwidth

where PMCH bandwidth is the broadcast bandwidth indicated by upper layer signaling pmch-Bandwidth in the MBSFN area (see TS 36.331 [7]).

### 5.4C.2	Channel raster, carrier frequency and EARFCN

The channel raster is 100 kHz for all bands, which means that the carrier centre frequency must be an integer multiple of 100 kHz.

The EARFCN for applicable bands designated for LTE based 5G broadcast according to Table 5.2C-1 are specified in Table 5.4C.2-1.

Table 5.4C.2-1: E-UTRA channel numbers for LTE based 5G broadcast in TS 36.102 and TS 36.108

| Operating Band | Downlink |  |  | Uplink |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | FDL_low (MHz) | NOffs-DL | Range of NDL | FUL_low (MHz) | NOffs-UL | Range of NUL |
| 252 | 1467 | 228521 | 228501 – 228500 | N/A |  |  |
| NOTE 1: The channel numbers that designate carrier frequencies so close to the operating band edges that the broadcast carrier with PMCH bandwidth extends beyond the operating band edge shall not be used. |  |  |  |  |  |  |

# 6. References

- RP-250788, “New WID: New band for 5G Broadcast Utilizing Geosynchronous Satellite”, Astrum Mobile,    Qualcomm, RAN#107, March, 2025