3GPP TSG-RAN WG4 Meeting #114bis				                  R4-2504067

# Wuhan, Hubei, China, 07th – 11th April, 2025

Source:	Sony

Title:	OCC scheme on the RF requirements for IoT NTN

Agenda item:	7.30.2

Document for:	Discussion

# Background

In RAN4#113, the impact of OCC scheme on IoT NTN RF requirements has been further discussed. Based on the progress in RAN1 and RAN4, it has been concluded that there will be no RF requirement impact for OCC-2 with the symbol level for 3.75 kHz SCS, and the UE RF requirement impact from 15 kHz single-tone NPUSCH with slot-level OCC2 is pending on RAN1 progress of resource mapping.

In addition, two other issues, including the gap in the middle of the OCC scheme and the mechanism to allow UE to resync to avoid high and opposite frequency inaccuracy between a pair of OCC UEs, were discussed, with the following agreements made [1].:

#### Issue 2-1: UE RF requirement impact from 15 kHz single-tone NPUSCH with slot-level OCC2

- Agreement:

  - No RF impact on OCC2 for IoT NTN.

#### Issue 2-2: Whether there would be UE RF requirement impact when OCC schemes would cross gap in UL transmission.

- Agreement:

  - No RF impact for <1ms gap length.

  - FFS for longer gaps.

#### Issue 2-3: Whether UE would need to re-synchronize its TX frequency periodically to avoid high and opposite frequency inaccuracy between a pair of OCC UEs.

- Agreement:

  - Further discuss whether solutions (e.g., CFO grouping) for solving frequency inaccuracy between OCC UEs would have potential RF impact.

Meanwhile, a RAN1 LS was sent to RAN4 [2] asking about the feasibility to introduce support of TDM DMRS for 3.75kHz SCS NPUSCH.

In this contribution, we provide our views on the issues discussed above.

# TDM DMRS for 3.75 kHz SCS

In the RAN1 LS to RAN4 (R1-2501621), RAN1 has made the following working assumption on the DMRS pattern for OCC for 3.75kHz SCS OCC for NPUSCH format 1, and RAN1 is asking from a RAN4 perspective, if the proposed scheme is feasible.

For 3.75kHz SCS OCC for NPUSCH format 1, support TDM DMRS over 4 slots where DMRS are transmitted in the first 2 slots and DMRS REs are blanked in the next 2 slots, or vice-versa, where the DMRS REs are as in legacy NB-IoT and the guard period within the slot is as in legacy NB-IoT.

Based on the FL summary in RAN1 [3], the scheme described above can be illustrated as in Fig. 1:

Fig. 1. TDM DMRS scheme 2 UEs.

In our understanding, the proposed scheme has been well studied in RAN1, and a performance loss compared to CDM DMRS has been identified due to lower DMRS energy. It is understood that the TDM DMRS scheme that RAN1 settled on is a compromise that accounts for this performance loss as well as other factors (including coexistence, specification impact etc.). The main issue for RAN4 to investigate is whether there would be a phase continuity issue over a blanked symbol.

Observation 1: The main issue for RAN4 to discuss on the TDM DMRS is on the phase continuity over the blanked symbol.

The blanked DMRS symbol creates a gap within the continuous transmission, which causes concern on the phase continuity over this gap. Based on the study from previous releases on DMRS bundling for NR coverage enhancement [4], it was concluded that for non-back-to-back transmission with non-zero gap in-between adjacent transmissions, the 13-symbol gap is the maximum length for the gap to keep the phase continuity across the gap, which equals to 1ms in terms off time duration. Such a gap duration is also specified in TS 38.214.

In this case, due to the short duration of the gap, it is likely that the UE will keep the Tx chain on and thus no phase jump would occur over the gap.

Observation 2: During the Rel-17 coverage enhancement WI, it was concluded that 13-symbol (< 1ms) gap is the maximum length for a non-zero gap in-between adjacent transmissions to keep the phase continuity across the gap.

Although the previous study in RAN4 was for NR devices, considering that a DMRS symbol is only about 275us long with 3.75 kHz SCS, which is much shorter than the 1ms gap length that RAN4 concluded on for NR, we don’t foresee any issue with respect to the phase continuity over the blank symbol.

Observation 3: It is feasible from the UE perspective to keep the phase continuity across the blank DMRS symbol.

However, one difference compared to the previous coverage enhancement WI is that it was agreed that from the test perspective, the test equipment (mimicking a gNB) should correct the CFO from the UE in a slot-by-slot manner (see below) in the coverage enhancement WI. However, since the DMRS in 2 consecutive slots will be blanked now with TDM DMRS, it is unclear if the network can still estimate the UE phase accurately even if there is no phase jump from the UE side. In particular, the large gap (6 ms) in between slot 2 and slot 5 as in Fig. 1 can cause a phase wrap up issue as the worst-case phase error will go beyond the 360 degree (e.g., 6ms*0.1ppm*2GHz*360 degree = 432 degree).

However, per our knowledge, this issue has been studied in RAN1 and contributions from companies in RAN1 [5] has explained that the wrap around issue over the blanked DMRS slots can be resolved by using the information of consecutive slots with DMRS. For example, taking the configuration in Fig. 1 as an example, the network can measure the phase difference between slot 1 and 2, and also phase difference between slot 5 and 6 of UE1, to understand how fast the phase changes with CFO. Then, it can use this information to estimate the CFO on slot 3 and slot 4, with unwrapped phase.

Observation 4: the phase wrap issue over DMRS blanked slots has been considered in RAN1 discussion when designing TDM DMRS and it can be resolved by combining with estimation from consecutive slots with DMRS.

Therefore, based on the understanding above, the following reply is suggested:

Proposal 1: provide the following reply to RAN1:

RAN4 has studied the feasibility of TDM DMRS.

- RAN4 has concluded that it is feasible for the UE to maintain the phase continuity across the blank DMRS symbol in TDM DMRS scheme.

- RAN4 also discussed the phase wrapping issue on CFO estimation of the slots with blanked DMRS, but it is understood that there can be solutions from network side to resolve this issue.

Therefore, from RAN4 side, no feasibility issue is identified and RAN4 respectfully asks RAN1 to take into account the information above when determining the final feasibility of TDM DMRS.

# OCC scheme across the gap

In RAN4#114, it has been concluded that a small gap that is <1ms in length has no impact on the RF requirement. For larger gaps that span over several ms, it can be expected that the phase continuity would not be able to be maintained over a gap unless the UE has some dedicated design. For example, the UE may need to buffer the phase information before the gap and use it to calibrate the phase of the UL transmission after the gap. In either case, this may not be generally applied to all UEs, and thus, a UE capability is needed if OCC crosses such gaps would be needed.

Observation 5: For larger gaps over several ms, dedicated UE implementation would be needed to support the phase continuity across the gap.

Proposal 2: If gap larger than 1ms needs to be supported for OCC scheme, dedicated UE capability and requirements need to be defined.

# Resynchronize TX frequency periodically

Whether the UE would need to resync with network was discussed in RAN4#114, and the need of such an operation is questioned if the network can group the UE with similar CFO. In our understanding, to perform such a CFO grouping, the network needs to predict the CFO from UEs that would be grouped.

Observation 6: In order to group the UE with similar CFO, the network needs to predict the CFO of UEs.

As discussed in section 1 already, the current assumption is that the network estimates the CFO of UEs in a slot-by-slot manner and it is unclear how network can estimate the UE CFO performances before grouping them into the one OCC pair. To fully understand the feasibility and potential impact on the RF requirements, it is proposed that RAN4 shall clarify the operation mechanism of CFO from UE.

Proposal 3: RAN4 needs to clarify the operation mechanism of CFO grouping to understand if there is any RF requirement impact.

On the other hand, if the UE is allowed to resync with the network time to time, it can minimize the relative error between the paired UEs to avoid the large and opposite frequency error, and also without requiring any advanced estimation algorithm from the network side. For example, suppose a transmission consists of 16 OCC codewords, and resynchronization occurs between the first set of 8 codewords and the second set of 8 codewords. In that case, it is unlikely that both the first set and second set of OCC codewords would observe high and opposite frequency inaccuracy (e.g. the first set of OCC codewords might have inaccuracy of +/- 0.1ppm and the second set of OCC codewords might have an inaccuracy of 0/0ppm). The FEC and interleaving across the OCC codewords would then reduce the probability of block error.

Observation 7: allowing the UE to resync with the network from time to time can avoid large and opposite frequency errors without requiring more advanced estimation of CFO from the network side.

A small sync gap is needed to perform such a resynchronization. As discussed in Section 1, a small gap below 1 ms would not affect the OCC scheme to cross the gap. Then, the next question is how often such a resync is needed. For legacy NB-IoT UEs, a 40 ms measurement gap is scheduled after a 256 ms transmission. The 40 ms is based on the typical behavior of the frequency error estimator, so the frequency error could be corrected low enough. Some analysis has been given [6]. Therefore, with the assumption that a 40 ms measurement is needed to fix the frequency error accumulated over 256 ms, to have a gap of less than 1 ms, an accumulated error over a periodicity with 256/40 = 6.4 ms can be considered.

Proposal 4: Define a resynchronized gap of 1 ms with a periodicity of about 6.4 ms to avoid high and opposite frequency inaccuracy between a pair of OCC UEs.

Please note that there can also be other methods, for example, the UE determines two independent estimates of the required UL frequency before NPUSCH transmission and the UE then switches its output frequency between those two estimates during the transmission, to randomize the CFO and reduce the probability of large and opposite frequency error. So, they can be considered if resync is considered to be not feasible.

# Conclusion

In this contribution, we make the following observations and conclusions:

Observation 1: The main issue for RAN4 to discuss on the TDM DMRS is on the phase continuity over the blanked symbol.

Observation 2: During the Rel-17 coverage enhancement WI, it was concluded that 13-symbol (< 1ms) gap is the maximum length for a non-zero gap in-between adjacent transmissions to keep the phase continuity across the gap.

Observation 3: It is feasible from the UE perspective to keep the phase continuity across the blank DMRS symbol.

Observation 4: the phase wrap issue over DMRS blanked slots has been considered in RAN1 discussion when designing TDM DMRS and it can be resolved by combining with estimation from consecutive slots with DMRS.

Observation 5: For larger gaps over several ms, dedicated UE implementation would be needed to support the phase continuity across the gap.

Observation 6: In order to group the UE with similar CFO, the network needs to predict the CFO of UEs.

Observation 7: allowing the UE to resync with the network from  time to time can avoid large and opposite frequency errors without requiring more advanced estimation of CFO from the network side.

Proposal 1: provide the following reply to RAN1:

RAN4 has studied the feasibility of TDM DMRS.

- RAN4 has concluded that it is feasible for the UE to maintain the phase continuity across the blank DMRS symbol in TDM DMRS scheme.

- RAN4 also discussed the phase wrapping issue on CFO estimation of the slots with blanked DMRS, but it is understood that there can be solutions from network side to resolve this issue.

Therefore, from RAN4 side, no feasibility issue is identified and RAN4 respectfully asks RAN1 to take into account the information above when determining the final feasibility of TDM DMRS.

Proposal 2: If gap larger than 1ms needs to be supported for OCC scheme, dedicated UE capability and requirements need to be defined.

Proposal 3: RAN4 needs to clarify the operation mechanism of CFO grouping to understand if there is any RF requirement impact.

Proposal 4: Define a resynchronized gap of 1 ms with a periodicity of about 6.4 ms to avoid high and opposite frequency inaccuracy between a pair of OCC UEs.

# References

- R4-2500576	Topic summary for [114][315] IoT_NTN_Ph3 	MediaTek

- R4-2502373	Way Forward for [114][315] IoT_NTN_Ph3 	MediaTek

- R1-2501622 LS on TDM DMRS for 3.75kHz SCS OCC

- R4-2103393 Reply LS on PUCCH and PUSCH repetition

- R1-2501176	IOT-NTN uplink capacity/throughput enhancement	Qualcomm Incorporated

- R4-163255	Simulation results of UCG parameters for NB-IoT	 Intel Corporation