3GPP TSG-RAN WG4 Meeting #114bis	R4-2504071

Wuhan, China, April 7th – April 11th, 2025

Source:	Nokia

Title:	On event-triggered L1 reporting for LTM

Agenda item:	7.27.2.1

Document for:	Discussion

Introduction

In this contribution we continue the discussion for Rel-19 event-triggered L1 reporting for LTM.

Discussion

Event-triggered reporting delay

In the past RAN4 meetings, the following agreements have been made regarding the event-triggered reporting delay:

- Starting point in event triggered L1 reporting requirements is the time when a condition exists at the UE which will trigger the reporting.

- Ending point in event triggered L1 report requirements is defined as the point when the UE starts to transmit the first UL transmission to report measurement result over the air interface.

- Additional time TSSB_time_index for SSB index acquisition is needed, unless deriveSSB-IndexFromCell is enabled or UE has reported SSB index in L3 measurement report of the same cell.

- No need to explicitly capture TTT in core requirements.

- RAN4 should start work assuming no L1 filtering.

- For event triggered L1 measurement reporting, measurement reporting delay excludes a delay which is caused by no UL resources being available for UE to send the measurement report on, which is to follow the approach for event triggered L3 measurement reporting.

What remains to be open is how to define the measurement evaluation delay from when the reporting condition becomes fulfilled until the UE realizes the condition is fulfilled and triggers the report. In the last meeting this definition was discussed under issue 2-2 [1]:

Issue 2-2: Event triggered reporting delay for L1 reporting Agreement: - Except for periodic MR: - Option 1: The event triggered measurement reporting delay shall be no larger than the maximum L1-RSRP measurement period on the cells corresponding to the event. - The measurement period on the LTM candidate cells is based on the Rel-18 requirement, with the assumption of Treport is 0. - Option 2: The event triggered measurement reporting delay shall be no larger than the L1-RSRP measurement period corresponding to the Cell(s) which triggers the event on the cells corresponding to the event. - The measurement period on the LTM candidate cells is based on the Rel-18 requirement, with the assumption of Treport is 0. - For periodic MR: - Option 1: For the first report after condition is met, the event triggered measurement reporting delay shall be no larger than the maximum L1-RSRP measurement period on the cells corresponding to the event. - RAN4 define delay requirement for the first report after condition is met. After that, UE follow the network configuration on the reporting interval. FFS how to capture this agreement into the CR. - Option 2: For the first report after condition is met, the event triggered measurement reporting delay shall be no larger than the L1-RSRP measurement period corresponding to the Cell(s) which triggers the event on the cells corresponding to the event. - RAN4 define delay requirement for the first report after condition is met. After that, UE follow the network configuration on the reporting interval. FFS how to capture this agreement into the CR.

In our understanding there are no differing views in the group about the UE behavior regarding the event evaluation and the measurement that triggers the event after the condition becomes fulfilled. Depending on when the condition becomes met over the air in relationship to the ongoing measurements, UE may need to measure either serving cell/beam, candidate cell/beam, or both before it can realize the condition for reporting is fulfilled. UE shall trigger the report based on the first measurement that indicates that the condition is fulfilled. There do not seem to be differing views about this. Hence, the discussion is purely about how to capture the delay.

In our view, all the above scenarios can be covered by simply defining the delay based on the measurement that will trigger the report, let it then be serving or candidate cell measurement. If both measurements need to be performed, the delay is still the time until the measurement that triggers the delay. Therefore, we would propose to conclude the issue by defining the condition evaluation delay as:

The SSB-based event triggered measurement reporting delay shall be less than TL1-RSRP_Measurement_Period_SSB_intra (clause 9.14.5) or TL1-RSRP_Measurement_Period_SSB_Inter (clause 9.15.5), where Treport = 0 and TSSB_NBC is the periodicity of the (serving or candidate cell) SSB that triggers the report.

Approach of proposal 1 can be followed for CSI-RS based event-triggered reporting delay once the CSI-RS measurement requirements are concluded.

RAN4 can then consider in the test case discussion which of the scenarios to test.

For event-triggered periodic reporting, we propose to define the requirements similar as for L3 event-triggered periodic reporting i.e. by simply defining delay requirements for the first report, and for the following periodic reports just define the accuracy requirements:

#### 9.2.4.2 Event-triggered Periodic Reporting Reported RSRP, RSRQ, and RS-SINR measurements contained in event-triggered periodic measurement reports shall meet the requirements in clauses 10.1.2.1 (RSRP for FR1), 10.1.3.1 (RSRP for FR2), 10.1.7.1 (RSRQ for FR1), 10.1.8.1 (RSRQ for FR2), 10.1.12.1 (RS-SINR for FR1) and 10.1.13.1 (RS-SINR for FR2). The first report in event triggered periodic measurement reporting shall meet the requirements specified in clause 9.2.4.3.

For event-triggered periodic reporting, apply the event-triggered reporting requirements for the first report, and for the following periodic reports simply specify the measurement accuracy requirements the reported measurements shall fulfil (similar to L3 event-triggered periodic reporting).

Impact on other RRM requirements

In the last meeting RAN4 agreed the following about cell switch and TCI state activation delay requirement applicability when event-triggered L1 reporting is configured:

Issue 2-10: Impact on other LTM requirements Tentative agreement: - UE has sent an event-triggered L1 report and the reporting condition continues to be met at the time of receiving the cell switch command or TCI state activation command, [and leaving condition of the event is configured and corresponding condition is not met], the related cell switch delay and TCI state activation requirements are applicable. - Further discuss how to capture this in RRM spec. Other proposals for further discussion: - Proposal (Ericsson): RAN4 to define TCI state known condition considering the L3 and L1 event triggered reporting.

How to capture this agreement and whether the known TCI state condition is to be updated remained open.

The applicability of cell switch delay requirements is according to the latest agreed CR R4-2502620 the following:

The requirements in this section apply when - the target cell is known, and - the target joint UL/DL TCI state or separate UL and DL TCI states in the MAC-CE LTM cell switch command are known, or - the target cell is an FR1 cell, and the UE has reported L3-RSRP measurement result with SSB index associated to the target TCI state within 5 seconds before receiving the LTM cell switch command, and SNR of the SSB associated to the target TCI state ≥ -3dB, - PL-RS is QCLed with RS of the TCI state indicated by LTM cell switch command, and - For CFRA cell switch, RS of the TCI state indicated by LTM cell switch command is QCLed with the SSB indicated for CFRA.

The applicability of TCI state activation delay requirements according to the CR R4-2502622 agreed in the last meeting is:

The requirements in this clause apply for - Known downlink TCI state in FR1 and FR2 specified in clause 8.25.2, and - Unknown downlink TCI state in FR1 specified in clause 8.25.2, provided the following conditions are met: - UE has reported L3 measurement result with the associated SSB index of the downlink TCI state within 5 seconds before the LTM TCI state activation command. - SNR of the associated SSB is above -3 dB.

Regarding the agreement that was made in the last meeting, we do not think known TCI state condition needs to be changed. The existing conditions can continue to apply as it was earlier indicated by some companies that the conditions should be the same throughout the specification, which is why the L3-condition was also captured separately. We would propose to follow the same approach for the new requirement, which should be added on top of the other conditions.

For example, for cell switch delay applicability:

The requirements in this section apply when - the target cell is known, and - the target joint UL/DL TCI state or separate UL and DL TCI states in the MAC-CE LTM cell switch command are known, or - the target cell is an FR1 cell, and the UE has reported L3-RSRP measurement result with SSB index associated to the target TCI state within 5 seconds before receiving the LTM cell switch command, and SNR of the SSB associated to the target TCI state ≥ -3dB, or - the UE has sent event-triggered L1 measurement report for RS associated to the target TCI state before receiving the LTM cell switch command, and the reporting condition continues to be fulfilled at the time of reception of the cell switch command, and - PL-RS is QCLed with RS of the TCI state indicated by LTM cell switch command, and - For CFRA cell switch, RS of the TCI state indicated by LTM cell switch command is QCLed with the SSB indicated for CFRA.

Whether the leave condition that is now in brackets in the agreement needs to be specifically mentioned is a matter of taste in our view. “Condition continues to be fulfilled” in our view covers also this part, but if some companies prefer to specifically mention the leave report/condition, this is also acceptable to us. However, it should be noted that report on leave is also a measurement report, and based on the existing conditions the delay requirements would still apply for 1280 ms after this report, as the target TCI state is known also based on report on leave.

Capture the agreement about cell switch and TCI state activation delay applicability under issue 2-10 of RAN4#114 WF as a separate applicability condition i.e. do not edit the existing known TCI state condition.

If UE sends report on leave, this leads to an associated TCI state being known for 1280 ms after the report similar as any other L1 report i.e. requirements do not stop applying immediately when report on leave is sent.

To describe the proposals in this contribution better, we have prepared a text proposal in our draft CR in R4-2504072 to show how we think the requirements can be captured.

Conclusion

The following Observations and Proposals were made:

Proposal 1: The SSB-based event triggered measurement reporting delay shall be less than TL1-RSRP_Measurement_Period_SSB_intra (clause 9.14.5) or TL1-RSRP_Measurement_Period_SSB_Inter (clause 9.15.5), where Treport = 0 and TSSB_NBC is the periodicity of the (serving or candidate cell) SSB that triggers the report.

Proposal 2: Approach of proposal 1 can be followed for CSI-RS based event-triggered reporting delay once the CSI-RS measurement requirements are concluded.

Proposal 3: For event-triggered periodic reporting, apply the event-triggered reporting requirements for the first report, and for the following periodic reports simply specify the measurement accuracy requirements the reported measurements shall fulfil (similar to L3 event-triggered periodic reporting).

Proposal 4: Capture the agreement about cell switch and TCI state activation delay applicability under issue 2-10 of RAN4#114 WF as a separate applicability condition i.e. do not edit the existing known TCI state condition.

Proposal 5: If UE sends report on leave, this leads to an associated TCI state being known for 1280 ms after the report similar as any other L1 report i.e. requirements do not stop applying immediately when report on leave is sent.

- References

- WF for Part 1:  R4-2502601, Apple, RAN4#114