3GPP TSG-RAN WG4 Meeting #114bis	R4-2504076

Wuhan, China, April 7th – April 11th, 2025

Source:	Nokia, Nokia Shanghai Bell

Title:	On performance part of L3 measurement delay reduction

Agenda item:	7.17.4

Document for:	Discussion

Introduction

In this contribution we discuss the RRM performance requirements for FR2-1 SSB based L3 measurement delay reduction, considering both CSSF optimization and FBS reduction through multi-Rx.

Test cases for L3 beam sweeping factor reduction

Within the core part work for L3 beam sweeping factor reduction through multi-Rx capability, RAN4 has agreed to enhance the following FR2-1 requirements with the reduced beam sweeping factor:

- Handover delay

- RRC re-establishment delay

- L3 intra- and inter-frequency measurement delay with and without measurement gaps

In the performance part, RAN4 will define the corresponding test cases. Considering that the core part is not finished yet, the details of all the test cases cannot yet be discussed. However, RAN4 can as a starting point reuse the legacy test cases as the baseline for FBS test cases, by replacing the legacy beam sweeping factor with the reduced beam sweeping factor. The exact delay requirement that the UE needs to pass can be updated when the core part is concluded.

Use legacy test cases as the baseline for L3 FBS test cases, where the delay to be tested is defined with the reduced beam sweeping factor.

On high level, we propose to consider the following test case list to cover testing of the agreed core part requirements:

For L3 fast beam sweeping, consider the following test case list:
-   Intra- and inter-frequency handover to unknown FR2-1 cell
-   Intra- and inter-frequency RRC re-establishment to FR2-1 cell
-   Intra- and inter-frequency FR2-1 L3 measurements without measurement gaps
-   Intra- and inter-frequency FR2-1 L3 measurements with measurement gaps

Test cases for CSSF optimization

On CSSF optimization, RAN4 has agreed on the following solutions to improve the SSB based measurement delay:

- Measuring one serving CC per FR2 band

- 3 searchers

In the performance part, RAN4 needs to define the corresponding test cases. For solution 1, the discussion has been assuming FR2 band hence we should define new test cases for SSB-based measurement outside gaps in FR2 SA and FR1+FR2 scenarios. As RAN4 agreed to further discuss if the CSSF optimization solution is applicable to FR1, the test cases for FR1 SA can be further added based on the progress.

For the solution of measuring one serving CC per band, RAN4 to define new test cases for SSB-based measurement outside gaps in FR2 SA and FR1+FR2 scenarios. The test cases in FR1 SA is up to the progress of core requirements discussion.

For Solution 3, it was agreed to apply the 3-searcher solution to FR1 only and FR1+FR2 scenarios. Both need to be considered in the test cases.

For 3 searcher solution, RAN4 to define new test cases for SSB-based measurement outside gaps in FR1 only and FR1+FR2 scenarios.

In last meeting, it was further agreed to apply the enhanced CSSF to all the measurement procedures. Hence the test cases shall be defined in the following SSB-based measurement procedures outside gaps [1]:

- Intra-frequency SSB-based measurement without MG

- Inter-frequency SSB-based measurement without MG

- Inter-RAT SSB based measurement without MG

The test cases shall be defined in the following measurement procedures outside gaps:

- Intra-frequency SSB-based measurement without MG

- Inter-frequency SSB-based measurement without MG

- Inter-RAT SSB based measurement without MG

In addition, the test cases need to be defined considering all applicable scenario i.e. CA, EN-DC, NR-DC AND NE-DC scenarios. To reduce the test efforts, the test cases can be distributed among these scenarios.

The test cases shall be defined considering all applicable scenarios i.e. CA, EN-DC, NR-DC AND NE-DC scenarios and can be distributed among these scenarios to reduce test efforts.

Conclusion

The following Observations and Proposals were made:

L3 beam sweeping factor reduction:

Proposal 1: Use legacy test cases as the baseline for L3 FBS test cases, where the delay to be tested is defined with the reduced beam sweeping factor.

Proposal 2: For L3 fast beam sweeping, consider the following test case list: 
-   Intra- and inter-frequency handover to unknown FR2-1 cell 
-   Intra- and inter-frequency RRC re-establishment to FR2-1 cell 
-   Intra- and inter-frequency FR2-1 L3 measurements without measurement gaps 
-   Intra- and inter-frequency FR2-1 L3 measurements with measurement gaps

CSSF optimization:

Proposal 3: For the solution of measuring one serving CC per band, RAN4 to define new test cases for SSB-based measurement outside gaps in FR2 SA and FR1+FR2 scenarios. The test cases in FR1 SA is up to the progress of core requirements discussion.

Proposal 4: For 3 searcher solution, RAN4 to define new test cases for SSB-based measurement outside gaps in FR1 only and FR1+FR2 scenarios.

Proposal 5: The test cases shall be defined in the following measurement procedures outside gaps:

-	Intra-frequency SSB-based measurement without MG

-	Inter-frequency SSB-based measurement without MG

-	Inter-RAT SSB based measurement without MG

Proposal 6: The test cases shall be defined considering all applicable scenarios i.e. CA, EN-DC, NR-DC AND NE-DC scenarios and can be distributed among these scenarios to reduce test efforts.