3GPP TSG-RAN WG4 Meeting #114bis	R4-2504112

Wuhan, China, 7th – 11th, April 2025

Source: 	Ericsson

Title:  	Discussion on RedCap RRM requirements for NTN for NR Phase 3

Agenda Item:	7.29.4.1

Document for:	Discussion

# Introduction

In the light of the WI NR Non-Terrestrial Networks (NTN) for NR Phase 3, the outcomes of the last meeting's debate, encompassing consensuses and unresolved issues, were documented in WF [1].

This contribution outlines our point of view on on the underlying impacts to RedCap RRM requirements mandated by the issues listed in the WF.

# Discussion

## Topic #1: General consideration on RRM requirements

Issue 1-1-1: Issue 1-1-2: SDT requirements

Agreement: - For SDT requirement in RRC_INACTIVE state for (e)RedCap with NTN, TA validation based on the RSRP change criterion for RedCap in TN shall be removed and RAN4 can refer to the requirements on TA validation for the PUR requirements for IoT-NTN, that is, - Define the requirement based on cg-SDT-timeAlignmentTimer and valid values for NTA, NTA,offset, NTA,adjcommon and NTA,adjUE , specific approach can refer to the PUR requirements for IOT-NTN. - Further discuss whether to send RAN2 LS

#### Issue 1-1-2: The Requirements to be further determined

- Background R17 NTN UE capabilities related to 'in parallel' are copied as follows: The agreements in the last meeting: RAN4#113: <Agreements>: - RAN4 to define RRM requirements for multiple SMTC-s for a single frequency carrier for (e)RedCap UEs. - For whether to support parallel measurements on multiple NGSO satellites within a SMTC (Feature 25-5) for (e)RedCap UEs? - Option 1 (MTK, Xiaomi, Samsung, ZTE, Ericsson, LG, QC): Not support - Option 2 (HW, Nokia): FFS - Further discuss the supported number of SMTCs, the number of cells and SSB, the number of supported satellites, the proximity distance between two SMTCs. - Further discuss the specific measurement requirements, i.e, whether and how to extend measurement period. - Further discuss other R17 NTN UE capabilities (i.e, Feature 25-2/25-3) related to 'in parallel'..

#### Issue 1-2-3-1: Feature 25-1(parallelSMTC)

<Way Forward>: Options for further discussion: If not differentiate 1Rx and 2Rx RedCap UEs, the baseline for UE capability is: - Option 1: [1 SMTC or] 2 SMTCs not in parallel - Option 2: 2 parallel SMTCs - ‘Parallel’ means the SMTCs are TDM with proximity distance.

Since this serves as the baseline, we shall allow a simplified UE implementation while maintaining a degree of flexibility to accommodate the need for measuring both the serving satellite and at least one neighboring satellite. The following options are supported:

- Two SMTCs: The UE can measure two SMTCs that are not in parallel.

- Two SMTCs: The UE can measure two parallel SMTCs that are time-division multiplexed (TDM) within a proximity distance.

Proposal 1: RedCap UE is able to measure SMTCs in below scenarios:

- When the 2 SMTCs are not in parallel

- When the 2 SMTCs are in parallel, with the scaled measurement delay.

#### Issue 1-2-3-1a: The proximity distance between two SMTCs

Agreement: - RAN4 to define the proximity distance between two SMTCs for (e)RedCap UEs in NTN. - The distance between two SMTC occasions is defined as the time difference between the ending point of the earlier SMTC occasion and the starting point of the later SMTC occasion. - For whether to consider relaxation for proximity when (e)RedCap UE is configured with more than one SMTC on a SAN carrier, - Option 1: No. RAN4 to reuse the legacy collision condition. - Option 2: Yes. RAN4 to consider relaxation for proximity, i.e., - When (e)RedCap UE is configured with more than one SMTC on a SAN carrier, two SMTC occasions outside measurement gap are considered colliding if at least one of the following conditions is met: - the two SMTC occasions are fully or partially overlapping in time domain, or - the magnitude of the distance between two SMTC occasions in time domain is less than or equals to (4+x) ms. (x is the relaxation for (eRedCap UE)), x can be FFS. (1ms as starting point)

Option 2 provides a relaxation for (e)RedCap UE, and we may consider accepting this relaxation.

Proposal 2: For (e)RedCap UE , relaxation of the proximity distance is acceptable, 1 or 2 ms can be added to the existing 4ms, which shall be same to RedCap and eRedCap.

#### Issue 1-2-3-3: The number of cells/SSBs/supported satellites

<Way Forward>: - For the number of supported satellites, - Proposal 1: For the number of target LEO satellites the UE can monitor per carrier including serving satellite, legacy UE capability (Feature 25-7, maxNumber-LEO-SatellitesPerCarrier-r17) can be reused for RedCap UE. - Proposal 2: - RedCap UEs shall support at least one NGSO satellite per SMTC configured in a frequency layer in addition to the serving satellite. - The total number of supported satellites for measurement in a frequency layer is equal to N+1 where N is the number of configured SMTCs, if the serving satellite is to be measured in this frequency layer and N if the serving satellite is not to be measured in this frequency layer. - For the number of cells and SSB, - Proposal 1: RedCap NTN reuses the number of cells and SSB for NR NTN, i.e., - Proposal 1a: For each intra-frequency layer, during each layer 1 measurement period, the UE shall be capable of performing SS-RSRP, SS-RSRQ, and SS-SINR measurements for at least: - 8 identified cells, and - 8 SSBs with different SSB index and/or PCI on the intra-frequency layer, where the number of SSBs in the serving cell is not smaller than the number of configured RLM-RS SSB resources. - 4 SSBs with different SSB index and/or PCI from neighbour cells in GEO deployment. - In NGSO deployments: cells from 2 satellites including the satellite serving the PCell if UE does not support the capability maxNumber-NGSO-SatellitesPerCarrier-r17; or cells from 3 or 4 satellites satellites including the satellite serving the PCell, depending on the value indicated in maxNumber-NGSO-SatellitesPerCarrier-r17.

Regarding the number of supported satellites, our opinion is that maxNumber-LEO-SatellitesPerCarrier-r17already provides a well-defined representation of UE capability. Therefore, there is no need to delve into the maximum total number of supported satellites as proposed in Proposal 2.

Proposal 3: For sake of simplification, we prefer Proposal 1, i.e., reusing maxNumber-LEO-SatellitesPerCarrier-r17.

Taking the number of cells and number of SSB for an intra-frequency measurement layer as example, the ones for RedCap for TN, NR NTN and NR TN as follow.

The number of cells and number of SSB supported by RedCap for TN is duplicated as follows.

9.2B.3 Number of cells and number of SSB 9.2B.3.1 Requirements for FR1 For each intra-frequency layer, during each layer 1 measurement period, the UE shall be capable of performing SS-RSRP, SS-RSRQ, and SS-SINR measurements for at least: - 8 identified cells, and - 14 SSBs with different SSB index and/or PCI on the intra-frequency layer, where the number of SSBs in the serving cell is not smaller than the number of configured RLM-RS SSB resources.

The number of cells and number of SSB supported NR TN is duplicated as follows.

9.2.3 Number of cells and number of SSB 9.2.3.1 Requirements for FR1 For each intra-frequency layer, during each layer 1 measurement period, the UE shall be capable of performing SS-RSRP, SS-RSRQ, and SS-SINR measurements for at least: - 8 identified cells, and - 14 SSBs with different SSB index and/or PCI on the intra-frequency layer, where the number of SSBs in the serving cell (except for the SCell) is not smaller than the number of configured RLM-RS SSB resources.

The number of cells and number of SSB supported by NR NTN is duplicated as follows.

9.2C.3 Number of cells and number of SSB 9.2C.3.1 Requirements for FR1 For each intra-frequency layer, during each layer 1 measurement period, the UE shall be capable of performing SS-RSRP, SS-RSRQ, and SS-SINR measurements for at least: - 8 identified cells, and - 8 SSBs with different SSB index and/or PCI on the intra-frequency layer, where the number of SSBs in the serving cell (except for the SCell) is not smaller than the number of configured RLM-RS SSB resources. - 4 SSBs with different SSB index and/or PCI from neighbour cells in GEO deployment. - In NGSO deployments: cells from 2 satellites including the satellite serving the PCell if UE does not support the capability maxNumber-NGSO-SatellitesPerCarrier-r17; or cells from 3 or 4 satellites satellites including the satellite serving the PCell, depending on the value indicated in maxNumber-NGSO-SatellitesPerCarrier-r17.

It can be observed that, compared to RedCap for TN, the number of SSBs supported by NR NTN is reduced due to the consideration of satellite beams. Similarly, it appears that RedCap NTN may follow the same approach.

Proposal 4: For the number of cells and SSB, we support Proposal 1: For the number of target LEO satellites the UE can monitor per carrier including serving satellite, legacy UE capability (Feature 25-7, maxNumber-LEO-SatellitesPerCarrier-r17) can be reused for RedCap UE.

#### Issue 1-2-3-5: Feature 25-3(parallelMeasurementGap)

<Way Forward>: - For whether to define RRM requirements for Feature 25-3(parallelMeasurementGap-r17) for (e)RedCap UEs, - Proposal 1: Support to define. - Proposal 1a: The UE capability is applicable, and the support of the optional features is left to UE implementation following the same principle of the existing non-RedCap requirements. - Proposal 1b: The capability can be optionally supported by (e)RedCap UEs with 2Rx, can not be supported by (e)RedCap UEs with 1Rx.

The measurement gap may haven’t strong relationship with Rx number.

Proposal 5: We lean toward Proposal 1a regarding parallelMeasurementGap, i.e., : The UE capability is applicable, and the support of the optional features is left to UE implementation following the same principle of the existing non-RedCap requirements.

#### Issue 1-2-7: NCD-SSB

<Way Forward>: - Proposal 1: - Use the terminology of SSB without differentiating CD-SSB/NCD-SSB, and add a clarification in a separate clause that the requirements are only applicable to CD-SSB. - Proposal 2: - Use the terminology of SSB without differentiating CD-SSB/NCD-SSB, no need to add additional requirement applicability about CD-SSB/NCD-SSB.

A concern to Proposal 2 is RedCap TN UE may have different requirements for NCD-SSB and CD-SSB, e.g. handover. The lack of applicability clarification could lead to potential misunderstandings. We support Proposal 1. If Proposal 2 is not considered to introduce ambiguity, it may also be acceptable. However, ensuring clear applicability is preferred to avoid potential misunderstandings.

Proposal 6: Use the terminology of SSB without differentiating CD-SSB/NCD-SSB, and add a clarification in a separate clause that the requirements are only applicable to CD-SSB.

#### Issue 1-2-8: Other features

#### Issue 1-2-7: Other features <Way Forward>: - For SMTC adjustment in idle/inactive, - Option 1: Keep mandatory for RedCap UE. - Option 2: Keep optional for RedCap UE. - For Location-based measurement trigger, - Option 1: Keep optional for RedCap UE. - Option 2: RAN4 not to further discuss applicability of location-based measurement report trigger.

We have concerns on Option 2: Keep optional for RedCap UE. If SMTC adjustment is optional, the measurement requirements must be limited. One possible limitation is the UE only can measure GSO satellites, including the serving and neighbor cells. Another possible limitation is the UE can measure NGSO satellites but cannot guarantee measurement requirements.

Proposal 7: Regarding SMTC adjustment, we prefer Option 1, i.e., SMTC adjustment in idle/inactive is mandatory.

## Topic #2: Reduction in the number of UE Rx branches

#### Issue 2-1-2: Cell re-selection requirements of inter-RAT E-UTRAN cells with TN carrier for 1Rx/ 2Rx (e)RedCap UE with FR1-NTN

<Way Forward>: - Proposal 1: The existing requirements for measurements of inter-RAT E-UTRAN cells with TN carrier can be used as baseline. - Proposal 2: For inter-RAT E-UTRAN TN carrier cell re-selection measurement requirements, reuse the legacy requirements defined for normal UEs for (e)Redcap UE not configured with eDRX_IDLE cycle, reuse the Redcap Inter-RAT cell reselection requirement defined in TS 38.133 Table 4.2B.2.5-2 for (e)Redcap UE configured with eDRX_IDLE cycle. - Proposal 3: For cell re-selection measurement requirements of inter-RAT E-UTRAN cells with TN carrier for (e)RedCap UE with FR1-NTN bands, - For 2Rx UE, the existing cell reselection requirements in Rel-18 (e)NTN can be reused for (e)RedCap 2Rx UE with FR1-NTN bands. - For 1RX UE, reuse the same margin of RSRP/RSRQ as for 2Rx.

After observation, we suppose that Proposal 2 and Proposal 3 are same from requirements perspective. the only difference is what chapters/tables shall be referred to.

Proposal 8: Support Proposal 2, because the referred table is more suitable, i.e., For inter-RAT E-UTRAN TN carrier cell re-selection measurement requirements, reuse the legacy requirements defined for normal UEs for (e)Redcap UE not configured with eDRX_IDLE cycle, reuse the Redcap Inter-RAT cell reselection requirement defined in TS 38.133 Table 4.2B.2.5-2 for (e)Redcap UE configured with eDRX_IDLE cycle.

## Topic #3: The impact of HD-FDD

#### Issue 3-1-1: General HD-FDD related impact

<Way Forward>: - Proposal 1: The legacy requirements and applicable conditions can be as the baseline for defining requirements of RedCap UE in NTN scenario. - Proposal 1a: To following the TN, the following requirements will be affected by HD-FDD for (e)RedCap UE with FR1-NTN: - Paging reception requirements in RRC_IDLE/ RRC_INACTIVE state - Handover interruption time - Random access - SA: RRC Connection Release with Redirection - Minimum requirement for L1 indication for RLM and LR - MAC-CE/DCI/RRC based uplink spatial relation switch delay - Scheduling availability of UE performing intra/inter measurements - Scheduling availability of UE during L1-RSRP measurement - Proposal 1b: For scheduling availability, need further RAN1 progress for detailed requirements of HD-FDD UEs. - Proposal 1c: - Adopt TN requirements as baseline for clarifying the requirements listed in P1a for HD-FDD RedCap UE except for - MAC-CE/DCI/RRC based uplink spatial relation switch delay - Scheduling availability of UE during L1-RSRP measurement - Clarify that the requirements for CSI-RS and PRS apply provided that no CSI-RS and PRS resource is dropped due to collision with UL transmission during the measurement period. - Proposal 1d:RAN4 to check whether existing HD-FDD related requirements and applicable conditions can be reused or not after RAN1 reach further conclusions. - Proposal 2: For NTN UEs with HD-FDD, the requirements are met provided that the SSB of at least one of the satellites to be measured in this SMTC period is available at the UE side. - Proposal 3: For HD RedCap and eRedCap UEs, RAN4 to adopt a high level statement that the requirement does not count resources, channels, SSB, etc if they overlap with the other direction as specified a valid case in RAN1 specification. - Proposal 4: RAN4 should further discuss measurement behavior for collision between SMTC and UL when network indicates UL overriding DL in HD-FDD.

To our understanding, the RAN1’s discussion isn’t completed.

Proposal 9: We prefer Proposal 1b: For scheduling availability, need further RAN1 progress for detailed requirements of HD-FDD UEs.

## Topic #4: Others

#### Issue 4-2-1: How to declare the capabilities for (e)RedCap UE with FR1-NTN bands?

<Way Forward>: - Proposal 1: Declare the capabilities for (e)RedCap UE with FR1-NTN bands through implicitly by declaring support of RedCap (supportOfRedCap-r17 or supportOfERedCap-r18) and support of NTN (nonTerrestrialNetwork-r17). - Proposal 2: Introduce new capability to declare support over RedCap over NTN. - Proposal 3: Both proposals, i.e., a new capability declaring (e) RedCap for NTN or combining (e)RedCap and NTN, are fine with us. It may be determined later.

We understand Proposal 1 and Proposal 2 both can work for distinguishing RedCap in NTN from the two following scenarios: NR in NTN and RedCap in TN.

Proposal 10: We slightly prefer Proposal 1, but we are also fine with Proposal 2, as it may help avoid misunderstandings.

# Conclusion

Proposal 1: RedCap UE is able to measure SMTCs in below scenarios:

- When the 2 SMTCs are not in parallel

- When the 2 SMTCs are in parallel, with the scaled measurement delay.

Proposal 2: For (e)RedCap UE , relaxation of the proximity distance is acceptable, 1 or 2 ms can be added to the existing 4ms, which shall be same to RedCap and eRedCap.

Proposal 3: For sake of simplification, we prefer Proposal 1, i.e., reusing maxNumber-LEO-SatellitesPerCarrier-r17.

Proposal 4: For the number of cells and SSB, we support Proposal 1: For the number of target LEO satellites the UE can monitor per carrier including serving satellite, legacy UE capability (Feature 25-7, maxNumber-LEO-SatellitesPerCarrier-r17) can be reused for RedCap UE.

Proposal 5: We lean toward Proposal 1a regarding parallelMeasurementGap, i.e., : The UE capability is applicable, and the support of the optional features is left to UE implementation following the same principle of the existing non-RedCap requirements.

Proposal 6: Use the terminology of SSB without differentiating CD-SSB/NCD-SSB, and add a clarification in a separate clause that the requirements are only applicable to CD-SSB.

Proposal 7: Regarding SMTC adjustment, we prefer Option 1, i.e., SMTC adjustment in idle/inactive is mandatory.

Proposal 8: Support Proposal 2, because the referred table is more suitable, i.e., For inter-RAT E-UTRAN TN carrier cell re-selection measurement requirements, reuse the legacy requirements defined for normal UEs for (e)Redcap UE not configured with eDRX_IDLE cycle, reuse the Redcap Inter-RAT cell reselection requirement defined in TS 38.133 Table 4.2B.2.5-2 for (e)Redcap UE configured with eDRX_IDLE cycle.

Proposal 9: We prefer Proposal 1b: For scheduling availability, need further RAN1 progress for detailed requirements of HD-FDD UEs.

Proposal 10: We slightly prefer Proposal 1, but we are also fine with Proposal 2, as it may help avoid misunderstandings.

# References

- R4-2502604, The WF on RRM requirements for Rel-19 NR NTN phase3, CATT