3GPP TSG-RAN WG4 Meeting #114bis	R4-2504113

Wuhan, China, 7th – 11th, April 2025

Source: 	Ericsson

Title:  	Discussion on FR2-1 L3 measurement delay by optimizing Rx beam sweeping factor

Agenda Item:	7.17.2.1

Document for:	Discussion

# Introduction

In the light of the WI of NR Radio Resource Management (RRM) enhancements (WID in [1]) for Rel-19, the outcomes of the last meeting's debate, encompassing consensuses and unresolved issues, were documented in WF [2].

This contribution outlines our point of view on FR2-1 L3 measurement delay by optimizing Rx beam sweeping factor as follows.

# Discussion

## Topic #1: FR2-1 SSB based L3 measurement delay reduction for connected mode

Issue 1-1-1: Conditions to trigger UE to activate L3 measurement delay reduction by optimizing Rx BSF

Agreement in ad-hoc session: - Discuss if additional conditions are needed to activate FBS-based L3 measurements. - Do not consider additional conditions beside the agreements from RAN4#113 meeting. Agreement in RRM session (Monday): - Adopt FBS triggering condition of serving cell is worse than a threshold, including RSRP, RSRQ threshold. - Further discuss whether to limit the serving cell to FR2 serving cell. - Further discuss whether to add the hysteresis and timeToTrigger parameters. - Note: the scenario related to single carrier or CA/DC is discussed in another issue. - The signaling design is up to RAN2.

During the previous meeting, a use case was presented and discussed in which a serving cell, such as an FR1 (serving) cell, triggers another FR2 (serving or neighboring) cell.

For the case of FR1 serving cell triggering FBS on FR2 neighbour cell, it is considered valid, as the FR2 neighbour cell measurement is regarded as inter-frequency measurement.

For the case of FR1 serving cell triggering FBS on FR2 serving cell, the only potentially relevant scenario would involve a PSCell change, where FBS is applied to the FR2 serving cell while maintaining the FR1 cell as serving the PCell.

Assuming there’re a network configuring two serving cells, a FR1 cell and a FR2 cell. Below configurations to the two cells may exist,

- Option 1: FR1 cell is configured with FBS condition, FR2 cell isn’t.

- Option 2: FR1 cell is configured with FBS condition, FR2 cell is configured with FBS condition.

- Option 3: FR1 cell isn’t, FR2 cell is configured with FBS condition.

It appears that Option 1 is the use case that initially triggered the issue. But Option 2 is also feasible in implementation.

At least, in our opinion,

- Option 2 and Option 3 may better match the PSCell change (within SCG) purpose, compared with Option 1. We don’t see the reason to choose Option 1 expect that the FR2 cell can’t configure FBS.

- For PCell change (handover to new MCG), Option 1 may occur, similar to the case of FR1 serving cell triggering FBS on FR2 neighbour cell.

Proposal 1: RAN4 to agree that the FBS condition can be applied to a FR1 serving cell, which may trigger FBS for inter-frequency measurement on a FR2 cell.

Observation 1: For the case of a FR1 serving cell triggering FBS on a FR2 serving cell, the use cases may be:

- PSCell change (within SCG) to another cell from the FR2 serving cell

- PCell change (handover to new MCG) to the FR2 serving cell.

Proposal 2:  Satisfying the FBS condition on a FR1 serving cell may trigger FBS on a FR2 serving cell depending on NW plan, e.g., PCell change (handover to new MCG) to the FR2 serving cell.  We don’t oppose the case, but there are some unclear aspects need to be clarified, and the aspects cause more complexity:

- The FR1 serving cell is limited to Pcell only or not.

- If more than one FR1 serving cell configures the FBS condition, each of them can trigger FBS on the a FR2 serving cell or not.

The associated parameters regarding the FBS condition may have the below options to our understanding.

- Hysteresis: It’s not crucial for measurements.

- TimeToTrigger: Similarly, the robustness of enabling FBS is not crucial for measurements. Earlier enabling FBS, even the condition could not be consistently met in a time period, doesn’t result to any problem or negative effect to mobility.

Proposal 3: No strong view on hysteresis parameter, it may be removed.

Proposal 4: Not to define TimeToTrigger parameter.

To avoid too frequent ping-pong between FBS and non-FBS, a minimal duration time for FBS/ non-FBS shall be defined.

- FBS deactivation restriction: Before the minimum duration expires, FBS shall not be deactivated, even if the activation condition changes.

- FBS activation restriction: Similarly, if non-FBS is activated, FBS shall not be re-activated unless the non-FBS state has persisted for at least the defined minimum duration.

Proposal 5: Instead of TTT, RAN4 to introduce ‘time to deactivate’ parameter, which limits the minimal FBS measurement time after activated/enabled.

Proposal 6: NW is aware of UE enabling FBS, it may need the information regarding the UE's activation or entry into the FBS state. One potential solution is for the UE to report when it meets the FBS condition or it enables FBS.

Issue 1-1-2: Conditions to trigger UE to quit “L3 measurement delay reduction by optimizing Rx BSF”:

Agreement in ad-hoc session: - UE is not required to perform FBS based fast L3 measurements when threshold is not configured to UE for scenario 1/2/3/4. Further discuss this issue.

Firstly, certain conditions may prevent the UE from entering the FBS state. One such condition that can be observed is overheating, which may inhibit FBS activation.

Proposal 7: Overheating indication may stop the UE entering FBS even the FBS condition is met.

Under certain conditions, the UE may exit FBS without an explicit indication from the network after FBS has been enabled. In our view, these conditions should be explicitly defined in the requirements. Without clear specifications, the UE might deactivate FBS arbitrarily, leading to inconsistencies in network configuration and scheduling.

For instance, consider a scenario where the FBS activation condition remains valid, and the UE has completed at least one measurement in the FBS state. Under such circumstances, the UE may determine that there is no urgent need to continue operating in FBS mode, for example, if the UE remains in a stationary state or if there are no qualified neighboring cells available for switching. In such cases, the UE may autonomously exit FBS.

Proposal 8: UE may quit FBS if the UE has completed at least one measurement in the FBS way and the UE identifies that there isn’t any urgent task to continue FBS, e.g., the UE remains a stationary state or no qualified neighbour cells are available for switching.

Proposal 9: UE shall notify NW of exiting FBS.

Proposal 10: UE may quit FBS in certain case with or without other conditions, but at least after the minimal time period ‘time to deactivate’.

A leftover is, if the UE has quit FBS, whether the UE shall re-evaluate the FBS activation condition to enable FBS?

We understand the issue can be resolved in more than one way. Among them, a simple solution is that UE shall re-evaluate the FBS activation condition which is maintained. But, as described in the last proposal, a minimal during time in FBS/ non-FBS, to avoid too frequent ping-pong between FBS and non-FBS.

Proposal 11: After deactivating FBS, the UE may re-evaluate the condition of enabling FBS, at least after a minimal time interval ‘time to active’ after deactivated or disabled.

An achieved agreement about the handover procedure, which is repeated in the table below, specifies that FBS must always be used during Tsearch. We understand the purpose is that handover command is able to activate FBS for fast measurement and speeding up mobility.

For UE supporting Rel-19 BSF reduction capability, faster beam sweeping always applies during handover (Tsearch).

For the CHO delay requirements, Tmeasure is the measurement time stated in clause 6.1.4.4.2 of TS38.133. Regarding the description, ‘The measurement time delay is defined from the end of TEvent_DU until UE executes a handover to a target cell and interruption time starts.’, it implies that Tmeasure is later than the time instant when the event is met. In this case, it’s reasonable to adopt FBS as well to speed up the CHO delay.

Proposal 12: For CHO, UE always applies FBS during Tmeasure.

Issue 1-2-1: Solutions to apply/specify L3 measurement delay reduction by optimizing Rx BSF

- Proposals - Option 1a: (ZTE, CATT, Ericsson, CT, CMCC) - Introducing the UE capability of reduced N, the candidates could be 2, 4 or 6. - Option 1b: (QC) - At least, support reduction of beam sweeping factor from 8 to 4 for multi-RX capable UEs in Rel-19. Support of other beam sweeping factors can be FFS. - Option 1c: (MTK) - On the solution to achieve reduced beam sweeping factor (BSF) as UE capability, RAN4 to decide to use either: - Single value for reduced BSF (e.g., 6 or 4) - Multiple candidate values for BSF (e.g., 6 and 4) - Option 2: (vivo) - RAN4 to consider the following approaches as solutions: - Introduce the UE capability to indicate the reduced M value (M=sample*BSF), e.g., 12, 14, 16, etc. - Introduce the UE capability to indicate the fraction value for reduction, e.g., 1/2, 1/3, etc. - Introduce the UE capability to indicate the BSF value, the value needs to be discussed under a common assumption for the sample value, for example, sample = 4. - Option 3: (HW) - The UE capability design shall allow enough flexibility on supporting this feature considering: - Different BC - Number of Carriers/MOs to measured - Intra-frequency/inter-frequency - Option 4: (LGE) - Introduce scaling value (Pmulti-Rx) < 1 to reduce measurement delay - Recommended WF - Introducing the UE capability of reduced N to indicate the BSF value(s) or fraction value(s). FFS the value(s). - Discuss option 3 in regard to the UE capability design: - Different BC - Number of Carriers/MOs to measured - Intra-frequency/inter-frequency

Proposal 13:  We recommend initially limiting the consideration of FBS to a single FR2 serving cell and excluding the scenario of multiple serving cells from the BSF capability assessment at the first step.

Proposal 14: For UE supporting Multi-Rx operation for L3 measurement and working with single carrier configuration, the Rx BSF may be to 2, 4 or 6 according to the single value indicated by UE capability.

# Conclusion

Observation 1: For the case of a FR1 serving cell triggering FBS on a FR2 serving cell, the use cases may be:

- PSCell change (within SCG) to another cell from the FR2 serving cell

- PCell change (handover to new MCG) to the FR2 serving cell.

Proposal 1: RAN4 to agree that the FBS condition can be applied to a FR1 serving cell, which may trigger FBS for inter-frequency measurement on a FR2 cell.

Proposal 2:  Satisfying the FBS condition on a FR1 serving cell may trigger FBS on a FR2 serving cell depending on NW plan, e.g., PCell change (handover to new MCG) to the FR2 serving cell.  We don’t oppose the case, but there are some unclear aspects need to be clarified, and the aspects cause more complexity:

- The FR1 serving cell is limited to Pcell only or not.

- If more than one FR1 serving cell configures the FBS condition, each of them can trigger FBS on the a FR2 serving cell or not.

Proposal 3: No strong view on hysteresis parameter, it may be removed.

Proposal 4: Not to define TimeToTrigger parameter.

Proposal 5: Instead of TTT, RAN4 to introduce ‘time to deactivate’ parameter, which limits the minimal FBS measurement time after activated/enabled.

Proposal 6: NW is aware of UE enabling FBS, it may need the information regarding the UE's activation or entry into the FBS state. One potential solution is for the UE to report when it meets the FBS condition or it enables FBS.

Proposal 7: Overheating indication may stop the UE entering FBS even the FBS condition is met.

Proposal 8: UE may quit FBS if the UE has completed at least one measurement in the FBS way and the UE identifies that there isn’t any urgent task to continue FBS, e.g., the UE remains a stationary state or no qualified neighbour cells are available for switching.

Proposal 9: UE shall notify NW of exiting FBS.

Proposal 10: UE may quit FBS in certain case with or without other conditions, but at least after the minimal time period ‘time to deactivate’.

Proposal 11: After deactivating FBS, the UE may re-evaluate the condition of enabling FBS, at least after a minimal time interval ‘time to active’ after deactivated or disabled.

Proposal 12: For CHO, UE always applies FBS during Tmeasure.

Proposal 13:  We recommend initially limiting the consideration of FBS to a single FR2 serving cell and excluding the scenario of multiple serving cells from the BSF capability assessment at the first step.

Proposal 14: For UE supporting Multi-Rx operation for L3 measurement and working with single carrier configuration, the Rx BSF may be to 2, 4 or 6 according to the single value indicated by UE capability.

# References

- RP-240830, New WID: WI resulting from discussion on RRM enhancements, vivo (Moderator)

- R4-2502593, WF for NR_RRM_Ph5_part2, CATT