3GPP TSG-RAN WG4 Meeting # 114bis					R4-2504124

Wuhan, China, Apr. 7th ‒ Apr. 11th, 2025

Source:	Nokia

Title:	On LS response to RAN1 on UE capability for non-simultaneous RxTx FDD-FDD inter-band CA

Agenda item:	6.3.3

Document for:	Discussion

Introduction

At RAN4#113 sent an LS to RAN2 [1] which RAN2 have responded to in [2] with the content copied in below for reference.

RAN2 thanks RAN4 for the LS on UE capability for FDD-FDD inter-band CA simultaneous Rx/Tx.

Regarding whether to make necessary modifications in RAN2 specification to enable the described functionality, RAN2 agreement is as below:

RAN2 understands that for the example band the 2UL/2DL inter-band FDD-FDD CA_n5-n8 with the scheduling restriction of non-concurrent n5 DL and n8 UL is feasible, and there is no RAN2 spec impact (i.e., the necessary scheduling restriction is up to network implementation)

Regarding whether to define UE capability signalling, RAN2 didn't reach consensus due to the lack of details on simultaneous Rx/Tx in inter-band FDD-FDD CA_n5-n8. Please note that RAN2 only defines UE capability signalling for well-defined behaviour/features. RAN2 therefore would like to ask RAN4 the following questions for better understanding:

- Whether it’s possible for a UE to support simultaneous Rx/Tx in inter-band FDD-FDD CA_n5-n8 without any scheduling restriction, or whether there may be a case where a UE can support simultaneous Rx/Tx in some part(s) of the overlapping FDD bands? If possible, from which release version?

- If a UE may be able to support simultaneous Rx/Tx in inter-band FDD-FDD CA_n5-n8 (or some other inter-band FDD-FDD CA combinations) in some form, whether it requires additional UE capabilities for the support/requirement of other features, e.g., full-duplex, switching time, or some frequency gap/guard band?

ACTION: 	RAN2 respectfully asks RAN4 to take the above information into account and provide feedback on the questions.

In this contribution we will address the two questions from RAN2.

Discussion

The FDD-FDD case discussed in RAN4 is illustrated in Figure 2-1 with CA_n5-n8, leading to the original LS [1]. The issue is that since the operating bandwidths of the bands (partially) overlap, the UE’s DL filter for n5 overlaps with n8; so, despite the configured bandwidths of n5DL and n8UL not overlapping, it is still possible that a UE could experience self-interference in n5 reception when it was transmitting on n8. In this case, the UE can always support n5UL with n8DL, but n5DL cannot be supported concurrently with n8UL.

Figure 2-1: FDD-FDD inter-band CA_n5-n8

If simultaneousRxTxInterBandCAPerBandPair is applied to FDD-FDD CA_n5-n8, the UE could only indicate to the network that n5 and n8 cannot be used concurrently, but then the network would never configure n5 and n8 together, despite both configurations shown @t1 and @t2 in Figure 2-1 being valid.

To further illustrate the point, we can consider an example with more overlapping bands. For example, FDD-FDD CA_nA-nB-nC-nD where band B partially overlaps with bands C and D as shown in Figure 2-2. (Since RAN4 already supports band combinations with many bands and is expected to support even larger band combinations, we think this type of scenario will occur in the near future.) In this case neither simultaneous (nBUL, nDDL) nor simultaneous (nBDL nCUL) are supported, even though simultaneous (nBDL, nDUL) and simultaneous (nBUL, nCUL) are supported. If the legacy capability simultaneousRxTxInterBandCAPerBandPair was applied to this band combination, the network would understand that band B can only be configured in CA_nA-nB, even though other configurations including band B could be valid.

| A UL |  | A DL |  |  |  |  |  |  | B UL |  | B DL |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | D UL |  |  | D DL |  |  |  |  |  | C UL |  |  | C DL |
| A UL |  | A DL |  |  |  |  |  |  | B UL |  | B DL |  |  |  |  |  |
|  |  |  |  | D UL |  |  | D DL |  |  |  |  |  | C UL |  |  | C DL |

Figure 2-2: FDD-FDD inter-band CA_nA-nB-nC-nD

For FDD-FDD band pairs, simultaneousRxTxInterBandCAPerBandPair cannot differentiate between cases where a UE supports simultaneous Rx/Tx of a band combination but does not support simultaneous Tx/Rx of this band combination.

Even RAN2 have reached the conclusion that by applying scheduling restrictions left for network implementation there should be no impact to the RAN2 specification it is still recommended that RAN2 introduces corresponding capability to indicate any UE restriction to allow the network applying any needed scheduling restrictions.

UE capability signalling is needed to indicate when band pairs of an FDD-FDD inter-band CA combination do not support concurrent operation. This to allow the network applying any needed scheduling restrictions.

Currently RAN4 and RAN2 have focused on CA_n5-n8 but as shown with the example in relation to Figure 2-2 more than two band CA scenarios should be considered.

RAN4 shall inform RAN2 to consider any new capability to indicate when band pairs of an inter-band CA combination do not support concurrent operation to cover more than two band CA.

As an example of the use of this can be when referring to the inter-band reference sensitivity exceptions of TS 38.101-1, Table 7.3A.5-2 which shows a number of cases where intermodulation interference can be caused by two bands in the UL can seriously desensitize the DL of another band.

For example, a 31.5 dB n77 maximum sensitivity degradation (MSD) is allowed for band n3DL with UL CA in bands n1 and n77, as specified for CA_n1-n3-n77 in Table 7.3A.5-2, rendering the band n3DL unusable together with UL CA in bands n1 and n77, even though the UE might otherwise support n3DL with n1UL or n77UL separately (i.e. when n1 and n77 are not configured with 2UL). However, there would be no way to indicate this with the current capability signalling.

There are cases where UE could separately support simultaneous Rx/Tx in bands A/B and simultaneous Rx/Tx in A/C, but would not support Rx in band A when Tx is configured in bands B and C at the same time.

Question 1 - Simultaneous Rx/Tx in overlapping FDD bands

RAN2 are asking the question:

- Whether it’s possible for a UE to support simultaneous Rx/Tx in inter-band FDD-FDD CA_n5-n8 without any scheduling restriction, or whether there may be a case where a UE can support simultaneous Rx/Tx in some part(s) of the overlapping FDD bands? If possible, from which release version?

From our understanding the scenario in question can be visualized in Figure 2.1-1 where the difference to Figure 2-1 is whether it would be possible to have concurrent transmission/reception in band n5 and n8 potential with the restriction that sufficient separation/guardband is provided between the scheduled UL and DL resource blocks.

Figure 2.1-1: Simultaneous Rx/Tx for inter-band CA_n5-n8

Given the long discussion enabling CA_n5-n8 as a configuration it seems unlikely that a “normal” UE would support simultaneous Rx/Tx in inter-band FDD-FDD CA_n5-n8 without any restriction. This does not mean that a UE would not be able to receive while transmitting on the two overlapped bands, just not within the same overlapped frequency span at the same time.

It seems unlikely that a “normal” UE would support simultaneous Rx/Tx on overlapped parts in inter-band FDD-FDD CA_n5-n8 without any restriction.

From this perspective it seems unnecessary to introduce specific capability signalling for this scenario.

RAN4 shall inform RAN2 that it is not necessary for them to introduce specific capability signalling for simultaneous Rx/Tx in inter-band FDD-FDD CA_n5-n8 with concurrent transmission/reception in the overlapped band range.

Question 2 - Additional UE capability information

RAN2 are asking the question:

- If a UE may be able to support simultaneous Rx/Tx in inter-band FDD-FDD CA_n5-n8 (or some other inter-band FDD-FDD CA combinations) in some form, whether it requires additional UE capabilities for the support/requirement of other features, e.g., full-duplex, switching time, or some frequency gap/guard band?

Given the discussion related to question 1 in section 2.1, where it is questioned if this enhanced support for what could be considered a corner case scenario is justified, it seems unnecessary to discuss this in depths.

Given the proposed answer to question 1 there is no need to discuss question 2 in depths in our view.

What is needed to assist the network applying any needed scheduling restrictions is which bands can’t have concurrent transmissions. This is on our view the core UE restriction which needs to be taken into account when bands are either overlapped or have other restrictions.

RAN4 shall respectfully ask RAN2 to introduce capability signalling allowing the UE to report any restrictions to concurrent transmission/reception in the supported CA band combinations.

Conclusion

In this contribution the following Observations and Proposals were made:

Observation 1: For FDD-FDD band pairs, simultaneousRxTxInterBandCAPerBandPair cannot differentiate between cases where a UE supports simultaneous Rx/Tx of a band combination but does not support simultaneous Tx/Rx of this band combination.

Observation 2: UE capability signalling is needed to indicate when band pairs of an FDD-FDD inter-band CA combination do not support concurrent operation. This to allow the network applying any needed scheduling restrictions.

Proposal 1: RAN4 shall inform RAN2 to consider any new capability to indicate when band pairs of an inter-band CA combination do not support concurrent operation to cover more than two band CA.

Observation 3: There are cases where UE could separately support simultaneous Rx/Tx in bands A/B and simultaneous Rx/Tx in A/C, but would not support Rx in band A when Tx is configured in bands B and C at the same time.

Observation 4: It seems unlikely that a “normal” UE would support simultaneous Rx/Tx on overlapped parts in inter-band FDD-FDD CA_n5-n8 without any restriction.

Proposal 2: RAN4 shall inform RAN2 that it is not necessary for them to introduce specific capability signalling for simultaneous Rx/Tx in inter-band FDD-FDD CA_n5-n8 with concurrent transmission/reception in the overlapped band range.

Observation 5: Given the proposed answer to question 1 there is no need to discuss question 2 in depths in our view.

Proposal 3: RAN4 shall respectfully ask RAN2 to introduce capability signalling allowing the UE to report any restrictions to concurrent transmission/reception in the supported CA band combinations.

- References

- R4-2420410	LS on UE capability for FDD-FDD inter-band CA simultaneous Rx/Tx

- R4-2503120	Reply LS on UE capability for FDD-FDD inter-band CA simultaneous Rx/Tx (R2-2501572)