3GPP TSG-RAN WG4 Meeting # 114bis					R4-2504127

Wuhan, China, Apr. 7th ‒ Apr. 11th, 2025

Source:	Nokia

Title:	On future HPUE Basket Arrangement

Agenda item:	7.1.1.1.3

Document for:	Discussion

Introduction

In this contribution we share our view on the need for R20 HPUE baskets because this has become very relevant topic in R19 MSD simplification discussion. R19 HPUE MSD simplification has direct impact on structure of R20 basket framework.

Discussion

Even in Rel-17 there were HPUE (i.e PC2) specific basket WIs and already when moving from Rel-17 to Rel-18 it was argued in e.g. [8], that since generic/core requirements for PC2 band combinations had already been specified in RAN4 specifications the PC3 baskets could also contain these HPUE combinations. This is the only difference is a delta in the MSD analysis and the needed co-existence analysis are the same.

The co-existence analysis for HPUE (i.e. PC1/1.5/2) band combinations is the same as already done for PC3.

MSD look up table (LUT) work is assumed to be completed in September 2025 at the end of R19.

Observation 2: HPUE MSD does not require too much effort as MSD lookup tables (LUT) are defined

Hence, we have not seen any reasons to have HPUE specific/dedicated basket WIs once generic/core requirements are completed. This means that as soon as a specific type of HPUE band combination has been completed with related example band combinations the remaining and/or additional combinations requested by operators should be completed within the existing basket framework. This is by making all Rel-20 band combination baskets power class agnostic.

Rel-20 basket WIs should be power class agnostic.

This is not aligned with the WF from RAN4 but it seems to be a good improvement to reduce the number of places to track band combinations requested by different operators.

In specific it is proposed that the following Rel-19 HPUE baskets are merged into the Rel-20 proposed “normal” baskets as shown in table 3.

| Rel-19 Baskets: | Rel-20 Basket(s) |
| --- | --- |
| HPUE_DC_LTE_NR_R19 | HPUE objective is added into DC_R20_xBLTE_yBNR |
| HPUE_NR_CADC_SUL_R19 | HPUE objective is added into NR_CADC_SUL_R20 |
| HPUE_NR_FR1_bands_R19 | Will have R20 version |
| HPUE_LTE_bands_R19 | Not needed in R20 |

Table 3 –Basket Rel-20 proposal

RAN shall consider merging some of the Rel-19 HPUE baskets into the “normal” Rel-20 baskets.

It can also be considered whether the NTN work can be included into the Rel-20 baskets. In specific, it may make sense to include both TN and NTN bands into the HPUE_NR_bands_R20 basket since the work will be very similar.

It can also be considered whether the NTN work can be included into the Rel-20 baskets.

Conclusion

In this contribution, we discuss the study item on fragmented carriers. The following Observations and Proposals were made:

Observation 1: The co-existence analysis for HPUE (i.e. PC1/1.5/2) band combinations is the same as already done for PC3.

Proposal 1: Rel-20 basket WIs should be power class agnostic.

Proposal 2: RAN shall consider merging some of the Rel-19 HPUE baskets into the “normal” Rel-20 baskets.

Observation 2: It can also be considered whether the NTN work can be included into the Rel-20 baskets.