3GPP TSG-RAN WG4 Meeting # 114-bis					R4-2504135

Wuhan Meeting, Apr. 7th – Apr. 11th, 2025

Title:	TP to TR 38.719-02-01 CA_n2-n77

Source:	Nokia, Verizon

Agenda item:	6.3.3

Document for:	Approval

# 1	Introduction

This is a TP to TR 38.719-02-01 to add CA_n2A-n77C with CA_n2A-n77C in uplink BCS 4&5. Note that additional tables are added for TR completeness, regardless if the requirements have been placed in the specifications

************************************* Start of TP*****************************************

## 5.x	CA_n2-n77

### 5.x.1	Common for 1 band UL and 2 bands UL CA

#### 5.x.1.1	Operating bands for CA

Table 5.x.1.1-1: CA band combination of band n2+n77

| NR Band | Uplink (UL) band | Downlink (DL) band | Duplex mode |
| --- | --- | --- | --- |
|  | BS receive / UE transmit | BS transmit / UE receive |  |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| n2 | 1850-1910 | 1930-1990 | FDD |
| n77 | 3300-4200 | 3300-4200 | TDD |

#### 5.x.1.2	Channel bandwidths per operating band for CA

Table 5.x.1.2-1: Supported bandwidths per CA band combination of band n2+n77

| CA operating/channel bandwidth (MHz) |  |  |  |  |
| --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| CA_n2A-n77C | n778, 9 CA_n77C CA_n2A-n77A8 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n77 | CA_n77C_BCS0 |  |
|  | n778, 9 CA_n77C CA_n2A-n77A8 CA_n2A-n77C | n2 | n2 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n77 | CA_n77C_BCS 4 and 5 |  |
| CA_n2(2A)-n77C | n778, 9 CA_n77C CA_n2A-n77A8 | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n77 | CA_n77C_BCS1 |  |
|  | CA_n2A-n77A CA_n2A-n77C | n2 | CA_n2(2A)_BCS 4 and 5 | 4 and 5 |
|  |  | n77 | CA_n77C_BCS 4 and 5 |  |

#### 5.x.1.3	UE co-existence studies for 1 band UL

Table 5.x.1.3-1 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_n2-n77.

Table 5.x.1.3-1 UL/DL harmonics/harmonic mixing analysis

| UL/DL harmonics |  | n2 | UL12 | UL2 | UL33 | UL4 | UL5 | MSD type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | fLow | 1850 | 3700 | 5550 | 7400 | 9250 |  |
| n77 | fLow | fHigh | 1910 | 3820 | 5730 | 7640 | 9550 |  |
| DL1 | 3300 | 4200 | N/A | D | - | - | - | UL harmonic |
| DL23 | 6600 | 8400 | - | N/A | - | N/A | N/A | Harmonic mixing |
| DL34 | 9900 | 12600 | - | - | N/A | - | N/A |  |
| DL4 | 13200 | 16800 | - | N/A | N/A | N/A | N/A |  |
| DL54 | 16500 | 21000 | - | - | N/A | N/A | N/A |  |
| Analysis |  |  | There is an issue found at UL2/DL1 |  |  |  |  |  |
| UL/DL harmonics |  | n77 | UL14 | UL2 | UL33 | UL4 | UL5 | MSD type |
|  |  | fLow | 3300 | 6600 | 9900 | 13200 | 16500 |  |
| n2 | fLow | fHigh | 4200 | 8400 | 12600 | 16800 | 21000 |  |
| DL1 | 1930 | 1990 | N/A | - | - | - | - | UL harmonic |
| DL23 | 3860 | 3980 | D | N/A | - | N/A | N/A | Harmonic mixing |
| DL34 | 5790 | 5970 | - | - | N/A | - | N/A |  |
| DL4 | 7720 | 7960 | - | N/A | N/A | N/A | N/A |  |
| DL54 | 9650 | 9950 | - | - | N/A | N/A | N/A |  |
| Analysis |  |  | There is an issue found at UL1/DL2 |  |  |  |  |  |
| Note 1: ULx means UL xth harmonic frequency, and DLy means DL yth harmonic frequency range Note 2: When a collision is detected with an overlap >0Hz between the ULx with DLy frequency ranges, the ULx/DLy cell is marked “D” for direct hit. When the gap between ULx and DLy frequency range is from 0Hz to x*MinULCBW, the ULx/DLy cell is marked “N” for Near miss. Note 3: UL3/DL2 harmonic mixing direct hit case for PC3/5 only apply for DL>3GHz Note 4: For harmonic mixing, near-miss cases only apply for UL1 and odd DLy orders. |  |  |  |  |  |  |  |  |

The requirements are already found in the specification.

Table 5.x.1.3-2 summarizes frequency ranges where cross band isolation may occur for CA_n2-n77.

Table 5.x.1.3-2: Cross-band isolation analysis

| Bands | n2 |  | n77 |  |
| --- | --- | --- | --- | --- |
| Frequency limit | flow | fhigh | flow | fhigh |
| fUL (MHz) | 1850 | 1910 | 3300 | 4200 |
| fDL (MHz) | 1930 | 1990 | 3300 | 4200 |
| UL CBW (MHz)2 | Minimum CBW | Maximum CBW | Minimum CBW | Maximum CBW |
|  | 5 | 40 | 10 | 100 |
| ACLR1 range | fxULlow-maxULCBWx | fxULhigh+maxULCBWx | fyULlow-maxULCBWy | fyULhigh+maxULCBWy |
| ACLR1 (MHz) | 1810 | 1950 | 3200 | 4300 |
| ACLR2 range | fxULlow-2*maxULCBWx | fxULhigh+2*maxULCBWx | fyULlow-2*maxULCBWy | fyULhigh+2*maxULCBWy |
| ACLR2 (MHz) | 1770 | 1990 | 3100 | 4400 |
| ACLR3 range | fxULlow-3*maxULCBWx | fxULhigh+3*maxULCBWx | fyULlow-3*maxULCBWy | fyULhigh+3*maxULCBWy |
| ACLR3 (MHz) | 1730 | 2030 | 3000 | 4500 |
| ACLR4 range | fxULlow-4*maxULCBWx | fxULhigh+4*maxULCBWx | fyULlow-4*maxULCBWy | fyULhigh+4*maxULCBWy |
| ACLR4 (MHz) | 1690 | 2070 | 2900 | 4600 |
| ACLR5 range1 | fxULlow-5*maxULCBWx | fxULhigh+5*maxULCBWx | fyULlow-5*maxULCBWy | fyULhigh+5*maxULCBWy |
| ACLR5 (MHz) | 1650 | 2110 | 2800 | 4700 |
| Analysis | No issues found |  | No issues found |  |
| NOTE 1: Even if there is no overlap up to ACLR5, MSD beyond the ACLR5 range should be evaluated further if: - The UL aggressor band and DL aggressor band are part of the same or adjacent band group as described in table A.1 - If the DL band is above the UL band, it’s lower frequency edge must be below the UL lowest 2nd harmonic frequency - As an indicative threshold, if >45dB UL rejection at the DL band frequency can be guaranteed, assuming a -130dBm/Hz TX noise floor level, the transmitter noise floor related MSD should be negligible NOTE 2: The maximum UL channel bandwidth of the BCS (noted maxULCBW) is used to calculate the band ACLR ranges while the minimum DL channel bandwidth of the BCS (noted minDLCBW) is used for the DL band victim channel bandwidth. |  |  |  |  |

Table 5.x.1.3-3 summarizes frequency ranges where IMD products caused by one UL band with 2CC intra-band UL CA may occur for n5B

Table 5.x.1.3-3: Intra-band ULCA IMD overlap with the other DL band analysis.

| All in MHz | flow | fhigh | BB IMD range3 |  |  |
| --- | --- | --- | --- | --- | --- |
| n77 fUL | 3300 | 4200 | Order | flow | fhigh |
| n2 fDL | 1850 | 1910 |  |  |  |
| 2CCBW1 | Minimum | Maximum | IMD2 (1-1) | Min2CCBW | Max2CCBW |
|  | 45 | 200 |  | 45 | 200 |
| Close to UL IMD range2 |  |  | IMD4 (2-2) | 2*Min2CCBW | 2*Max2CCBW |
| Order | flow | fhigh |  | 90 | 400 |
| IMD3 (2-1) | fULlow-Max2CCBW | fULhigh+Max2CCBW | IMD6 (3-3) | 3*Min2CCBW | 3*Max2CCBW |
|  | 3100 | 4400 |  | 135 | 600 |
| IMD5 (3-2) | fULlow-2*Max2CCBW | fULhigh+2*Max2CCBW | Close to H2 IMD range4 |  |  |
|  | 2900 | 4600 | Order | flow | fhigh |
| IMD7 (4-3) | fULlow-3*Max2CCBW | fULhigh+3*Max2CCBW | IMD4 (3-1) | 2*fULlow-Max2CCBW | 2*fULhigh+Max2CCBW |
|  | 2700 | 4800 |  | 6400 | 8600 |
| IMD9 (5-4) | fULlow-4*Max2CCBW | fULhigh+4*Max2CCBW | IMD6 (4-2) | 2*fULlow-2*Max2CCBW | 2*fULhigh+2*Max2CCBW |
|  | 2500 | 5000 |  | 6200 | 8800 |
| IMD11 (6-5) | fULlow-5*Max2CCBW | fULhigh+5*Max2CCBW | Close to H3 IMD range4 |  |  |
|  | 2300 | 5200 | Order | flow | fhigh |
| IMD13 (7-6) | fULlow-6*Max2CCBW | fULhigh+6*Max2CCBW | IMD5 (4-1) | 3*fULlow-Max2CCBW | 3*fULhigh+Max2CCBW |
|  | 2100 | 5400 |  | 9700 | 12800 |
| Analysis | No issues found |  |  |  |  |
| NOTE 1: 2CCBW is the instantaneous transmit bandwidth of the two intra-band UL CCs: - The minimum 2CCBW for contiguous / non-contiguous intra-band ULCA is 0 / minimum UL channel bandwidth - The maximum 2CCBW for contiguous / non-contiguous ULCA is Min(maximum aggregated bandwidth / maximum separation bandwidth(600MHz),fULhigh-fULlow) NOTE 2: The close to UL IMD range is the most critical when the victim DL band in proximity to the UL band: - For contiguous/non-contiguous intra-band ULCA within a TDD band, IMD order up to 9/7 should be considered and MPR assumed - For intra-band ULCA within a FDD band, IMD order up to 13 should be considered for bands in the same band group and MPR is not assumed. If justified by poor filtering performance, higher order IMD may need to be specified. NOTE 3: The BB IMD range should only be considered if the DL band is below the UL band and for non-contiguous ULCA within a TDD band >3GHz (assuming CA with 450MHz bands is not considered) - IMD2 is not considered assuming CA with 450MHz bands is not considered - IMD4 is considered for FDD or SimRx/Tx TDD bands <1GHz - IMD6 is considered for FDD or SimRx/Tx TDD bands <1.68GHz NOTE 4: The harmonic 2 and 3 IMD ranges should only be considered if the DL band is above the UL band |  |  |  |  |  |

#### 5.x.1.4	∆TIB,c and ∆RIB,c values

For CA_n2-n77, the ∆TIB,c and ∆RIB,c values have already been specified.

#### 5.x.1.5	REFSENS requirements

There is no need for new REFSENS requirements.

#### 5.x.1.6	OOB blocking exception requirements

There is no need for OOB blocking exception.

### 5.x.2	Specific for 2 bands UL CA

#### 5.x.2.1	Maximum output power for inter-band CA

Table 5.x.2.1-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Power Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n2A-n77C | 23 | +2/-3 |

#### 5.x.2.2	UE co-existence studies for 2 bands UL

Table 5.x.2.2-1 lists Band n2 + Band n77 2 bands UL CA(2CC) 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 5.x.2.2-1: Band n2 and Band n77 for 2CC UL IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1390 - 2350 |  | 5150 - 6110 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 500 - 520 |  | 4690 - 6550 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7000 - 8020 |  | 8450 - 10310 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1350 - 2430 |  | 7990 - 10750 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 2780 - 4700 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 8850 - 9930 |  | 11750 - 14510 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 10300 - 12220 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 11290 - 14950 |  | 3200 - 4340 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 6080 - 8900 |  | 870 - 2850 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 15050 - 18710 |  | 10700 - 11840 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 13600 - 16420 |  | 12150 - 14130 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

The requirements are already found in the specification.

Table 5.x.2.2-2 lists Band n2 + Band n77 2 bands UL CA(3CC) triple beat IMD analysis for Band n2- Band n77.

Table 5.x.2.2-2: Band n2 and Band n77 for 3CC UL IMD products

| Band / CA1 | n2 |  | CA_n77C |  |
| --- | --- | --- | --- | --- |
| Frequency limit (all MHz) | fx_low | fx_high | fy_low | fy_high |
| fUL | 1850-1910 |  | 3300-4200 |  |
| fDL | 1930-1990 |  | N/A | N/A |
| 2CCBW2 | N/A | N/A | Minimum | Maximum |
|  |  |  | 0 -200 |  |
| IMD3 products | fxUL_low-max2CCBW | fxUL_low | fxUL_high | fxUL_high+max2CCBW |
| IMD3 (MHz) | 1650-1850 |  | 1910-2110 |  |
| Analysis | There is an issue of IMD3, but no requirements, since the bands are not in same or adjacent bandgroups |  |  |  |
| NOTE 1: If the two bands are not part of the same or adjacent band groups as defined in table A.1, the analysis can be ignored. NOTE 2: For contiguous intra-band ULCA, the minimum and maximum separation BW are 0MHz and Min(fy_high-fy_low, maximum aggregated BW) respectively. |  |  |  |  |

#### Based on Table 5.x.2.2-2:

The bands are not of the same or adjacent band groups, so Note 1 applies.

#### 5.x.2.3	REFSENS requirements

There is no need for new REFSENS requirements.

************************************* End of TP******************************************