3GPP TSG-RAN WG4 Meeting # 114-bis											R4-2504140

Wuhan, Apr. 7th ‒ Apr. 11th, 2025

Agenda item:			6.2.2

Source:	Nokia, Telefonica

Title:	TP to TR 37.719-11-11 addition of DC_40A_n28A

Document for:	Approval

# Introduction

This is a TP to TR 37.719-11-11 to add DC_40A_n28A.

************************************* Start of TP*****************************************

## 6.x.1	DC_ 40_n28

#### 6.x.1.1 	Configurations for DC

Table 6.x.1.1-1:  Inter-band EN-DC configurations of 1 LTE band + 1 NR band

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | Single UL allowed |
| --- | --- | --- |
| DC_40A_n28A | DC_40A_n28A | No |

#### 6.x.2.2	Maximum output power for DC

Table 6.x.2.2-1: Maximum output power for inter-band EN-DC of 1 LTE band + 1 NR band

| DC configuration | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_40A_n28A | 23 | +2/-3 |

#### 6.x.2.3	Spurious emission band UE co-existence for DC

Table 6.x.2.3-1: Spurious emissions for inter-band EN-DC of 1 LTE band + 1 NR band

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_ 40_n28 | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| NOTE 3: Applicable when co-existence with PHS system operating in 1884.5 - 1915.7 MHz. |  |  |  |  |  |  |  |

#### 6.x.2.4	Co-existence analysis for DC

Table 6.x.2.4-1 Band 40 and Band n28 UL/DL harmonics/harmonic mixing analysis

| UL/DL harmonics |  | n28 | UL12 | UL2 | UL33 | UL4 | UL5 | MSD type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | fLow | 703 | 1406 | 2109 | 2812 | 3515 |  |
| 40 | fLow | fHigh | 748 | 1496 | 2244 | 2992 | 3740 |  |
| DL1 | 2300 | 2400 | N/A | - | - | - | - | UL harmonic |
| DL23 | 4600 | 4800 | - | N/A | - | N/A | N/A | Harmonic mixing |
| DL34 | 6900 | 7200 | - | - | N/A | - | N/A |  |
| DL4 | 9200 | 9600 | - | N/A | N/A | N/A | N/A |  |
| DL54 | 11500 | 12000 | - | - | N/A | N/A | N/A |  |
| Analysis |  |  | No issues found. |  |  |  |  |  |
| UL/DL harmonics |  | 40 | UL14 | UL2 | UL33 | UL4 | UL5 | MSD type |
|  |  | fLow | 2300 | 4600 | 6900 | 9200 | 11500 |  |
| n28 | fLow | fHigh | 2400 | 4800 | 7200 | 9600 | 12000 |  |
| DL1 | 758 | 803 | N/A | - | - | - | - | UL harmonic |
| DL23 | 1516 | 1606 | - | N/A | - | N/A | N/A | Harmonic mixing |
| DL34 | 2274 | 2409 | D | - | N/A | - | N/A |  |
| DL4 | 3032 | 3212 | - | N/A | N/A | N/A | N/A |  |
| DL54 | 3790 | 4015 | - | - | N/A | N/A | N/A |  |
| Analysis |  |  | There is an issue found at UL1/DL3 |  |  |  |  |  |
| Note 1: ULx means UL xth harmonic frequency, and DLy means DL yth harmonic frequency range Note 2: When a collision is detected with an overlap >0Hz between the ULx with DLy frequency ranges, the ULx/DLy cell is marked “D” for direct hit. When the gap between ULx and DLy frequency range is from 0Hz to x*MinULCBW, the ULx/DLy cell is marked “N” for Near miss. Note 3: UL3/DL2 harmonic mixing direct hit case for PC3/5 only apply for DL>3GHz Note 4: For harmonic mixing, near-miss cases only apply for UL1 and odd DLy orders. |  |  |  |  |  |  |  |  |

Based on Table 6.x.2.4-1, there is UL1/DL3 into band n28.

Table 6.x.2.4-2: Band 40 and Band n28 cross-band isolation analysis

| Bands | n28 |  | 40 |  |
| --- | --- | --- | --- | --- |
| Frequency limit | flow | fhigh | flow | fhigh |
| fUL (MHz) | 703 | 748 | 2300 | 2400 |
| fDL (MHz) | 758 | 803 | 2300 | 2400 |
| UL CBW (MHz)2 | Minimum CBW | Maximum CBW | Minimum CBW | Maximum CBW |
|  | 3 | 30 | 5 | 40 |
| ACLR1 range | fxULlow-maxULCBWx | fxULhigh+maxULCBWx | fyULlow-maxULCBWy | fyULhigh+maxULCBWy |
| ACLR1 (MHz) | 673 | 778 | 2260 | 2440 |
| ACLR2 range | fxULlow-2*maxULCBWx | fxULhigh+2*maxULCBWx | fyULlow-2*maxULCBWy | fyULhigh+2*maxULCBWy |
| ACLR2 (MHz) | 643 | 808 | 2220 | 2480 |
| ACLR3 range | fxULlow-3*maxULCBWx | fxULhigh+3*maxULCBWx | fyULlow-3*maxULCBWy | fyULhigh+3*maxULCBWy |
| ACLR3 (MHz) | 613 | 838 | 2180 | 2520 |
| ACLR4 range | fxULlow-4*maxULCBWx | fxULhigh+4*maxULCBWx | fyULlow-4*maxULCBWy | fyULhigh+4*maxULCBWy |
| ACLR4 (MHz) | 583 | 868 | 2140 | 2560 |
| ACLR5 range1 | fxULlow-5*maxULCBWx | fxULhigh+5*maxULCBWx | fyULlow-5*maxULCBWy | fyULhigh+5*maxULCBWy |
| ACLR5 (MHz) | 553 | 898 | 2100 | 2600 |
| Analysis | No issues found |  | No issues found |  |
| NOTE 1: Even if there is no overlap up to ACLR5, MSD beyond the ACLR5 range should be evaluated further if: - The UL aggressor band and DL aggressor band are part of the same or adjacent band group as described in table A.1 - If the DL band is above the UL band, it’s lower frequency edge must be below the UL lowest 2nd harmonic frequency - As an indicative threshold, if >45dB UL rejection at the DL band frequency can be guaranteed, assuming a -130dBm/Hz TX noise floor level, the transmitter noise floor related MSD should be negligible NOTE 2: The maximum UL channel bandwidth of the BCS (noted maxULCBW) is used to calculate the band ACLR ranges while the minimum DL channel bandwidth of the BCS (noted minDLCBW) is used for the DL band victim channel bandwidth. |  |  |  |  |

Based on Table 6.x.2.4-2, there is no cross-band issue for this combination.

Table 6.x.2.4-3: Band 40 and Band n28 UL IMD products

| UE UL carriers |  | fx_low | fx_high | fy_low | fy_high |  |
| --- | --- | --- | --- | --- | --- | --- |
| 2nd order IMD products |  | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |  |
| IMD frequency limits (MHz) |  | 1552 - 1697 |  | 3003 - 3148 |  |  |
| Two-tone 3rd order IMD products |  | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |  |
| IMD frequency limits (MHz) |  | 804 - 994 |  | 3852 - 4097 |  |  |
| Two-tone 3rd order IMD products |  | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |  |
| IMD frequency limits (MHz) |  | 3706 - 3896 |  | 5303 - 5548 |  |  |
| Two-tone 4th order IMD products |  | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |  |
| IMD frequency limits (MHz) |  | 56 - 291 |  | 6152 - 6497 |  |  |
| Two-tone 4th order IMD products |  | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |  |
| IMD frequency limits (MHz) |  | 3104 - 3394 |  |  |  |  |
| Two-tone 4th order IMD products |  | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |  |
| IMD frequency limits (MHz) |  | 4409 - 4644 |  | 7603 - 7948 |  |  |
| Two-tone 4th order IMD products |  | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |  |
| IMD frequency limits (MHz) |  | 6006 - 6296 |  |  |  |  |
| Two-tone 5th order IMD products |  | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |  |
| IMD frequency limits (MHz) |  | 8452 - 8897 |  | 412 - 692 |  |  |
| Two-tone 5th order IMD products |  | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |  |
| IMD frequency limits (MHz) |  | 5404 - 5794 |  | 2356 - 2691 |  |  |
| Two-tone 5th order IMD products |  | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |  |
| IMD frequency limits (MHz) |  | 9903 - 10348 |  | 5112 - 5392 |  |  |
| Two-tone 5th order IMD products |  | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |  |
| IMD frequency limits (MHz) |  | 8306 - 8696 |  | 6709 - 7044 |  |  |
| NOTE 1: For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. NOTE 2: The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Based on Table 6.x.2.4-3, there is no IMD issue for this combination.

#### 6.x.2.5	∆TIB and ∆RIB values

The TIB,c and RIB,c values are reused from DC_28_n40 as specified in TS 38.101-3.

Table 6.x.2.5-1: ΔTIB,c

| Inter-band DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)1 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration2 |  |
| DC_40_n28 | 0.3 | 0.3 |
| NOTE 1: “-” denotes ΔTIB,c = 0. NOTE 2: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively. |  |  |

Table 6.x.2.5-2: ΔRIB,c

| Inter-band DC configuration | ΔRIB,c for NR band / E-UTRA band (dB)1 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration2 |  |
| DC_40_n28 | - | - |
| NOTE 1: “-” denotes ΔRIB,c = 0. NOTE 2: The component band order in the configuration should be listed by the order of NR band and E-UTRA band respectively. |  |  |

#### 6.x.2.6	Analysis of MSD requirements

The REFSENS requirement for DC_40_n28 due to harmonic mixing are reused from DC_28_n40 and are shown in table 6.x.2.6-1:

Table 6.x.2.6-1: Reference sensitivity exceptions (MSD) due to receiver harmonic mixing for EN-DC in NR FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| 40 | n28 | 10 | 15 | 25 | 5 | 37.8 | NOTE 4 | UL1/DL3 |
| 40 | n28 | 10 | 15 | 25 | 20 | 30.8 | NOTE 4 | UL1/DL3 |

************************************* End of TP*****************************************