# 3GPP TSG-RAN WG4 Meeting #114bis	R4-2504157
Wuhan, Hubei, China, 07th – 11th April, 2025

Agenda item:	7.10.2.3.1

Source: 	OPPO

Title: 	Discussion on Tx requirements for IoT-NTN HPUE

Document for:	Discussion

- Introduction

In last RAN4 meeting, the HPUE Tx RF requirements with IoT NTN operating in IoT-NTN bands were discussed and some agreements are approved in WF [1]:

Issue 1-1-1: ACLR for NB-Iot based Iot-NTN for PC1/2; Agreement: - Re-use current PC3 ACLR (UTRA_ACLR = 37dBc) for PC1/2. Issue 1-1-2: ACLR for eMTC based Iot-NTN for PC1/2; Agreement: o Option 1: Re-use PC3 eMTC based Iot-NTN ACLR 30dBc. o Option 2: 31dBc ACLR Issue 1-1-3: SEM for NB-Iot based NTN for PC1/2 Agreement: - Reusing SEM requirements defined for PC3 for NB-Iot based Iot-NTN Issue 1-1-4: SEM for eMTC based NTN for PC1/2 Agreement: - Reusing SEM requirements defined for PC3 for eMTC based Iot-NTN Issue 1-1-5: MPR/A-MPR assumptions for NB-Iot based Iot-NTN for PC1/2 Agreement: - For MPR/A-MPR assumptions for NB-Iot based Iot-NTN for PC1/2 - For NB-IoT PC2, for A-MPR evaluations assume 25 dBc IQ-image and LO-leakage and 60 dBc CIM3 - For NB-IoT PC1, for MPR and A-MPR evaluations assume [28~34]dBc IQ-image and LO-leakage and 60 dBc CIM3. - For NB-IoT PC2, as a starting point consider re-use of PC3 MPR - Editor’s Note: In RAN4#113 there was also a related agreement: “As the starting point, consider the center single tone allocation as MPR=0 reference point, and introduce MPR for edge single tone allocation.” Issue 1-1-6: MPR assumptions for eMTC based Iot-NTN for PC1/2 Agreement: - For MPR/A-MPR assumptions for eMTC based Iot-NTN for PC1/2 - For Cat M1, in simulations calibrate PA to just meet ACLR with [30~31] dBc ACLR and 1 dB MPR with 100 RB DFT-s-OFDM waveform. Issue 1-1-7: P-MPR definition for NB-Iot based Iot-NTN. - Proposals - For NB-Iot based Iot-NTN, discuss the proposal, confirm it is applicable for all the power classes, and discuss whether it can be defined from Rel-18. - Discuss whether P-MPR should be applied to handheld UEs only and whether this needs to be specified or not. - WF - Further discuss P-MPR definition Issue 1-1-8: The Basic MOP tolerance (Including eMTC and NB-Iot based) - Proposals - For PC2: - Option 1: +2 / -3dB - Option 2: +2 / -2dB - For PC1: - Option 1: +2 / -3dB - WF FFS Issue 1-1-9: On relaxing the upper limitation of nominal MOP or MOP tolerance for NTN HD-FDD operation. - Proposals - Option 1: Relaxation of TX MOP tolerance upper limitation for NTN HD-FDD EUTRA band; Class 2 (dBm); Tolerance (dB); Class 3 (dBm); Tolerance (dB). 256; 26; +2x, -2; 23; +2x, -2. 255; 26; +2x, -2; 23; +2x, -2. NOTE X: The upper power tolerance limit relaxation and related requirements in HD-FDD operation are specified in sub-clause [xxx].. - Option 2: Additional NW signal to control relaxation of TX MOP upper limitation for NTN HD-FDD. EUTRA band; Class 2 (dBm); Tolerance (dB); Class 3 (dBm); Tolerance (dB). 256; 26x; +2, -2; 23x; +2, -2. 255; 26x; +2, -2; 23x; +2, -2. NOTE X: The power limit relaxation and related requirements in HD-FDD operation are specified in sub-clause [xxx].. - Option 3: New sub power classes (e.g., PC2NTN-HD-FDD) to relax the TX output power upper limitation. - Option 4: Other ways are not excluded - FD-FDD can also be considered. - WF - FFS. Issue 1-1-8: The nominal MOP tolerance] Background: HD-FDD FE IL is obviously lower compared to FD-FDD IL, which can improve HD-FDD TX MOP with 0.8dB compared to FD-FDD TX MOP. From EC Decision 2016/687, at least a tolerance of up to +2 dB is to take account of operation under extreme environmental conditions and production spread. As for IoT-NTN eMTC and NB1/NB2, the actual NTN-IoT PC2 HD-FDD TX output power upper bound could be 28.8dBm (i.e., 26.8dBm+2dB) but exceeds conventional PC2 TX upper power bound of 28dBm. From ETSI EN 301 908, PC3 TX output power upper bound can be specified higher (i.e., 23dBm +2.7dB or 23dBm +3dB). - Proposals As for IoT-NTN eMTC and NB1/NB2, regarding solution for PC2 HD-FDD TX MOP upper bound higher than FD-FDD TX MOP upper bound, consider a specific or general solution for all IoT-NTN bands. The solution is applicable for solving the same issue in PC3 and PC5. The options are as follows. - Option1: Add additional note to increase the upper tolerance for NTN HD-FDD operation in applicable regions. EUTRA band; Class 2 (dBm); Tolerance (dB); Class 3 (dBm); Tolerance (dB); Class 5 (dBm); Tolerance (dB). 256; 26; +22; 23; +22, -2; 20; +22, -2. 255; 26; +22; 23; +22, -2; 20; +22, -2. 254; 26; +22; 23; +22, -2; 20; +22, -2. 253; 26; +22; 23; +22, -2; 20; +22, -2. 252; 26; +22; 23; +22, -2; 20; +22, -2. NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: When UEs recognizes and supports NTN HD-FDD operation in XXX region, the upper tolerance requirement can be increased from +2dB to [+2.7dB or +TT-NTN-HDFDD].. - Option 2: Add additional UE capability and NW signal to control NTN HD-FDD power increment. - Option 2-1: EUTRA band; Class 2 (dBm); Tolerance (dB); Class 3 (dBm); Tolerance (dB); Class 5 (dBm); Tolerance (dB). 256; 26; +22; 23; +22, -2; 20; +22, -2. 255; 26; +22; 23; +22, -2; 20; +22, -2. 254; 26; +22; 23; +22, -2; 20; +22, -2. 253; 26; +22; 23; +22, -2; 20; +22, -2. 252; 26; +22; 23; +22, -2; 20; +22, -2. NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: When IoT-NTN UE operates in HD-FDD and indicates support for UE optional capability x1-r19 and if IE x2-r19 is set to 1. The MOP upper tolerance is further increased by ΔPNTNHdFdd from 2dB.. - Option 2-2: EUTRA band; Class 2 (dBm); Tolerance (dB); Class 3 (dBm); Tolerance (dB); Class 5 (dBm); Tolerance (dB). 256; 262; +2; 232; +2, -2; 202; +2, -2. 255; 262; +2; 232; +2, -2; 202; +2, -2. 254; 262; +2; 232; +2, -2; 202; +2, -2. 253; 262; +2; 232; +2, -2; 202; +2, -2. 252; 262; +2; 232; +2, -2; 202; +2, -2. NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: When IoT-NTN UE operates in HD-FDD and indicates support for UE optional capability x1-r19 and if IE x2-r19 is set to 1. The reference power is increased by ΔPNTNHdFdd.. - Option 3: Add new power classes (e.g., PC4.5_NTN-HD-FDD, PC2.5_NTN-HD-FDD, PC1.75_NTN-HD-FDD). - Option 4: - Re-use LTE TN TS 36.101-1 nominal MOP and ±2 tolerance can be considered as starting point (same as for band n14) for PC2 for IoT-NTN HPUE. - WF - FFS Feasibility & Requirements Application Issue 1-2-1: Whether PC1 requirements can be applied to handheld UE for NB-Iot based Iot-NTN. - Proposals - Option 1: Yes - Option 2: No - WF - FFS. Issue 1-2-2: PC1 Nominal MOP for NB-Iot based Iot-NTN - Proposals - Considering lower nominal output power for NB-IoT based Iot-NTN PC1. - Not necessarily aligned and further improve feasibility - WF - FFS. Issue 1-2-3: The reference components need to be considered. Agreement: - The reference components need to be considered. - For NB-Iot based Iot-NTN, both GSM PA and NR 1Tx PC2 PA are taken into consideration in developing unified for one power class; - For eMTC-based IoT-NTN, HPUE for PC2 and NR 1Tx PC2 PA Issue 1-2-4: Whether PC1 requirements can be applied to handheld UE for eMTC based with half duplex. - Proposals - Option 1: Yes - Option 2: No - WF - FFS Current Status: (For Information) Table: Current status for requirements consideration and feasibility for Iot-NTN Power Class; Antenna Numbers; Handheld and/or non-handheld. NB-Iot based; PC1; 1Tx; FFS (non-handheld already agreed). PC2; 1Tx; Both. eMTC based with half duplex; PC1; 1Tx; FFS (non-handheld already agreed). PC2; 1Tx; Both. eMTC based with full duplex; PC1; 1Tx; Non-handheld. PC2; 1Tx; Both.

This contribution further discussed the UE Tx RF requirements for IoT NTN HPUE.

- Discussion

Issue 1-1-2: ACLR for eMTC based Iot-NTN for PC1/2;

Agreement:

o	Option 1: Re-use PC3 eMTC based Iot-NTN ACLR 30dBc.

o	Option 2: 31dBc ACLR

In our companion contribution for NR-NTN HPUE, we proposed the ACLR for NR-NTN HUE reuse the ACLR for NR-TN HUUE based on the MPR simulation results.

And based on the IoT NTN co-existence study results:

- For NB-IoT based IoT-NTN PC2, it is observed that the required ACLR is less than 20 dB with 1.5km isolation distance assumed based on the simulation results.

- For NB-IoT based IoT-NTN PC1, it is observed that the required ACLR is less than 26.2 dB with 1.5km isolation distance assumed based on the simulation results.

To facilitate IoT-NTN HPUE can reuse the same RF implementation with NR-NTN HPUE, the ACLR for eMTC based Iot-NTN HPUE can use the same ACLR value with NR_NTN HPUE.

Proposal 1: 31dB ACLR for eMTC based Iot-NTN for PC2.

Proposal 2: 37dB ACLR for eMTC based Iot-NTN for PC1.

Proposal 3: The Basic MOP tolerance (Including eMTC and NB-Iot based)

- For PC2: +2 / -3dB

- For PC1: +2 / -3dB

In the R-19, the purpose of IoT NTN HPUE is to introduce PC1 and PC2 for IoT NTN bands and

- PC2 will apply to both handheld and non-handheld UE

- PC1 are prioritized for non-handheld UE

In previous meeting, some companies proposed that it can reuse the GSM PA for PC1 handheld IoT NTN UE. Although the insertion loss of the duplexer in half-duplex mode does not need to be considered, the front-end filter still needs to be considered, it’s hard to get the 31dBm for PC1 handheld UE with 1Tx. In additional, it also needs consider GSM delisting issue. So, reusing GSM PA for PC1 handheld IoT NTN UE is not an optimal choice in the long term. Until a suitable PA for PC1 handheld NTN UE is found, PC1 with 1Tx is best for non-handheld IoT NTN UE only.

Proposal 4: PC1 with 1Tx only apply to non-handheld IoT NTN UE.

- Conclusion

In this paper we give further discussion on the TX requirement for IoT NTN HPUE. The proposals are as following:

Proposal 1: 31dB ACLR for eMTC based Iot-NTN for PC2.

Proposal 2: 37dB ACLR for eMTC based Iot-NTN for PC1.

Proposal 3: The Basic MOP tolerance (Including eMTC and NB-Iot based)

- For PC2: +2 / -3dB

- For PC1: +2 / -3dB

Proposal 4: PC1 with 1Tx only apply to non-handheld IoT NTN UE.

- Reference

[1] R4-2503006, WF on HPUE UE requirement for IoT-NTN, vivo, RAN4#114.