3GPP TSG-RAN WG4 Meeting #114bis		R4-2504166
Wuhan, China, 7– 11 April, 2025

Title:	R16 n1 NS_05 PC2 AMPR

Source:	OPPO

Agenda item:	5.3.1

Document for:	Approval

- Introduction

According to paper [1] the protection of PHS system will be relaxed due to shut down of this system in the near future in Japan. It will mainly impact the band n1 emission requirements, and AMPR for NS_05 and NS_05U which were defined because of PHS. Thus [1] requests companies to re-evaluate the AMPR for NS_05 and NS_05U.

In last meeting, WF [2] was agreed. This paper shares the simulation results for NS_05 AMPR in band n1 with PC2.

- Discussion

  - AMRP for PC2 NS_05

2.3.1 AMRP region definition

The configurations are:

- power class PC2

- One PC2 PA or 2xPA3 PA

- 5/10/15/20MHz CBW with 30khz scs

- CP-OFDM / DFT-S-OFDM

- Modulation pi/2 bpsk, qpsk, 16qam, 64qam, 256qam

- Different centre frequency locations

- To meet the updated emission requirements

The AMPR-MPR results can be found in annex A for NS_05 with different CBW and different centre frequencies. Based on these simulation results, the AMPR regions proposed to be defined are in table 1. And exact regions are as shown in following figure

Table 1 AMPR region definition

|  | Region A1 |  | Region A2 |  |
| --- | --- | --- | --- | --- |
|  | LCRB | Rbstart | LCRB | Rbstart |
| 5M@1922.5 | N/A | N/A | N/A | N/A |
| 10M@1925 | >= 1.25*RBstart + 5.31MHz/12/scs | >0 | <= 1.8MHz/12/scs | <= 1.8MHz/12/scs |
| 10M@1930 | N/A | N/A | N/A | N/A |
| 15M@1927.5 | >= RBstart*1.4 + 4.896MHz/12/scs | >0 | <= 3.6MHz/12/scs | <= 3.6MHz/12/scs |
| 15M@1932.5 | >= 12.96MHz/12/scs | <= 1.08MHz/12/scs | <= 2.16MHz/12/scs | <= 1.8MHz/12/scs |
| 20M@1930 | >= RBstart*1.44 + 4.52MHz/12/scs | >0 | <= 5.04MHz/12/scs | <= 5.04MHz/12/scs |

Proposal 1:         Consider Table 1 as the AMPR region definition for NS_05 AMPR requirements for PC2.

2.3.2 AMRP requirements for PC2 1Tx

Based on above AMPR region definition and the AMPR simulation results in Annex B, the AMPR requirements for NS_05 are summarized in table 2 for DFT-S-OFDM and CP-OFDM with implementation margin considered.

Table 2 AMPR for NS_05 PC2 with 1Tx

| PC2 NS_05 |  | 5M@ 1922.5 | 10M@ 1925 |  | 10M@ 1930 | 15M@ 1927.5 |  | 15M@ 1932.5 |  | 20M@ 1930 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | A1 | A2 |  | A1 | A2 | A1 | A2 | A1 | A2 |
| DFT-S-OFDM | Pi/2bpsk | 0 | 2.5 | 0 | 0 | 2.5 | 1.5 | 1.5 | 0 | 2.5 | 1.5 |
|  | QPSK |  | 3 | 1.5 |  | 3.5 | 2 | 2.5 | 1.5 | 3.5 | 2 |
|  | 16QAM |  | 3.5 | 2 |  | 4 | 2.5 | 2.5 | 2 | 4.5 | 3 |
|  | 64QAM |  | 3.5 | 2 |  | 4.5 | 3 | 3 | 2 | 4.5 (5.5 for 2T) | 3 (4 for 2T) |
|  | 256QAM |  | 3.5 | 2.5 |  | 4.5 | 3 | 3 | 2.5 | 4.5 | 3 |
| CP-OFDM | QPSK |  | 4.5 | 3.5 |  | 5.5 | 4 | 3.5 | 3 | 5.5 | 4 |
|  | 16QAM |  | 4.5 | 3.5 |  | 5.5 | 4 | 3.5 | 3 | 5.5 | 4 |
|  | 64QAM |  | 4.5 | 3.5 |  | 5.5 | 4 | 3.5 | 3 | 5.5 | 4 |
|  | 256QAM |  | 4.5 | 3.5 |  | 5.5 | 4 | 3.5 | 3 | 5.5 | 4 |

Proposal 2:         Consider Table 2 as the AMPR requirements for NS_05 with PC2 1Tx.

2.3.3 AMRP requirements for PC2 2Tx

For the FDD PC2, currently many UEs can only support PC2 with two PC3 PAs. To better understand the AMPR differences between PC2 1Tx and 2Tx, 2Tx AMPR for 20MHz@1930MHz with 64QAM and DFT has been conducted, and the AMPR is 1dB higher than AMPR for PC2 1Tx (marked red in table 2).

Observation 1:   2Tx AMPR is 1dB larger than 1Tx AMPR in the same configurations (64QAM DFT) for PC2 NS_05.

Proposal 3:         To accommodate 2Tx PA configurations and simplify the tables, define one set AMPR for n1 PC2 NS_05 covering both 1T and 2T. The AMPR needed for 2T in all configurations need FFS.

- Conclusion

This paper shared the AMPR simulation results for NS_05 in n1 with PC2 and got below observations and proposals.

Proposal 1:         Consider Table 1 as the AMPR region definition for NS_05 AMPR requirements for PC2.

Table 1 AMPR region definition

|  | Region A1 |  | Region A2 |  |
| --- | --- | --- | --- | --- |
|  | LCRB | Rbstart | LCRB | Rbstart |
| 5M@1922.5 | N/A | N/A | N/A | N/A |
| 10M@1925 | >= 1.25*RBstart + 5.31MHz/12/scs | >0 | <= 1.8MHz/12/scs | <= 1.8MHz/12/scs |
| 10M@1930 | N/A | N/A | N/A | N/A |
| 15M@1927.5 | >= RBstart*1.4 + 4.896MHz/12/scs | >0 | <= 3.6MHz/12/scs | <= 3.6MHz/12/scs |
| 15M@1932.5 | >= 12.96MHz/12/scs | <= 1.08MHz/12/scs | <= 2.16MHz/12/scs | <= 1.8MHz/12/scs |
| 20M@1930 | >= RBstart*1.44 + 4.52MHz/12/scs | >0 | <= 5.04MHz/12/scs | <= 5.04MHz/12/scs |

Proposal 2:         Consider Table 2 as the AMPR requirements for NS_05 with PC2 1Tx.

Table 2 AMPR for NS_05 PC2 with 1Tx

| PC2 NS_05 |  | 5M@ 1922.5 | 10M@ 1925 |  | 10M@ 1930 | 15M@ 1927.5 |  | 15M@ 1932.5 |  | 20M@ 1930 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | A1 | A2 |  | A1 | A2 | A1 | A2 | A1 | A2 |
| DFT-S-OFDM | Pi/2bpsk | 0 | 2.5 | 0 | 0 | 2.5 | 1.5 | 1.5 | 0 | 2.5 | 1.5 |
|  | QPSK |  | 3 | 1.5 |  | 3.5 | 2 | 2.5 | 1.5 | 3.5 | 2 |
|  | 16QAM |  | 3.5 | 2 |  | 4 | 2.5 | 2.5 | 2 | 4.5 | 3 |
|  | 64QAM |  | 3.5 | 2 |  | 4.5 | 3 | 3 | 2 | 4.5 (5.5 for 2T) | 3 (4 for 2T) |
|  | 256QAM |  | 3.5 | 2.5 |  | 4.5 | 3 | 3 | 2.5 | 4.5 | 3 |
| CP-OFDM | QPSK |  | 4.5 | 3.5 |  | 5.5 | 4 | 3.5 | 3 | 5.5 | 4 |
|  | 16QAM |  | 4.5 | 3.5 |  | 5.5 | 4 | 3.5 | 3 | 5.5 | 4 |
|  | 64QAM |  | 4.5 | 3.5 |  | 5.5 | 4 | 3.5 | 3 | 5.5 | 4 |
|  | 256QAM |  | 4.5 | 3.5 |  | 5.5 | 4 | 3.5 | 3 | 5.5 | 4 |

Observation 1:   2Tx AMPR is 1dB larger than 1Tx AMPR in the same configurations (64QAM DFT) for PC2 NS_05.

Proposal 3:         To accommodate 2Tx PA configurations and simplify the tables, define one set AMPR for n1 PC2 NS_05 covering both 1T and 2T. The AMPR needed for 2T in all configurations need FFS.

- Reference

[1] R4-2415439 NR A-MPR reevaluation scenarios on n1 for Japan, KDDI, NTT Docomo, #112bis 2024-10

[2] R4-2502980 WF on PHS protection update, KDDI, OPPO, #114 2025-02

Annex A: PC2 NS_05 AMPR-MPR simulation results

Annex A.1 5M@1922.5 DFT-S-OFDM

[pi/2 BPSK]													 [QPSK]

[16QAM]													 [64QAM]

[256QAM]

Annex A.2 5M@1922.5 CP-OFDM

[QPSK]													 [16QAM]

[64QAM]													 [256QAM]

Annex A.3 10M@1925 DFT-S-OFDM

[pi/2 BPSK]													 [QPSK]

[16QAM]													 [64QAM]

[256QAM]

Annex A.4 10M@1925 CP-OFDM

[QPSK]													 [16QAM]

[64QAM]													 [256QAM]

Annex A.5 10M@1930 DFT-S-OFDM

[pi/2 BPSK]													 [QPSK]

[16QAM]													 [64QAM]

[256QAM]

Annex A.6 10M@1930 CP-OFDM

[QPSK]													 [16QAM]

[64QAM]													 [256QAM]

Annex A.7 15M@1927.5 DFT-S-OFDM

[pi/2 BPSK]													 [QPSK]

[16QAM]													 [64QAM]

[256QAM]

Annex A.8 15M@1927.5 CP-OFDM

[QPSK]													 [16QAM]

[64QAM]													 [256QAM]

Annex A.9 15M@1932.5 DFT-S-OFDM

[pi/2 BPSK]													 [QPSK]

[16QAM]													 [64QAM]

[256QAM]

Annex A.10 15M@1932.5 CP-OFDM

[QPSK]													 [16QAM]

[64QAM]													 [256QAM]

Annex A.11 20M@1930 DFT-S-OFDM

[pi/2 BPSK]													 [QPSK]

[16QAM]													 [64QAM] 1T

[64QAM] 2T 												[256QAM]

Annex A.12 20M@1930 CP-OFDM

[QPSK]													 [16QAM]

[64QAM]													 [256QAM]

Annex B: PC2 NS_05 AMPR simulation results

Annex B.1 5M@1922.5 DFT-S-OFDM

[pi/2 BPSK]													 [QPSK]

[16QAM]													 [64QAM]

[256QAM]

Annex B.2 5M@1922.5 CP-OFDM

[QPSK]													 [16QAM]

[64QAM]													 [256QAM]

Annex B.3 10M@1925 DFT-S-OFDM

[pi/2 BPSK]													 [QPSK]

[16QAM]													 [64QAM]

[256QAM]

Annex B.4 10M@1925 CP-OFDM

[QPSK]													 [16QAM]

[64QAM]													 [256QAM]

Annex B.5 10M@1930 DFT-S-OFDM

[pi/2 BPSK]													 [QPSK]

[16QAM]													 [64QAM]

[256QAM]

Annex B.6 10M@1930 CP-OFDM

[QPSK]													 [16QAM]

[64QAM]													 [256QAM]

Annex B.7 15M@1927.5 DFT-S-OFDM

[pi/2 BPSK]													 [QPSK]

[16QAM]													 [64QAM]

[256QAM]

Annex B.8 15M@1927.5 CP-OFDM

[QPSK]													 [16QAM]

[64QAM]													 [256QAM]

Annex B.9 15M@1932.5 DFT-S-OFDM

[pi/2 BPSK]													 [QPSK]

[16QAM]													 [64QAM]

[256QAM]

Annex B.10 15M@1932.5 CP-OFDM

[QPSK]													 [16QAM]

[64QAM]													 [256QAM]

Annex B.11 20M@1930 DFT-S-OFDM

[pi/2 BPSK]													 [QPSK]

[16QAM]													 [64QAM] 1T

[64QAM] 2T 													[256QAM]

Annex B.12 20M@1930 CP-OFDM

[QPSK]													 [16QAM]

[64QAM]													 [256QAM]