3GPP TSG RAN WG4 Meeting #114bis	R4-2504175

Wuhan, China, 7th – 11th April 2025

Source:		Rohde & Schwarz

Title:	Discussion on CDL channel model

Agenda Item:	7.16.2

Document for:	Approval

# Introduction

During the last RAN4 meeting a way to create the CDL channel model was discussed and an initial approach was captured in the Annex of [1]. In this contribution we share our views on the topic and possible restrictions from TE side.

# Discussion

The way forward in [1] captures a description of how to derive the CDL channel models in Annex B:

Step 1: The CDL model from TR 38.901 is the starting point. Step 2: Generate the spatially filtered profile assuming single AAV (subarray or single element) is used at the transmitter and at the receiver. This step ensures that the power-based tap reduction will consider the AAV spatial response in the power based tap reduction. Step 3: Re-normalize cluster powers in the spatially filtered profile such that the strongest cluster is at 0dB. Step 4: Find the clusters in the spatially filtered profile with power below [-30] dB, and remove the corresponding cluster index from the CDL model. Step 5: If the number of clusters in the spatially filtered profile is larger than 12, find the cluster with the lowest power, and remove the corresponding cluster index from the CDL model. Repeat this step until the number of clusters equals 12. - Do not consider the spatially filtered profile further after this step Step 6: Calculate the effective delay spread after truncation of the CDL model DStruncated (ns) to be used in the next step by computing the root mean squared (rms) delay spread of the remaning clusters, using the following expressions: τ=iτipiipi DStruncated=iτi2piipi-τ2 Where: - τi is the normalized delay value of the i-th cluster in the CDL profile - pi is the power value of the i-th cluster in the CDL profile Step 7: Perform delay scaling according to the procedure described below, based on the scaling procedure from clause 7.7.3 in TR 38.901 but considering the impact of truncation using τrmstruncated. τn, scaled= τn, model ∙DSdesiredDStruncated Where: - τn, scaled is the delay value of the i-th cluster in the truncated CDL profile - DSdesired is the wanted delay spread in ns Step 8: Apply the quantization to the delay resolution of 5 ns (for FR1) and [TBD] ns for FR2. This is done simply by rounding the tap delays to the nearest multiple of the delay resolution. - To ensure that there is reduced impact from the use of a regularly spaced grid in the delay position, further improvements to delay quantization (e.g. dithering) can be considered Step 9: If the delay spread has slightly changed due to the delay quantization, adjust the final delay spread by increasing or decreasing the power of the last cluster so that the delay spread matches the pre-quantization value. Step 10: Round the amplitude of the last cluster to one decimal (e.g. -8.78 dB -8.8 dB) Note to the cluster reduction procedure: - The resulting truncated CDL Angular and Delay profile should be used according to the procedure described in TR 38.753, including spatial filtering with AAV, Angular Spread Scaling, etc

Some of the points to be further clarified after offline discussions are the truncation to 12 clusters (Step 5) and the delay resolution (Step 8).

With respect to the number of clusters the computational complexity of the whole TC scenario needs to be considered. When running a test case, the test system does not only need to emulate the fading scenario, but has to emulate the whole scenario. This includes things like emulating (multiple) cells (including up to x8 MIMO), fade those (multiple) cells, take care of all the signalling, etc. While it may be feasible to emulate fading scenarios with more than 12 clusters, it becomes increasingly compex when taking into account all the other computational tasks that need to be performed.

Therefore, the overall computational complexity of the test cases needing to be performed needs to be considered when defining the channel models. From this point of view, the channel models should to be limited to 12 clusters.

Observation 1:  Computational complexity needs to be considered for channel model emulation in the test equipment.

Proposal 1: CDL channel models shall be limited to 12 clusters.

TR 38.901 [2] defines PDPs with normalized delays. The delays that have to be applied in a simulator depend on the delay spread (DS) that should be emulated. They can be determined from the normalized delays via denormalization with the DS according to equation 7.7-1 in [2]. This denormalization in general leads to delays with a resolution of 1 pico second.

The receiver’s ability to resolve different paths in time is limited by the system bandwidth. An accepted metric to determine path resolvability is the inverse of the system BW [3]. Thus a delay resolution of 1 ps can be considered as an extra effort that is not justified from the test purpose.

As has been discussed previously already for earlier releases, it would be sufficient to have a delay grid with a 
∆T≤1BW

using the bandwidth of the signal to be faded. To avoid different grids for different channel bandwidths, a grid of ∆T=5ns was chosen, since this is sufficient for channel bandwidths up to 200 MHz Since for FR1 the maximum channel bandwidth is 100 MHz and the maximum channel bandwidth for test cases which require fading is 40 MHz, the same 5 ns grid can be reused.

Observation 2:  5ns delay grid is sufficient for FR1 requirements, taking into account the channel bandwidth to be tested and corresponding receiver resolution.

Proposal 2: Reuse the established 5ns delay grid also for CDL channel models.

As discussed above due to the limited resolvability it makes no difference for the receiver whether a given path i is emulated with delay τi or with delay τi' satisfying

τi-12BW≤τi'≤τi+12BW

Thus, as seen from the receiver the channel properties are properly reproduced as long as the delays are emulated with an accuracy of  12BW , i.e.

τi'-τi≤12BW

Therefore, from a TE emulation point of view any delay inside the tolerance mentioned before results in the same channel model for the DUT. Thus it should be allowed to shift the anywhere inside the tolerance anywhere within the range to ease TE implementation.

Proposal 3: The required emulation accuracy is τi'-τi≤12BW for any given cluster in the channel model.

# Summary

In this contribution we shared our views on open issues for the doppler and delay profiles for NGSO satellites. We have the following observation and proposals:

Observation 1:  Computational complexity needs to be considered for channel model emulation in the test equipment.

Proposal 1: CDL channel models shall be limited to 12 clusters.

Observation 2:  5ns delay grid is sufficient for FR1 requirements, taking into account the channel bandwidth to be tested and corresponding receiver resolution.

Proposal 2: Reuse the established 5ns delay grid also for CDL channel models.

Proposal 3: The required emulation accuracy is τi'-τi≤12BW for any given cluster in the channel model.

# References

[1]	R4-2502378; “Way Forward [114][322] NR_SCM”; Moderator (Nokia); RAN4#114 February 2025

[2]	TS 38.901 v.18.0.0

[3]	J. Proakis, M. Salehi, Digital Communications, New York: McGraw-Hill, 5th edition