3GPP TSG-RAN WG4 Meeting #114bis	R4-2504176

Wuhan, China, 7-11 April, 2025

Source: 	Qualcomm Incorporated

Title: 	Views on SCM for demod

Agenda item:	 7.16.2

Document for:	Agreement

- Introduction

In the previous RAN4 Meeting #114, we contributed to the SCM Study Item sharing considerations on the Study Item on the use spatial channel model for demodulation performance requirements.

According to the outcome of the discussion during the meeting and based on the approved Way Forward, we provide in this contribution further observations as part of the ongoing study.

# Views on SCM

## Random versus Fixed PMI in CDL Models

In our results we observed that the choice of PMI index for both PDSCH and SU PMI simulations has a large impact on the throughput results. Particularly unfortunate PMI choices may also result in scenarios not achieving full throughhput (e.g. 8RX/8L when using CDL channel option A3).

It follows that when using random PMI the result of the simulation will average out all various conditions experienced by the signal, resulting in a metric that might not reflect any actual performance expectation in real world conditions.

This was not the case in existing TDL requirements, where the PMI choice did not meaningfully impact the SNR performances.

Observation 1: Differently from existing TDL setup, when using CDL models the choice of different PMI indexes result in different throughput performances vs SNR. The results for random PMI provide an averaged performance that may not properly reflect a realistic operating performance.

To that end, we believe that for the purposes of both alignment and comparison of SCM models RAN4 should consider a collectively chosen fixed PMI as the best way forward, with company bringing their set of proposed indices, and a decision based on the minimum common denominator set.

Proposal 1: RAN4 to consider for the purposes of SCM alignment and comparison for PDSCH simulations to use a collectively chosen fixed PMI index. Each company provides their best N (e.g. 3/4 PMI indexes), and the most common indexes are chosen as the final set.

Proposal 2: Similarly, RAN4 should consider to use as Gamma denominator for PMI simulations the fixed PMI throughput.

## CDL Model Details

### Taps Reduction

In the previous meeting, we submitted a proposal for the procedure of taps reduction to limit the number of taps, based on the implementation concerns supported by a few companies, including test equipment vendors. We see the value in reducing the complexity of CDL modeling by using a reduced number of taps, though we are open to considering the impact this has on the spatial channel diversity.

Proposal 3: RAN4 to support the introduction of the taps reduction procedure included in Annex B in this document.

### SNR Normalization

We believe that it’s not necessary to define a formula for the normalization factor, and RAN4 can leave this up to implementation, with a clear goal of the normalization objective which is to not introduce signal gain over noise.

Proposal 4: RAN4 not to specify an SNR normalization factor for CDL.

### AAV Configurations

Our results show that AAV(8,1) is always outperformed by AAV(1,1).

We have a couple potential reasons, one being that the expected advantage of  a more spatially selective AAV configuration is not realized due to the SINR normalization implemented by RAN4, while another pointing to the fact that if the BS antenna virtualizer is not chosen appropriately, its effect of cluster selection could be more detrimental than advantageous.

To that end, in previous meetings we have explored multiple choices of BS Antenna Virtualizer (see below, from "R4-2416591 Way Forward Minutes for [112bis][318] NR_SCM")

- Option 1: Down Tilt Virtualiser : -10 degrees

- Option 2: Broadside

- Option 3: Max Receive Power

- Option 4: DFT Virtualiser according to TR 36.897 with application of panning and tilting angles (as per Table 7.8-1 of 38.901)

If the broadside option does not provide sufficient performance, we suggest to consider first the option that maximizes the received power, assuming beam directed toward the strongest cluster,  while keeping the DFT option also in consideration.

Proposal 5: RAN4 to consider options for the BS antenna virtualizer that maximize the received power (beam directed towards the strongest cluster)

### Options for alignment for CDL Models

In the previous meeting there was discussion to reduce the coverage of the alignment simulations, to reduce the burden of simulation for the companies. To that end, we have identified option A1 as the least interesting for the prosecution of the study item

Proposal 6: RAN4 not to further consider profile option A1 for the purposes of alignment, and only consider profiles for option A2 (CDL-C table based on 38.901) and A3 (A2 + Taps Reduction)

### CDL Profile Tables

Proposal 7: RAN4 to consider the CDL profile table for option A3 included in Annex A in this document.

### Ray splitting

In the inter-meeting email discussions, there were discussions on the ray splitting procedure for sub-clusters, and on the interpretation of previous agreements in RAN4.

In our view, our agreements to treat all clusters as weak cluster implies that no sub-cluster should be created. The implications is that certain clusters could have identical doppler shift and correlated fading, however the impact to performances is not clear. As such, to reduce the complexity of the implementation and the impact to the randomness reduction framework, we recommend to not consider ray splitting and use 20 rays per each cluster regardless of the cluster delay and orientation.

Proposal 8: To reduce complexity and modifications to the randomness reduction framework, RAN4 should not consider ray splitting and use 20 rays per each cluster regardless the cluster delay and orientation.

## TDL Extensions Model

We don't provide results in this meeting with extended TDL, however we support the choice to included this in the evaluation. If there's downselection, we believe Option 2 Multi-beams TDL channel with implementation details in R4-2501738 should be considered, in view of its more favourable implementation complexity.

Proposal 9: Prioritize Option 2 as TDL extension model for alignment and comparison.

## Simulation Assumptions

The current simulation assumptions specify periodicity and offset (10,0) for CSI-RS for CSI acquisition. However, to avoid conflict with SSB slot, we propose to use offset = 1 slot.

Proposal 10: RAN4 to assume slot offset 1 for CSI-RS for CSI acquisition.

- Conclusions

Observation 1: Differently from existing TDL setup, when using CDL models the choice of different PMI indexes result in different throughput performances vs SNR. The results for random PMI provide an averaged performance that may not properly reflect a realistic operating performance.

Proposal 1: RAN4 to consider for the purposes of SCM alignment and comparison for PDSCH simulations to use a collectively chosen fixed PMI index. Each company provides their best N (e.g. 3/4 PMI indexes), and the most common indexes are chosen as the final set.

Proposal 2: Similarly, RAN4 should consider to use as Gamma denominator for PMI simulations the fixed PMI throughput.

Proposal 3: RAN4 to support the introduction of the taps reduction procedure included in Annex B in this document, with the number of taps tentatively set to 12. If necessary, the number of taps can be increased.

Proposal 4: RAN4 not to specify an SNR normalization factor for CDL.

Proposal 5: RAN4 to consider options for the BS antenna virtualizer that maximize the received power (beam directed towards the strongest cluster)

Proposal 6: RAN4 not to further consider profile option A1 for the purposes of alignment, and only consider profiles for option A2 (CDL-C table based on 38.901) and A3 (A2 + Taps Reduction)

Proposal 7: RAN4 to consider the CDL profile table for option A3 included in Annex A in this document.

Proposal 8: To reduce complexity and modifications to the randomness reduction framework, RAN4 should not consider ray splitting and use 20 rays per each cluster regardless the cluster delay and orientation.

Proposal 9: Prioritize Option 2 as TDL extension model for alignment and comparison.

Proposal 10: RAN4 to assume slot offset 1 for CSI-RS for CSI acquisition.

- References

[1] RP-241610, “Study on spatial channel model for demodulation performance requirements”, 3GPP TSG RAN Meeting #104, Shanghai, China, 17th – 20th June, 2024;

[2] 3GPP TR 38.827 V16.8.0 (2022-09), “3rd Generation Partnership Project; Technical Specification Group Radio Access Network; Study on radiated metrics and test methodology for the verification of multi-antenna reception performance of NR User Equipment (UE); (Release 16)”

[3] R4-2502378, “Way Forward [114][322] NR_SCM”, 3GPP TSG-RAN WG4 Meeting #114, Athens, Greece, 17th – 21st February 2025

# ANNEX A: CDL-C CDP Tables for Information

| CLID | Delay, nsec | Power, dB | AoD, deg | AoA, deg | ZoD, deg | ZoA, deg |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | -4.4 | -38.2 | -96.9 | 96.8 | 98.1 |
| 2 | 75 | -1.2 | -21.8 | 118.9 | 98.5 | 71.1 |
| 3 | 80 | -3.5 | -21.8 | 118.9 | 98.5 | 71.1 |
| 4 | 85 | -5.2 | -21.8 | 118.9 | 98.5 | 71.1 |
| 5 | 80 | -2.5 | -34.2 | -124.4 | 100.9 | 67.6 |
| 6 | 230 | 0 | -5.9 | 171.2 | 99.2 | 76.6 |
| 7 | 235 | -2.2 | -5.9 | 171.2 | 99.2 | 76.6 |
| 8 | 240 | -3.9 | -5.9 | 171.2 | 99.2 | 76.6 |
| 9 | 240 | -7.4 | 44.2 | 51.9 | 106.4 | 62.8 |
| 10 | 290 | -7.1 | -50.6 | 63.4 | 94.5 | 56.6 |
| 11 | 300 | -10.7 | 49.1 | -79.4 | 107.5 | 69.8 |
| 12 | 340 | -11.1 | -73.0 | 43.1 | 92.3 | 50.8 |
| 13 | 450 | -5.1 | -44.2 | 65.1 | 104.6 | 103.4 |
| 14 | 480 | -6.8 | -50.4 | -63.4 | 105.2 | 50.1 |
| 15 | 790 | -8.7 | -60.2 | 79.0 | 91.7 | 51.7 |
| 16 | 990 | -13.2 | 64.6 | 26.3 | 105.2 | 121.0 |
| 17 | 1555 | -13.9 | 62.2 | 33.4 | 94.0 | 53.9 |
| 18 | 1680 | -13.9 | 55.0 | -1.6 | 91.8 | 61.6 |
| 19 | 2005 | -15.8 | -76.3 | 36.2 | 90.7 | 37.5 |
| 20 | 2045 | -17.1 | 57.4 | 4.5 | 108.2 | 53.1 |
| 21 | 2300 | -16 | 58.1 | 0.2 | 91.7 | 35.8 |
| 22 | 2425 | -15.7 | 67.3 | 13.8 | 91.6 | 52.9 |
| 23 | 2570 | -21.6 | 76.2 | 65.1 | 106.4 | 46.4 |
| 24 | 3160 | -22.8 | -91.4 | 29.3 | 109.5 | 44.7 |

| CASD in [deg] | CASA in [deg] | CZSD in [deg] | CSA in [deg] | XPR in [dB] |
| --- | --- | --- | --- | --- |
| 1.4 | 15.6 | 3.6 | 12.2 | 7 |

## A.1: Option A3: CDL-C Cluster Delay Profile (CDP) after spatial filtering and delay scaling

| CLID | Delay, nsec | Power, dB | AoD, deg | AoA, deg | ZoD, deg | ZoA, deg |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | -4.4 | -38.2 | -96.9 | 96.8 | 98.1 |
| 2 | 225 | -1.2 | -21.8 | 118.9 | 98.5 | 71.1 |
| 3 | 240 | -3.5 | -21.8 | 118.9 | 98.5 | 71.1 |
| 4 | 250 | -5.2 | -21.8 | 118.9 | 98.5 | 71.1 |
| 5 | 235 | -2.5 | -34.2 | -124.4 | 100.9 | 67.6 |
| 6 | 685 | 0 | -5.9 | 171.2 | 99.2 | 76.6 |
| 7 | 690 | -2.2 | -5.9 | 171.2 | 99.2 | 76.6 |
| 8 | 705 | -3.9 | -5.9 | 171.2 | 99.2 | 76.6 |
| 9 | 705 | -7.4 | 44.2 | 51.9 | 106.4 | 62.8 |
| 10 | 850 | -7.1 | -50.6 | 63.4 | 94.5 | 56.6 |
| 11 | 1315 | -5.1 | -44.2 | 65.1 | 104.6 | 103.4 |
| 12 | 1405 | -6.8 | -50.4 | -63.4 | 105.2 | 50.1 |

| CASD in [deg] | CASA in [deg] | CZSD in [deg] | CSA in [deg] | XPR in [dB] |
| --- | --- | --- | --- | --- |
| 1.4 | 15.6 | 3.6 | 12.2 | 7 |

# ANNEX B: Tap Reduction Procedure

The Clustered Delay Line (CDL) profiles are simplified from the TR 38.901 CDL models. The simplification steps are shown below for information. These steps are only used when new delay profiles are created.

Step 0: The CDL profile from TR 38.901 is the starting point.

Step 1: The first modification to the CDL profile is the application of angle spread scaling (ASdesired ) and angle rotation (μϕ,desired) according to the agreed procedure. Compute the updated CASA, ASD, ZSA, ZSD according to the applied angle spread scaling according to the following equation:

CASx=ASxdesiredASxmodelCASx

Step 2: Apply spatial filtering to the CDL profile generated in Step 1 assuming single AAV (subarray or single element) is used at the transmitter and at the receiver, considering also AAS orientation (downtilt, bearing, slant if applicable). This step ensures that the power-based tap reduction will consider the AAV spatial response in the power based tap reduction.

Step 3: Re-normalize cluster powers in the spatially filtered profile generated in Step 2 such that the strongest cluster is at 0dB.

Step 4: Find the clusters in the spatially filtered profile generated in Step 2 with power below [-30] dB, and remove the corresponding cluster index from the CDL profile generated in Step 1.

Step 5: If the number of clusters in the spatially filtered profile after Step 4 is larger than 12, find the cluster with the lowest power, and remove the corresponding cluster index from the CDL profile generated in Step 1. Repeat this step until the number of clusters equals 12.

- Do not consider the spatially filtered profile generated in Steps 2-3 further after this step. The following modifications are to be applied on the remaining clusters in the CDL profile generated in Step 1.

Step 6: Calculate the effective delay spread of the CDL profile generated in Step 1 after truncation, DStruncated (ns), to be used in the next step by computing the root mean squared (rms) delay spread of the remaning clusters, using the following expressions:

τ=iτipiipi

DStruncated=iτi2piipi-τ2

Where:

- τi is the normalized delay value of the i-th cluster in the CDL profile

- pi is the power value of the i-th cluster in the CDL profile

Step 7: Perform delay scaling according to the procedure described below, based on the scaling procedure from clause 7.7.3 in TR 38.901 but considering the impact of truncation using τrmstruncated.

τn, scaled= τn, model ∙DSdesiredDStruncated

Where:

- τn, scaled is the delay value of the i-th cluster in the truncated CDL profile

- DSdesired is the wanted delay spread in ns

Step 8: Apply the quantization to the delay resolution of 5 ns (for FR1) and [TBD] ns for FR2. This is done simply by rounding the tap delays to the nearest multiple of the delay resolution.

- To ensure that there is reduced impact from the use of a regularly spaced grid in the delay position, further improvements to delay quantization (e.g. dithering) can be considered

Step 9: If the delay spread has slightly changed due to the delay quantization, adjust the final delay spread by increasing or decreasing the power of the last cluster so that the delay spread matches the pre-quantization value.

Step 10: Round the amplitude of the last cluster to one decimal (e.g. -8.78 dB  -8.8 dB)

Note to the cluster reduction procedure:

- The resulting truncated CDL Angular and Delay profile should be used according to the procedure described in TR 38.753, including spatial filtering with AAV, Angular Spread Scaling, etc