3GPP TSG RAN WG4 #114bis	R4-2504192

Wuhan, China, April 7 – 11, 2025

Agenda item:		7.13.4

Source:	Nokia

Title:	On TX co-existence spurious emissions requirements

Document for:		Discussion

# Introduction

Work item description [1] was updated in RAN#106 regarding transmitter co-existence spurious emission requirements. This contribution further elaborates Nokia’s views on the topic.

# Discussion

## TX co-existence spurious emissions requirements justification

The WID states the following on the justification for this work:

Liaison Statement (LS) from ETSI MSG TFES [1] was received with a request to re-evaluate co-existence transmitter spurious emission requirement levels for BS. RAN4 have acknowledged the LS in [2]. The current requirement levels were originally determined based on old assumptions summarized in contribution [3] presented at RAN4#113. These assumptions were relevant for UTRA operating at 2 GHz before the concept of BS classes was introduced. The re-evaluation would include new aspects such as new bands at higher frequencies and AAS BS to be considered. Proper and relevant requirement levels are essential to guarantee operation of multiple IMT networks in the same geographical area with reasonable complexity and cost. [1] RP-242432, “Transmitter spurious emissions: co-existence requirement limits for high frequency bands”, ETSI TC MSG/TFES [2] R4-242447, “Reply LS to ETSI MSG TFES on Tx spurious emissions: co-existence requirements limits for high frequency bands”, Huawei [3] R4-2418396, “On the topic of the need to reevaluate requirement levels for BS transmitter co-existence spurious emission for bands in upper region of FR1”, Ericsson

As stated above, there is a clear justification for this work in RAN4. ETSI has sent out an LS with a request to re-evaluate the limits for transmitter co-existence spurious emissions. RAN4 needs to re-evaluate the current requirements for high frequency bands.

## Objectives for TX co-existence spurious emissions requirements

The WID states the following objectives for this work:

Objective of SI or Core part WI or Testing part WI Transmitter co-existence spurious emission requirements - Evaluate co-existence transmitter spurious emission limits according to the request from ETSI/TFES - Improve transmitter co-existence spurious emission requirements if needed. - Note: start the work after March, 2025 and focus on bands in the upper region of FR1 with priority above 5.9 GHz. Objective of Performance part WI Transmitter co-existence spurious emission requirements - Conformance testing corresponding to co-existence spurious emission requirements, if needed.

As described in the objective, the target is to evaluate co-existence transmitter spurious emission limits according to the LS from ETSI/TFES and focus on the frequency bands in the upper region of FR1 with priority above 5.9GHz. The WID mentions the following: “These assumptions were relevant for UTRA operating at 2 GHz before the concept of BS classes was introduced. The re-evaluation would include new aspects such as new bands at higher frequencies and AAS BS to be considered. Proper and relevant requirement levels are essential to guarantee operation of multiple IMT networks in the same geographical area with reasonable complexity and cost.” Taking into consideration the previous text and the WID, the intended focus should be in bands n96 (5925-7125MHz), n102 (5925-6425MHz) and n104 (6425-7125MHz) as priority is with bands above 5.9GHz. The requirements cannot change in the middle of the band so therefore bands that start before 5.9GHz but extend to over 5.9GHz are excluded. And as n96 and n102 are unlicenced bands, n104 should be prioritized.

Requirements cannot change between bands’ starting and ending frequency.

n96 and n102 are unlicensed bands.

Focus on band n104 as first priority.

## Evaluation of TX co-existence spurious emissions requirements

## We feel that there is a need to clarify the definition of minimum coupling loss. In TR 25.942 it is stated: “Minimum Coupling Loss (MCL) is defined as the minimum distance loss including antenna gain measured between antenna connectors;” which would mean that digital beamforming is not taken into consideration. And if Tx and Rx beamforming are not considered, it would mean that minimum coupling loss is only based on free space path-loss.

There is a need to clarify the definition of minimum coupling loss.

## There are options on how to proceed with this work, but we see that it would be most beneficial to proceed with system level simulations. We don’t believe using link budget calculation is a good approach. Different scenarios with specific path-loss equations, slow-fading impact, Tx and Rx beamforming need to be defined. System level simulations need to be run according for different scenarios to get feasible results for the higher FR1 bands. 99th percentile of the CDF could be used as a baseline since it is commonly used in similar evaluations.

Consider using system level simulations for MCL evaluation. Define different scenarios with specific path-loss equations, slow-fading impact, Tx and Rx beamforming etc.

# Conclusion

This document shares Nokia’s views on the Tx co-existence spurious emissions requirements. There were following observations and proposals listed:

- Requirements cannot change between bands starting and ending frequency.

n96 and n102 are unlicensed bands.

There is a need to clarify the definition of minimum coupling loss.

- Focus on band n104 as first priority.

Consider using system level simulations for MCL evaluation. Define different scenarios with specific path-loss equations, slow-fading impact, Tx and Rx beamforming etc.

# References

- RP-243270, “Revised WID: NR base station (BS) RF requirement evolution for FR1/FR2 and testing”, RAN#106, Madrid, Spain 9th – 12th December, 2024.