3GPP TSG RAN WG4 #114bis	R4-2504194

Wuhan, China, April 7 – 11, 2025

Agenda item:		7.23.2.1

Source:	Nokia

Title:	On potentially new BS RF requirements for SBFD operation

Document for:		Approval

# Introduction

A Way Forward was agreed in RAN4#114 [1] regarding evolution of NR duplex operation. There were some agreements made that have impact on the potentially new BS RF requirements for SBFD operation. Also, the open items are listed in the WF. This contribution displays Nokia’s view on the potentially new BS RF requirements for SBFD operation.

# Discussion

## Requirements for transient period between SBFD and non-SBFD

The following was agreed in RAN4#114:

Agreement: - RAN4 reach the conclusion of the location of transient periods: - Case-A: in TA_offset in SBFD symbol(s); - Case-B: in guard symbol(s) reserved before UL non-SBFD symbols - FFS guard symbol(s) in this case is SBFD symbol or not. - Case-C: FFS in guard symbol(s) reserved in non-SBFD DL symbol(s) or blanked SBFD symbols in the beginning of SBFD UL transmission: - RAN4 recognize that it can be dependent on RAN1 discussion for guard period design. - If still no RAN4 agreement can be achieved in RAN4#114bis, RAN4 shall - either send LS to RAN1 to ask for clarification, - or conclude the location of transient period between SBFD and non-SBFD as gNB implementation issue without RAN4 requirement impact. - Case-D: in TA_offset in non-SBFD UL symbol(s). - Note: The illustrative figure for Case A, B, C and D is provided below: ##### Issue 3-1-3: Transient period requirement applicability on SBFD subbands Way Forward: - Text proposal below is taken as the baseline for further discussion, with the understanding that the content can be further updated based on the future agreement on the location of transient period between SBFD and non-SBFD: =================== Start of Text Proposal =================== 6.4.2B Transmitter transient period for SBFD 6.4.2.1B General Transmitter transient period for SBFD requirements apply only to SBFD operation of the BS. The transmitter transient period for SBFD is the time period during which the transmitter is changing from the transmitter OFF period to the transmitter ON period or vice versa. The transmitter transient period is illustrated in figure 6.4.2.1-1. The transmitter transient period for SBFD shall be measured in the transmission bandwidth configuration for SBFD DL subband, UL subband and guardband, depending on different SBFD cases: Table 6.4.2.1B-1: Transmitter transient period for SBFD Frequency regions for transition; BS transmitter behavior during transition. Case-A (SBFD symbol to DL symbol); SBFD UL subband and guardband(s); BS transmitter OFF-to-ON. Case-B (SBFD symbol to UL symbol); SBFD DL subband; BS transmitter ON-to-OFF. Case-C (DL symbol to SBFD symbol); SBFD UL subband and guardband(s); BS transmitter ON-to-OFF. Case-D (UL symbol to SBFD symbol); SBFD DL subband; BS transmitter OFF-to-ON. For BS type 1-H this requirement shall be applied at each TAB connector supporting transmission in the operating band. =================== End of Text Proposal ===================

On case C, there is a FFS on whether the guard symbol(s) should be in in non-SBFD DL symbols or blanked SBFD symbols in the beginning of SBFD UL transmission: while we have some slight preference to place the guard symbol(s) in DL-only symbols at the end of the DL slot, the actual location of the transient period can be left to implementation. The text proposal in the way forward can therefore be agreed as it is as it captures what is necessary regarding the transient period.

On the transmitter transient period for SBFD, adopt the text proposal from the way forward.

## In-channel adjacent subband leakage ratio requirements

The following was agreed in RAN4#114:

Agreement: - New requirement for In-channel adjacent subband leakage ratio: - In-channel adjacent subband leakage ratio requirement is defined with the existing ACLR requirement in absolute PSD limit value as baseline (i.e., Table 6.6.3.2-2 in TS 38.104 for FR1 and Table 9.7.3.3-2 for FR2 in TS 38.104) - FFS the value for absolute PSD limit - FFS the naming of this requirement, by considering the absolute PSD limit used instead of the ratio. Note that the current naming in TS 38.104 is ACLR absolute limit. ##### Issue 3-2-2: In-channel adjacent subband leakage ratio: Configuration Way Forward: - Further study the configuration of the in-channel adjacent subband leakage ratio requirement, by taking the following table as the baseline: BS DL subband bandwidth BWSBFD,DL (MHz); BS UL subband bandwidth BWSBFD,UL (UL); Filter on UL subband and corresponding filter bandwidth; In-Channel Adjacent Subband Leakage Ratio limit. BWSBFD,DL as configured by gNB; BWSBFD,UL as configured by gNB; Square (BWSBFD,UL); TBD.

RAN4 agreed in RAN4#113 that a new requirement for in-channel adjacent subband leakage ratio should be defined. It was also agreed in RAN4#114 that an absolute PSD limit would be used as a baseline. The value for absolute PSD limit was still left for further study. In our view, the existing ACLR absolute basic limit values can be used for in-channel adjacent subband leakage ratio requirements as described in Tables 6.6.3.2-2 and Table 9.7.3.3-2 in TS 38.104:

Table 6.6.3.2-2: Base station ACLR absolute basic limit

| BS category / BS class | ACLR absolute basic limit |
| --- | --- |
| Category A Wide Area BS | -13 dBm/MHz |
| Category B Wide Area BS | -15 dBm/MHz |
| Medium Range BS | -25 dBm/MHz |
| Local Area BS | -32 dBm/MHz |

Table 9.7.3.3-2: BS type 2-O ACLR absolute limit

| BS class | ACLR absolute limit |
| --- | --- |
| Wide area BS | -13 dBm/MHz |
| Medium range BS | -20 dBm/MHz |
| Local area BS | -20 dBm/MHz |

Use the existing ACLR absolute limits as the new in-channel adjacent subband leakage ratio requirements.

As it was agreed in the WF, the naming of the requirement could be different. It is a bit confusing to mention ratio if the actual requirement is an absolute PSD limit. We feel that the simplest way to name the new requirement would be to just remove the ratio from the name and have it as in-channel adjacent subband leakage.

Change the name of the in-channel adjacent subband leakage ratio to in-channel adjacent subband leakage.

## In-channel adjacent subband blocking requirements

The following way forward was agreed in RAN4#114:

Way Forward: - RAN4 continue to study In-channel adjacent subband blocking requirement, by following the agreement achieved in RAN4#113.

RAN4 also agreed in RAN4#113 that a new requirement for in-channel adjacent subband blocking should be defined. In Nokia’s view, this is very important to ensure that blocking requirement is also covered in the presence of co-site inter-sector and inter-site interference. The starting point for the requirement should be without gNB-to-gNB CLI handling schemes as there are many limitations on applying CLI mitigation. Also, as we would like to make sure the performance is guaranteed in all cases, we should define the requirement as close to worst case as possible. This would also mean to select the expected gNB-to-gNB isolation to be the worst case as well. It will be very difficult to agree what is reasonable expectation here. If reasonable compromise is not reached, there may be a need to add a note saying that the performance is achieved with some limitations.

Starting point for the new requirement should be without any gNB-to-gNB CLI handling schemes.

Reasonable expected gNB-to-gNB isolation should be the worst-case scenario to ensure functionality in the field.

# Conclusion

In this contribution, our views on the potentially new BS RF requirements for SBFD operation are presented. The following proposals were listed:

- On the transmitter transient period for SBFD, adopt the text proposal from the way forward.

- Use the existing ACLR absolute limits as the new in-channel adjacent subband leakage ratio requirements.

- Change the name of the in-channel adjacent subband leakage ratio to in-channel adjacent subband leakage.

Starting point for the new requirement should be without any gNB-to-gNB CLI handling schemes.

Reasonable expected gNB-to-gNB isolation should be the worst-case scenario to ensure functionality in the field.

# References

- R4-2502310 “Way Forward for [114][306] NR_duplex_evo_General”, RAN4#114, Athens, Greece, 17th-21st February, 2025