# 3GPP TSG-RAN WG4 Meeting #114bis	R4-2504227

Wuhan, Hubei, China, 07th – 11th April, 2025

Agenda Item:	7.25.2.2

Source: 	Ericsson

Title:	Discussion on RS and channel adaptation in Rel-19 NES

Document for:	Discussion

# Introduction

In last RAN4 meeting, we achieved some agreements as follow[1].

### Sub-topic 1-1 SSB adaptation – Scenario Issue 1-1-1: Scenario clarification based on RAN1 progress Agreement: - Further clarification on the scope of RRM impact for SSB adaption: - No RRM impacts for SSB adaptation requirement for PCell: - Frequency location is same before and after SSB adaptation. - No change to spatial relation (in terms of QCL assumption) for the same SSB index before and after SSB adaptation - Note: Above could be further updated based on further RAN1 progress if any. ### Sub-topic 1-2 SSB adaptation – L1 Requirements impacts Issue 1-2-1: Applicable L1 requirements for SSB adaptation Agreement: - No impact of SSB adaptation: - RLM - BFD - Define requirements if SSB adaption happens: - L1-RSRP for serving cell - L1-SINR - CBD - Whether and how to define requirements at transition is discussed in other issues. Issue 1-2-2: Transition requirements applicability & Issue 1-2-3: Transition requirements – details Agreement: - For L1-RSRP/L1-SINR/CBD - The measurement/evaluation period based on SSB periodicity after adaptation apply from Tfirst_SSB. - Tfirst_SSB is the uncertainty time from the timepoint when SSB adaptation occurs till the first SSB after SSB adaptation. ### Sub-topic 1-3 SSB adaptation – L3 Requirements impacts Issue 1-3-1: L3 requirements for SSB adaptation – general Agreement: Deactivated SCell measurement: - Option 1: For SSB adaption, reuse the same conclusion from OD-SSB case#2 (scenario#4) discussion to define RRM requirements. - At least for the case when the SSB periodicity is adapted to be shorter than before - Option 2: Same as legacy - Other options are not precluded. The requirement can be updated based on further progress in other WG or other aspects are identified that the conclusion can not be directly reused.

After RAN2 #128 meeting, RAN2 sent an LS to RAN4 about the SMTC for SSB adaptation as follow.

- RAN2 preference is to keep SMTC based L3 RRM framework and to introduce additional SMTC configuration according to SSB adaptation for L3 RRM measurement on SCell with SSB adaptation.

- For L3 measurement, RAN2 assumes the adapted SSB on neighbor cell is measured based on legacy SMTC.

In this contribution, we continue to discuss on the Rel-19 NES operation in Obj. 3 from RAN4 perspective.

# Adaptation of common signal/channel transmissions (CS/CC adaptation)

L3 Requirement

In last meeting, an LS related to SSB adaptation in L3 measurement was sent out from RAN2, RAN2 agreed that the adapted SSB on neighbour cell is measured based on legacy SMTC. In other words, UE shall measure neighbour cell following legacy SMTC if it’s an SSB adaptation cell. In our understanding, UE will perform neighbour cell measurement with legacy SMTC in the SSB adaptation SCell’s frequency.

At the same time, RAN2 agreed to introduce additional SMTC for SCell L3 RRM meas with SSB adaptation. However, it’s unclear the additional SMTC is configured per frequency or per cell. If the additional SMTC is configured for the SCC, UE shall follow the new SMTC to perform L3 measurement after SSB adaptation. If the additional SMTC is configured for the SCell only, UE only needs to perform SCell’s measurement based on the additional SMTC, but the neighbour cells in the same frequency can still follow the legacy SMTC. RAN4 can wait RAN2’s further clarification about how to configure the additional SMTC according to the SSB adaptation.

Proposal 1: The L3 neighbour cell measurement follows legacy SMTC requirement.

Proposal 2: RAN4 to wait RAN2’s further clarification on how to configure the additional SMTC for SCell.

- Summary

In this contribution, we have discussed the RS adaptation requirements. Based on the discussions, we made the following proposals and observations:

Proposal 1: The L3 neighbour cell measurement follows legacy SMTC requirement.

Proposal 2: RAN4 to wait RAN2’s further clarification on how to configure the additional SMTC for SCell.

- References

- R4-2500538, “WF on RRM requirements for Netw_Energy_NR_enh_Part2”, Huawei