3GPP TSG-RAN WG4 Meeting #114bis	R4-2504231

Wuhan, China, April 7th – April 11th, 2025

Source:	Nokia

Title:	Testing issues for CSI compression two-sided models

Agenda item:	7.20.2

Document for:	Discussion

Introduction

In this contribution, we discuss the testing issues for CSI compression use case, which is a two-sided models use case.

The discussion on the test and reference encoder/decoder design continued during RAN4#114 ‎[1] meeting and based on the agreements in RAN4#113 ‎[2], RAN4#112bis ‎[3] and RAN4#111 ‎[4] meetings. In this contribution, we provide further results for Option 3 Track 1 following agreements in the previous meeting.

Furthermore, following Option 3 Track 2 agreement from RAN4#114, we also provide results using the new agreed mixed dataset, and using Samsung’s and Nokia’s frozen decoder.

We also provide our view on Option 4 and suggested some further insights and proposed updates on the agreed procedure for Option 4a and 4b.

Finally, we also provide our view on the selection criterion for reference encoder and we also discussed specification aspects of reference encoder.

Discussion

Refinement of Option 3 Track 1

Following agreements were reached in RAN4#114 meeting regarding Option 3 Track 1:

## Sub-topic 1: Refinement of option 3 track 1 Agreements on quantization: Companies shall provide decoders with 2 bit quantization, and with no quantization - No quantization aware training when model is unquantized - Quantization aware training when model is quantized For the model with quantization: Agree on a scaling and quantization codebook - Include Sigmoid in model file in the encoder, and inverse Sigmoid in the decoder - Codebook (1/8, 3/8, 5/8, 7/8) in model file - Do not including mapping to 2 bits. Quantizer function of converting to 2 bits should not be in the model file For the model without quantization: For no quantization models, do not include sigmoid at the model output and no inverse Sigmoid in the decoder Agreements regarding the input / output dimensionality: define Encoder Input and Decoder Output as data dimension (n, 2, 13, 32) with data type float32 where n is the dynamic batch size define Encoder Output and Decoder Input as data dimension (n, 32) with data type float No additional inputs How to continue with track 1 until RAN4#114bis Companies should provide quantized and unquantized models according to the advice above, in particular Samsung, CATT and Huawei. Each company is encouraged to train own encoder using the fixed decoder and their own dataset, and then test with every companies datasets, in order to check for consistency in results

We utilize NOKIA’s dataset containing 504K samples for training, and the testing part in RAN4 #113 mixed dataset for testing, which includes 10K samples per company (70K samples in total). Four reference decoders have been selected for training our encoder: Nokia’s, Samsung’s, CATT’s, and Huawei’s decoders. After training the encoders with each selected decoder, the trained NOKIA encoders and the frozen decoders are evaluated on the test dataset.

During testing, a consistent batch size of 256 is applied to all encoder-decoder pairs. It is important to note that CATT’s reference decoder requires the test data to be evenly divisible by the batch size. Consequently, the actual number of test samples for NOKIA encoder with CATT’s reference decoder is slightly less than 10K (10K//256 * 256 = 9984).

The SGCS results of Option 3 Track 1 are summarized in Table 1. Option 3 Track 1 results aren’t very encouraging. We think the reason may be that the test decoder is trained using a different dataset then the encoder to be tested. One exception in our results is Oppo’s dataset with which Nokia’s encoder is performing sufficiently well in testing phase for at least 3 difference test decoders. However, this may not be generalized since the performances with other companies’ datasets are not consistent. The observed SGCS performance degradation may be attributed to differences in datasets between the companies or potential unobserved variations in the selected encoder architecture.

Table 1. Option 3 Track 1 Performance Summary with Different Frozen Decoders and Testing datasets.

| Encoder | Decoder | Testing dataset | Testing SGCS without quantization | Testing SGCS with quantization |
| --- | --- | --- | --- | --- |
| Nokia (trained with Nokia’s dataset) | Nokia's frozen decoder of Option 3 Track1 | Nokia | 0.772 | 0.720 |
|  |  | Mediatek | 0.651 | 0.585 |
|  |  | OPPO | 0.768 | 0.723 |
|  |  | Vivo | 0.733 | 0.677 |
|  |  | Ericsson | 0.738 | 0.688 |
|  |  | CATT | 0.544 | 0.477 |
|  |  | Qualcomm | 0.532 | 0.487 |
| Nokia (trained with Nokia’s dataset) | Samsung’s frozen decoder | Nokia | 0.755 | 0.707 |
|  |  | Mediatek | 0.661 | 0.579 |
|  |  | OPPO | 0.760 | 0.716 |
|  |  | Vivo | 0.719 | 0.668 |
|  |  | Ericsson | 0.723 | 0.676 |
|  |  | CATT | 0.568 | 0.515 |
|  |  | Qualcomm | 0.536 | 0.501 |
| Nokia (trained with Nokia’s dataset) | CATT’s frozen decoder | Nokia | 0.736 | 0.684 |
|  |  | Mediatek | 0.610 | 0.530 |
|  |  | OPPO | 0.740 | 0.692 |
|  |  | Vivo | 0.704 | 0.647 |
|  |  | Ericsson | 0.721 | 0.668 |
|  |  | CATT | 0.479 | 0.418 |
|  |  | Qualcomm | 0.487 | 0.418 |
| Nokia (trained with Nokia’s dataset) | Huawei’s frozen decoder | Nokia | 0.637 | 0.604 |
|  |  | Mediatek | 0.489 | 0.470 |
|  |  | OPPO | 0.649 | 0.621 |
|  |  | Vivo | 0.584 | 0.554 |
|  |  | Ericsson | 0.605 | 0.570 |
|  |  | CATT | 0.452 | 0.434 |
|  |  | Qualcomm | 0.481 | 0.460 |

Even with further alignment on simulations, the performance of the testing phase in Option 3 Track 1 is not consistent while testing is done using different companies’ datasets.

The SGCS values obtained in Option 3 Track 1 are not acceptable for several tests with different companies’ datasets, particularly with testing datasets from CATT and Qualcomm.

Based on our results, Option 3 Track 1 does not look like a feasible option. Therefore, RAN4 should down prioritize Option 3 Track 1.

Refinement of Option 3 Track 2

Following agreements were reached in RAN4#114 meeting regarding Option 3 Track 2:

## Sub-topic 2: Refinement of option 3 track 2 Dataset agreement: Take mixed dataset comprising Mediatek, Ericsson, Vivo, Oppo, Nokia How to continue with track 2 until RAN4#114bis: The mixed dataset will be revised such that it comprises data from the 5 above companies and uploaded to the folder for RAN4#114. Companies requested to provide new models based on this mixed dataset together with SGCS within 2 weeks The model with median SGCS will be selected for the frozen decoder. If an even number, choose the model below the median Once the frozen decoder is selected, companies are invited to train their own encoders using the mixed dataset or/and private dataset and test using the mixed test dataset

Nokia's quantized encoder model was trained with a mixed dataset in conjunction with Samsung's frozen quantized decoder model from Option 3 Track 2. Following training, this encoder model, paired with Samsung's frozen quantized decoder, was evaluated using the entire mixed dataset (500k samples) and 70K samples from Nokia’s dataset.

Similarly, Nokia's unquantized encoder model underwent training using the mixed dataset alongside Samsung's frozen unquantized decoder model from Option 3 Track 2. Post training, this encoder model and Samsung's frozen unquantized decoder were evaluated using the entire mixed dataset (500k samples) and 70K samples from Nokia's dataset. These procedures were also repeated with Nokia’s frozen decoder model, adhering to the architectural agreements. The results are documented in Table 2.

Table 2. Option 3 Track 2 Performance Summary with Different Frozen Decoders and Testing datasets.

| Decoder | Without quantization |  |  | With quantization |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Training SGCS | Testing SGCS on mixed Dataset | Testing SGCS on Nokia’s Dataset | Training SGCS | Testing SGCS on mixed Dataset | Testing SGCS on Nokia’s Dataset |
| Samsung’s frozen decoder | 0.768 | 0.7544 | 0.7642 | 0.7142 | 0.7132 | 0.7141 |
| Nokia’s frozen decoder | 0.778 | 0.7538 | 0.7536 | 0.7114 | 0.7089 | 0.7101 |

The SGCS results obtained in Option 3 Track 2 show a similar performance for the two selected decoders (Samsung’s and Nokia’s decoders trained on mixed dataset) when tested using own Nokia’s dataset or the mixed dataset.

Compared to the SCGS performance from Option 3 Track 1, it is evident that Option 3 Track 2 results are more consistent among different companies’ models.

Since the performance when using a mixed dataset for the training of both encoder and decoder is better than when using different datasets, RAN4 should pursue Track 2 over Track 1 for the feasibility study of test decoder Option 3.

In the conclusion of the feasibility study of Option 3, and based on our results, we suggest selecting Nokia’s decoder from Track 2 as the test decoder together with the agreed mixed dataset

Refinement of Option 4

Option 4a

The Option 4a feasibility procedure was updated in RAN4#114 meeting as follows:

Option 4a (Dataset based) for 2-sided model - Step 1-3: Reuse results of Option 3 - Step 4: Select one or more Eigenvalue dataset(s) for further analysis based on option 3 - Selection criteria: select the dataset(s) generated from the selected encoder and decoder pair(s) from Option 3 track 1 - Or mixed dataset from track 2 - If multiple datasets are selected (as was the case of Option 3 track 1), the subsequent steps of the feasibility procedure will be applied on each one of the selected datasets separately - Step 5: Label selected dataset with encoder input/output using encoder corresponding to the selected test decoder from option 3 - Step 6: Companies bring results for training of “own encoder(s) and decoder(s)” with selected dataset(s) - Performance alignment to be checked/discussed - Step 7: Conclude on overall feasibility of Option 4a - feasibility criteria to be discussed (e.g. Perform testing of all the UE encoders using all test decoders (replicating the process of testing UE’s against RAN4 requirements with different TE vendor decoders, others given in R4-2415376, etc.). - Dataset for feasibility evaluation to be discussed; may be common test dataset or own test dataset

This procedure is illustrated in Figure 1, where the figure has been composed from the viewpoint of utilizing Option 4a in practice.  The figure is annotated with references to the steps in the above feasibility study procedure.  Since Option 4a assumes a standardized dataset is used for creating the test decoder, Steps 1-4 of this procedure are shown as 3GPP development with the final product being the labelled dataset.  The labelled dataset includes both the input CSI (eigenvectors) and the compressed CSI (the encoder output).  This dataset is used both by the UE vendor to train its encoder and by the test equipment (TE) vendor to train its test decoder.  Under the assumptions of Option 4a in practice, the UE and TE vendors do not know the architecture or parameters of the model being developed by the other vendor.  They also do not know the training process being used by the other vendor.  For example, if the UE vendor decides to train a decoder before training its encoder, the TE vendor will not know this.  The final step is the testing process, where the encoder can be tested by the decoder of any TE vendor.

Figure 1.  An illustration of the use of Option 4a in practice

During offline discussions, the moderator made the following proposal regarding the above feasibility procedure:

- For option 4a:

  - Take Samsung encoder and use it to “label” the training dataset. Train a new decoder using the labelled dataset. Then train an “own” encoder with the newly trained decoder

  - Then all companies share their “new” decoders that have been trained with the Samsung labelled dataset

  - Test “own” encoder vs. All other companies decoders from step 2. (The other companies decoders here represent different TE vendors...)

In this proposal, the Samsung encoder is from Option 3 (Steps 1-3) and the dataset labelling occurs in Step 5 after selection in Step 4.  Since the decoder which is trained in the first sub-bullet of the moderator’s proposal is also used to train the encoder, this decoder must be created by the UE vendor and cannot be known by the TE vendor.  However, since this decoder is also shared as indicated in the second sub-bullet and then used for performance evaluation, it must also be the test decoder trained by the TE vendor.  Thus, the decoder is functioning both as a decoder trained by the UE vendor and as the test decoder trained by the TE vendor, which contradicts how Option 4a would work in practice.  Since there are multiple companies training encoders and decoders as part of this feasibility study, let us accept the dual role of this decoder to determine the consequences for the feasibility study.

In the final sub-bullet, the encoders are tested against the shared test decoders.  The goal of this study is to determine if allowing TE vendors to train their own test decoders can produce consistent test results.  In other words, an encoder should be able to be tested by any test decoder and consistently pass or fail the test no matter which decoder is used in the test.  This can be judged by observing the spread of the measured performance obtained by testing a single encoder against all test decoders.  We do this for multiple encoders to better represent variation in the encoder design as well.  For any encoder tested using the moderator’s proposal, there is always one decoder which does not represent reality – the decoder trained by the same company.  This decoder could produce an outlier in performance since it is closely related to the encoder under test.  If we exclude the performance of each encoder with its own decoder, then no encoder is tested with the same set of test decoders.  If we include that performance, then the spread of the results may not represent what would occur in practice.  We can either accept this situation in the feasibility study or adjust the procedure to avoid it by separating the training of a decoder for the purpose of training the encoder at the UE vendor from the training of a decoder at the TE vendor.

Using a decoder trained for the purpose of training the UE vendor’s encoder as the test decoder violates the separation of knowledge between the UE and TE vendors.  This creates potential problems in the determination of feasibility of Option 4a.

Clarify in Step 6 of the Option 4a feasibility procedure that the training of the “own decoder(s)” for the purpose of using it as a test decoder must be separated from the training of the “own encoder(s)” since the actions of different vendors (TE vendors and UE vendors, respectively) are being mimicked in this step.

- Step 6: Companies bring results for independent training of “own encoder(s) and decoder(s)” with the selected dataset(s)

  - A decoder may be trained to support the training of the own encoder, but this decoder is distinct from the TE decoder used for performance assessment of Option 4a.

  - Performance alignment to be checked/discussed

Note that the above proposal is not meant to prevent the training of a decoder which is used to train the “own encoder” for the UE vendor, but to prevent that decoder from being used as a TE vendor test decoder due to the separation of knowledge assumed between the vendors.

Option 4b

Option 4b procedure was updated in RAN4#114 meeting as follows:

Option 4b (encoder based) for 2-sided model - Step 1-3: Reuse results of Option 3 - Step 4: Select one or more encoder(s) for further analysis based on option 3 - Selection criteria: select the encoder(s) from the selected encoder and decoder pair(s) of Option 3 track 1 - Or encoder(s) from the selected encoder and decoder pair(s) of Option 3 track 2 - Step 5: Company brings results for training of “own decoder(s)” with selected encoder(s) - Performance alignment to be checked/discussed - FFS what is the dataset assumed for the training - Step 6: Company trains “own encoder(s)” with “own decoder(s)” from step 5 - FFS what is the dataset assumed for the training - Step 7: Conclude on overall feasibility of Option 4b - Feasibility criteria to be discussed (e.g. Perform testing of all the UE encoders using all test decoders (replicating the process of testing UE’s against RAN4 requirements with different TE vendor decoders, others given in R4-2415376, etc.). - Dataset for feasibility evaluation to be discussed; may be common test dataset or own test dataset Encoder in step 5 should at least have the agreed structure. Companies can also bring analysis/results with other encoders (using different structures)

As we did with Option 4a, the above procedure for Option 4b is illustrated in Figure 2, where the figure has been composed from the viewpoint of utilizing Option 4b in practice.  The figure is annotated with references to the steps in the above feasibility study procedure.  Since Option 4b assumes partial specification via the reference encoder, Steps 1-4 of this procedure are shown as 3GPP development of the reference encoder, and the reference encoder is passed forward to both the UE and TE vendors.  This reference encoder allows both vendors to generate their own datasets for training a UE encoder and the TE decoder.  As with Option 4a, the UE and TE vendors do not know the architecture or parameters of the model being developed by the other vendor.  It is also possible for the UE vendor to train a decoder before training its encoder, but the TE vendor will not know this.  Again, the final step is the testing process, where the encoder can be tested by the decoder of any TE vendor.

The key idea being tested by this feasibility study is the ability of the UE and TE vendors to independently train interoperable models based on the information provided through the standard.  In this case, that information is the reference encoder.  If we look closely at the Option 4b procedure, we see that strictly speaking the UE and TE vendors are not operating independently.  The procedure indicates that an “own decoder” should be trained in Step 5, and that this decoder is used to train an “own encoder” in Step 6.  To maintain independence, the “own decoder” in Step 6 must be the UE-side decoder used to assist the training of the UE-side encoder.  However, then there is no test decoder in the procedure since the test decoder cannot be used to train the UE-side encoder as the TE vendor does not have knowledge of the UE-side models.  In Option 4b procedure, the test decoder in Step 6 is assumed to come from the training in Step 5, which means that the UE encoder and TE decoder are not being trained independently.

Use of the decoder in Step 5 of the Option 4b feasibility study procedure as both the TE test decoder and for training the UE encoder violates the assumption of independent training of the test decoder and UE encoder.

This is the same issue we encountered in Option 4a with the same alternative solutions and consequences:  We can ignore the violation of UE vendor/TE vendor training independence and realize the effect on the final performance comparison, or we can adjust the procedure at this stage.  The solution we select has a bearing on the resolution of the FFS items in Steps 5 and 6 on the datasets assumed in the training.  As with Option 4a, here we also prefer to adjust the feasibility study procedure to maintain the independence of the training of the UE encoder and TE decoder.  The solution is to adjust Step 6 to only reflect the training of the UE encoder without the link to the decoder in Step 5.  It is still possible to train a decoder to aid in the training of the UE encoder in Step 6, but this decoder is not the TE decoder used in Step 5.  The decoders will be distinguished by the datasets used in their training.  Therefore, the dataset used in Step 5 must be different from the dataset used in Step 6.  In practice, the datasets would be determined individually by the UE vendor (in Step 6) and the TE vendor (in Step 5).  Interoperability is maintained through the use of the reference encoder to label each dataset.

The dataset used in Step 5 of the feasibility study procedure for Option 4b is the company’s own dataset.

Adjust Step 6 in the feasibility study procedure for Option 4b to:

- Step 6: Company trains “own encoder(s)”

  - A decoder may be trained to support the training of the own encoder, but this decoder is distinct from the decoder in Step 5.

  - The dataset used for the training is the company’s own dataset, but a dataset different from the one used in Step 5.

Figure 2.  An illustration of the use of Option 4b in practice

Option 4 results

Table 3 summarizes the training and testing we did for Option 4. We choose Samsung (SAM) and NOKIA (NOK) uploaded reference encoders from Option 3 Track 1 (T1) and Track 2 (T2). The bolded rows with mixed training dataset and “SAM T2 frozen Enc” used to create labelled dataset match the aforementioned procedure for Option 4a. Then in that row, the “OWN ENC” and “OWN DEC” are the trained encoder and decoder at the UE vendors. The “Ref Dec” is the decoder corresponding to the reference encoder, “SAM T2 Ref Dec” in this case. If we want to choose NOKIA’s reference encoder to create the labelled dataset, we can also use the row in Table 3 with mixed training dataset and “NOK T2 frozen Enc”.

Table 3. Option 4 Performance Summary with Training on our own (NOKIA) dataset and the Mixed dataset, with different frozen reference encoders from Option 3 Track 1 and 2.

| Training |  | Testing Enc + Dec |  | Testing SGCS without quantization | Testing SGCS with quantization |
| --- | --- | --- | --- | --- | --- |
| Dataset | Ref Encoder | Testing Enc | Testing Dec | Testing on the corresponding Train Dataset |  |
| Nokia | SAM T2 frozen Enc (Option 4b) | Own Enc | Ref Dec | 0.710 | 0.701 |
|  |  | Ref Enc | Own Dec | 0.713 | 0.691 |
|  | SAM T1 frozen Enc (Option 4b) | Own Enc | Ref Dec | 0.737 | 0.698 |
|  |  | Ref Enc | Own Dec | 0.745 | 0.689 |
|  | NOK T2 frozen Enc (Option 4b) | Own Enc | Ref Dec | 0.758 | 0.699 |
|  |  | Ref Enc | Own Dec | 0.759 | 0.689 |
|  | NOK T1 frozen Enc (Option 4a) | Own Enc | Ref Dec | 0.753 | 0.705 |
|  |  | Ref Enc | Own Dec | 0.753 | 0.694 |
| Mixed | SAM T2 frozen Enc (Option 4a) | Own Enc | Ref Dec | 0.747 | 0.688 |
|  |  | Ref Enc | Own Dec | 0.739 | 0.686 |
|  | SAM T1 frozen Enc (Option 4b) | Own Enc | Ref Dec | 0.720 | 0.665 |
|  |  | Ref Enc | Own Dec | 0.744 | 0.677 |
|  | NOK T2 frozen Enc (Option 4a) | Own Enc | Ref Dec | 0.741 | 0.696 |
|  |  | Ref Enc | Own Dec | 0.747 | 0.683 |
|  | NOK T1 frozen Enc (Option 4b) | Own Enc | Ref Dec | 0.726 | 0.673 |
|  |  | Ref Enc | Own Dec | 0.738 | 0.682 |

For Option 4b, according to our description above, the UE vendor and TE vendor should choose different datasets to train their encoder and test decoder with the same reference encoder, respectively. Assuming the mixed dataset is used at the UE vendor, we can choose the reference encoders from Option 3 Track1. Then the “Own Enc” and “Own Dec” can be treated as the UE vendor trained encoder and decoder, as indicated in Table 3. Then the TE vendor should choose another dataset to train its test decoder with the reference encoder, and the dataset chosen by the TE is better to be different from the mixed dataset and the dataset used to train the reference encoder. Similarly, if we choose NOKIA dataset as the UE vendor dataset, any reference encoder other than NOKIA T1 encoder can be used as the reference encoder for Option4b.

What would be standardized for Option 3

Agreement: To verify the performance of CSI compression test decoder for calibration of test equipment, agree to define the reference encoder including encoder structure and parameters in RAN4 - FFS on whether to specify it in TS or TR, and capture the conclusion in the TR conclusion part - FFS on criterion for selection of reference encoder - FFS on decision whether RAN4 or RAN5 will specify it Way forward: There was also a discussion on whether to capture a training dataset, but no agreements. Some open questions from the discussion (not agreements): - Whether it is useful to capture a training dataset - Where and how the training dataset would be captured (TR, TS, some other public place…)

Criterion for Selection of reference encoder:

In our view, the selection criterion would depend which option is finally agreed in RAN4.

For Option 3, the corresponding encoder to the selected test decoder should be the candidate for reference encoder.

For Option 4b, encoder based approach, the selected encoder (from Option 3) should be the candidate for reference encoder.

For Option 4a, RAN4 needs to further discuss the selection criterion for reference encoder.

Specification of reference encoder:

In our view, reference encoder would be required not only for test decoder verification but potentially for the definition of RAN4 requirements. Therefore, it should be captured in RAN4 specifications.

To verify test decoder and to define performance requirements, RAN4 needs to specify reference encoder. It should be captured in the TR conclusion as well as in the TS.

If RAN1 also specifies a reference encoder, there should be alignment between RAN1 and RAN4 reference encoder.

Conclusion

In this paper, we share our further views and simulation results on the two-sided CSI compression use case. Specifically, we cover the following aspects:

- Derivation of test decoder and encoder with Option 3

- Derivation of test decoder and encoder with Option 4

- Standardization of reference encoder

In the paper, the following Observations and Proposals were made:

Observation 1: Even with further alignment on simulations, the performance of the testing phase in Option 3 Track 1 is not consistent while testing is done using different companies’ datasets.

Observation 2: The SGCS values obtained in Option 3 Track 1 are not acceptable for several tests with different companies’ datasets, particularly with testing datasets from CATT and Qualcomm.

Proposal 1: Based on our results, Option 3 Track 1 does not look like a feasible option. Therefore, RAN4 should down prioritize Option 3 Track 1.

Observation 3: Compared to the SCGS performance from Option 3 Track 1, it is evident that Option 3 Track 2 results are more consistent among different companies’ models.

Proposal 2: Since the performance when using a mixed dataset for the training of both encoder and decoder is better than when using different datasets, RAN4 should pursue Track 2 over Track 1 for the feasibility study of test decoder Option 3.

Proposal 3: In the conclusion of the feasibility study of Option 3, and based on our results, we suggest selecting Nokia’s decoder from Track 2 as the test decoder together with the agreed mixed dataset

Observation 4: Using a decoder trained for the purpose of training the UE vendor’s encoder as the test decoder violates the separation of knowledge between the UE and TE vendors.  This creates potential problems in the determination of feasibility of Option 4a.

Proposal 4: Clarify in Step 6 of the Option 4a feasibility procedure that the training of the “own decoder(s)” for the purpose of using it as a test decoder must be separated from the training of the “own encoder(s)” since the actions of different vendors (TE vendors and UE vendors, respectively) are being mimicked in this step.

- Step 6: Companies bring results for independent training of “own encoder(s) and decoder(s)” with the selected dataset(s)

  - A decoder may be trained to support the training of the own encoder, but this decoder is distinct from the TE decoder used for performance assessment of Option 4a.

  - Performance alignment to be checked/discussed

Observation 5: Use of the decoder in Step 5 of the Option 4b feasibility study procedure as both the TE test decoder and for training the UE encoder violates the assumption of independent training of the test decoder and UE encoder.

Proposal 6: Adjust Step 6 in the feasibility study procedure for Option 4b to:

- Step 6: Company trains “own encoder(s)”

  - A decoder may be trained to support the training of the own encoder, but this decoder is distinct from the decoder in Step 5.

  - The dataset used for the training is the company’s own dataset, but a dataset different from the one used in Step 5.

Proposal 7: For Option 3, the corresponding encoder to the selected test decoder should be the candidate for reference encoder.

Proposal 8: For Option 4b, encoder based approach, the selected encoder (from Option 3) should be the candidate for reference encoder.

Proposal 9: For Option 4a, RAN4 needs to further discuss the selection criterion for reference encoder.

Proposal 10: To verify test decoder and to define performance requirements, RAN4 needs to specify reference encoder. It should be captured in the TR conclusion as well as in the TS.

Proposal 11: If RAN1 also specifies a reference encoder, there should be alignment between RAN1 and RAN4 reference encoder.

- References

- R4-2503953, “WF on AI/ML air interface study”, Ericsson, RAN4#114 meeting, 17th – 24th February 2025, Athens, Greece.

- R4-2420513, “WF on AI/ML air interface part 1”, Qualcomm Incorporated, RAN4#113 meeting, 18th – 22nd November 2024, Orlando, USA.

- R4-2417212, “WF on requirements for AI/ML air interface”, Qualcomm Incorporated, RAN4#112bis meeting, 14th – 18th October 2024, China.

- R4-2410570, WF on AI/ML, RAN4#111, Fukuoka city, Japan, May 20th – 24th, 2024.