3GPP TSG-RAN4 Meeting #114bis	R4-2504241

# Wuhan, China, 7th – 11th April, 2025

Source:	LG Electronics

Title: 	HPUE MPR for intra-band UL CA

Agenda Item:	7.1.1.1.2

Document for:	Discussion and Approval

# Introduction

RAN4 has been discussing requirements for PC1.5 intra-band contiguous and non-contiguous UL CA. Way forward [1] was agreed in RAN WG4 Meeting #114 in Athens in February 2025.

This WF captures following agreement for ULCA with contiguous RB allocation.

## 1.1 MPR values for intra-band contiguous ULCA with contiguous RB allocation (with TxD)

Agreement:

- Define separate MPR/A-MPR requirements for handheld and FWA

- Companies are encouraged to bring A-MPR values in following meetings

- The MPR values of intra-band contiguous ULCA PC1.5 are supposed to be higher than single CC PC1.5, as well as intra-band contiguous ULCA PC2

- Having the following PC1.5(2Tx) MPR values for equal PSD as starting point to be further checked in upcoming meetings

  - Companies can further evaluate the unequal PSD (6dB) impacts in next meeting.

  - The MPR values for PC3(1Tx) and PC2(2Tx) listed in the below table is just for reference

| Modulation |  | PC3 (1Tx) |  | PC2 (2Tx) |  | PC1.5 (2Tx) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Handheld |  | Handheld |  | Handheld |  | FWA1 |  |
|  |  | inner | outer | inner | outer | inner | outer | inner | outer |
| DFT-s-OFDM | PI/2 BPSK | 2.5 | 7 | 3.5 | 8 | [4.5] | [9] | [4] | [8.5] |
|  | QPSK | 2.5 | 7 | 3.5 | 8 | [4.5] | [9] | [4] | [8.5] |
|  | 16QAM | 2.5 | 7 | 3.5 | 8 | [4.5] | [9] | [4] | [8.5] |
|  | 64QAM | 5 | 7 | 6 | 8 | [7] | [9] | [6.5] | [8.5] |
|  | 256QAM | 7 | 7.5 | 8 | 8.5 | [9] | [9.5] | [8.5] | [9] |
| CP-OFDM | QPSK | 3.5 | 8 | 4 | 8.5 | [5-5.5] | [9.5] | [4.5-5] | [9] |
|  | 16QAM | 3.5 | 8 | 4 | 8.5 | [5-5.5] | [9.5] | [4.5-5] | [9] |
|  | 64QAM | 5 | 8 | 5.5 | 8.5 | [6-6.5] | [9.5] | [5.5-6] | [9] |
|  | 256QAM | 7 | 8 | 7.5 | 8.5 | [8.5] | [9.5] | [8.5] | [9] |
|  |  |  |  |  |  |  |  | NOTE 1: This table is targeted to large FWA form factor with 20 dB or above antenna isolation. |  |

# Discussion

The agreement in [1] indicates that about 1dB higher MPR is needed for Handheld PC1.5 UE vs. Handheld PC2 UE and these values are highlighted in the table below. Similar about 1dB delta between PC2 and PC1.5 has been indicated also in [2], where delta MPR to already specified power class was estimated based on the OIP3 derived from existing transmitter requirements and generally known IM3 equations. The theoretical analysis in [2] was done for NCCA against the MPR to meet -13dBm and -30dBm emission mask as this scenario matches fully with the two-tone formulas and is easy to explain, but similar relative difference applies also for other cases where the UE performance is limited by the transmitter third order non-linearity.

Observation 1: The relative MPR delta between the PC1.5 and PC2 UE is about 1dB both 1.) based on the company views for intra-band contiguous ULCA with contiguous RB allocation (with TxD) and 2.) based on the theoretical analysis.

| Modulation |  | PC3 (1Tx) |  | PC2 (2Tx) |  | PC1.5 (2Tx) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Handheld |  | Handheld |  | Handheld |  | FWA1 |  |
|  |  | inner | outer | inner | outer | inner | outer | inner | outer |
| DFT-s-OFDM | PI/2 BPSK | 2.5 | 7 | 3.5 | 8 | [4.5] | [9] | [4] | [8.5] |
|  | QPSK | 2.5 | 7 | 3.5 | 8 | [4.5] | [9] | [4] | [8.5] |
|  | 16QAM | 2.5 | 7 | 3.5 | 8 | [4.5] | [9] | [4] | [8.5] |
|  | 64QAM | 5 | 7 | 6 | 8 | [7] | [9] | [6.5] | [8.5] |
|  | 256QAM | 7 | 7.5 | 8 | 8.5 | [9] | [9.5] | [8.5] | [9] |
| CP-OFDM | QPSK | 3.5 | 8 | 4 | 8.5 | [5-5.5] | [9.5] | [4.5-5] | [9] |
|  | 16QAM | 3.5 | 8 | 4 | 8.5 | [5-5.5] | [9.5] | [4.5-5] | [9] |
|  | 64QAM | 5 | 8 | 5.5 | 8.5 | [6-6.5] | [9.5] | [5.5-6] | [9] |
|  | 256QAM | 7 | 8 | 7.5 | 8.5 | [8.5] | [9.5] | [8.5] | [9] |
|  |  |  |  |  |  |  |  | NOTE 1: This table is targeted to large FWA form factor with 20 dB or above antenna isolation. |  |

At the same time as RAN4 is defining the new requirements for PC1.5 UL CA UE there’s discussion ongoing on simplification of the specification where MPR requirements are one of the areas where improvements would be welcome. We therefore propose to define the PC1.5 UL CA UE MPR requirements, when possible (as it is for Handheld devices), as a delta requirement to PC2 (2TX) requirement. We consider these PC2 (2TX) requirements to be mature enough to serve as solid base and a delta requirement will guarantee that the PC1.5 (2TX) requirements will be consistent and aligned with the earlier work and ensure that the targeted system level performance improvements are guaranteed.

For PC1.5 (2TX) FWA device there’s no proper reference existing, like another power class 2TX FWA UE. Therefore the delta approach to another FWA device cannot be used, but on the other hand, the MPR is still driven by the transmitter third order non-linearity, which is a function of the transmitter OIP3 and hence another delta value to other device type should also work.

Observation 2: For PC1.5 (2TX) FWA device there’s no proper reference existing, like another power class 2TX FWA UE. However, MPR is still driven by the transmitter third order non-linearity, which is a function of the transmitter OIP3 and hence another delta value to other device type should also work.

Proposal 1: It is proposed to define the PC1.5 UL CA UE MPR requirements, when possible (as it is for Handheld devices), as a delta requirement to PC2 (2TX) requirement.

Proposal 2: It is proposed to use the 1dB as MPR delta value between PC1.5 2TX Handheld and PC2 2TX Handheld UEs for both intra-band C CA and intra-band NC CA.

Proposal 3:.It is proposed to use another delta MPR value, like [-0.5]dB between PC1.5 2TX FWA and PC1.5 2TX Handheld UEs for intra-band C CA.

Proposal 4:.ANNEX A proposes a way on how the delta MPR could be defined in the 38.101-1 [3] specification for Handheld UE using the MPR to meet -30dBm/MHz and -13dBm/MHz requirements as an example, where the new requirement consists of the reference to existing requirement and delta MPR value. For other device types, like FWA, another delta MPR value can be added.

For PC1.5 intra-band NC CA of (2TX) FWA with indicating dualPA, based on [4],

Proposal 5: For PC1.5 intra-band NC CA of (2Tx) FWA, it is proposed to define MPR in Table 2-1 to meet -13dBm/MHz and -30dBm/MHz respectively.

Table 2-1 : PC1.5 (2Tx) FWA MPR for intra-band NC CA with 2x26dBm+2LO

| B | MPR (dB) |  |
| --- | --- | --- |
|  | -13dBm/MHz | -30dBm/MHz |
| 0 ≤ B <0.54 | ≤ 8.5 | ≤ 9.0 |
| 0.54 ≤ B < 1.08 | ≤ 8.0 | ≤ 9.0 |
| 1.08 ≤ B < 2.16 | ≤ 8.0 | ≤ 9.0 |
| 2.16 ≤ B < 3.24 | ≤ 7.5 | ≤ 9.0 |
| 3.24 ≤ B < 5.4 | ≤ 7.5 | ≤ 9.0 |
| 5.4 ≤ B ≤ 10.8 | ≤ 7.0 | ≤ 9.0 |
| 10.8 < B | ≤ 7.0 | ≤ 9.0 |

Proposal 6: For PC1.5 intra-band non-contiguous UL CA, it is proposed to consider up to 6dB PSD imbalance in configured transmitted power.

- 6 dB ≤ { PCMAX,c1 – 10*log10(LCRB1*SCS1)} –  { PCMAX,c2 – 10*log10(LCRB2*SCS2)} ≤ 6 dB

The upper bound of  PCMAX,C1 and PCMAX,C2 is 26dBm.

# Conclusions

This document discusses the MPR requirements for PC1.5 intra-band contiguous and non-contiguous UL CA for Handheld and FWA devices.

Observation 1: The relative MPR delta between the PC1.5 and PC2 UE is about 1dB both 1.) based on the company views for intra-band contiguous ULCA with contiguous RB allocation (with TxD) and 2.) based on the theoretical analysis.

Observation 2: For PC1.5 (2TX) FWA device there’s no proper reference existing, like another power class 2TX FWA UE. However, MPR is still driven by the transmitter third order non-linearity, which is a function of the transmitter OIP3 and hence another delta value to other device type should also work.

Proposal 1: It is proposed to define the PC1.5 UL CA UE MPR requirements, when possible (as it is for Handheld devices), as a delta requirement to PC2 (2TX) requirement.

Proposal 2: It is proposed to use the 1dB as MPR delta value between PC1.5 2TX Handheld and PC2 2TX Handheld UEs.

Proposal 3:.It is proposed to use another delta MPR value, like [0.5]dB between PC1.5 2TX Handheld and PC2 2TX Handheld UEs.

Proposal 4:.ANNEX A proposes a way on how the delta MPR could be defined in the 38.101-1 [3] specification for Handheld UE using the MPR to meet -30dBm/MHz and -13dBm/MHz requirements as an example, where the new requirement consists of the reference to existing requirement and delta MPR value. For other device types, like FWA, another delta MPR value can be added.

Proposal 5: For PC1.5 intra-band NC CA of (2Tx) FWA, it is proposed to define MPR in Table 2-1 to meet -13dBm/MHz and -30dBm/MHz respectively.

Table 2-1 : PC1.5 (2Tx) FWA MPR for intra-band NC CA with 2x26dBm+2LO

| B | MPR (dB) |  |
| --- | --- | --- |
|  | -13dBm/MHz | -30dBm/MHz |
| 0 ≤ B <0.54 | ≤ 8.5 | ≤ 9.0 |
| 0.54 ≤ B < 1.08 | ≤ 8.0 | ≤ 9.0 |
| 1.08 ≤ B < 2.16 | ≤ 8.0 | ≤ 9.0 |
| 2.16 ≤ B < 3.24 | ≤ 7.5 | ≤ 9.0 |
| 3.24 ≤ B < 5.4 | ≤ 7.5 | ≤ 9.0 |
| 5.4 ≤ B ≤ 10.8 | ≤ 7.0 | ≤ 9.0 |
| 10.8 < B | ≤ 7.0 | ≤ 9.0 |

Proposal 6: For PC1.5 intra-band non-contiguous UL CA, it is proposed to consider up to 6dB PSD imbalance in configured transmitted power.

- 6 dB ≤ { PCMAX,c1 – 10*log10(LCRB1*SCS1)} –  { PCMAX,c2 – 10*log10(LCRB2*SCS2)} ≤ 6 dB

The upper bound of  PCMAX,C1 and PCMAX,C2 is 26dBm.

# References

[1] R4-2502863, WF on TN HPUE, Samsung, Skyworks, OPPO, LGE, Apple, TSG-RAN WG4 Meeting #114, Feb 2025

[2] R4-2412024, HPUE for intra-band UL CA, LGE, TSG-RAN4 Meeting #112, Aug 2024

[3] 3GPP TS 38.101-1, User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone (Release 18)

[4] R4-2502091, HPUE for intra-band UL CA, LGE, TSG-RAN4 Meeting #114, Feb 2025

# Annex A: Draft CR

< START OF CHANGE #1 >

#### 6.2A.2.2.1	MPR to meet -30dBm/MHz

##### 6.2A.2.2.1.1		PC3 with indicating dualPA-Architecture supported

MPR in this clause is for intra-band non-contiguous CA power class 3 for UEs indicating IE dualPA-Architecture supported. The allowed maximum output power reduction is defined as:

MPR=MAWhere MA is defined as follows

MA = 	15; 	0 ≤ B < 1.08

14.5; 	1.08 ≤ B < 2.16

13.5; 	2.16 ≤ B < 3.24

12.5;       3.24 ≤ B < 5.04

11.5; 	5.04≤ B < 10.08

10.5; 	10.08 ≤ B < 16.38

10;        16.38 ≤ B < 21.78

9; 	      21.78 ≤ B

##### 6.2A.2.2.1.2		PC2 with indicating dualPA-Architecture supported

MPR in this clause is for intra-band non-contiguous CA power class 2 for UEs indicating IE dualPA-Architecture supported. The allowed maximum output power reduction is defined as:

MPR=MAWhere MA is defined as follows

MA = 	15.5; 	0 ≤ B < 1.44

15.0; 	1.44 ≤ B < 2.88

14.0;    2.88 ≤ B < 5.76

12.0; 	5.76 ≤ B < 10.8

10.5; 	10.8 ≤ B < 23.04

9.0; 	 23.04 ≤ B

##### 6.2A.2.2.1.3		PC3 without indicating dualPA-Architecture supported

MPR in this clause is for intra-band non-contiguous CA power class 3 for UEs without indicating IE dualPA-Architecture supported. The allowed maximum output power reduction is defined as:

MPR=MAWhere MA is defined as follows

MA = 	17.5; 	0 ≤ B < 1.08

17.0; 	1.08 ≤ B < 2.16

16.5; 	2.16 ≤ B < 3.24

16;      3.24 ≤ B < 5.04

15; 	5.04≤ B < 10.08

14.5; 	10.08 ≤ B < 36

10;      36 ≤ B < 56.88

9; 	56.88 ≤ B

##### 6.2A.2.2.1.4		PC2 without indicating dualPA-Architecture supported

MPR in this clause is for intra-band non-contiguous CA power class 2 for UEs without indicating IE dualPA-Architecture supported. The allowed maximum output power reduction is defined as:

MPR=MAWhere MA is defined as follows

MA = 	19.5; 	0 ≤ B < 1.08

19; 	1.08 ≤ B < 2.16

18; 	2.16 ≤ B < 5.04

16.5; 	5.04≤ B < 10.08

16; 	10.08 ≤ B < 36

12;      36 ≤ B < 56.88

10.5; 	56.88 ≤ B

##### 6.2A.2.2.1.5		PC1.5 with indicating dualPA-Architecture supported

MPR in this clause is for intra-band non-contiguous CA power class 1.5 for UEs with indicating IE dualPA-Architecture supported. The allowed maximum output power reduction is defined as:

MPR= MA,total , where MA,total is defined as follows

MA,total= MA + MA,delta , where MA equals to MA defined in sub clause 6.2A.2.2.1.2 and MA,delta is [1]dB for Handheld UE.

#### 6.2A.2.2.2	MPR to meet -13dBm/MHz

##### 6.2A.2.2.2.1		PC3 with indicating dualPA-Architecture supported

MPR in this clause is for intra-band non-contiguous CA power class 3 for UEs indicating IE dualPA-Architecture supported. The allowed maximum output power reduction is defined as:

MPR=MA

Where MA is defined as follows

MA = 	9	;	 0 ≤ B < 0.54

8	;	 0.54 ≤ B < 1.08

7	; 	1.08 ≤ B < 2.16

6.5	; 	2.16 ≤ B < 3.24

5.5	; 	3.24 ≤ B < 5.4

4	; 	5.4 ≤ B

##### 6.2A.2.2.2.2		PC2 with indicating dualPA-Architecture supported

MPR in this clause is for intra-band non-contiguous CA power class 2 for UEs indicating IE dualPA-Architecture supported. The allowed maximum output power reduction is defined as:

MPR=MA

Where MA is defined as follows

MA = 	9	;	 0 ≤ B < 0.54

8	;	 0.54 ≤ B < 1.08

7	; 	1.08 ≤ B < 2.16

6.5	; 	2.16 ≤ B < 3.24

6	; 	3.24 ≤ B < 5.4

5.5	; 	5.4 ≤ B ≤ 10.8

4	; 	10.8 < B

##### 6.2A.2.2.2.3		PC3 without indicating dualPA-Architecture supported

MPR in this clause is for intra-band non-contiguous CA power class 3 for UEs without indicating IE dualPA-Architecture supported. The allowed maximum output power reduction is defined as:

MPR=MAWhere MA is defined as follows

MA = 	11; 	0 ≤ B < 1.08

10.5; 	1.08 ≤ B < 2.16

10; 	2.16 ≤ B < 3.24

9.5; 	3.24≤ B < 5.04

8.5; 	5.04 ≤ B < 10.08

7.5;      10.08 ≤ B < 36

7; 	36 ≤ B

##### 6.2A.2.2.2.4		PC2 without indicating dualPA-Architecture supported

MPR in this clause is for intra-band non-contiguous CA power class 2 for UEs without indicating IE dualPA-Architecture supported. The allowed maximum output power reduction is defined as:

MPR=MAWhere MA is defined as follows

MA = 	14; 	0 ≤ B < 1.08

12; 	1.08 ≤ B < 2.16

11.5; 	2.16 ≤ B < 3.24

11; 	3.24≤ B < 5.04

9.5; 	5.04 ≤ B < 10.08

8.5;      10.08 ≤ B < 36

6.5; 	36 ≤ B

##### 6.2A.2.2.2.5		PC1.5 with indicating dualPA-Architecture supported

MPR in this clause is for intra-band non-contiguous CA power class 1.5 for UEs with indicating IE dualPA-Architecture supported. The allowed maximum output power reduction is defined as:

MPR= MA,total , where MA,total is defined as follows

MA,total= MA + MA,delta , where MA equals to MA defined in sub clause 6.2A.2.2.2.2 and MA,delta is [1]dB for Handheld UE.

< END OF CHANGE #1 >