# 3GPP TSG-RAN WG4 Meeting #114bis	R4-2504264

Wuhan, China, 7th-11th April 2025

Source:			Ericsson

Title:				On co-existence simulation results for NR NTN Ku band

Agenda item:		7.11.2

Document for:		Information

# 1	Introduction

In 3GPP RAN#104, a new Work Item (WI) was approved for Introduction of Ku band for NR NTN. The WI was further updated in RAN#106 [1]. One of the objectives of the WI is to –

- Conduct coexistence study. Reuse Ka band coexistence study assumptions as much as possible, with modifications where necessary, e.g. for coexistence scenario, frequency range and subcarrier spacing. Conduct new co-existence studies based on adjacent services.

WF [2] captured initial agreements on co-existence simulation assumptions in the last meeting.

This contribution provides co-existence simulation results based on the agreements so far.

# 2	Discussion

In the following sections, we present our simulation results based on the agreed simulation parameters in WF [2]. This section is further divided based on the NTN scenarios – GEO and LEO600 for GSO.

## 2.1 	GEO

In this sub-section, we share the simulation results for Case 1-4 for GEO NTN scenario. The simulation assumptions have been captured in the Annex of our companion contribution [3].

Some other assumptions to be considered are listed below –

- Antenna elevation angle is 90 degrees, 25 degrees and 35 degrees.

  - Additional information for 35 degrees is presented in [3].

- Mobile VSAT

  - Mobile VSAT is L-ESIM.

- VSAT NF is 2.5 dB

- VSAT antenna pattern is parabolic.

| ACIR |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Elevation angle |  |  |
|  |  |  |  | 90 degrees | 25 degrees | 35 degrees |
|  |  |  |  | Mobile VSAT | Mobile VSAT | Mobile VSAT |
| Aggressor | Victim | Scenario | Frequency | 2.5 dB NF | 2.5 dB NF | 2.5 dB NF |
| VSAT UL | TN UE UL | 1 | 14 GHz | 1 | 12 |  |
| TN UL | VSAT UL | 2 | 14 GHz | 16 | 22 |  |
| VSAT UL | TN DL | 3 | 14 GHz | 0 | 0 |  |
| TN DL | VSAT UL | 4 | 14 GHz | 6 | 36 | 26 |
| TN DL | SAN DL | 5 | 11 GHz | 36 | 32 | 30 |
| SAN DL | TN DL | 6 | 11 GHz | 0 | 0 | 0 |
| SAN DL | TN UE DL | 7 | 11 GHz | 0 | 0 | 0 |
| TN UL | SAN DL | 8 | 11 GHz | 22 | 16 | 16 |

In scenario 4, higher throughput is observed at 35 degrees elevation angle, resulting in the ACIR value to drop by 10 dB at 35 degrees when compared to 25 degrees elevation angle.

## 2.2	LEO600

In this sub-section, we share the simulation results for Case 1-4 for LEO600 NTN scenario. The simulation assumptions have been captured in the Annex of our companion contribution [3].

Some other assumptions to be considered are listed below –

- Antenna elevation angle is 90 degrees, 25 degrees and 35 degrees.

  - Additional information for 35 degrees is presented in [3].

- Mobile VSAT

  - Mobile VSAT is L-ESIM.

- VSAT NF is 2.5 dB.

- VSAT antenna pattern is parabolic.

| ACIR |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Elevation angle |  |  |
|  |  |  |  | 90 degrees | 25 degrees | 35 degrees |
|  |  |  |  | Mobile VSAT | Mobile VSAT | Mobile VSAT |
| Aggressor | Victim | Scenario | Frequency | 2.5 dB NF | 2.5 dB NF | 2.5 dB NF |
| VSAT UL | TN UE UL | 1 | 14 GHz | 0 | 8 |  |
| TN UL | VSAT UL | 2 | 14 GHz | 16 | 12 |  |
| VSAT UL | TN DL | 3 | 14 GHz | 0 | 0 |  |
| TN DL | VSAT UL | 4 | 14 GHz | 9 | 31 | 22 |
| TN DL | SAN DL | 5 | 11 GHz | 35 | 33 | 30 |
| SAN DL | TN DL | 6 | 11 GHz | 0 | 0 | 0 |
| SAN DL | TN UE DL | 7 | 11 GHz | 0 | 0 | 0 |
| TN UL | SAN DL | 8 | 11 GHz | 26 | 21 | 20 |

In scenario 4, higher throughput is observed at 35 degrees elevation angle, resulting in the ACIR value to drop by 9 dB at 35 degrees when compared to 25 degrees elevation angle.

# Conclusion

In the previous sections we made the following observations:

Observation 1	In scenario 4, higher throughput is observed at 35 degrees elevation angle, resulting in the ACIR value to drop by 10 dB at 35 degrees when compared to 25 degrees elevation angle.

Observation 2	In scenario 4, higher throughput is observed at 35 degrees elevation angle, resulting in the ACIR value to drop by 9 dB at 35 degrees when compared to 25 degrees elevation angle.

# References

- RP-243314, “Revised WID: Introduction of Ku bands for NR NTN”, 3GPP RAN#105

- R4-2502290, “WF for NR_NTN_Ku_Band_General”, 3GPP RAN4#114, Moderator

- R4-2504263, “On Co-existence simulation assumptions for NR NTN Ku band”, Ericsson, 3GPP RAN4#114bis