3GPP TSG-RAN WG4 Meeting #114bis                                    R4-2504287

Wuhan, Hubei, China, 07th – 11th April, 2025

Source:	Samsung

Title:              General discussion on AI mobility regarding RAN4 requirements impact

Agenda item:			7.21.2.1

Document for:	Discussion

Introduction

Some AI mobility RAN4 requirements impact were discussed extensively in last meeting. In this meeting, we will give some further discussions for some open issues captured in the agreed WF [1].

2 Discussion

2.1 Impacts on beam/cell level measurements

- Based on the following clarifications in TR 38.744,

## 5.2 RRM measurement prediction ### 5.2.1 Evaluation methodology, metrics and assumptions Measurement prediction accuracy for cell-level RRM measurement prediction is defined as average L3 RSRP difference between predicted L3 filtered cell-level measurement result and actual L3 filtered cell-level measurement result of the same cell for all RRM sub-use cases.

it is necessary and reasonable to define L3 cell level RSRP prediction accuracy requirements and define L3 measurement prediction accuracy requiremente at point C

- Regarding whether to define accuracy requirement at point A1. First, based on our simulation results presented in RAN2, the necessity of L1 filtering can be observed and verified. Second, from our understanding, if L1 beam level prediction (Sub-use-case 1) is considered, it is better to define accuracy requirements at A1, since the L3 filtered RSRP will be derived based on measured L1 beam results and the predicted beam L1 results following cell-level metrics derivation and L3 filtering procedure defined in TS 38.331 Section 5.5. However, the predicted beam qualities are different from measured beam qualities, in this sense, the beam measurement prediction accuracy should be defined to ensure the AI model can learn the best and can provide the acceptable and accurate predictions directly. And to define the accuracy at A1 actually aligns with our traditional non-AI L3 RSRP requirement definition logic.

- However, we have some concerns that if we to define accuracy requirement at A1, whether it is necessary to define an additional prediction accuracy requirement [on top of the requirement at C]. Since: I.  Signalling/memory/measurement overhead and computational complexity for model training/inference; II.  Accuracy evaluation. The beam prediction accuracy requirement defined at A1 may be rigorous for cell-level prediction, which complies with beam management rather than system performance evaluation, in other words, L3 mobility performance including neighbout cell; III. For case B, the applicability of sliding and non-sliding; IV. RSRP difference interpretion, are different by comparing directly leveraging L3 to predict L3 and indirectly leveraging L1 to predict L3.

- Furthermore, regarding whether to define accuracy requirement at E, we suggest to deprioritize the case, since the model may need to learn the logic of the beam consolidation mechanism, which is different from the ones discussed for A1/C.

Proposal 1:

- RAN4 to prioritize studying the impacts on requirements for L3 cell level RSRP (Point C) prediction

- RAN4 to deprioritize studying the impacts on requirements for L3 beam-level (Point E) prediction, aligning with RAN2 prioritization

Proposal 2:

- If it is proposed to consider L1 beam level prediction, RAN4 to study the impacts on requirements for L1 beam level RSRP (Point A1) prediction

  - FFS: Whether to/How to define an additional prediction accuracy requirement on top of the requirement at C

2.2 Core requirements

For our understanding, for inter-frequency prediction, RAN4 can to study the impact on MG reduction in measurement period/scheduling restriction based on the measurement overhead reduction goal. Besides, since measurements of the co-located cell (s) can be reduced, the CSSF requirements can be enhanced.

Proposal 3: RAN4 to discuss the RRM impact of measurement gap reduction in measurement period/ scheduling restriction and CSSF enhancement for inter-frequency prediction

Furthermore, the prediction can be used to reduce the inter-frequency measurement the UE needs to take, so I. The measurement latency for the certain MO (s) can be reduced, since in the legacy, MG is used to perform RF tuning, and the simplest way to complete measurements of all the measurement objects in in turn; II. The data interruption time can be reduced and scheduling restriction can be relaxed, since in the legacy, during MGs, the UE can not receive/transmit data of the serving cell.

Proposal 4: For the RRM impact on prediction window, at least the following may need to be discussed

- Scheduling restriction

- Measurement latency for MO (s), MO (s) reduction

2.3 RAN4 requirements for Measurement event prediction use case

- From our understanding, the PW is used for mobility event and/or handover determination by the UE/network. And the ultimate aim for 1st goal is not to evaluate measurement overhead reduction, but to, take NW-side mode as example, recover the skipped RRM measurements based on the predicted results what UE reports in order to mitigate the occurrence of unintended events (e.g., ping-pong HO, too late/early/wrong HO, short ToS, RLF due to too late HO, etc.) or to make accurate HO decision by foreseeing the RRM status of UE in mobility. From this, the measurement reduction should keep the mobility performance from degrading.

- However, the legacy L3 HO mechanism provides a relatively long HO complete latency, which may also require a huge measurement effort, and even result in unnecessary event happens. In this sense, comparing to the legacy HO mechanism that NW decides HO after Event A3 trigger conditions are fulfilled during TTT, using AI/ML event predictions may reduce the HO execution time to some extent

As per RAN2 agreement guidance, please kindly find below

Agreements on Measurement event case A - Most of the results show that the F1 score for prediction of measurement event is very good. - For handover decision option 3 and 2 AI/ML performs better than baseline (legacy) in terms of HO failure rate. Other agreements Inter-frequency measurement event prediction will be considered for the specification impact study but no explicit simulations will be required. Agreements on direct measurement event prediction - F1 score for direct measurement is very good based on the current simulation results.

In this sense,

Proposal 5: RAN4 to study if HO requirements including delay, interruption, side conditions and accuracy are needed to be updated for AI/ML-assisted mobility, temporal domain prediction case A can be taken as starting point considering verified handover decision option 3 and 2

2.4 RAN4 requirements for Measurement event prediction use case

An AI/ML model needs to be developed, deployed, and managed during the entire lifecycle. The overall procedure of AI/ML model LCM was discussed in Rel-18 AI/ML BM RAN1, and potential AI mobility specific enhancement, e.g., LCM was agreed in RAN2 scope.

In Rel-18 AI/ML for NR air interface, the necessity and feasibility of defining requirements/tests for LCM was extensively discussed and confirmed. And during the discussions, the agreements on LCM related requirements/functional tests were achieved sequentially. Based on the WFs agreed in Rel-18, the following LCM related requirements are agreed to define based on the specific BM and positioning use cases:

Issue 1-5: LCM Requirements Agreement: - RAN4 considers defining RRM requirements for LCM procedures based on the specific use cases (beam management and positioning), and candidates are: - identification - selection - activation - deactivation - switching - fallback to non-AI operation - performance monitoring - Others are not precluded

From this, although the above LCM related requirements and functional tests are AI/ML PHY use cases specific, the aim, necessity, feasibility and even the principle of defining the LCM related requirements and functional tests should be inherited to AI/ML mobility use cases.

Proposal 6: LCM related requirements and functional tests should to be defined based on the specific AI/ML mobility use cases and sub-use cases

- The LCM procedures agreed in AI/ML for NR air interface RRM need to be considered as starting point

  - Prefer to use the legacy framework for RRC/MAC-CE/DCI based core requirements as the baseline for LCM procedures

3 Conclusion

In this contribution, we provided our viewpoints on general discussion on AI mobility regarding RAN4 requirements impact. The following observations and proposals are obtained:

Proposal 1:

- RAN4 to prioritize studying the impacts on requirements for L3 cell level RSRP (Point C) prediction

- RAN4 to deprioritize studying the impacts on requirements for L3 beam-level (Point E) prediction, aligning with RAN2 prioritization

Proposal 2:

- If it is proposed to consider L1 beam level prediction, RAN4 to study the impacts on requirements for L1 beam level RSRP (Point A1) prediction

  - FFS: Whether to/How to define an additional prediction accuracy requirement on top of the requirement at C

Proposal 3: RAN4 to discuss the RRM impact of measurement gap reduction in measurement period/ scheduling restriction and CSSF enhancement for inter-frequency prediction

Proposal 4: For the RRM impact on prediction window, at least the following may need to be discussed

- Scheduling restriction

- Measurement latency for MO (s), MO (s) reduction

Proposal 5: RAN4 to study if HO requirements including delay, interruption, side conditions and accuracy are needed to be updated for AI/ML-assisted mobility, temporal domain prediction case A can be taken as starting point considering verified handover decision option 3 and 2

Proposal 6: LCM related requirements and functional tests should to be defined based on the specific AI/ML mobility use cases and sub-use cases

- The LCM procedures agreed in AI/ML for NR air interface RRM need to be considered as starting point

  - Prefer to use the legacy framework for RRC/MAC-CE/DCI based core requirements as the baseline for LCM procedures

4 References

[1] R4-2502594 “WF on AI/ML Mobility”, Nokia