3GPP TSG-RAN WG4 Meeting #114bis	R4-25xxxxx

Wuhan, Hubei, China, 07th – 11th April, 2025

Title: 	Simulation results on 8Rx MMSE-IRC requirements with interference

Source: 	Huawei, HiSilicon

Agenda item:	7.18.2

Document for:	Discussion

Background

This contribution provides our simulation results on 8Rx MMSE-IRC requirements with interference based on the simulation assumptions in [1].

- PDSCH for inter cell interference

Table 2-1 summarizes the simulation assumptions for inter cell interference

Table 2-1 Simulation assumptions for inter cell interference

| Parameter |  | Unit | Value |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 | Cell 3 |
|  |  |  | Enabled | Enabled | Enabled for HomoNet Disabled for HetNet |
| Duplex mode |  |  | FDD |  |  |
| Active DL BWP index |  |  | 1 |  |  |
| Physical cell ID |  |  | 0 | 1 | 2 |
| Transmission rank |  |  | 1,2,4 | Random rank with 70% and 30% probability for rank 1 and rank 2 | Random rank with 70% and 30% probability for rank 1 and rank 2 for HomoNet N/A for HetNet |
| Time offset to Cell1 |  | us | N/A | 3 | -1 |
| Frequency offset to Cell 1 |  | Hz | N/A | 300 | -100 |
| Interference Model |  |  | N/A | As specified in B.6.2 in 38.101-4 |  |
| INR (Note 2) |  | dB | N/A | 7.77 for HomoNet 7.58 for HetNet | 2.29 for HomoNet N/A for HetNet |
| PDSCH configuration | Mapping type |  | Type A |  |  |
|  | k0 |  | 0 |  |  |
|  | Starting symbol (S) |  | 2 |  |  |
|  | Length (L) |  | 12 |  |  |
|  | PDSCH aggregation factor |  | 1 |  |  |
|  | PRB bundling type |  | Static |  |  |
|  | PRB bundling size |  | 2 |  |  |
|  | Resource allocation type |  | Type 0 |  |  |
|  | RBG size |  | Config2 |  |  |
|  | VRB-to-PRB mapping type |  | Non-interleaved |  |  |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |  |  |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |  |  |
|  | Number of additional DMRS |  | 1 |  |  |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |  |  |
| Number of HARQ Processes |  |  | 4 |  |  |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | 2 |  |  |
| Note1: Cell 1 is the serving cell; Cells 2, 3 are interfering cells Note 2: INR is defined in Annex B.6.1 in 38.101-4 |  |  |  |  |  |

Table 2.2 provides simulation results for 8Rx with inter-cell interference.

Table 2.2 Simulation results for 8Rx with inter cell interference

| Duplex Mode | SNR(dB)@70% of max TP |  |  |  |
| --- | --- | --- | --- | --- |
|  | Rank2 HomNet |  | Rank4 HetNet |  |
|  | Baseline Receiver | Simplified Receiver | Baseline Receiver | Simplified Receiver |
| FDD | 7.3 | 10.8 | 7.8 | 12.4 |
| TDD | 7.7 | 10.3 | 7.9 | 12.0 |

- PDSCH for MU-MIMO

Table 3-1 summarizes the simulation assumptions under MU-MIMO scenario

Table 3-1: Simulation assumptions for MU-MIMO scenario

| Parameter |  | Unit | Target UE | Co-scheduled UE |
| --- | --- | --- | --- | --- |
| Duplex mode |  |  | FDD,TDD |  |
| Active DL BWP index |  |  | 1 |  |
| PDSCH configuration | Mapping type |  | Type A |  |
|  | k0 |  | 0 |  |
|  | Starting symbol (S) |  | 2 |  |
|  | Length (L) |  | 12 |  |
|  | PDSCH aggregation factor |  | 1 |  |
|  | PRB bundling type |  | Static |  |
|  | PRB bundling size |  | 2 |  |
|  | Resource allocation type |  | Type 0 |  |
|  | RBG size |  | Config2 |  |
|  | VRB-to-PRB mapping type |  | Non-interleaved |  |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |  |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |  |
|  | Number of additional DMRS |  | 1 |  |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |  |
|  | Antenna ports indexes |  | 1000,1001 | 1002,1003 |
|  | Number of PDSCH DMRS CDM group(s) without data |  | 2 | 2 |
| PDSCH & PDSCH DMRS Precoding configuration |  |  | Single Panel Type I, Randomized precoder selection for every PRB bundle and updated per slot, with equal probability of each applicable i1/i2 combination or codebook Index, chosen from section 5.2.2.2.1 of TS 38.214 [12]. | Single Panel Type I, Select the precoder to ensure any column of precoder is orthogonal to any column of precoder for the target PDSCH |
| MU-MIMO Beamforming Model |  |  | As specified in B.4.2 in 38.101-4 |  |
| Number of HARQ Processes |  |  | 4 | N/A |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | 2 | N/A |
| Note 1: The DMRS scrambling ID is same for both target UE and Co-scheduled UE. |  |  |  |  |

Table 3-2 provides simulation results for 8Rx under MU-MIMO scenario.

Table 3.2 Simulation results for 8Rx under MU-MIMO scenario

| Duplex Mode | SNR(dB)@70% of max TP |  |
| --- | --- | --- |
|  | Baseline Receiver | Simplified Receiver |
| FDD | 9.3 | 12.3 |
| TDD | 9.4 | 12.3 |

- CQI requirements for inter cell interference

Table 4-1 summarizes simulation results for CQI requirements. Rank2 with baseline receiver are assumed,

Table 4-1: Simulation results for CQI requirements

| SNR(dB) | FDD |  | TDD |  |
| --- | --- | --- | --- | --- |
|  | Gama | BLER | Gama | BLER |
| -4 | 2.71 | 0.04 | 2.60 | 0.02 |
| -3 | 2.58 | 0.08 | 2.30 | 0.08 |
| -2 | 2.50 | 0.05 | 2.52 | 0.08 |
| -1 | 2.36 | 0.12 | 2.45 | 0.12 |
| 0 | 2.63 | 0.01 | 2.52 | 0.03 |
| 1 | 2.57 | 0.01 | 2.64 | 0.02 |
| 2 | 2.53 | 0.04 | 2.80 | 0.05 |
| 3 | 2.49 | 0.08 | 2.70 | 0.02 |
| 4 | 2.70 | 0.09 | 2.65 | 0.05 |
| 5 | 3.04 | 0.07 | 3.11 | 0.03 |
| 6 | 2.91 | 0.12 | 3.19 | 0.09 |
| 7 | 3.20 | 0.02 | 3.53 | 0.02 |
| 8 | 2.78 | 0.03 | 3.10 | 0.03 |
| 9 | 2.13 | 0.04 | 2.67 | 0.02 |
| 10 | 2.01 | 0.02 | 2.21 | 0.08 |

- Conclusion

This contribution provides our simulation results for 8Rx requirements with interference.

- Reference

[1]  R4-2502275 - WF for [114][324] NR_demod_Ph5_Part2_UE. Nokia