3GPP TSG-RAN WG4 Meeting #114bis	R4-25xxxxx

Wuhan, Hubei, China, 07th – 11th April, 2025

Title: 	On CA performance for less than 5MHz

Source: 	Huawei, HiSilicon

Agenda item:	7.3.4

Document for:	Discussion

Background

A new WID [1] was agreed in RAN 107 meeting, the motivation of which is to introduce NR CA/DC UE RF performance with 3MHz CBW in the one band and 5MHz or 10MHz in the other band. One objective of performance part is to define the performance requirements if necessary. This contribution provides our views on the demodulation performance requirements definition.

- Discussions

The current applicability rule for inter band CA test is step1: select the CA configuration with the maximum number of CC, step2: select any one of CA configuration, which contain CA bandwidth combination with the largest aggregated channel bandwidth, step3: select the CA configurations with largest number of bands and with the maximum number of CCs, step4, select any one of CA configurations, which contain CA bandwidth combination with the largest aggregated channel bandwidth.

From above it can be observed that under the condition with same number of CCs, CA configuration with largest aggregated channel bandwidth will be selected, noted that the aggregated channel bandwidth may be the largest among all configurations even one of CCs is 3MHz. Hence 3MHz carrier will likely be selected and it is necessary to define CA requirements with 3MHz component carrier.

Proposal 1: RAN4 to define CA requirements with 3MHz component carrier.

It’s proposed to reuse the existing CA test setup, hence we propose the following:

Proposal 2: RAN4 to use following test setups for 3MHz component carrier:

16QAM, 0.48

TDLA30-10

2x2, ULA Low

- Conclusion

- This contribution provides our views on CA performance requirements with 3MHz component carrier. The proposals are:

Proposal 1: RAN4 to define CA requirements with 3MHz component carrier.

Proposal 2: RAN4 to use following test setups for 3MHz component carrier:

MCS: 16QAM, 0.48

Propagation conditions: TDLA30-10

Correlation and antenna configuration: 2x2, ULA Low

- Reference

[1] RP-250781 Revised WID: NR channel BW less than 5MHz for FR1 Phase 2. Intel Corporation.