3GPP TSG-RAN WG4 Meeting # 114-bis	R4-2504333

# Wuhan, China, 7th - 11th April, 2025

Title:	Discussion on NR-NTN HPUE Tx requirements

Source:	Samsung

Agenda item:	7.10.2.2.1

Document for:	Approval

1	Introduction

In last #114 meeting, the WF[1] had progressed on down scope the HPUE architectures and UE types for different power classes. And also, the WF had narrowed down the remaining options for candidate ACLR for each of the power classes.

In this paper, we will continue to discuss the remaining open issues for Tx requirements of NR-NTN HPUE.

2	SEM and ACLR

From the last meeting WF [1], the meeting has concluded the following agreements on ACLR and SEM for NTN HPUE:

Issue 2-4-1: PC2 ACLR Agreement: - PC2 ACLR is [30 or 31] dBc for NR-NTN UE Issue 2-4-2: PC1.5 ACLR Agreement: - PC1.5 ACLR is [30 or 31] dBc for NR-NTN UE Issue 2-4-3: PC1 ACLR Agreement: - PC1 ACLR is [31 or 37] dBc for NR-NTN UE - Further study averaging value, i.e. 34. Issue 2-5: SEM Agreement: - Apply SEM from NR-NTN PC3 to other PCs for NR-NTN as baseline. - Further discuss whether and how to capture the FCC 47 CFR part 25 specification in clause 202

In last meeting discussions, the meeting had concluded the following agreements from co-ex study perspective:

Issue 1-2: NR-NTN Co-ex study results discussions Agreement: - For NR-NTN PC2, it is observed that the required ACLR is less than 20 dB with 1.5km isolation distance assumed based on the simulation results. Agreement: - For NR-NTN PC1.5, it is observed that the required ACLR is less than 20.7 dB with 1.5km isolation distance assumed based on the simulation results. Agreement: - For NR-NTN PC1, it is observed that the required ACLR is less than 21.6 dB with 1.5km isolation distance assumed based on the simulation results.

Observation 1: It’s our observation that from co-ex study results, the required ACLR is much less than the NR-NTN current PC3 ACLR requrements.

And in last meeting, we proposed to consider to re-use the NR-NTN PC3 ACLR, i.e. 30dBc, for NR-NTN PC2 and PC1.5, and the rationale is given in [2].

During the discussions, the most push-backs were mostly from the following concerns:

- Whether relaxed ACLR would result in stringent MPR, which further reduced available output power from NR-NTN HPUE;

- Whether relaxed ACLR would result in interference in operators in adjacent channel, which jeopardize the deployment of other networks.

- Whether relaxed ACLR would be beneficial to NTN HPUE implementation

In last meeting paper [3] and [4], and also in discussion, the common understanding is the limiting factor for MPR is SEM rather than ACLR, especially given the NTN UL have to utilize smaller RB allocations due to its link budget constraints. And from such understanding, the logic is that relaxing ACLR will not lead to a stringent MPR requirements, as the MPR requirements will be limited by SEM, which is agreed as un-changed from PC3 SEM.

Observation 2: Relaxing ACLR will not lead to stringent MPR which backfires the HPUE output power, due to the limiting factor of MPR is SEM in smaller RB allocations from constrained NTN UL link budget.

And we belive the Observation 1 and co-ex study conclusions should be sufficient to resovle the concerns over the interference to an operator in adjacent channel. The gap between required ACLR from co-ex to the 30dBc is quite large to guarantee the protection.

Observation 3: Relaxing ACLR from 31 to 30, will not jeopardize the operator in adjacent channel from co-existence conclusions.

From above observations, our understanding is that at least for PC2 and PC1.5 handheld UE, the ACLR could be relaxed from 31 to 30 dBc.

Proposal 1: For PC2 and PC1.5, we propose the meeting to agree on re-use PC3 ACLR which is 30dBc.

Moreover, in last meeting, one company proposed a different SEM which actually would impact the PC3 core requirements, and it will then further impact the HPUE Tx requirements if adopted for PC3. In order to understand the differences between the newly proposed SEM and existing SEM, we listed this new SEM proposal below:

47 C.F.R. § 25.202: (d) Frequency tolerance, Earth stations. The carrier frequency of each earth station transmitter authorized in these services shall be maintained within 0.001 percent of the reference frequency. (e) Frequency tolerance, space stations. The carrier frequency of each space station transmitter authorized in these services shall be maintained within 0.002 percent of the reference frequency. 47 C.F.R. § 25.202 (f): Emission limitations. Except for SDARS terrestrial repeaters and as provided for in paragraph (i), the mean power of emissions shall be attenuated below the mean output power of the transmitter in accordance with the schedule set forth in paragraphs (f)(1) through (f)(4) of this section. The out-of-band emissions of SDARS terrestrial repeaters shall be attenuated in accordance with the schedule set forth in paragraph (h) of this section. (1) In any 4 kHz band, the center frequency of which is removed from the assigned frequency by more than 50 percent up to and including 100 percent of the authorized bandwidth: 25 dB; (2) In any 4 kHz band, the center frequency of which is removed from the assigned frequency by more than 100 percent up to and including 250 percent of the authorized bandwidth: 35 dB; (3) In any 4 kHz band, the center frequency of which is removed from the assigned frequency by more than 250 percent of the authorized bandwidth: An amount equal to 43 dB plus 10 times the logarithm (to the base 10) of the transmitter power in watts; (4) In any event, when an emission outside of the authorized bandwidth causes harmful interference, the Commission may, at its discretion, require greater attenuation than specified in paragraphs (f) (1), (2) and (3) of this section.

In Rel-17 NR-NTN discussions, the above FCC requirements were adopted in TR 38.863[5] and agreed to re-use the existing SEM requirements from TN network considering the existing SEM was derived from previous regulation surveys.

Observation 4: The FCC regulatory requirements, CFR 25.202, was already considered and discussed in preivous Rel-17 PC3 core requirements discussions, and captured in TR 38.863.

Proposal 2: The consideration of this CFR 25.202 falls into the Rel-17 maintenance work, and in addition, it was already considered in Rel-17 and captured in TR 38.863.

3	MPR and A-MPR

In WF[1], the agreements were reached for candidate MPR as follows:

Issue 2-6: MPR/A-MPR Agreement: - For PC 2 and PC 1.5 MPR: - Option 1: Existing MPR requirements for PC2 and PC1.5 in TS 38.101-1 apply, if PC2/PC1.5 ACLR reused TN PC2/PC1.5 ACLR as 31 dBc. - Option 2: Aim for MPR analysis for PC2 and PC1.5 if PC/PC1.5 ACLR agreed as 30 dBc. - For PC 1 MPR: - Option 1: If PC1 ACLR reused TN PC1 ACLR as 37 dBc: - Option 1-1: Existing MPR requirements for PC1 in TS 38.101-1 applies - Option 1-2: More analysis is needed to revisit the MPR requirements from TN spec. - Option 2: Aim for MPR analysis for PC1 for other ACLR values [31 or 34] dBc - Aim for MPR/A-MPR analysis in #114bis for different power classes - For PA model calibration: - DFT-s-OFDM QPSK 20 MHz - 100RB0 - 4dB post PA loss - 1dB MPR - Carrier leakage 28dBc - IQ image:28 dBc - CIM3: 60 dBc - ACLR: PC2 [31]dB, PC1.5 [31]dB, PC1 [31]dB - SEM: existing PC3 SEM - EVM: 17.5%

And given in previous section of this paper, we had proposed to relax the ACLR of PC2 and PC1.5 to 30dBc, and also we had listed the observations from our side and also from several papers submitted to last meeting that relaxing ACLR will not lead to a stringent MPR due to the NR-NTN UL link budget will be constraint and probably can only schedule few RBs for UL.

Proposal 3: To re-use existing MPR requirements for PC2 and PC1.5 in TS 38.101-1, if PC/PC1.5 ACLR agreed as 30 dBc.

For PC1, given it’s already agreed that PC1 in this release will focus on non-handheld UE, our observation is that the Option 1-1, which is to re-use the existing MPR requirements for PC1 in TS 38.101-1 can be adopted.

Observation 5: Given only non-handheld UE is to be considered for PC1 in this release, the existing MPR requirements for PC1 in TS 38.101-1 can be re-used.

Proposal 4: To re-use existing MPR requirements for PC1 in TS 38.101-1, and use similar wording from TN spec to explicitly mention the form factor is non-handheld or vehicle based.

And regarding A-MPR, per previous regulatory survey, for n256, the A-MPR requirements for n65 could be as starting point. From TS 38.101-1, since the maximum carrier centre frequency of NS_05 and NS_05U is lower than 1960 MHz and the maximum carrier centre frequency of NS_51 is lower than 1980 MHz, it is proposed NS_05, NS_05U and NS_51 don’t need to be considered for n256. The requirements of NS_01, NS_24 and NS_100 specified in Table 6.2.3.1-1 in TS 38.101-1 for band n65 can be reused for n256.

Table 6.2.3.1-2: A-MPR for NS_100 (UTRA protection) (Power Class 3 and Power Class 2) Modulation/Waveform; Outer (dB). DFT-s-OFDM; Pi/2 BPSK; ≤ 2. QPSK; ≤ 2. 16 QAM; ≤ 2.5. 64 QAM; ≤ 3. 256 QAM; ≤ 4.5. CP-OFDM; QPSK; ≤ 4. 16 QAM; ≤ 4. 64 QAM; ≤ 4. 256 QAM; ≤ 6.5. NOTE 1: Void NOTE 2: Void. Table 6.5.2.4.2-1: UTRA ACLR requirement Power class 2; Power class 3. UTRAACLR1; 33 dB; 33 dB. UTRAACLR2; 36 dB; 36 dB. Table 6.2.3.15-1: A-MPR for NS_24 Channel Bandwidth, MHz; Carrier Centre Frequency, Fc, MHz; Region A; Region B; Region C. RBend*12*SCS MHz; LCRB*12*SCS MHz; A-MPR; RBend*12*SCS MHz; LCRB*12*SCS MHz; A-MPR; RBend*12*SCS MHz; LCRB*12*SCS MHz; A-MPR. 5MHz; 1987.5 < Fc ≤ 1992.5; >3.24; A7. 5MHz; 1992.5 < Fc ≤ 1997.5; >3.24; A4. 5MHz; 1997.5 < Fc ≤ 2002.5; >1.98; A1; >3.6; >1.08 ≤1.98; A2; ≤3.6; ≤1.98; A3. ≤1.08; A6. 10MHz; 1975 < Fc ≤ 1985; >5.4; A4. 10MHz; 1985 < Fc ≤ 1995; >4.32; A1; ≥7.2; >1.08 ≤4.32; A2; <7.2; ≤4.32; A3. ≤1.08; A6. 10MHz; 1995 < Fc ≤ 2000; ≥5.76; A5; <3.06; A5; ≥3.06 <5.76; >1.44; A6. 15MHz; 1972.5 < Fc ≤ 1987.5; >6.84; A1; ≥10.8; >1.08 ≤6.84; A2; <10.8; ≤6.84; A3. ≤1.08; A6. 15MHz; 1987.5 < Fc ≤ 1997.5; ≥8.64; A5; <3.78; A5; ≥3.78 <8.64; >1.44; A6. 20MHz; 1970 < Fc ≤ 1990; ≥12.96; A5; <4.68; A5; ≥4.68 <12.96; >2.16; A6. 20MHz; 1990 < Fc ≤ 1995; ≥11.52; A5; <5.58; A5; ≥5.58 <11.52; >1.44; A6. NOTE 1: The A-MPR values are listed in Table 6.2.3.15-2. NOTE 2: For any undefined region, MPR applies. Table 6.2.3.15-2: A-MPR for modulation and waveform type Modulation/Waveform; A1; A2; A3; A4; A5; A6; A7. Outer/Inner; Outer/Inner; Outer/Inner; Outer; Outer/Inner; Outer/Inner; Outer. DFT-s-OFDM PI/2 BPSK; ≤ 11; ≤ 5; ≤ 4; ≤ 8.5; ≤ 18; ≤ 10; ≤ 3.5. DFT-s-OFDM QPSK; ≤ 11; ≤ 5; ≤ 4; ≤ 8.5; ≤ 18; ≤ 10; ≤ 3.5. DFT-s-OFDM 16 QAM; ≤ 11; ≤ 5; ≤ 4; ≤ 8.5; ≤ 18; ≤ 10; ≤ 3.5. DFT-s-OFDM 64 QAM; ≤ 11; ≤ 5; ≤ 4; ≤ 8.5; ≤ 19; ≤ 10; ≤ 3.5. DFT-s-OFDM 256 QAM; ≤ 11; ≤ 5; ≤ 8.5; ≤ 20; ≤ 10. CP-OFDM QPSK; ≤ 13; ≤ 6.5; ≤ 4; ≤ 8.5; ≤ 19; ≤ 12; ≤ 5.5. CP-OFDM 16 QAM; ≤ 13; ≤ 6.5; ≤ 4; ≤ 8.5; ≤ 19; ≤ 12; ≤ 5.5. CP-OFDM 64 QAM; ≤ 13; ≤ 6.5; ≤ 4; ≤ 8.5; ≤ 19; ≤ 12; ≤ 5.5. CP-OFDM 256 QAM; ≤ 13; ≤ 6.5; ≤ 8.5; ≤ 20; ≤ 12. NOTE 1: The backoff applied is max(MPR, A-MPR) where MPR is defined in Table 6.2.2-1 NOTE 2: Outer and inner allocations are defined in clause 6.2.2. Table 6.5.3.3.13-1: Additional requirements for "NS_24" Frequency range (MHz); Channel bandwidth (MHz) / Spectrum emission limit (dBm); Measurement bandwidth. 5 MHz, 10 MHz, 15 MHz, 20 MHz. 2010 ≤ f ≤ 2025; -50; 1 MHz. NOTE 1: This requirement applies at a frequency offset equal or larger than 5 MHz from the upper edge of the channel bandwidth, whenever these frequencies overlap with the specified frequency band..

From existing NS_24 and NS_100, it can be observed that the A-MPR of NS_100 already cover to both PC3 and PC2, while the NS_24 may require further work to makes ure the emission limit is met for power class 2.

Proposal 5: For A-MPR of n256, take the requirements of NS_01, NS_24 and NS_100 specified in Table 6.2.3.1-1 in TS 38.101-1 for band n65.

4	Conclusion

For NR-NTN HPUE Tx requirements, we propose the following:

Observation 1: It’s our observation that from co-ex study results, the required ACLR is much less than the NR-NTN current PC3 ACLR requrements.

Observation 2: Relaxing ACLR will not lead to stringent MPR which backfires the HPUE output power, due to the limiting factor of MPR is SEM in smaller RB allocations from constrained NTN UL link budget.

Observation 3: Relaxing ACLR from 31 to 30, will not jeopardize the operator in adjacent channel from co-existence conclusions.

Proposal 1: For PC2 and PC1.5, we propose the meeting to agree on re-use PC3 ACLR which is 30dBc.

Observation 4: The FCC regulatory requirements, CFR 25.202, was already considered and discussed in preivous Rel-17 PC3 core requirements discussions, and captured in TR 38.863.

Proposal 2: The consideration of this CFR 25.202 falls into the Rel-17 maintenance work, and in addition, it was already considered in Rel-17 and captured in TR 38.863.

Proposal 3: To re-use existing MPR requirements for PC2 and PC1.5 in TS 38.101-1, if PC/PC1.5 ACLR agreed as 30 dBc.

Observation 5: Given only non-handheld UE is to be considered for PC1 in this release, the existing MPR requirements for PC1 in TS 38.101-1 can be re-used.

Proposal 4: To re-use existing MPR requirements for PC1 in TS 38.101-1, and use similar wording from TN spec to explicitly mention the form factor is non-handheld or vehicle based.

Proposal 5: For A-MPR of n256, take the requirements of NS_01, NS_24 and NS_100 specified in Table 6.2.3.1-1 in TS 38.101-1 for band n65.

5	Reference

[1] R4-2502954, WF on HPUE requirements for NR-NTN, Samsung

[2] R4-2501808, Discussion on NR-NTN HPUE Tx requirements, Samsung

[3] R4-2500090, Candidate TN PA for NR-NTN HPUE in smartphones, Skyworks Solutions, Inc.

[4] R4-2500293, Further considerations on the high-power UE Tx requirements, Apple

[5] TR 38.863