3GPP TSG-RAN WG4 Meeting # 114-bis	R4-2504337

# Wuhan, China, 7th - 11th April, 2025

Title:	Discussion on NTN Less than 5MHz UE RF requirements

Source:	Samsung

Agenda item:	7.10.3.2

Document for:	Approval

1	Introduction

In last RAN4 #114 meeting, the WF[1] had reached many agreements for each issues listed. And there are some of them are still ‘tentative’.

In this paper, we continue to discuss these issues and to further progress the work.

2	Discussions

One FFS in A-MPR is whether or not to document the analysis of MPR/AMPR anlaysis for introducing 3MHz in NTN to the TR 38.863.

Issue 2-1-7: A-MPR results for band n256 - FFS: - Consider capturing the 3MHz power back-off results with related considerations in TR 38.863.

In our view, the MPR/AMPR analysis is part of this WI, and in the impact TS/TR table, there’s no other places for n256, it should belong to the TR 38.863 if need to be documented.

And as this is for FR1-NTN, we believe the section 7.4.2.2.2	MPR/AMPR could be a suitable place for it.

Proposal 1: Consider to captrue the 3MHz MPR/AMPR analysis for n256 in section 7.4.2.2.2 in TR 38.863.

And for Refsens, the square bracket is still in the current WF[1] as below:

Issue 2-2-1: Reference sensitivity - Agreement - For band n254, [2.2] dB improvement compared with 5 MHz CBW; - For band n256, [1.7]dB improvement compared with 5 MHz CBW; - FFS for band n255

In our previous papers, we had presented the rationales, including Rx non-linearity and also the similar refsens improvement as in LTE b66, these are some additional factors to be considered to the simple scaling from RB numbers.

Proposal 2a: To agree on the n256 and n254 and remove the square bracket.

Proposal 2b: To have 1.7 dB improvement compared to 5MHz CBW for n255 as well.

3	Conclusion

In this contribution, we provide following proposals for NTN less than 5MHz UE RF aspects:

Proposal 1: Consider to captrue the 3MHz MPR/AMPR analysis for n256 in section 7.4.2.2.2 in TR 38.863.

Proposal 2a: To agree on the n256 and n254 and remove the square bracket.

Proposal 2b: To have 1.7 dB improvement compared to 5MHz CBW for n255 as well.

4	Reference

[1] R4-2502290, Way Forward for [114][313] NR_NTN_Ku_Band_General, Moderator (Eutelsat Group)

[2] R4-2502315, Co-existence simulation assumptions for NR NTN Ku band (Running document)