3GPP TSG-RAN WG4 Meeting #114bis	R4-2504350

Wuhan, Hubei, China, 07th – 11th April, 2025

Agenda item:	7.10.4.2

Source:	MediaTek Inc.

Title:	Discussions on demodulation performance requirements for NGSO channel model

Document for:	Discussion

# Introduction

According to the WID [1], RAN4 is tasked to specify UE demodulation performance requirement(s) of PDSCH for NGSO for NR-NTN and IoT-NTN in the FR1-NTN bands and the corresponding LTE bands based on the channel model with varying Doppler and delay shifts. In this contribution, we provide our views.

NTN testing for NGSO - Specify the TE-emulated channel model with varying Doppler and delay shifts for NR-NTN and IoT-NTN in the FR1-NTN bands and the corresponding LTE bands - Match the satellite motion trajectory based on the ephemeris for NGSO (Non-Geostationary Orbit) scenarios - Inform RAN5 to assist specifying RF frequency error tests, if needed - Checking if the existing frequency error requirement, can be met under the TE-emulated channel model without intention of modifying the existing core requirement - Specify UE RRM performance test of uplink timing for NGSO for NR-NTN and IoT-NTN in the FR1-NTN bands and the corresponding LTE bands based on the above channel model - Checking if the existing RRM uplink timing requirements, can be met under the TE-emulated channel model without intention of modifying the existing core requirement - Specify UE demodulation performance requirement(s) of PDSCH for NGSO for NR-NTN and IoT-NTN in the FR1-NTN bands and the corresponding LTE bands based on the above channel model - Minimize the number of demodulation performance requirements - Specify the applicability of the tests under the TE-emulated channel model

# Discussion

According to WID, RAN4 should minimize the number of demodulation performance requirements, we suggest applying the channel model with varying Doppler and delay shifts to some of existing NR-NTN and IoT-NTN PDSCH requirements.

In Rel-17, RAN4 introduced NR-NTN demodulation requirements in TS38.101-5, which are shown in Table 1.

Table 1: PDSCH requirements for NR-NTN

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | R.PDSCH.1-1.1 FDD | 10 / 15 | QPSK, 0.30 | NTN-TDLA100-200 | 1x2, ULA Low | 70 | 0.3 |
| 1-2 | R.PDSCH.1-2.1 FDD | 10 / 15 | 16QAM, 0.48 | NTN-TDLC5-200 | 1x2, ULA Low | 70 | 7.6 |
| 1-3 | R.PDSCH.1-1.1 FDD | 10 / 15 | QPSK, 0.30 | NTN-TDLC5-200 | 1x2, ULA Low | 70 | -0.4 |
| 1-4 | R.PDSCH.1-1.1 FDD(1) | 10 / 15 | QPSK, 0.30 | NTN-TDLA100-200 | 1x2, ULA Low | 70 | 1.1 |
| Note1: The Maximum throughput is based on the HARQ processes with HARQ feedback enabled. |  |  |  |  |  |  |  |

We propose to consider only test 1-1 and 1-2 as tests 1-3 and 1-4 need additional UE capabilities. Test 1-3 targets for 32 HARQ process which needs max-HARQ-ProcessNumber-r17 to be 32 and Test 1-4 targets for disable HARQ needs harq-FeedbackDisabled-r17.

As for IoT-NTN PDSCH requirements, RAN4 introduced requirements for Cat-M1 and NB-IoT in Rel-18, which are shown in Table 2 and 3. There is no conformance test introduced in RAN5 for IoT-NTN Cat-M1. Also, we believe that IoT-NTN NB-IoT is the more likely to be commercially compared to IoT-NTN Cat-M1. Therefore, we prefer not to apply time-varying Doppler shift and propagation delay model to Cat-M1. As for NB-IoT, based on the minimum testing time from RAN5, the minimum testing time for test 1 is around 244 seconds, which can be covered in TE-emulated channel is 30 degrees evaluation angle is considered. However, the minimum testing time for test 1 is around 602 seconds, which cannot be covered by TE-emulated channel simulation time.

Table 2: PDSCH requirements for IoT-NTN (Cat-M1)

| Test number | Bandwidth and MCS | Reference Channel | OCNG Pattern | Propagation Condition | Correlation Matrix and Antenna Configuration | Reference value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |  |
| 1 | 1.4MHz 16QAM 1/2 | R.1 FDD | OP.1 FDD | NTN-TDLC5-30 | 1x1 | 70 | 10.4 | M1 |
| 2 | 1.4MHz QPSK 1/3 | R.2 FDD | OP.1 FDD | NTN-TDLA100-200 | 1x1 | 70 | -4.2 | M1 |
| 3 | 1.4MHz QPSK 1/10 | R.3 FDD | OP.1 FDD | NTN-TDLA100-10 | 1x1 | 70 | -11.5 | M1 |

Table 3: PDSCH requirements for IoT-NTN (NB-IoT)

| Test number | Bandwidth | Carrier Type | Reference Channel | Repetition number | Propagation condition | Number of NRS ports | Antenna Configuration | Reference value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |  |
| 1 | 200kHz | Anchor | R.NB.1 FDD | 32 | NTN-TDLC5-200 | 1 | 1x1 | 70% | -4.7 | NB1, NB2 |
| 2 | 200kHz | Non-anchor | R.NB.2 FDD | 128 | NTN-TDLA100-10 | 1 | 1x1 | 70% | -10.6 | NB1, NB2 |

Observation 1: RAN4 should minimize the number of demodulation performance requirements according to the WID.

Proposal 1: For NR-NTN, only apply time-varying Doppler shift and propagation delay model to test 1-1 and 1-2 in TS 38.101-5 Table 8.2.1.2.2.1.1-3.

Proposal 2: For IoT-NTN, do not consider Cat-M1 as there is no conformance test defined in RAN5.

Proposal 3: For IoT-NTN, only apply time-varying Doppler shift and propagation delay model to test 1 in TS36.102 Table 8.3.1.1.1.1-2

# Conclusion

In this contribution, we provide our views on demodulation performance requirements for NGSO channel model. Observations and proposals are summarized as below.

Observation 1: RAN4 should minimize the number of demodulation performance requirements according to the WID.

Proposal 1: For NR-NTN, only apply time-varying Doppler shift and propagation delay model to test 1-1 and 1-2 in TS 38.101-5 Table 8.2.1.2.2.1.1-3.

Proposal 2: For IoT-NTN, do not consider Cat-M1 as there is no conformance test defined in RAN5.

Proposal 3: For IoT-NTN, only apply time-varying Doppler shift and propagation delay model to test 1 in TS36.102 Table 8.3.1.1.1.1-2

# Reference

- RP-242901, Revised WID on Enhanced requirements and conductive test methodology for NR NTN and IoT NTN, Apple, Thales, Inmarsat, Viasat, Globalstar