3GPP TSG-RAN WG4 Meeting#114bis                                R4-2504358

Wuhan, China, 07th – 11th April, 2025

Agenda item:	7.13.3

Source: 	ZTE Corporation, Sanechips

Title: 	Further discussion on OTA test enhancement

Document for:	Approval

Introduction

In the RAN#103 meeting, new WID for Rel-19 BS RF evolution has been approved and it’s expected to start the normative work from this April meeting. In last RAN4 meeting, we have some initial discussions on the necessity of improvement of co-location issues. In this contribution, we would like to share further views on the OTA co-location requirements.

Sub-topic 1-1: BS-to-BS isolation analysis Way forward: BS-to-BS isolation data collection to prioritize the following scenarios: - Horizontal domain prioritized (vertical BS stacking analyses not precluded); - Same-band analysis (inter-band analysis not precluded), - Array-to-array case (array to sub-array not precluded); - Same BS type for aggressor and victim (mixed cases of various BS types not precluded). Guidance for collection of future results: Companies to clarify on the statistical distribution of the results provided (i.e., worst case, CDF, etc.) For the derivation of the conclusion, [95-99th] percentile of CDF is expected to be used for simulations, or the worst-case results from measurements. BS-to-BS isolation data (simulations, or measurements) to be collected on top of the data already collected in the following table formats: - No beam steering consideration; same band; same BS type; A-to-A (unless otherwise stated): 2.6 GHz; 3.5 GHz; 4.9 GHz; 7 GHz. Isolation in vertical domain (dB); ZTE (M, R4-2501875): -54; ZTE (R4-2501875): -52; ZTE (M, R4-2501875): -56. Isolation in horizontal domain (dB); ZTE (M, R4-2501875): -31; ZTE (M, R4-2501875): -34 Huawei (M, R4-2502135): -46; ZTE (M, R4-2501875): -46. - S: simulations - M: measurements - A-to-A: array to array coupling - A-to-SA: array to sub-array coupling. - Beam steering consideration; same band; same BS type; A-to-A (unless otherwise stated): 2.6 GHz; 3.5 GHz; 4.9 GHz; 6.7 GHz --> 7 GHz. Isolation in vertical domain (dB). Isolation in horizontal domain (dB); S: Ericsson (R4-2418398): A-to-SA: -55±20 dB; S: Ericsson (R4-2502160): A-to-A: -77±25; A-to-SA: -70±20. For the deprioritized scenarios, reuse table templates as above. - Timeline for BS-to-BS isolation data collection: conclusion on the BS-to-BS isolation analysis to be reached no later than August 2025 meeting (RAN4#116). - Data collection for other carrier frequencies is not precluded. - Any future conclusion on isolation numbers is subject to availability of sufficient amount of data from multiple companies and can be decided case-by-case. Sub-topic 1-2: Specifications set baseline Way forward: Baseline set of specifications for implementation of the BS-BS isolation conclusions: - TS 37.105 - TS 37.145-1/-2 - TS 38.104 - TS 38.141-1/-2 - TS 38.174 - TS 38.176-1/-2 - TS 38.106 - TS 38.115-1/-2 Sub-topic 1-3: Requirements set baseline Way forward: Baseline set of requirements for implementation of the BS-BS isolation conclusions: - Transmitter OFF power requirement - Co-location spurious emission requirement/Protection of the BS receiver of own or different BS - Tx IMD requirement - Co-located OOBB requirement Sub-topic 1-4: TX IMD co-location Way forward: Companies are encouraged to provide inputs on the handling of the intra-system TX IMD requirements. Sub-topic 1-5: Capturing CLTA-related agreements Way forward: As there is a dedicated CLTA section in TR 37.941 on RF conformance testing background for OTA BS requirements, consider TR 37.941 as the placeholder for the CLTA-related agreements collection. Rel-19 findings to be ca in a brand new sub-clause. Sub-topic 1-6: Horn antenna aspects Way forward: Companies to investigate applicable frequency range for horn antenna studies (i.e., where the reference CLTA may not be available). Sub-topic 1-7: Assumptions for the beam steering scenario analyses Way forward: To improve simulation results alignment, the following assumptions are taken for the beam steering consideration: Data collection to prioritize the following scenarios: - Beam steering range +/- [60] degrees - Reuse CLTA arrangements, e.g., same plane orientation; 10 cm offset enclosure to enclosure, etc.

Discussion

As we discussed in the last RAN4 meeting, the following issues could be further discussed to identify the necessity for co-location reference antenna improvement.

  - .Coupling loss between two AAS BS in FR1 high bands

First of all, NR BS coexisting with non-AAS BS in FR1 high bands are not very common. Instead, two AAS BS coexisting with each other are more ubiquitous which are one candidate issue captured in the agreed WF [1]. Based on the above consideration from the practical deployment perspective, we conducted some coupling loss measurement shown as following:

- (left figure) Two AAS BS are separated in the vertical domain (10cm separation as worst assumption).

- (Right figure) Two AAS BS are separated in the horizontal domain (10cm separation as worst assumption).

Figure 2.1-1. coupling loss measurement for FR1 high bands with separation distance as 10cm from edge to edge

NOTE: coupling loss measurement with CW source signal is done in the OTA chamber without considering the different beam steering steering. Beam of two AAS BSs are steering towards the boresight direction.

Figure 2.1-2a. antenna ports for coupling loss measurement with vertical separation [expected worst coupling loss]

Figure 2.1-2b. antenna ports for coupling loss measurement with horizontal separation [expected worst coupling loss]

According to the above testing conditions/assumptions, some initial measurement results for FR1 high bands are provided in the following Table 2.1-1. As shown in the following table, if two NR AAS BS are placed with vertical separation which is quite typical for the mast installation, then the coupling loss between sub-array would be larger than 50dB under the vertical isolation (closet antenna elements as shown in Figure 2.1-2a). However if two NR BS are placed with horizontal separation which is typical for roof based installation, the coupling loss between sub-array is still larger than 30dBc (closet antenna elements as shown in Figure 2.1-2b). It also depends on the BS’s implementation, for different BS RF architecture, coupling loss among different antenna ports are varying from 31dB to 46dBc.

Table 2.1-1. summary of measured coupling loss different frequency ranges for FR1 WA BS

| Antenna type | 2.6G-BS1 | 2.6G-BS2 | 3.5G-BS1 | 3.5G-BS2 | 4.9G-BS1 |
| --- | --- | --- | --- | --- | --- |
| Isolation in vertical domain(dB) | <-54 | <-57 | <-52 | <-53 | <-56 |
| Isolation in horizontal domain | <-31 | <-40 | <-34 | <-35 | <-46 |

Observation 1:if two NR AAS BS are placed with vertical separation which is quite typical for the mast installation, then the coupling loss between sub-array would be larger than 50dB with10cm separation distance.

Observation 2: if two NR BS are placed with horizontal separation which is typical for roof based installation, the coupling loss is still larger than 30dBc and the coupling loss among different antenna ports are varying from 31dB to 46dBc.

In the last RAN4 meeting, there were some discussions on the impacts of beam steering on coupling loss, from our understanding, it’s true that the beam steering from antenna array of aggressive BS towards sub-array of victim BS and from antenna array of aggressive BS towards antenna array of victim BS would have the impacts. More measurement results or simulation results by EM software platform could be provided later on to figure out the exact coupling loss in different deployment scenario.

  - . The set of impacted specific requirement

In the previous discussions, there were some discussions on the impacted specific requirement, we would like to review the background for the following requirement firstly.

### 2.2.1.Transmitter OFF power requirement

For NR BS transmit OFF power is defined in terms of PSD in 1MHz, calculated as following:

[Transmit OFF power] < [Thermal noise] + [Noise Figure] – 6dB + [Minimum coupling loss (MCL)] + 10log[1MHz]

< -174 + 5 – 6 + MCL+60

< -115dBm/MHz + MCL

Considering the worst case is that eNBs are co-located, the minimum coupling loss(MCL) is assumed 30dB. The transmit OFF power is -85dBm/MHz.

### 2.2.2.Co-location spurious emission requirement/Protection of the BS receiver of own or different BS

- The Spurious Emission Level of WA BS should be:

- Spurious Emission LevelWA ＜ kT0 + NFWA  + 10*log10(100kHz) －7dB + MCL

- = -174dBm/Hz + 5dB + 50dB－7dB + MCL

- Where :

- MCL refer to the minimum coupling loss which is 30dB in the past for 3GPP.

- The BS of different classes will have different Noise figures. Generally, in most 3GPP spec, we assume that   the WA NF = 5dB, MR NF = 10dB, and LA NF = 13dB.

### 2.2.3.Tx IMD requirement

- Interfering signal level is equal to Rated total output power (Prated,t,AC) in the operating band – 30 dB where MCL refer to the minimum coupling loss which is 30dB in the past for 3GPP.

Table 6.7.2.1-1: Interfering and wanted signals for the co-location transmitter intermodulation requirement

| Parameter | Value |
| --- | --- |
| Wanted signal type | NR single carrier, or multi-carrier, or multiple intra-band contiguously or non-contiguously aggregated carriers, with NB-IoT operation in NR in-band if supported. |
| Interfering signal type | NR signal, the minimum BS channel bandwidth (BWChannel) with 15 kHz SCS of the band defined in clause 5.3.5. |
| Interfering signal level | Rated total output power (Prated,t,AC) in the operating band – 30 dB |
| Interfering signal centre frequency offset from the lower/upper edge of the wanted signal or edge of sub-block inside a sub-block gap | , for n=1, 2 and 3 |
| NOTE 1: Interfering signal positions that are partially or completely outside of any downlink operating band of the base station are excluded from the requirement, unless the interfering signal positions fall within the frequency range of adjacent downlink operating bands in the same geographical area. In case that none of the interfering signal positions fall completely within the frequency range of the downlink operating band, TS 38.141-1 [5] provides further guidance regarding appropriate test requirements. NOTE 2: In Japan, NOTE 1 is not applied in Band n77, n78, n79. |  |

### 2.2.4.Co-located OOBB requirement

- Interfering signal level for WA, MR and LA is based on the following calculation assumption:

- 46dBm-MCL;

- 38dBm-MCL;

- 24dBm-MCL;

- Where MCL refer to the minimum coupling loss which is 30dB in the past for 3GPP.

Table 7.5.3-1: Blocking performance requirement for NR BS when co-located with BS in other frequency bands.

| Frequency range of interfering signal | Wanted signal mean power for WA BS (dBm) | Interfering signal mean power for WA BS (dBm) | Interfering signal mean power for MR BS (dBm) | Interfering signal mean power for LA BS (dBm) | Type of interfering signal |
| --- | --- | --- | --- | --- | --- |
| Frequency range of co-located downlink operating band | PREFSENS +6dB (Note 1) | +16 | +8 | x (Note 2) | CW carrier |
| NOTE 1: PREFSENS depends on the BS channel bandwidth as specified in Table 7.2.2-1, 7.2.2-2, and 7.2.2-3. NOTE 2: x = -7 dBm for NR BS co-located with Pico GSM850 or Pico CDMA850 x = -4 dBm for NR BS co-located with Pico DCS1800 or Pico PCS1900 x = -6 dBm for NR BS co-located with UTRA bands or E-UTRA bands or NR bands NOTE 3: The requirement does not apply when the interfering signal falls within any of the supported uplink operating band(s) or in ΔfOOB immediately outside any of the supported uplink operating band(s). |  |  |  |  |  |

The concept of co-location reference antenna is defined due to the measurement capability issue in the OTA chamber. E..g. -85dBm/MHz OFF power,  -96dBm/100KHz co-location spurious emission requirement is too low to be measured in the OTA chamber, using the alternative co-location reference antenna could help replace OTA measurement as the conducted interface measurement with much better measurement capability.

For receiver OOBB requirement, to generate the CW signal with 46dBm/38dBm is also very challenging somehow, to use the co-location reference antenna could also alleviate the measurement risk.

During the last RAN4 meeting, we have already agreed to consider the following requirement based on the MCL results, from our understanding, if we could focus on the MCL within single band analysis, then the transmitter OFF power and Tx IMD requirement should be treated as higher priority.

- Transmitter OFF power requirement

- Co-location spurious emission requirement/Protection of the BS receiver of own or different BS

- Tx IMD requirement

- Co-located OOBB requirement

- Proposal 1: if RAN4 agrees to focus on the MCL analysis within the same band in Rel-19, propose to consider  Tx OFF power and Tx IMD requirement only.

  - .Difference between AAS BS vs AAS BS and AAS vs co-location reference antenna

In the previous discussions, one of the critical issue is to identify the difference between AAS vs AAS BS and AAS vs co-location reference antenna. In the following section,we would like to provide the further analysis from both transmitter and receiver requirements perspective.

| Clause number | Requirement | Co-location reference antenna operation | Type |
| --- | --- | --- | --- |
| 6.5 | OTA transmit ON/OFF power for FR1 | Measure emission | Mandatory |
| 6.7.5.3 6.7.5.5 | OTA spurious emission: Protection of the BS receiver of own or different BS | Measure emission | Optional based on declaration |
|  | OTA spurious emission: Co-location with other base stations |  |  |
| 6.8 | OTA transmitter intermodulation | Inject the interferer signal | Mandatory |
| 7.6.3 | OTA out-of-band blocking: Co-location with other base stations | Inject the interferer signal | Optional based on declaration |

Figure 2.3-1. the legacy co-location reference antenna for OFF power and spurious emission requirement

For the legacy co-location reference antenna, the received power at co-location reference antenna output should be Ptx+G_Tx-Coupling_loss+G_Rx_CLTA.

Figure 2.3-2. the proposed new co-location reference antenna for OFF power and spurious emission requirement

For the proposed new co-location reference antenna for FR1 high bands, received power at co-location reference antenna output should be Ptx+G_Tx-Coupling_loss+G_Rx_sub-array.

The main difference for OFF power and spurious emission measurement would be G_Rx_CLTA and G_Rx_sub-array pattern. These two received gain for passive antenna of CLTA and AAS sub-array would be quite different. In other words, the measured spurious emission or transmitter OFF power from CLTA cannot be representative for AAS BS co-locating with AAS BS at sub-array level.

Observation 3: These two received gain for passive antenna of CLTA and AAS sub-array would be quite different. In other words, the measured spurious emission or transmitter OFF power from CLTA cannot be reused or applicable for AAS BS co-locating with AAS BS at sub-array level.

Figure 2.3-3. the legacy co-location reference antenna for Tx intermodulation and co-location OOBB requirement

For the legacy co-location reference antenna, the received power at sub-array of AAS BS should be Ptx+G_Tx_CLTA -Coupling_loss+G_Rx_sub_array

Figure 2.3-4 the proposed new co-location reference antenna for Tx intermodulation and co-location OOBB requirement

For the proposed new co-location reference antenna for FR1 high bands, received power at sub-array of AAS BS should be Ptx+G_Tx_AAS -Coupling_loss+G_Rx_sub_array.

The main difference for Tx intermodulation and co-location reference antenna would be G_Tx_CLTA and G_Tx_AAS antenna pattern. These two passive antenna and AAS BS pattern would be quite different. G_Tx_CLTA should provide much more stringent requirement for it since it provides the much wider beam in azimuth plane towards the victim AAS BS with narrow beam. In other words, from Tx intermodulation and co-location OOBB requirement, it seems that the legacy CLTA requirement would provide more stringent conformance testing for it.

Observation 4: These two passive antenna and AAS BS pattern would be quite different, from Tx intermodulation and co-location OOBB requirement, it seems that the legacy CLTA requirement would provide more stringent conformance testing for it.

Conclusions

In this contribution, we shared some further views on co-location reference antenna concept in RAN4 and proposals are made as following:

Observation 1:if two NR AAS BS are placed with vertical separation which is quite typical for the mast installation, then the coupling loss between sub-array would be larger than 50dB with10cm separation distance.

Observation 2: if two NR BS are placed with horizontal separation which is typical for roof based installation, the coupling loss is still larger than 30dBc and the coupling loss among different antenna ports are varying from 31dB to 46dBc.

- Proposal 1: if RAN4 agrees to focus on the MCL analysis within the same band in Rel-19, propose to consider  Tx OFF power and Tx IMD requirement only.

Observation 3: These two received gain for passive antenna of CLTA and AAS sub-array would be quite different. In other words, the measured spurious emission or transmitter OFF power from CLTA cannot be reused or applicable for AAS BS co-locating with AAS BS at sub-array level.

Observation 4: These two passive antenna and AAS BS pattern would be quite different, from Tx intermodulation and co-location OOBB requirement, it seems that the legacy CLTA requirement would provide more stringent conformance testing for it.

References

- RP-240829,New WID: WI resulting from discussion on BS RF requirement evolution, Approved.

- R4-2319398, Background and technical challenges with BS RF co-location requirements in relation to development of NCR RF co-location requirements, Ericsson, Noted

- R4-1703389, “On BS to BS co-location/co-siting background”, Ericsson

- R4-1706766,Co-location isolation between different frequency bands, Ericsson, Noted

- R4-2406088,Way Forward for [110bis][311] NR_BS_RF, ZTE, approved.

- R4-2404259,OTA test enhancements - CLTA, Nokia

- R4-2409955, Way forward for [111][311] NR_BS_RF, Approved.

- R4-2416574,Way Forward for [112bis][303] NR_BS_RF_Part2_CLTA, Huawei, approved.

- R4-2419797,Way Forward for [113][303] NR_BS_RF_Part2_CLTA, Approved

- RP-243270,Revised WID: NR base station (BS) RF requirement evolution for FR1/FR2 and testing, approved.

- R4-2502279, Way Forward for [114][303] NR_BS_RF_Part2_CLTA, Huawei, approved.