3GPP TSG-RAN WG4 Meeting#114bis                                R4-2504369

Wuhan, China, 07th – 11th April, 2025

Agenda item:	7.23.2.1

Source: 	ZTE Corporation

Title: 	Discussion on potentially new requirements for SBFD operation

Document for:	Approval

Introduction

In the RAN#102 meeting, new WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD) has been approved. It’s expected to start the RAN4 meeting from April meeting. Since we have already reached good progress on the SBFD BS RF impacts in SI phase and outcome was captured in TR 38.858. In this contribution, we would like to share some further views on these remaining open issues.

- Sub-topic 3-2: In-channel adjacent subband requirements - Issue 3-2-1: In-channel adjacent subband leakage ratio: General Agreement: - New requirement for In-channel adjacent subband leakage ratio: - In-channel adjacent subband leakage ratio requirement is defined with the existing ACLR requirement in absolute PSD limit value as baseline (i.e., Table 6.6.3.2-2 in TS 38.104 for FR1 and Table 9.7.3.3-2 for FR2 in TS 38.104) - FFS the value for absolute PSD limit - FFS the naming of this requirement, by considering the absolute PSD limit used instead of the ratio. Note that the current naming in TS 38.104 is ACLR absolute limit. - Issue 3-2-2: In-channel adjacent subband leakage ratio: Configuration Way Forward: - Further study the configuration of the in-channel adjacent subband leakage ratio requirement, by taking the following table as the baseline: BS DL subband bandwidth BWSBFD,DL (MHz); BS UL subband bandwidth BWSBFD,UL (UL); Filter on UL subband and corresponding filter bandwidth; In-Channel Adjacent Subband Leakage Ratio limit. BWSBFD,DL as configured by gNB; BWSBFD,UL as configured by gNB; Square (BWSBFD,UL); TBD. - Issue 3-2-4: In-channel adjacent subband blocking requirement Way Forward: - RAN4 continue to study In-channel adjacent subband blocking requirement, by following the agreement achieved in RAN4#113.

Discussion

## 2.1. Potential adjacent sub-band leakage

During the last RAN4 meeting, there were the agreement to define the in-band leakage requirement with some specific absolute limit e.g. CACLR limit as baseline. In the following section, we would like to share some further understanding on it and share some basic ground how these requirement were defined in the past.

To use the following example non-contiguous spectrum to clarify its background how this requirement was specified in the past. In order to comply with less stringent requirement between absolute and relative ACLR limit, the minimum Wgap was agreed to make sure the 1st carrier and 2nd carrier to be available to be measured, otherwise it will fail its purpose to verify the corresponding requirement. If Wgap is not too big (e.g. wider than 60MHz for ACLR1 conformance testing in FR1), then the impacts from other carrier leakage would be also negligible from the testing perspective.

Figure 2.1.1. the example CACLR requirement for 40+40MHz case in FR1

Figure 2.1.2. the example CACLR requirement for 100+200MHz case in FR2

Based on the above consideration and typical DUD (40,20,40) or DU(80,20) configuration in FR1, the existing CACLR requirement could be also applied for DUD case and DU case, however for FR2 with DUD as (100,100, 200MHz) case, it seems that CACLR requirement for 200MHz is challenging to be met due to the narrow UL sub-band size.

Figure 2.1.3. the example CACLR requirement for 40+40MHz case in FR1

Figure 2.1.4. the example CACLR requirement for 100+100+200MHz case in FR2

Based on the above consideration and typical DUD (40,20,40) or DU(80,20) configuration in FR1, the existing CACLR requirement could be also applied for DUD case and DU case, however for FR2 with DUD as (100,100, 200MHz) case, it seems that CACLR requirement for 200MHz is challenging to be met due to the narrow UL sub-band size.

Proposal 1: propose to reuse the absolute CACLR limit as staring point for SBFD in-band carrier leakage requirement and further discuss how to address the following two cases:

- if the UL sub-band size is too narrow for the DL sub-band ACLR1 measurement compared with the legacy CACLR requirement defined in TS38.104

- absolute ACLR limit is too stringent compare with relative ACLR requirement.

## 2.2. Potential adjacent receiver requirements

During the last RAN4 meeting, there were some discussions on the necessity of in-channel adjacent sub-band blocking, it was agreed to define this requirement, however the exact value for adjacent sub-band leakage requirement is not confirmed yet. From our understanding, the DL sub-band signal from the aggressor BSs within the single carrier is orthogonal to the UL sub-band signal reception. In other words, there are no such kind of selectivity needed at all. However based on some comments received online, this DL sub-band interference might cause some nonlinearity to LNA, other RF component similar as in-band blocking signal. Based on our initial simulation results as shown in the following figures, IoT level increase due to BS2BS CLI problem in the DL sub-band region if SBFD BS is capable of receiving the whole carrier instead of UL sub-band only and without considering the BS CLI coordination within the carrier is larger than legacy 25dBc (e.g. 16dBc Iot+9dB for 16QAM demod) for FR1 ICS requirement and also 14dBc (e.g. 5dBc Iot+9dB for 16QAM demod) for FR2 ICS requirement except for FR2 WA scenario; However if so, the BS2BS CLI power level is still expected to lower than in-band blocking power level according to the following evaluation results. The absolute power for in-band blocking level could be provided later on.

FR1:

FR2-1:

Observation 1: increasing Iot level due to BS2BS CLI problem in the DL sub-band region if SBFD BS is capable of receiving the whole carrier instead of UL sub-band only and without considering the BS CLI coordination within the carrier is larger than legacy 25dBc for FR1 ICS requirement and also 14dBcfor FR2 ICS requirement except for FR2 WA scenario.

Observation 2: BS co-channel coordination .e.g. beam nulling or beam coordination could alleviate the increasing IoT level for SBFD BS reception.

Proposal 2a: if any further evaluation for in-channel blocking requirement are needed, then some beam coordination among different BSs should be considered otherwise we failed to the CLI coordination information exchange defined in RAN3.

Proposal 2b: if in-channel blocking requirement is higher than legacy in-band blocking requirement, propose to check the antenna gain of each interference link between aggressor and victim BS and also check the in-channel blocking alleviation if beam coordination among BSs are considered. [More simulation results to be provided later on]

Proposal 3: for the feasibility of in-channel blocking requirement, analog filter could be assumed for FR1 WA SBFD and no analog filter could be assumed for FR1 MR, LA SBFD and FR2 SBFD BS.

Conclusions

In this contribution, we want to share some further views on RF requirements for SBFD BS and proposals/observations are made as following:

Proposal 1: propose to reuse the absolute CACLR limit as staring point for SBFD in-band carrier leakage requirement and further discuss how to address the following two cases:

- if the UL sub-band size is too narrow for the DL sub-band ACLR1 measurement compared with the legacy CACLR requirement defined in TS38.104

- absolute ACLR limit is too stringent compare with relative ACLR requirement.

Proposal 2a: if any further evaluation for in-channel blocking requirement are needed, then some beam coordination among different BSs should be considered otherwise we failed to the CLI coordination information exchange defined in RAN3.

Proposal 2b: if in-channel blocking requirement is higher than legacy in-band blocking requirement, propose to check the antenna gain of each interference link between aggressor and victim BS and also check the in-channel blocking alleviation if beam coordination among BSs are considered.

Proposal 3: for the feasibility of in-channel blocking requirement, analog filter could be assumed for FR1 WA SBFD and no analog filter could be assumed for FR1 MR, LA SBFD and FR2 SBFD BS.

References

- RP-234035,New WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD), Huawei, Samsung, Approved.

- TR 38.858

- R4-2321091, TP to TR 38.858 Impact on BS RF requirements, ZTE, Agreed

- R4-2413514, Way Forward for [112][307] NR_duplex_evo_General, Samsung

- R4-2416584,Way forward for [112bis][306] NR_duplex_evo_General, Samsung

- R4-2502310	Way Forward for [114][306] NR_duplex_evo_General, approved.

- R4-2502380	Way Forward for [114][307] NR_duplex_evo_BSRF, approved.