3GPP TSG-RAN WG4 Meeting #114bis	R4-2504376

Wuhan, China, 7th – 11th April, 2025

Agenda Item:	7.10.2.2.1

Source:	MediaTek Inc.

Title:	Discussion on NR-NTN HPUE TX RF requirements

Document for:	Approval

- Introduction

In RAN#103 and RAN#106 plenary meetings, the WID on enhanced requirements and test methodology for NR and IoT NTN were approved in [1][2]. One of the main objectives is to enable high power UE (HPUE) with defining RF requirements targeting at support of IoT-NTN UEs for FR1 single carrier scenario.

In this contribution, our views about HPUE TX RF requirements for IoT-NTN UE are provided for discussion.

- Discussion

2.1		High power UE (HPUE) for NTN

Some of the main Rel-19 WID [2] objectives for enhanced requirements and test methodology for NR and IoT NTN are shown below for reference.

3 Justification High power UE (HPUE) for NTN Until Rel-18, the 3GPP specifications limit maximum non-terrestrial network (NTN) UE transmission (Tx) power to the level of 23dBm, which may not allow to use the full potential of satellite communication. On the other hand, power class 2 (PC2 26dBm) and power class 1.5 (PC1.5 29dBm for TDD only) are widely supported for terrestrial network (TN), and power class 1 (PC1 31dBm) is also supported for particular device for public safety. 4.1 Objective of Core part WI The objectives of core part for enhanced requirements and test methodology for NR and IoT NTN include: High power UE (HPUE) for NTN - Conduct the coexistence analysis based on TR 38.863 for the above example bands and corresponding power classes - Evaluate the impact on the uplink performance of TN (Terrestrial network) - Example bands include bands n256 and 256, n255 and 255 - PC2, PC1.5 (for NR-NTN only) and PC1 will be evaluated for ACLR and ACS - For NR-NTN and IoT-NTN: coexistence studies shall be carried out under the same conditions, including geographical separation between NTN UE and Terrestrial Networks (TN), as the co-existence studies in Rel-17 and Rel-18 - Specify high power UE (HPUE) for NR-NTN (Non-Terrestrial Networks) and IoT-NTN (NB-IoT and eMTC based NTN) in FR1-NTN bands and the corresponding LTE NTN bands for the single uplink (UL) carrier scenario - Support UE with power class 2 (PC2, 26 dBm) that applies to both handheld and non-handheld based on the co-existence study. - Support other power classes based on the co-existence and feasibility study - Non-hand held UE is prioritized - Specify RF requirements for all considered power classes for NR-NTN and IoT-NTN in FR1-NTN bands and the corresponding LTE NTN bands - Necessary Tx requirements, at least including - Investigate and if necessary, specify the solution to address potential SAR (Specific Absorption Rate restriction) requirements for handheld UE - Specify MPR and A-MPR for the example bands considering the regulation for the corresponding bands, if necessary - Necessary Rx requirements, at least including - Specify the RSD requirements on the example bands, if necessary - Specify the necessary signaling to support the above objectives

2.2		Previous Meeting Agreements

In RAN4#114 meeting, the agreed WF [2] provides the following works for specifying TX requirements for NR-NTN under HPUE operation. The partial contents are shown as follows.

NR-NTN HPUE Tx requirements Issue 2-1: NR-NTN PC2 Agreement: - Define the requirements for NR-NTN PC2 covering the following cases and try to define the common requirements as much as possible - 1Tx handheld - 2Tx handheld - 1Tx non-handheld - 2Tx non-handheld - If the difficulty is identified to define the requirements for the cases above, discuss how to down-select the cases in Rel-19 Issue 2-3: NR-NTN PC1.5 Agreement: - Define the requirements for PC1.5 for non-handheld NR-NTN UE with 2Tx in Rel-19. - FFS on other cases if time is allowed Issue 2-3: NR-NTN PC1 Agreement: - RAN4 to consider non-handheld UE with 1Tx and [4Tx] in Rel-19. Issue 2-4-1: PC2 ACLR Agreement: - PC2 ACLR is [30 or 31] dBc for NR-NTN UE Issue 2-4-2: PC1.5 ACLR Agreement: - PC1.5 ACLR is [30 or 31] dBc for NR-NTN UE Issue 2-4-3: PC1 ACLR Agreement: - PC1 ACLR is [31 or 37] dBc for NR-NTN UE - Further study averaging value, i.e. 34. Issue 2-5: SEM Agreement: - Apply SEM from NR-NTN PC3 to other PCs for NR-NTN as baseline. - Further discuss whether and how to capture the FCC 47 CFR part 25 specification in clause 202 Issue 2-6: MPR/A-MPR Agreement: - For PC 2 and PC 1.5 MPR: - Option 1: Existing MPR requirements for PC2 and PC1.5 in TS 38.101-1 apply, if PC2/PC1.5 ACLR reused TN PC2/PC1.5 ACLR as 31 dBc. - Option 2: Aim for MPR analysis for PC2 and PC1.5 if PC/PC1.5 ACLR agreed as 30 dBc. - For PC 1 MPR: - Option 1: If PC1 ACLR reused TN PC1 ACLR as 37 dBc: - Option 1-1: Existing MPR requirements for PC1 in TS 38.101-1 applies - Option 1-2: More analysis is needed to revisit the MPR requirements from TN spec. - Option 2: Aim for MPR analysis for PC1 for other ACLR values [31 or 34] dBc - Aim for MPR/A-MPR analysis in #114bis for different power classes - For PA model calibration: - DFT-s-OFDM QPSK 20 MHz - 100RB0 - 4dB post PA loss - 1dB MPR - Carrier leakage 28dBc - IQ image:28 dBc - CIM3: 60 dBc - ACLR: PC2 [31]dB, PC1.5 [31]dB, PC1 [31]dB - SEM: existing PC3 SEM - EVM: 17.5%

2.3		UE RF requirements

2.3.1	NR-NTN PC2 HPUE

Proposal 1: Regarding NR-NTN PC2 HPUE, 1Tx MPR Table is not reused to become 2Tx MPR table.

Proposal 2: Regarding NR-NTN PC2 HPUE, discuss the TX ACLR and MPR requirements as shown below.

| NR-NTN PC2 HPUE | ACLR (dBc) | MPR |
| --- | --- | --- |
| 1Tx handheld | 31 | Existing MPR requirements for PC2 in TS 38.101-1 Table 6.2.2-2 apply |
| 2Tx handheld (2 PC3-PAs) | 30 | Existing MPR requirements for PC2 in TS 38.101-1 Table 6.2D.2-1 apply |
| 1Tx non-handheld | 31 | Existing MPR requirements for PC2 in TS 38.101-1 Table 6.2.2-2 apply |
| 2Tx non-handheld (2 PC3-PAs) | 30 | Existing MPR requirements for PC2 in TS 38.101-1 Table 6.2D.2-1 apply |

2.3.2	NR-NTN PC1.5 HPUE

Proposal 3: Regarding NR-NTN PC1.5 HPUE, discuss whether the TX ACLR and MPR requirements as shown in table below could be used.

| NR-NTN PC2 HPUE | ACLR (dBc) | MPR |
| --- | --- | --- |
| 2Tx non-handheld (2 PC2-PAs) | 31 | Existing MPR requirements for PC1.5 in TS 38.101-1 Table 6.2D.2-3 apply |

2.3.3	NR-NTN PC1 HPUE

Observation 1: As indicated in NTN HPUE WID[2], power class 1 (1Tx PC1 31dBm) is supported for particular device for public safety for terrestrial network (TN).

Proposal 4: Regarding NR-NTN PC1 HPUE, 1Tx MPR Table is not reused to become 4Tx MPR table.

Proposal 5: Regarding NR-NTN PC1 HPUE, discuss the TX ACLR and MPR requirements as shown below.

Table 1: NR-NTN PC1 non-handheld HPUE ACLR and MPR requirements

| NR-NTN PC1 HPUE | ACLR (dBc) | MPR |
| --- | --- | --- |
| 1Tx non-handheld (Particular device) | 37 | Existing MPR requirements for PC1 in TS 38.101-1 Table 6.2.2-4b apply |
| 4Tx non-handheld (4 PC2-PAs) | [31] | TBD |

Observation 2: It is expected that PC1 4Tx MPR would be inferior to PC1 1Tx MPR.

3 Conclusion

In this paper, our views on HPUE TX RF requirements for NR-NTN UE are provided as follows:

Proposal 1: Regarding NR-NTN PC2 HPUE, 1Tx MPR Table is not reused to become 2Tx MPR table.

Proposal 2: Regarding NR-NTN PC2 HPUE, discuss the TX ACLR and MPR requirements as shown below.

| NR-NTN PC2 HPUE | ACLR (dBc) | MPR |
| --- | --- | --- |
| 1Tx handheld | 31 | Existing MPR requirements for PC2 in TS 38.101-1 Table 6.2.2-2 apply |
| 2Tx handheld (2 PC3-PAs) | 30 | Existing MPR requirements for PC2 in TS 38.101-1 Table 6.2D.2-1 apply |
| 1Tx non-handheld | 31 | Existing MPR requirements for PC2 in TS 38.101-1 Table 6.2.2-2 apply |
| 2Tx non-handheld (2 PC3-PAs) | 30 | Existing MPR requirements for PC2 in TS 38.101-1 Table 6.2D.2-1 apply |

Proposal 3: Regarding NR-NTN PC1.5 HPUE, discuss whether the TX ACLR and MPR requirements as shown in table below could be used.

| NR-NTN PC2 HPUE | ACLR (dBc) | MPR |
| --- | --- | --- |
| 2Tx non-handheld (2 PC2-PAs) | 31 | Existing MPR requirements for PC1.5 in TS 38.101-1 Table 6.2D.2-3 apply |

Observation 1: As indicated in NTN HPUE WID[2], power class 1 (1Tx PC1 31dBm) is supported for particular device for public safety for terrestrial network (TN).

Proposal 4: Regarding NR-NTN PC1 HPUE, 1Tx MPR Table is not reused to become 4Tx MPR table.

Proposal 5: Regarding NR-NTN PC1 HPUE, discuss the TX ACLR and MPR requirements as shown below.

Table 1: NR-NTN PC1 non-handheld HPUE ACLR and MPR requirements

| NR-NTN PC1 HPUE | ACLR (dBc) | MPR |
| --- | --- | --- |
| 1Tx non-handheld (Particular device) | 37 | Existing MPR requirements for PC1 in TS 38.101-1 Table 6.2.2-4b apply |
| 4Tx non-handheld (4 PC2-PAs) | [31] | TBD |

Observation 2: It is expected that PC1 4Tx MPR would be inferior to PC1 1Tx MPR.

4 Reference

- RP-240857, “New WID: Enhanced requirements and test methodology for NR and IoT NTN”, Huawei

- RP-242901, “Revised WID on Enhanced requirements and conductive test methodology for NR NTN and IoT NTN”, Apple, Thales, Inmarsat, Viasat, Globalstar

- R4-2502954, “WF on HPUE requirements for NR-NTN”, Samsung