3GPP TSG-RAN WG4 Meeting #114-bis	R4-2504384

# Wuhan, China, April 7th – 11st 2025

Source:	Ericsson

Title:	General view on ATG enhancement BS demodulation requirements

Agenda item:	7.12.6

Document for:	Discussion

# Introduction

In 3GPP release 18, RAN4 has completed the standardization work for the air-to-ground (ATG) network WI, the main work focused on the FR1 single carrier scenario. To enhance the throughput to meet growing ATG capacity demand and fully utilize operators’ spectrum, the RAN plenary meeting #103 has approved a new WI on enhancements for the air-to-ground network for NR [1].

### 4.2 Objective of Performance part WI NOTE: Leave empty if the WI proposal does not contain a RAN performance part. The performance part of the work item includes: - Specify corresponding RRM performance and demodulation requirements for intra-band co-located and inter-band co-located DL CA. - Specify corresponding demodulation requirements for support of UL MIMO with 2TX for single CC for UE with dual-polarization antenna. - Specify demodulation requirements for support of DL MIMO for UE with limited test cases. - Specify conformance testing and conformance requirements for BS.

In this paper, we provided our views on how to define UE demodulation requirements for intra-band co-located and inter-band co-located DL CA enhancement, and the necessary of UL MIMO 2Tx for BS demodulation requirements.

# Discussion

Air-to-ground (ATG) networks provide in-flight connectivity by utilizing ground-based cellular towers ("ATG BS") to transmit signals to an aircraft’s onboard ATG terminal ("ATG UE"). As the aircraft moves through different airspace regions, the onboard ATG terminal dynamically connects to the cell with the signal depending on the traffic with the strongest signal, much like how a mobile phone switches between towers on the ground. This system establishes a direct radio link between the ground base station and the CPE-type user equipment installed in the aircraft.

Due to its advantages, including high data throughput, low propagation delay, and cost-effectiveness, ATG has attracted significant interest from operators and the aviation industry, leading to strong demand for its deployment. In May 2023, China’s Ministry of Industry and Information Technology allocated a 4.9 GHz trial spectrum to CMCC for ATG technology testing.

## 2.1 	UE demodulation

Within Release 18, RAN4 initiated standardization efforts specifically for the FR1 single-carrier scenario. Moving into Release 19, two key aspects are being addressed:

- Improving throughput to meet ATG capacity requirements.

- Maximizing the utilization of operators’ spectrum, particularly fragmented spectrum resources.

To facilitate the aspects above, one objective of performance part W (as per the WID [1]) for the DL is to specify corresponding demodulation requirements for support of intra-band co-located and inter-band co-located DL CA. Test configurations for single carrier ATG requirements can be reused:

- Antenna: 2x2, 2x4

- Propagation condition: AWGN + Doppler

- MIMO: 1 layer

Following the structure of the CA chapter in TS38.101-4, we propose to define ATG CA requirements with the following cases (take 2Rx as an example; cover both 2Rx and 4Rx):

Table 5.2A.2.x-1: Single carrier performance for FDD 15 kHz SCS for CA configurations

| Bandwidth (MHz) | Reference channel | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 5 | TBD | 16QAM, 0.48 | AWGN+220Hz | 2x2 | 70 |  |
| 10 | TBD | 16QAM, 0.48 | AWGN+220Hz | 2x2 | 70 |  |
| 15 | TBD | 16QAM, 0.48 | AWGN+220Hz | 2x2 | 70 |  |
| 20 | TBD | 16QAM, 0.48 | AWGN+220Hz | 2x2 | 70 |  |
| 25 | TBD | 16QAM, 0.48 | AWGN+220Hz | 2x2 | 70 |  |
| 30 | TBD | 16QAM, 0.48 | AWGN+220Hz | 2x2 | 70 |  |
| 35 | TBD | 16QAM, 0.48 | AWGN+220Hz | 2x2 | 70 |  |
| 40 | TBD | 16QAM, 0.48 | AWGN+220Hz | 2x2 | 70 |  |
| 45 | TBD | 16QAM, 0.48 | AWGN+220Hz | 2x2 | 70 |  |
| 50 | TBD | 16QAM, 0.48 | AWGN+220Hz | 2x2 | 70 |  |

Table 5.2A.2.x-2 Single carrier performance for TDD 15 kHz SCS for CA configurations

| Bandwidth (MHz) | Reference channel | Modulation format and code rate | Propagation condition | antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 5 | TBD | 16QAM, 0.48 | AWGN+500Hz | 2x2 | 70 |  |
| 10 | TBD | 16QAM, 0.48 | AWGN+500Hz | 2x2 | 70 |  |
| 15 | TBD | 16QAM, 0.48 | AWGN+500Hz | 2x2 | 70 |  |
| 20 | TBD | 16QAM, 0.48 | AWGN+500Hz | 2x2 | 70 |  |
| 25 | TBD | 16QAM, 0.48 | AWGN+500Hz | 2x2 | 70 |  |
| 30 | TBD | 16QAM, 0.48 | AWGN+500Hz | 2x2 | 70 |  |
| 40 | TBD | 16QAM, 0.48 | AWGN+500Hz | 2x2 | 70 |  |
| 50 | TBD | 16QAM, 0.48 | AWGN+500Hz | 2x2 | 70 |  |

Table 5.2A.2.x-3 Single carrier performance for TDD 30 kHz SCS for CA configurations

| Bandwidth (MHz) | Reference channel | Modulation format and code rate | Propagation condition | antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 5 | TBD | 16QAM, 0.48 | AWGN+500Hz | 2x2 | 70 |  |
| 10 | TBD | 16QAM, 0.48 | AWGN+500Hz | 2x2 | 70 |  |
| 15 | TBD | 16QAM, 0.48 | AWGN+500Hz | 2x2 | 70 |  |
| 20 | TBD | 16QAM, 0.48 | AWGN+500Hz | 2x2 | 70 |  |
| 25 | TBD | 16QAM, 0.48 | AWGN+500Hz | 2x2 | 70 |  |
| 30 | TBD | 16QAM, 0.48 | AWGN+500Hz | 2x2 | 70 |  |
| 40 | TBD | 16QAM, 0.48 | AWGN+500Hz | 2x2 | 70 |  |
| 50 | TBD | 16QAM, 0.48 | AWGN+500Hz | 2x2 | 70 |  |
| 60 | TBD | 16QAM, 0.48 | AWGN+500Hz | 2x2 | 70 |  |
| 80 | TBD | 16QAM, 0.48 | AWGN+500Hz | 2x2 | 70 |  |
| 90 | TBD | 16QAM, 0.48 | AWGN+500Hz | 2x2 | 70 |  |
| 100 | TBD | 16QAM, 0.48 | AWGN+500Hz | 2x2 | 70 |  |

Proposal 1	Define CA requirements for ATG by reusing the configurations of Rel-18 ATG demodulation requirements, covering FDD, TDD and 2Rx, 4Rx.

Regarding the CA combination, according to the WID, intra-band co-located and inter-band co-located DL CA. This means we can assume following three combinations as same as the existing CA demodulation requirements: FDD 15kHz + FDD 15kHz, TDD 30kHz + TDD 30kHz, FDD 15kHz + TDD 30KHz.

Proposal 2	Define CA requirements with FDD 15kHz + FDD 15kHz, TDD 30kHz + TDD 30kHz, and FDD 15kHz + TDD 30kHz.

Another objective is to specify demodulation requirements for support of DL MIMO for UE with limited test cases.

RAN4 only considered 1 layer for ATG PDSCH demodulation requirements in the previous release since companies had concern regarding potential inter-layer interference brought by multiple layers transmission due to the agreed high correlation channel. Therefore, we propose to evaluate the performance of 2 layers first, configuring with both 2Rx and 4Rx.

Followings are the example cases for initial evaluation:

| Test num. | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Frequency offset | antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1 | 10 / 15 | 16QAM, 0.48 | AWGN (Note 1) | 220Hz | 2x2 | 70 |  |
| 2 | 10 / 15 | 64QAM, 0.65 | AWGN (Note 1) | 220Hz | 2x2 | 70 |  |
| 3 | 10 / 15 | 256QAM, 0.82 | AWGN (Note 1) | 220Hz | 2x2 | 70 |  |
| 4 | 10 / 15 | 16QAM, 0.48 | AWGN (Note 1) | 220Hz | 2x4 | 70 |  |
| 5 | 10 / 15 | 64QAM, 0.65 | AWGN (Note 1) | 220Hz | 2x4 | 70 |  |
| 6 | 10 / 15 | 256QAM, 0.82 | AWGN (Note 1) | 220Hz | 2x4 | 70 |  |

Note 1: Apply the static propagation condition specified in TS 38.101-4 B.1.

Proposal 3	Propose to evaluate the performance of 2-layer in the first stage, configuring 2Rx and 4Rx.

## 2.2	BS demodulation

The ATG UE normally is considered as with large antenna array mounted on aircraft. Generally, this kind of UE could support higher layer transmission easily due to possibility of low channel correlation.

In Rel-18 ATG discussion, the number of Tx/Rx for PUSCH demodulation requirements were discussed. The final agreement is that only 1Tx 2Rx is considered as “extra” test case for ATG BS on top of Rel-15 PUSCH demodulation requirements. The reason behind is that the Rel-15 requirements are defined with fading channel which could be more restrict than AWGN channel used in ATG scenario. The ATG BS performance can be secured once it passes Rel-15 requirements. To avoid no specified requirements for ATG BS, Rel-18 ATG introduced new FRC tables with higher MCS (than Rel-15).

As for 2Tx PUSCH demodulation requirements, the similar view could be followed that the 2Tx performance of ATG BS could be secured once it passed Rel-15 PUSCH requirements.

ATG BS should pass Rel-15 BS demodulation requirements which include 1Tx/2Tx antenna configurations.

The 2Tx performance could be secured if ATG BS could pass Rel-15 2Tx PUSCH demodulation requirements regarding fading channel is used in Rel-15.

The remaining issue is if new MCS should be considered as in Rel-18 discussion. Rel-15 normal PUSCH consider MCS 2/16 in Table 1 for 2Tx. In Rel-18 ATG PUSCH demodulation requirements, MCS 28 (64QAM) in Table 1 and MCS 22 (256QAM) in Table 2 are introduced regarding to high throughput and capacity demand in ATG scenario. The corresponding target SNR in the requirements for these two new MCS have been 20dB which is high for the tests. For 2Tx, the lower MCS should be used compared to 1Tx regarding to keep similar target SNR. In that case, 256QAM seems not practical for 2Tx ATG tests. Lower 64QAM MCS, such as MCS 20 with higher Rx configuration, could be considered but the benefit is very limited because both capability of 2Tx and performance of MCS 20 have been covered by previous requirements. Furthermore, only BS support high Rx could be tested with this configuration which is not a good requirement coverage.  To avoid more test effort and specification complexity, it is recommended not to consider new demodulation requirement for 2Tx PUSCH in ATG scenario.

The target SNR of 2Tx with high MCS would be very high which might not be feasible in tests.

The test coverage and applicability would not be good if consider 2Tx with high MCS with only high Rx configuration.

The benefit is very limited if consider 2Tx with low MCS since the low MCS performance have been checked by legacy requirements.

Proposal 4	Do not introduce new 2Tx PUSCH requirements for ATG scenario.

# Conclusions

In the previous sections we made the following observations:

Observation 1	ATG BS should pass Rel-15 BS demodulation requirements which include 1Tx/2Tx antenna configurations.

Observation 2	The 2Tx performance could be secured if ATG BS could pass Rel-15 2Tx PUSCH demodulation requirements regarding fading channel is used in Rel-15.

Observation 3	The target SNR of 2Tx with high MCS would be very high which might not be feasible in tests.

Observation 4	The test coverage and applicability would not be good if consider 2Tx with high MCS with only high Rx configuration.

Observation 5	The benefit is very limited if consider 2Tx with low MCS since the low MCS performance have been checked by legacy requirements.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Define CA requirements for ATG by reusing the configurations of Rel-18 ATG demodulation requirements, covering FDD, TDD and 2Rx, 4Rx.

Proposal 2	Define CA requirements with FDD 15kHz + FDD 15kHz, TDD 30kHz + TDD 30kHz, and FDD 15kHz + TDD 30kHz.

Proposal 3	Propose to evaluate the performance of 2-layer in the first stage, configuring 2Rx and 4Rx.

Proposal 4	Do not introduce new 2Tx PUSCH requirements for ATG scenario.

# References

[1] 	RP-243285: Revised WID on Enhancements for Air-to-ground network for NR, CMCC, ZTE Corporation, CATT, Huawei, HiSilicon, Verizon, China Telecom, China Unicom, RAN #106, Madrid, Spain