3GPP TSG-RAN WG4 Meeting # 114-bis														R4-2404390

Wuhan, China, 7th – 11th April, 2025

Agenda item:			7.22.2

Source:	Samsung

Title:	UE RF requirements of NR MIMO Phase 5

Document for:	Discussion

1	Introduction

RAN4 has discussed UE RF aspects of Rel-19 NR MIMO Phase 5 focusing on the objective of 3-antenna-port transmissions in the WID [1]. RAN4 has achieved considerable progress, but still has a few remaining issues which are tricky from the beginning as captured in the latest WF [2]. In this contribution, we would like to provide our views on the rest of the issues to conclude them in this meeting.

2	Discussion

2.1	Power class for 3Tx

The applicable power class for 3Tx, legacy power classes such as PC3/2/1.5 were discussed, and RAN4 has done most of power class by allowing PC3 and PC2 except PC1.5 for both FWA and handheld UE. For PC1.5, thanks to companies’ efforts on the compromise, it is supposed to be decided in this meeting as below [2].

<Sub-topic 1-1> Power class <Way forward>: PC for 3Tx requirement - For PC3 and PC2, specify requirements for FWA and handheld UE - FFS on PC1.5 with 3Tx, will be decided in the next meeting - Option 1: Consider it in the next release - Option 2: Specify requirements in Rel-19

Observation 1:	For 3Tx, RAN4 agreed to introduce PC3 and PC2 for both FWA and handheld UE, but not for PC1.5 yet.

As discussed in past meetings, PC1.5 cannot be supported at current level of maximum single PA output, e.g., 26dBm, in consideration of the mode 0. This is because, in our understanding, mode 0 also means to support a target power class with a single full rated port, which is also confirmed by the reply LS from RAN1 [3]. Moreover, as a common understanding agreed in multiple WFs before, the WI does not consider Tx diversity, which would mean that the power scaling mode with 3-port antenna transmission for PC1.5 could not be in the scope of the WI. Even though the Tx diversity is considered, based on [3], the UE needs to be capable of three 26dBm PA for single band given the equal power splitting, which is not that practical until Rel-19 at least. Given such restriction of this WI, it is desired for RAN4 to consider PC1.5 in the next release for 3Tx.

Observation 2:	Mode 0 requires a single full-rated port to meet the target power class, which is confirmed by the reply LS from RAN1. Therefore, supporting PC1.5 in this WI is unrealistic.

Proposal 1:		Considering the limitations of this WI, it is reasonable for RAN4 to consider PC1.5 in future releases for 3Tx.

2.2	MPR for 3Tx

Regarding maximum power reduction (MPR) for 3Tx, RAN4 has reached a consensus to reuse the single carrier MPR for each power class of 3Tx for PC3 and PC2 although it is not yet clear which MPR can be referred to for 3Tx PC2 between 1Tx and 2Tx. For PC1.5 MPR, it could be determined if it use the same MPR as PC1.5 with 2Tx in case PC1.5 can be introduced for 3Tx as below [2].

## <Sub-topic 1-2> MPR <Way forward>: PC2 for 3Tx - Option 1: Use the same MPR as MPR for PC2 with 2Tx, as it’s for UL-MIMO and from simulation results - Option 2: Use the same MPR as MPR for PC2 with 1Tx, as it’s the worst case

Observation 3:	RAN4 has reached a consensus to reuse the single carrier MPR for each power class of 3Tx for PC3 and PC2 although it is not yet clear which MPR can be referred to for 3Tx PC2 between 1Tx and 2Tx.

Since the gap between 1Tx and 2Tx MPR for PC2 is minimal even for higher modulation, it is reasonable to adopt the value for 2Tx for 3Tx PC2 MPR. This has been another background not to have a simulation campaign for 3Tx from the beginning in our understanding even though the value of 2Tx has also been confirmed by a simulation result from a company. Moreover, it could be nature that the MPR for multi-Tx mostly has a larger number than single-Tx given the previous discussion in RAN4. In that case, 3Tx PC2 MPR should follow the value for PC2 with 2Tx, as it is technically justified.

Proposal 2:		3Tx PC2 MPR should follow the value for PC2 with 2Tx, as it is technically justified.

2.3	Asymmetric DL sTRP/UL mTRP

For another persistent topic, asymmetric DL sTRP/UL mTRP, RAN4 had discussed whether it has impact on the UE RF requirement in Rel-19, but could not reach any consensus unfortunately. According to [2], RAN4 is recommended to make a decision between two options in this meeting as below.

<Sub-topic 2-1> UE RF impact <Way forward>: RAN4 will decide between following options in the next meeting - Option 1: No UE RF impact of asymmetric DL sTRP/UL mTRP - Option 2: Adopt following dCR, where the companies are encouraged to propose wording improvements in the next meeting ## 6.6M Beam correspondence for [UEs with different TCI states for UL and DL] For a UE that supports [[future_sTRP_2TA_capability] and the IE unifiedTCI-StateType is configured as ‘separate’, and the QCL type D reference signals associated with the UL and DL TCI states are different,] the requirements of clause 6.6 apply. Side-conditions in clause 6.6 apply to each of the TCI states.

Two umbrella options have been kept on the table from the beginning. Option 1 is not to define any FR2 UE RF requirement for the scenario, which is never discussed in RAN1 for this WI. For option 2, it is based on a proposed change of beam correspondence requirements for the asymmetric DL sTRP/UL mTRP limited to the scenario only if UE is not configured with PL offset in joint/UL TCI state(s) based on the reply LS from RAN1 as below [x].

Reply for Question 1: From the perspective of UE: if UE is configured with PL offset in joint/UL TCI state(s), UE does not expect to receive SSB from UL TRP(s), else, UE may expect to receive SSB from UL TRP(s).

Observation 4:	Option 1 is not to define any FR2 UE RF requirement for the scenario, which is never discussed in RAN1 for this WI. On the other hand, Option 2 is based on a proposed change of beam correspondence requirements for the asymmetric DL sTRP/UL mTRP limited to the scenario only if UE is not configured with PL offset in joint/UL TCI state(s).

To move forward, also to understand Option 2 correctly, it is required to look into the scenario of “else” in more details by looking back at the progress of mTRP related enhancements together with of beam correspondence requirement discussions in previous releases. The mTRP transmission with non-coherent JT was enabled from Rel-16, and continued enhanced until Rel-18 from various aspects such as mDCI, unified UL/DL TCI states, 2TA, CSI, and UL/DL RSs. In Rel-19, it has been extended to the asymmetric DL sTRP/UL mTRP deployments which include the TRP without DL signals under the mTRP scenario. In other words, once the TRP is capable of DL transmission, it could not be “UL TRP” anymore, but rather the “else TRP” enabled from previous releases, and this is an obvious reason that RAN1 has not discussed the “else” TRP in Rel-19.

Observation 5:	mTRP transmission with non-coherent JT was enabled from Rel-16, and continued enhanced until Rel-18 from various aspects.

Observation 6:	Once the TRP is capable of DL transmission, it could not be “UL TRP” anymore, but rather the “else TRP” enabled from previous releases, and this is an obvious reason that RAN1 has not discussed the “else” TRP in Rel-19.

On the other hand, the beam correspondence requirement was defined in RAN4 to evaluate the ability of the UE to select a suitable beam for UL transmission based on DL measurements with or without relying on UL beam sweeping. In general, as defined in Rel-15, it is fulfilled if the UE meets the corresponding minimum peak EIRP and spherical coverage requirement for that power class with its autonomously chosen UL beams and without uplink beam sweeping, but based on downlink reference signals, e.g., SSB or CSI-RS. Hence, in RAN4, the beam correspondence requirement has never been enhanced or changed according to the mTRP enhancement as long as the UE can receive the DL RS for a UL TCI state.

Observation 7:	Beam correspondence requirement has never been enhanced or changed according to the multi-TRP enhancement as long as the UE can receive the DL RS for a UL TCI state.

Figure 1 briefly summarizes what learnt from the observations for the mTRP enhancements and its beam correspondence requirement, including the “else” scenario.

Figure 1: Beam correspondence under multi-TRP operations

In short, as RAN4 has long discussed, the proposed change in Option 2 under Scenario 4 is no longer necessary in terms of the beam correspondence requirement since it is the same as other previous mTRP scenarios which are anyhow based on a DL RS for a UL TCI state from the requirement as shown in Figure 1. In addition, it should be noted that the UE still can support the asymmetric DL sTRP/UL mTRP of Rel-19 by following the legacy beam correspondence requirement. It is not required for RAN4 to make a list of the UE capabilities in UE RF specification. Therefore, RAN4 should refrain from considering asymmetric DL sTRP/UL mTRP in UE RF discussions unless they are going to have a suffix/clause for every single feature/capability.

Observation 8:	Scenario 4 is no longer necessary in terms of the beam correspondence requirement since it is the same as other previous mTRP scenarios which are anyhow based on a DL RS for a UL TCI state from the requirement as shown in Figure 1.

Observation 9:	UE still can support the asymmetric DL sTRP/UL mTRP of Rel-19 by following the legacy beam correspondence requirement.

Observation 10:	It is not required for RAN4 to make a list of the UE capabilities in UE RF specification.

Proposal 3:		RAN4 should refrain from considering asymmetric DL sTRP/UL mTRP in UE RF discussions.

3	Conclusion

This contribution focuses on remaining open issues from the outcome of the last meeting [2]. Following observations and proposals were derived for consideration in this meeting.

Observation 1:	For 3Tx, RAN4 agreed to introduce PC3 and PC2 for both FWA and handheld UE, but not for PC1.5 yet.

Observation 2:	Mode 0 requires a single full-rated port to meet the target power class, which is confirmed by the reply LS from RAN1. Therefore, supporting PC1.5 in this WI is unrealistic.

Proposal 1:		Considering the limitations of this WI, it is reasonable for RAN4 to consider PC1.5 in future releases for 3Tx.

Observation 3:	RAN4 has reached a consensus to reuse the single carrier MPR for each power class of 3Tx for PC3 and PC2 although it is not yet clear which MPR can be referred to for 3Tx PC2 between 1Tx and 2Tx.

Proposal 2:		3Tx PC2 MPR should follow the value for PC2 with 2Tx, as it is technically justified.

Observation 4:	Option 1 is not to define any FR2 UE RF requirement for the scenario, which is never discussed in RAN1 for this WI. On the other hand, Option 2 is based on a proposed change of beam correspondence requirements for the asymmetric DL sTRP/UL mTRP limited to the scenario only if UE is not configured with PL offset in joint/UL TCI state(s).

Observation 5:	mTRP transmission with non-coherent JT was enabled from Rel-16, and continued enhanced until Rel-18 from various aspects.

Observation 6:	Once the TRP is capable of DL transmission, it could not be “UL TRP” anymore, but rather the “else TRP” enabled from previous releases, and this is an obvious reason that RAN1 has not discussed the “else” TRP in Rel-19.

Observation 7:	Beam correspondence requirement has never been enhanced or changed according to the multi-TRP enhancement as long as the UE can receive the DL RS for a UL TCI state.

Observation 8:	Scenario 4 is no longer necessary in terms of the beam correspondence requirement since it is the same as other previous mTRP scenarios which are anyhow based on a DL RS for a UL TCI state from the requirement as shown in Figure 1.

Observation 9:	UE still can support the asymmetric DL sTRP/UL mTRP of Rel-19 by following the legacy beam correspondence requirement.

Observation 10:	It is not required for RAN4 to make a list of the UE capabilities in UE RF specification.

Proposal 3:		RAN4 should refrain from considering asymmetric DL sTRP/UL mTRP in UE RF discussions.

Reference

[1] 	RP-242394,	WID revision: NR MIMO Phase 5, RAN#105

[2] 	R4-2503037,	WF on UE RF requirements for MIMO enhancement, RAN4#114

[3] R4-2500005,	Reply to RAN4 LS on UE RF issues for Rel-19 MIMO Enhancement, RAN4#114