# 3GPP TSG-RAN WG4 Meeting # 114-bis					R4-2504398

# Wuhan, China,7th–11th April 2025

Source:		Ericsson

Title:			A-IoT BS RF impact overview

Agenda item:		7.24.2.1

Document for:		Approval

# Introduction

In this paper, we present our overview for A-IoT BS RF requirements based on WF [1].

# Discussion

## Transmitter requirements

In WF [1], the output power agreement is quoted below:

### Issue 1-1: Base station output power

- Agreement:

  - A-IoT BS output power, Prated,c,AC≤ [38]dBm

The output power simulated in study item phase is 33 dBm, the reason to propose the 38 dBm is due to legacy type 1-C BS maximum output power limitation for micro BS. Though the higher output power is needed to increase the downlink coverage, the UL coverage needs to be checked so the UL and DL coverage match. For D1T1-B, the DL coverage and UL coverage is dictated by the A-IoT BS output power and CW node power respectively. This is illustrated in Figure 1, where the received power at device and BS is calculated using the parameters defined in TR 38.769. The ISD models the distance between the CW node to the A-IoT BS, e.g. ISD= 20m corresponding to the coexisting simulation of D1T1-B. It can be observed that the received power difference at device is around 5 dB for ISD of 20 meters. If the device sensitivity limits the range, for example, -36 dBm corresponding to 67 meters between CW and BS, it is beneficial to increase the A-IoT BS output power. However, if the device sensitivity does not limit the R2D range, to support higher R2D power is not justified. In this sense, there is a tradeoff between BS output power and device sensitivity. It is not clear what is the device sensitivity for now. Therefore, we can delay the decision on the BS output power till the device sensitivity is decided. As the DL and UL coverage should be matched so the BS sensitivity should be also related to decide the maximum ISD range. The BS sensitivity will be relating to both CW power and REFSENS discussion. From the aspect, there is a need to discuss whether the maximum range of the ISD should be used for the discussion of the output power and RESENS. In TR 38.769, the maximum distance is quoted below. RAN4 should discuss whether to use this parameter in discussing the min RF requirement. For example, there is no need of 38 dBm output power if the device sensitivity would be better than -20 dBm as illustrated in Figure 2.

Applicable maximum distance target value(s): The maximum distance targets are set separately for device 1, device 2a, device 2b, respectively. The distance target is compared to the evaluation results which assume the agreed mandatory evaluation assumptions.

For device 1 in D1T1: 15 m

The DL and UL coverage should be matched. DL coverage is tradeoff between BS output power and device sensitivity and UL coverage is relating to both CW power and BS REFSENS.

The maximum distance for device 1 is 15 meters in TR 38.769.

RAN4 to decide if the maximum distance of 15 m for device 1 should be used to specify the min RF requirements for A-IoT BS and device.

Figure 1: The received power for device and BS with the different CW and BS output power

Figure 2: Received power at antenna of device and BS with ISD of 20 meters

### Issue 1-2-1: RE power control dynamic range

- Agreement:

  - Check whether RAN1 supports RE granularity operation for R2D transmission or not.

### Issue 1-2-2: Total power dynamic range

- Agreement:

  - Check whether RAN1 supports PRB granularity operation for R2D transmission or not

For RE power dynamic range and total power dynamic range, as the OFDM transmitter is agreed in RAN1, we think the legacy requirement about these two can be reused. There may be a dependency on the channel bandwidth for total power dynamic range. However, if the A-IoT BS always transmit in full RB, then RE dynamic range and power dynamic range would not be needed.

Total power dynamic range and RE dynamic range may not be needed if A-IoT BS always transmit the full RB.

### Issue 1-3-1: Transmitter OFF power

- Agreement:

  - Define Transmitter OFF power.

  - values FFS

### Issue 1-3-2: Transmitter transient period

- Agreement:

  - Define Transmitter transient period.

  - FFS values.

The transient period and OFF power requirement in legacy requirement relates to TDD operation. As it is agreed to introduce the HD-FDD operation in BS, it is relevant to define such requirement to switch ON PA during the R2D transmission and to switch OFF PA during the D2R receiving. In EPC specification, the transient time for RF envelop is 2000 us (Tr + T1) as illustrated in Figure 3. While the legacy requirement is 10 us for transient time, we think the legacy requirement could be reused so the spectrum utilization can be improved for A-IoT compared to EPC transient time. Currently RAN1 also discuss the CP handling and in our opinion, as the CP is 4.69us and transient period is 10us, it should be expected that BS will transmit the command after the transient time. Otherwise, the transient time may introduce uncertainty on the CP handling scheme which would be agreed in RAN1. Due to this, A-IoT device should not be expected to receive the R2D message after the last D2R transmission during the transient time. For Tx OFF power, we think the legacy requirement for TDD operation can be reused. However, the protection of the own receiver for FDD band, the requirement is -91 dBm/100kHz which is much tighter than TDD OFF power level of -85 dBm/MHz (or -75 dBm/100kHz). This requirement can be further discussed when REFSENS is specified for A-IoT BS, so a suitable level of protection is specified.

Define the transient time as the same with legacy requirement.

Discuss the TX OFF power later when REFSENS level of the A-IoT BS is defined.

Figure 3: EPC specification [2] on transient time

### Issue 1-4-2: Modulation quality

- Agreement:

  - FFS whether to define requirements such as modulation Depth, RF Envelope Ripple, RF Envelope Rise Time, RF Envelope Fall Time and RF Pulse width for A-IoT BS.

The RF envelop rise/fall time is discussed in Tx transient and for modulation depth, Ripple defined in EPC specification in [2], it is observed from Figure 4 to Figure 5 that the modulation depth and ripple are relating to the baseband synthesized signal. For example, Figure 4 to Figure 5 shows the ripple performance for DFT and ideal brick waveform with different subcarrier number and M segments in one OFDM symbol. It can be observed that the modulation depth is relating to the algorithm used to synthesize the OOK and also the ripple performance is relating to the configuration of the number of the subcarrier and M-Chip per OFDM symbol for baseband signal. For the ripple performance, the step response of channel filter can also introduce the ripple in time domain as illustrated in Figure 6. With a baseband signal input to the transmitter, the output will be convolution of both baseband signal and the impulse response of the transmitter’s transfer function. Channel filter is designed to attenuate the unwanted emission in frequency domain but not the ripple in time domain. It is illustrated in Figure 6 for the ripple performance of two LPF filter of different order filter. It can be observed that higher order filter with high attenuation may introduce higher ripple in step response compared to a lower order filter. From this analysis, we suggest not considering the ripple performance at A-IoT transmitter.

Modulation depth and ripple defined in EPC specification [2] relating to the algorithm used to synthesize the OOK waveform.

Ripple performance in time domain relates to both baseband signal and also the RF transmitter transfer function.

Not to define the modulation depth but depends on the RAN1 defined signal progressing for OOK signal generation.

Not to define the ripple performance.

Figure 4: Ripple and modulation depth with M=32 and different size of K

Figure 5: modulation depth and ripple with different M size

Figure 6: Step response of the two LPF filter

As it is not relevant to define the time domain requirement as specified in EPC specification in [2]. We think there is a need to align the OOK signal quality requirement with LP-WUS. Some of the analysis is illustrated below for what has been done based on the LP-WUS signal design. As the NR signal quality requirement are tested in frequency domain referring to the reference point definition below, so the analysis also done in frequency domain to evaluate the signal quality distortion for OOK-M signal.

Figure B.1-1 [3]: Reference point for EVM measurement

Figure 7:Constellation mapping for multi-bit OOK WUS (4-bit OOK case mapped to 64QAM).

Figure 8: Impact of mapping WUS constellation to existing QAM on OOK detection performance.

Figure 9: Impact of mapping WUS constellation to existing QAM on OFDM sequence detection performance.

- The tolerance of the distortion of the frequency domain data of OOK-4 is tested by remapping the frequency constellation of overlaid ZC sequence to one of existing modulation scheme.  Mapping the generated frequency domain values (before IFFT) to existing sequences/QAM modulations could be considered to reduce the impact on gNB. As one example, for OOK4 waveform, the higher modulation (e.g. 64QAM) than QPSK may be used to map the frequency domain OOK4 symbols (left constellation in  Figure 7) to 64QAM constellation points (right constellation in  Figure 7) for CP-OFDM symbols as illustrated in Figure 7.  Figure 8 and Figure 9 illustrates the initial LLS result to evaluate the impact of OOK and OFDM sequence detection due to the remapping, It can be observed that the performance loss of such mapping on WUS detection performance (OOK and OFDM) is around 0.2 dB when mapping to regular data structure generated from 64QAM and around 0.6 dB when mapped to 16QAM.

The performance loss for OOK signal detection at UE receiver due to transmitter remapping constellation to existing QAM constellation is around 0.2 dB for 64QAM remapping and 0.6 dB for 16QAM remapping.

From the constellation re-mapping, it can be observed that the time domain signal quality of OOK-4 is quite robust and can be maintained without big degradation of SNR when the frequency domain OOK-4 data is distorted by the amount of EVM equivalent to the 64QAM. The frequency domain data of OOK-4 is degraded if it is distorted by the amount of the EVM equivalent to the 16QAM.  As the A-IoT BS may not transmit the NR signal, therefore, a 64QAM test vector is needed to test 64QAM EVM instead.

The time domain OOK-4 signal can tolerate a distortion in frequency domain equivalent to and less than the EVM of 64QAM before iFFT.

Specify the OOK-M signal quality requirement with equivalent EVM requirement better than 64QAM.

For ACLR, there is no coexisting result concern for the assumed ACLR in Table 6.6.2.2-2 in TR 38.769 below. RAN4 confirm this value can be specified for A-IoT BS for 1 RB transmission bandwidth definition. We think maybe RAN4 should wait RAN1 further progress and see whether other bandwidths would be defined.

Use the ACLR 1 (40 dB) and ACLR2 (50 dB) for one RB transmission bandwidth.

Table 6.6.2.2-2 [2]: A-IoT reader RF parameters

| A-IoT reader parameters | Values for evaluation |
| --- | --- |
| A-IoT reader total Tx power | 33 dBm |
| A-IoT reader receiver Noise Figure (dB) | 10 |
| A-IoT reader antenna gain (dBi) | 6 |
| ACLR | ACLR of legacy NB -IOT gNB (i.e. ACLR1:40dB，ACLR2:50dB) |
| ACS | Same as legacy NR BS |
| Antenna configuration | Antenna Array Geometry：equals to omni-directional antenna pattern in GCG in horizontal Antenna pattern (horizontal): AHϕ=-min12ϕϕ3dB2,Am, φ3dB = 90°, Am = 15 dB Antenna pattern (horizontal): AVθ=-min12θ-90°θ3dB2,SLAv, θ3dB= 90°, SLAv = 15 dB Combining method in 3D antenna pattern: Aφ,θ=-min⁡{-AHφ+AVθ, Am} BS antenna gain (dBi) (including feeder loss): 6 |

It is agreed that only outdoor NR UE is assumed and in TR 38.769, it is assumed that the network will handover the NR UE if UE move to indoor accidently and this needs a NR BS coverage in the same A-IoT BS coverage. Such NR BS can be deployed in a separate band other than A-IoT BS band for this purpose. It can be assumed that such NR BS can be co-located with A-IoT BS in the worst case and from this perspective, co-located requirement can be specified for A-IoT. Adding the co-located requirement in later release may be problematic for the deployed A-IoT BS.

Specify the NR BS co-location requirement as the deployment of A-IoT BS with other co-located BS is not excluded.

For Tx intermodulation requirement, it can be discussed whether the A-IoT BS can be co-located with another A-IoT BS transmitting in another sector. From the LLS simulation, it is observed that the R2D receiving will not be impacted when interfering signal is below -22 dB while the test of the Tx inter-modulation is -30 dB assuming the antenna isolation between antenna of wanted signal and antenna of the interfering signal as illustrated in Figure 10. So it is possible to deploy a NR BS with adjacent channel with lower output power. If the co-located NR BS only is deployed with another band than the A-IoT band, the need of such requirement may be further discussed according to the note in legacy requirement:

NOTE 1:	Interfering signal positions that are partially or completely outside of any downlink operating band of the base station are excluded from the requirement, unless the interfering signal positions fall within the frequency range of adjacent downlink operating bands in the same geographical area. In case that none of the interfering signal positions fall completely within the frequency range of the downlink operating band, TS 38.141-1 [5] provides further guidance regarding appropriate test requirements.

Tx intermodulation should be defined if the NR BS is deployed within the same A-IoT band.

Figure 10: BLER performance with 49 RB NR signal

## Receiver requirement

For REFSENS requirement, the CW received power should be considered when testing the REFSENS. In Figure 11, the received CW power and D2R signal power is illustrated with different ISD distance between CW node and A-IoT BS. It can be observed that the CW received power is higher (e.g -15.5 dBm) with CW Tx of 33 dBm than the received CW power of -25.5 dBm with CW Tx power of 23 dBm for ISD of 20 meter. Both CW received power is much higher than legacy ACS interferer and IBB interferer (-47 dBm and -38 dBm respectively). Such high CW power may saturate the LNA and also no signal will be received. In this sense, several options could accommodate this.

- a CW cancellation function would be expected at A-IoT BS receiver before LNA,

- or the CW transmission power should be lowered in specification,

- or the ISD distance between CW node and A-IoT BS increased.

The CW cancellation is discussed in study phase but this needs additional functionality in hardware to tolerate higher CW power. However, from the link budget analysis as illustrated in Figure 1, if the maximum range of device to BS is limited, it seems there is no need of high CW power. For example, the received D2R power is -71 dBm when CW Tx power is 33 dBm and received D2R power is -81 dBm when CW power is 23 dBm when ISD distance is 20 meters.

Received CW power of -15.5 dBm may saturate the LNA of BS receiver when CW Tx power is 33 dBm.

With the distance of 20 meter between CW node and BS in coexisting simulation, CW Tx power does not need to be higher than 23 dBm.

Figure 12 shows the relation between LNA IIP3, desensitization and received CW signal at antenna using the modelling in TR 38.769. It can be observed that the desensitization will occur for a certain IIP3 and received CW signal and higher the received CW signal, more demanding on the linearity requirement of the LNA for a certain desensitization. Nevertheless, this analysis only considers the linearity impact, further desensitization may occur considering the reciprocity mixing of phase noise in analogue domain and also the power leakage in the digital domain when a filter is used to remove CW signal. Based on above analysis, we suggest adding X dB relaxation of the REFSENS. At the same time, discuss a CW received power Y dBm and RAN4 further discuss how to reduce the CW received power level.

Adding X dB relaxation on the REFSENS accounting the desensitization due to linearity, reciprocity mixing and power leakage from digital filter.

Specify Y dBm CW received power for REFSENS, FFS on value of Y.

Discuss further factors to specify the CW power received at BS antenna:

  - CW cancellation

  - Increasing the CW node to BS distance

  - Reconfiguring CW Tx power and CW Tx should be less than a maximum Tx power

Figure 11: BS received D2R power and CW power with different CW TX power

Figure 12: LNA IIP3 and desensitization by CW signal

Based on the simulation assumption, BLER is illustrated in Figure 13. The residue SFO is used to D2R simulation and it can be observed that that the SNR performance depends on the residue SFO. Therefore, the SFO estimation and compensation should be needed and as the D2R amble is still under RAN1 design, RAN4 can wait RAN1 progress or make reasonable residue SFO.

SNR is 10.2 dB with SFO residue of 10^2 to 10^3 ppm.

RAN4 can wait RAN1 progress or make reasonable residue SFO.

Figure 13: BLER with different residue SFO

For ACS and IBB, it is agreed that coexisting simulation will be needed to decide the interference level.

##### 6.7.2.2.3 In-band selectivity and blocking

Regarding ACS requirement and in-band blocking requirement, RAN4 reached the following consensus:

- ACS requirement and the interference level shall be determined by RAN4 co-existence study and wanted signal configuration and its corresponding configuration could be postponed to WI phase.

- In-band blocking requirement and the interference level shall be determined by RAN4 co-existence study, the wanted signal configuration and its corresponding configuration could be postponed to WI phase.

Last meeting it is agreed only UE outdoor is considered. However, in our previous analysis on co-location requirement, a NR UE operating in adjacent channel can roam into the indoor A-IoT coverage before being handover to another frequency. Therefore, we think ACS and IBB legacy requirement could apply.

NR UE operating in adjacent channel can roam into the A-IoT coverage before handover occurs.

Reuse the ACS and IBB requirement in legacy NR BS.

Intermodulation response rejection is a measure of the capability of the receiver to receive a wanted signal on its assigned channel frequency at the antenna connector for BS type 1-C in the presence of two interfering signals which have a specific frequency relationship to the wanted signal.  As discussed in ACS and IBB, a NR UE operating in adjacent channel can roam into the A-IoT coverage. Another case is multiple reader activation at the same time (e.g. 2 active readers is assumed in coexisting study). In such case, the BS may receive two different D2R signal in two channels at the same time, which is similar with the legacy NR BS. Therefore, the intermodulation requirement could be kept for receiver linearity requirement. There is no other requirement for receiver linearity but when receiver receiving a CW signal together with a weak backscattered signal, the linearity requirement will be needed anyway. From this perspective, it will be good to retain the same IM requirement.

Reuse the RX intermodulation requirement.

For ICS requirement, RAN1 has agreed FDMA for multiple devices, so different device can be multiplexed in the frequency domain at the same time. All the D2R signals from different devices are backscattered signals using the same CW signal but with different frequency offset. As there is no UL power control, the received power from different device may vary but still the highest received power should be CW signal. As the legacy ICS requirement allows the desensitization of the wanted signal in presence of the higher PSD in-band interfering signal, REFSENS tested with CW presence may be deemed as in-channel selectivity test.

RAN4 can discuss whether the REFSENS requirement tested with a CW signal presence is good enough for the in-channel selectivity. When multiple device D2R signal multiplexed in a D2R channel, the additional relaxation should be considered as multiple D2R signals from difference device may worsen the IM performance. We suggest further discussion on in-channel selectivity test with two cases:

- REFSENS tested with CW signal

- ICS with CW and multiple D2R receiving.

REFSENS tested with CW presence can be regarded as one ICS test case.

Further discuss on ICS of multiple devices receiving.

Agreement

Support FDMA with Y≥1 D2R transmissions for Msg.1 from multiple devices in response to a R2D transmission triggering random access.

- The maximum value of Y >1 is determined in AI 9.4.2, based on feasible values of small frequency shift to be decided in AI 9.4.2.

- Signalling details should be discussed and determined in AI 9.4.4.

# Conclusions

In this contribution, we present our overview on the A-IoT BS RF with below proposal and observations:

Observation 1 The DL and UL coverage should be matched. DL coverage is tradeoff between BS output power and device sensitivity and UL coverage is relating to both CW power and BS REFSENS.

Observation 2 The maximum distance for device 1 is 15 meters in TR 38.769.

Proposal-1: RAN4 to decide if the maximum distance of 15 m for device 1 should be used to specify the min RF requirements for A-IoT BS and device.

Proposal-2: Total power dynamic range and RE dynamic range may not be needed if A-IoT BS always transmit the full RB.

Proposal-3: Define the transient time as the same with legacy requirement.

Observation 3 Modulation depth and ripple defined in EPC specification [2] relating to the algorithm used to synthesize the OOK waveform.

Observation 4 Ripple performance in time domain relates to both baseband signal and also the RF transmitter transfer function.

Proposal-4:Not to define the modulation depth but depends on the RAN1 defined signal progressing for OOK signal generation.

Proposal-5: Not to define the ripple performance.

Observation 5 The performance loss for OOK signal detection at UE receiver due to transmitter remapping constellation to existing QAM constellation is around 0.2 dB for 64QAM remapping and 0.6 dB for 16QAM remapping.

Observation 6 The time domain OOK-4 signal can tolerate a distortion in frequency domain equivalent to and less than the EVM of 64QAM before iFFT.

Proposal-6: Specify the OOK-M signal quality requirement with equivalent EVM requirement better than 64QAM.

Proposal-7: Use the ACLR 1 (40 dB) and ACLR2 (50 dB) for one RB transmission bandwidth.

Proposal-8: Specify the NR BS co-location requirement as the deployment of A-IoT BS with other co-located BS is not excluded.

Proposal-9: Tx intermodulation should be defined if the NR BS is deployed within the same A-IoT band.

Observation 7 Received CW power of -15.5 dBm may saturate the LNA of BS receiver when CW Tx power is 33 dBm.

Observation 8 With the distance of 20 meter between CW node and BS in coexisting simulation, CW Tx power does not need to be higher than 23 dBm.

Proposal-10:Adding X dB relaxation on the REFSENS accounting the desensitization due to linearity, reciprocity mixing and power leakage from digital filter.

Proposal-11: Specify Y dBm CW received power for REFSENS, FFS on value of Y.

Proposal-12: Discuss further factors to specify the CW power received at BS antenna:

Observation 10 NR UE operating in adjacent channel can roam into the A-IoT coverage before handover occurs.

Observation 9 SNR is 10.2 dB with SFO residue of 10^2 to 10^3 ppm.

Proposal-13: RAN4 can wait RAN1 progress or make reasonable residue SFO.

Proposal-14: Reuse the ACS and IBB requirement in legacy NR BS.

Proposal-15: Reuse the RX intermodulation requirement.

Observation 11 REFSENS tested with CW presence can be regarded as one ICS test case.

Proposal-16: Further discuss on ICS of multiple devices receiving.

# References

- R4-2502859, WF on requirements for A-IoT BS and CW, Huawei

- EPC™ Radio-Frequency Identity Protocols Generation-2 UHF RFID