# 3GPP TSG-RAN WG4 Meeting # 114-bis					R4-2504401

# Wuhan, China,7th–11th April 2025

Source:		Ericsson

Title:			CW node RF impact overview

Agenda item:		7.24.2.3

Document for:		Approval

# Introduction

In this paper, we present our overview for RF requirements for a CWT node.

# Discussion

Below is the recap of the RAN agreement relating to CW in general.

RAN#103:

Proposal 3v2 (endorsed) - Regarding the objective in the SID: Study necessary characteristics of carrier-wave waveform for a carrier wave provided externally to the Ambient IoT device, including for interference handling at Ambient IoT UL receiver, and at NR basestation. - This objective allows studying CW waveform characteristics which would need control of the CW node(s), e.g. waveform characteristics that impact interference such as when CW is transmitted or not transmitted, power, bandwidth, spectrum, etc.

From the RAN1 and RAN agreements, it can be observed that the CW waveform is defined with unmodulated sinusoid waveform, and maximum number of CW signal is limited to two. As the CW transmission is controlled in terms of the timing, power, bandwidth, and spectrum, CW signal is relevant and RAN4 can continue to specify the RF characteristic of CW signal based on these basic assumptions.

CW is unmodulated sinusoid signal and can be one or two tones with FFS frequency gap.

CW transmission’s timing, power, bandwidth, spectrum is controlled.

### Issue 3-1: CW Output power

- Agreement:

  - CW output power [33] dBm.

Figure 1 illustrates the link budget for D1T1-A2 and Figure 2 illustrated the link budget for D1T1-B.

It can be observed in Figure 1 that output power of CW of 23 dBm is not good enough for D1T1-A considering a device sensitivity of -24 dBm. While for D1T1-B as illustrated in Figure 2, the CW node power is only relevant to the UL as CW node is located at ISD distance away from A-IoT BS. It can be observed in Figure 2 that CW node power 23 dBm is good enough if the BS sensitivity is better than -80 dBm.

In A-IoT BS RF discussion paper, it is also observed that higher CW power leads to higher received CW power at BS at the same time when receiving a weak backscattered signal. This is illustrated in Figure 3. Higher CW power received before LNA will demand a better linearity and this will not desensitize the receiver too much. Therefore, it is preferably to specify a lower CW Tx power to not saturate the BS receiver for 20 meters distance case from CW node to A-IoT BS. As the CW is infrastructure node, so its power should be declarable. It is agreed that CW node power is controlled in RAN plenary so at least CW node output power should be configurable in a semi-dynamic way so fixed output power is specified.

CW Tx power of 23 dBm is not good enough for D1T1-A but is good enough for D1T1-B if BS receiver sensitivity is better than -80 dBm.

CW Tx output power is a fixed output power configurable based on manufacture declaration.

Specify also CW Tx power of 23 dBm if only fixed power level to be specified.

Alternatively, the CW Tx power can be specified with a maximum output power the same as BS and manufacture declare the related CW power.

Figure 1: device 1 range considering the energy harvesting with CW transmission power of 33 dBm and 23 dBm for D1T1-A

Figure 2: BS received D2R power with CW Tx power of 23 dBm and 33 dBm for D1T1-B

Figure 3: BS received simultaneously the CW signal power and backscattered signal power

### Issue 3-2: ON/OFF power

- Agreement:

  - FFS for OFF power

The OFF power could be reused from the legacy ON/OFF mask. Together with OFF power, the transient time needs to be specified and the same timing mask for ON/OFF power can be reused from legacy BS.

As per RAN plenary, the CW transmission power ON /OFF and timing to turn ON/OFF is controlled. Therefore, it can be expected that the CW transmission does not overlap with the R2D signal transmission from BS. If this would occur, for device near the CW node, the received power of R2D can be more than 20 dB below the received power of CW as illustrated in Figure 4. Though it is assumed 10 MHz RF bandwidth for device 1 during study phase, the antenna selectivity at stop band may be around 20 dB for printed filtering dipole antenna [3]. Therefore, the device 1 may not decode the R2D when it is located near the CW node and far from A-IoT BS. In this case, the CW node transmission should be ON after the R2D signal transmission and colliding with R2D transmission in time domain should be avoided.

The attenuation at stop band for device antenna may not be good enough for D1T1-B and CW and R2D simultaneously transmission may cause outage of the R2D receiving at device.

In TR 38.769, there is also a RF BPF in device 1 architecture and this can enable the out of band blocking performance of device 1. The OOB for device 1 is not considered so far. If the CW transmission can occur simultaneously with R2D, RAN4 needs to discuss OOB for device 1 also.

Figure 4: Received power at antenna of device and BS with ISD of 20 meters

A relevant issue for CW ON/OFF mask is the synchronization accuracy between the CW node and BS. As illustrated in Figure 5, if the CW node is switched on too early or switched off too late, the R2D signal may be corrupted at device. Therefore, a reasonable synchronization accuracy between CW node and BS should be expected. For TDD network, 3 us is a known synchronization requirement between the TDD node. We propose to reuse such requirement for CW node and A-IoT BS. As discussed above, if CW and R2D operate simultaneously, the OOB requirement needs to be specified for device 1. We think maybe it will be better network guarantee the CW and R2D not be transmitted simultaneously rather than adding device complexity as passive device is sensitive to the power consumption and cost.

Introduce the synchronization requirement of 3 us between CW node and A-IoT BS.

OOB performance of device 1 needs to be specified if CW and R2D should be transmitted simultaneously at network

Figure 5: CW node switch too early or too late

The output power should be measured within a certain bandwidth where 99% emitted power is confined. To study the needed bandwidth, a simulation with CW signal with phase noise profile in TR 38.803 is illustrated in Figure 6 and it can be observed that CW occupied bandwidth is around 100Hz. The channel bandwidth is under discussion, as CW always transmit a CW tone, a min channel bandwidth can be chosen for CW channel bandwidth.

A bandwidth should be defined together with CW output power so that the power can be measured with OCC bandwidth.

A minimum channel bandwidth can be defined for CW node.

Figure 6: Occupied BW for a CW signal

For ON/OFF requirement, as the CW transmission will be during the D2R transmission, it will be beneficial to have predefined transient time so the receiving time of the D2R would be expected. This will also make sure the interference between CW ramp up time and R2D receiving to be minimal.

Transient time for R2D signal ramp down, transient time for the CW power ramp up and the synchronization time error between the A-IoT BS and CW node may overlap with each other and the total time between R2D signal ends to CW start may decide the TR2D_min in RAN1 agreement.

ON/OFF TX requirement should be specified.

For the phase noise, there are concerns that the close-in phase noise of the CW transmission may distort the D2R receiving for the outside topology. It is true that the unwanted emission generated by the phase noise and the transmitter non-linearity may appear higher than the backscatter signal at the antenna when CW is transmitted at the same A-IoT BS. This is illustrated in Figure 7. The phase noise profile in TR 38.803 is used and it can be observed that around -50 dBm at the frequency offset of the 300kHz. If the reader REFSENS is around -70 dBm, such level of emission will drown the backscatter signal. From Figure 3, it can be observed that the relative level between received CW power and backscatter signal is 57 dB (e.g. -25 dBm – (-82 dBm)), further discussion is needed to check the phase noise profile and the frequency offset of the first sidelobe.

Further discuss whether a inband emission mask should be needed for a certain CW Tx power and ISD distance.

Figure 7: the CW transmission with phase noise

Using the TR 38.803 phase noise profile, the CW transmission and the unwanted emission by phase noise is simulated and illustrated in Figure 8. The ACLR1 is around 40 dBc and ACLR2 is around 50 dBc.  Adding the PA non-linearity margin of 3 dB, we think the unwanted emission if modelled as the ACLR1 and ACLR2 should be 37 dBc and 47 dBc. As the PA is not modelled and 3 dB can be added to account PA noise.

CW unwanted emission can be modeled with 40 dBc for ACLR1 and 50 dBc for ACLR2.

Figure 8: the CW transmission with phase noise

For inter-modulation requirement, there is a need to discuss whether the co-location with other NR BS would be a valid case to consider. As CW node can only transmit the CW signal, so the requirement would not reuse the NR BS requirement if there is a need to define.

Discuss if the co-location with other BS is a valid scenario before discussing the inter-modulation requirement.

# Conclusions

In this contribution, we present our overview on the A-IoT BS RF with below proposal and observations:

Observation 1 CW is unmodulated sinusoid signal and can be one or two tones with FFS frequency gap.

Observation 2 CW transmission’s timing, power, bandwidth, spectrum is controlled.

Observation 3 CW Tx power of 23 dBm is not good enough for D1T1-A but is good enough for D1T1-B if BS receiver sensitivity is better than -80 dBm.

Observation 4 CW Tx output power is a fixed output power configurable based on manufacture declaration.

Proposal-1: Specify also CW Tx power of 23 dBm if only fixed power level to be specified.

Proposal-2: Alternatively, the CW Tx power can be specified with a maximum output power the same as BS and manufacture declare the related CW power.

Observation 5 The attenuation at stop band for device antenna may not be good enough for D1T1-B and CW and R2D simultaneously transmission may cause outage of the R2D receiving at device.

Proposal-3:Introduce the synchronization requirement of 3 us between CW node and A-IoT BS.

Observation 6OOB performance of device 1 needs to be specified if CW and R2D should be transmitted simultaneously at network

Proposal-4: A bandwidth should be defined together with CW output power so that the power can be measured with OCC bandwidth.

Proposal-5: A minimum channel bandwidth can be defined for CW node.

Proposal-6: ON/OFF TX requirement should be specified.

Proposal-7: Further discuss whether a inband emission mask should be needed for a certain CW Tx power and ISD distance.

Proposal-8:CW unwanted emission can be modeled with 40 dBc for ACLR1 and 50 dBc for ACLR2.

Proposal-9:Discuss if the co-location with other BS is a valid scenario before discussing the inter-modulation requirement.

# References

- RP-243326, “New Work Item: Solutions for Ambient IoT (Internet of Things) in NR”, RAN1 Vice-chair (Huawei), 3GPP TSG RAN Meeting #106, Dec. 2024.

- R4-2502859, WF on requirements for A-IoT BS and CW, Huawei

- “Printed Filtering Dipole Antenna With Compact Size and High Selectivity” Xiyao Liu; Ken Ning etc, IEEE Transactions on Antennas and Propagation (Volume: 72, Issue: 3, March 2024)