3GPP TSG-RAN WG4 Meeting # 114bis                                             	                   R4-2504426

# Wuhan, China, 7th - 11th, Apr 2025

Source: 	Samsung

Title: 	Link budget calculation results for fragmented carriers

Agenda Item:	7.5.3

Document for:	Discussion

- Introduction

This paper captures the link budget results for delta RIBNC, ACS 1/2 and IBB 1/2.

- 2. Discussion/Conclusion

Proposal 1: Following results are from link budget calculation.

- For n25(2A), 5M+5M with Wgap=55MHz

- For n2(2A), 5M+5M with Wgap=50MHz

- For n3(2A), 5M+5M with Wgap=65MHz

|  |  | Additional relaxation |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band and basic assumption | Cell | ΔRIBNC |  | ACS1 w/o Image |  | ACS1 w/ Image |  | ACS2 w/o Image |  | ACS2 w/ Image |  | IBB 1 w/o Image |  | IBB 1 w/ Image |  | IBB 2 w/o Image |  | IBB 2 w Image |  |
| n25 APF attenuation=8.6 NF increase due to AGC adjustment=9.2 | PCC | 10.8 | 15.4 | 2.4 | 4.2 | 2.4 | 4.2 | 1 | 1 | 1 | 1 | 6.3 | 10 | 6.3 | 10 | 6.3 | 10 | 6.3 | 10 |
|  | SCC | 7 | 10.9 | 0 | 0 | 2.1 | 2.8 | 0 | 0 | 6.5 | 6.5 | 1 | 4.9 | 6.1 | 7.7 | 1 | 4.9 | 16.6 | 16.8 |
| n2 APF attenuation=13 NF increase due to AGC adjustment=5 | PCC | 6.8 | 12.2 | 1.6 | 2.8 | 1.6 | 2.8 | 1 | 1 | 1 | 1 | 3.9 | 7.3 | 3.9 | 7.3 | 3.9 | 7.3 | 3.9 | 7.3 |
|  | SCC | 4.4 | 8.1 | 0 | 0 | 1.8 | 2.2 | 0 | 0 | 6.5 | 6.5 | 0 | 2.1 | 6.7 | 7.5 | 0 | 2.1 | 18 | 18.1 |
| n3 APF attenuation=7.9 NF increase due to AGC adjustment=9.3 | PCC | 11.1 | 16.3 | 2.5 | 4.7 | 2.5 | 4.7 | 1 | 1 | 1 | 1 | 6.5 | 10.8 | 6.5 | 10.8 | 6.5 | 10.8 | 6.5 | 10.8 |
|  | SCC | 7.5 | 12 | 0 | 0 | 2.4 | 3.3 | 0 | 0 | 6.5 | 6.5 | 1.5 | 6 | 6.8 | 8.7 | 1.5 | 6 | 17.4 | 17.6 |
| Note: Blue values assuming Tx leakage mapping to 0dBFs (PAPR is not considered) Black values assuming Tx leakage mapping to -12dBFS (PAPR is considered) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

- 3. Reference

[1] R4-2502873, WF on NR FR1 DL fragmented carriers study, Mediatek, RAN4#114