3GPP TSG-RAN WG4 Meeting #114bis	R4-2504428

Wuhan, China, 7th – 11th April, 2025

Agenda Item:	7.29.2.1

Source:	MediaTek Inc.

Title:	Discussion on Rel-19 NR-NTN RedCap UE RF requirements

Document for:	Approval

- Introduction

In RAN#102 plenary meeting, the Rel-19 WID of Non-Terrestrial Networks (NTN) for NR Phase 3 was approved in [1]. One of the main objectives is to support Rel-17 RedCap and Rel-18 eRedCap UEs with NR NTN operating in FR1-NTN band.

In RAN4#114 meeting, the WF [2] for guiding further discussion on residual issues was proposed. In this contribution, our views on the residual issues are provided.

- Discussion

2.1		Guidance for Rel-19 phase 3 NR-NTN UE RF requirements

The Rel-19 WID [1] objectives indicate that NTN for NR Phase 3 supports Redcap and eRedcap with NR NTN operating in FR1-NTN band. The objectives are shown below.

4.1 Objective of SI or Core part WI or Testing part WI Support of Rel-17 RedCap and Rel-18 eRedCap UEs with NR NTN operating in FR1-NTN bands [RAN4, RAN1] - For full-duplex FDD RedCap and eRedCap UEs, define the RF and RRM requirements [RAN4] - For HD-FDD RedCap UEs and eRedCap UEs, check whether any essential changes are needed for their support (i.e. focusing on HD collision rules) by end of Q2/2024 [RAN1] - Depending on feasibility assessment above, define the RF and RRM requirements [RAN4] Notes for this objective: GNSS (Global Navigation Satellite Systems) capabilities and simultaneous GNSS and NR-NTN operation is supported in RedCap/eRedCap UE.

The Rel-19 NR_NTN_Ph3_UERF WF [2] in RAN4#114 provided the guidance as follows.

Issue 1-1: PC3 output power for (e)RedCap Agreement: - Rel-18 power boosting is applicable for NR NTN including (e)RedCap - No reference sensitivity specification impact (e.g. RSD) from power boosting. - No changes to the PC3 tolerance for HD-FDD and FD-FDD compared non-RedCap UE NR NTN - FFS on solutions on additional new output power enhancement - supported modulation orders - potential downselection between HD-FDD and FD-FDD - simultaneous support with rel-18 power boosting - Background: output power enhancement (e.g., FE IL improvement) - Other solutions not precluded Note: No change for (e)RedCap MOP tolerance to enhance the output power does not preclude any solution in other Rel-19 WI.

2.2		PC3 output power for (e)RedCap

It is observed that the HD-FDD RF front-end insertion loss (FE IL) is lower compared to FD-FDD FE IL and can improve the RX REFSENS and TX maximum output power (MOP) simultaneously.

Proposal 1: The new output power enhancement for PC3 NTN (e)RedCap due to HD-FDD insertion loss(IL) improvement compared to FD-FDD IL can be applicable to all modulation order.

Observation 1: The Rel-18 TN and NTN specification have the power boosting framework in TS 38.101-1 clause 6.2.4 and TS 38.101-5 clause 6.2.4 respectively.

Proposal 2: The Rel-18 NTN power boosting framework in TS 38.101-5 clause 6.2.4 can be used as the starting point to add TX MOP increment due to HD-FDD insertion loss(IL) improvement compared to FD-FDD IL.

- Option 1: Add the NTN HD-FDD TX MOP increment scheme by extending TS 38.101-1/TS 38.101-5 power boosting framework as staring point.

| NR satellite band | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| n256 | 23x | +2, -2 |
| n255 | 23x | +2, -2 |
| NOTE X: The MOP limit increment requirements for HD-FDD operation are specified in clause of configured transmitted power. |  |  |

Proposal 3: It may be possible to have TX MOP increment due to FD-FDD IL improvement, but it is necessary to study what the comparison baseline is.

3 Conclusion

In this paper, our views on NTN-RedCap UE RF requirements in HD-FDD mode are provided as follows:

Proposal 1: The new output power enhancement for PC3 NTN (e)RedCap due to HD-FDD insertion loss(IL) improvement compared to FD-FDD IL can be applicable to all modulation order.

Observation 1: The Rel-18 TN and NTN specification have the power boosting framework in TS 38.101-1 clause 6.2.4 and TS 38.101-5 clause 6.2.4 respectively.

Proposal 2: The Rel-18 NTN power boosting framework in TS 38.101-5 clause 6.2.4 can be used as the starting point to add TX MOP increment due to HD-FDD insertion loss(IL) improvement compared to FD-FDD IL.

- Option 1: Add the NTN HD-FDD TX MOP increment scheme by extending TS 38.101-1/TS 38.101-5 power boosting framework as staring point.

| NR satellite band | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| n256 | 23x | +2, -2 |
| n255 | 23x | +2, -2 |
| NOTE X: The MOP limit increment requirements for HD-FDD operation are specified in clause of configured transmitted power. |  |  |

Proposal 3: It may be possible to have TX MOP increment due to FD-FDD IL improvement, but it is necessary to study what the comparison baseline is.

4 Reference

- RP-234078, “New WID: Non-Terrestrial Networks (NTN) for NR Phase 3”

- R4-2502281, “Way Forward for [114][310] NR_NTN_Ph3_UE_RF”, Qualcomm