3GPP TSG-RAN WG4 Meeting #114-bis	R4-2504432

Wuhan, China, 7 – 11th April 2025

Source:	BT

Title:	Channel Properties Comparison Methodologies

Agenda item:	7.16.2

Document for:	Discussion

Introduction

The Study on Spatial Channel Modelling for Demodulation Performance Requirements was approved at RAN#104 [1], and RAN4#112, RAN4#112b, RAN4#113 and RAN4#114 approved way forward agreements [2-5] respectively.

In this contribution we propose a set of spatial and temporal characteristics for comparison of the properties of channels generated by SCM candidates. The intended approach is to ensure that adopted SCM is robust to future requirements by capturing the key physical properties of measured channels.

Channel Properties Comparison

The spatial channel model should be robust and reliable enough to support both current known use cases (such as SU and MU PDSCH, and SU PMI), as well as future use cases yet to be identified (e.g. MU PMI, multi-cell, AI-related) without major modifications to the model. One way to achieve the robustness and reliability of the model is to ensure it reflect the physical properties of modelled channels with a reasonable accuracy, and therefore produces explainable results that are aligned with engineering expectations.

Measurement results highlighting properties of MIMO channels in commercial deployments been provided by several companies in support of the SCM study. For ease of reference, Table 1 below lists and summarizes some contributions to the study to date, and relevant channel properties reported.

| Tdoc number | Channel properties reported | Observations |
| --- | --- | --- |
| R4-2402277, Sec. 2.5 | Singular value distributions of measured 4x4 MIMO channels | Variability in the quality of spatial layers observed. |
| R4-2411557 | Long-term stability of singular values, cross-polarisation discrimination | Variations in channel eigenmodes characterised, long-term stability consistent with expected behaviour in a measurement scenario is observed. |
| R4-2415673 | Power delay profile | The power delay profiles from reflections of the transmitted reference signals exhibit stability across time (~250ms in this specific measurement sample) for static scenarios |
| R4-2419338 | Angle of Arrival | MIMO radio channels for stationary UE in typical deployments exhibit multiple long-term stable spatial directions of arrivals |
| R4-2501745 | Eigenmodes for 32x4 channels from commercial deployments; Angular spread | In typical MIMO deployments, dominant eigenmodes remain within a narrow high-gain range, while weaker eigenmodes are more spread out, leading to performance disparities among codewords |
| R4-2504431 | PMI statistics for sample measured 32x4 channels | The UE's preference for a specific PMI subset is observed in measured MIMO channels from typical deployments |

Measured MIMO channels from sample deployments exhibit distinct and observable spatial and temporal properties.

One approach to comparison of SCM candidates would consider alignment of the properties of the modelled channel to the properties of measured channels. A natural approach is to group the properties of the channels into spatial and temporal properties.

Therefore, we have the following proposals for the channel properties comparison methodologies:

SCM to produce channel samples with explainable and physically consistent properties, aligned with observations from field deployments.

Channel properties comparison to use at least spatial and temporal properties.

Spatial and temporal properties used for comparison to include at least:

  - Spatial properties:

    - Relative difference between singular of MIMO channels

    - Angles of arrivals/departure as generated by the model and their properties

    - Spatial preferences experienced by transmitter and receiver

    - Correlation, XPD

  - Temporal properties

    - Stability of channel features in time

    - Consistency and continuity of phase and amplitude of modelled channel in time and frequency domains

Conclusion

This contribution provides throughput simulation results for the use cases in [4]. Updated results will be submitted to RAN4#114 meeting through the results collection template in R4-2501743.

- Measured MIMO channels from sample deployments exhibit distinct and observable spatial and temporal properties.

- SCM to produce channel samples with explainable and physically consistent properties, aligned with observations from field deployments.

Channel properties comparison to use at least spatial and temporal properties.

Spatial and temporal properties used for comparison to include at least:

- Spatial properties:

- Relative difference between singular of MIMO channels

- Angles of arrivals/departure as generated by the model and their properties

- Spatial preferences experienced by transmitter and receiver

- Correlation, XPD

- Temporal properties

- Stability of channel features in time

- Consistency and continuity of phase and amplitude of modelled channel in time and frequency domains

References

- RP-241610, “3GPP Work Item Description: Study on spatial channel model for demodulation performance requirements”, Nokia, BT Plc, AT&T, Bell Mobility, Bouygues Telecom, China Telecom, CMCC, Deutsche Telekom, Ericsson, Intel Corporation, KDDI, Keysight, KT Corp., MediaTek, NTT Docomo, Orange, Qualcomm, Rohde & Schwarz, Samsung, SK Telecom, Spark NZ, Telecom Italia, Telefonica, Telenor, Telia Company, Telstra, T-Mobile USA, Verizon, Vodafone, ZTE Corporation

- R4-2413606, Way Forward for [112][327] FS_NR_demod_SCM, RAN4#112, Aug 2024.

- R4-2416591, Way Forward for [112b][328] FS_NR_demod_SCM, RAN4#112b, Oct 2024.

- R4-2419782, Way Forward for [113][320] NR_SCM, RAN4#113, Nov 2024.

- R4-2501745, “Measurement results for MIMO channels in band n78”, BT Plc, Ericsson