3GPP TSG-RAN WG4 Meeting #114bis	R4-2504447

Wuhan, China, April 7th – April 11th, 2025

Source:	Nokia

Title:	Mobility aspects related to extended SSB periodicity in NR over NTN

Agenda item:	7.29.4.2

Document for:	Discussion

Introduction

In [1], RAN plenary has approved a new WI for NTN, which will progress the effort of specification for NTN systems in 3GPP technologies in Rel-19 (NR_NTN_PH3). The WID contain a handful of objectives and among them, there is one topic on Downlink Coverage Enhancements. One of the main features discussed to be implemented as part of this feature is an extended SSB periodicity for the main (SA) cell in NTN, to match the periodic “active” and “inactive” periods of the cell. The switching between “active” and “inactive” is created in NTN to handle the limited power availability in the satellite.

The discussions of this WI in RAN4 were initiated in the meeting RAN4 #112. Discussions ensued in the following meetings, including RAN4#114 and as the outcome of the discussion a way-forward (WF) document was approved which contains a set of agreements and other way-forward (for further discussion) guidance for currently identified open issues [3].

In this document we provide our views related to current open issues on “Downlink Coverage Enhancements”.

Discussion

In this document we will discuss our views on some open issues related to the NTN mobility and other RRM aspects, when the minimum SSB periodicity is extended from 20 ms to 160 ms. Our discussions are based to a great extent on the open issues provided in the WF [3], in special the following list of issues listed there:

- Issue 1-1: Cell Reselection

- Issue 1-2-A: RRC Re-establishment (default TSMTC,i)

- Issue 1-2-B: RRC Re-establishment (SSB Ês/Iot < -8dB and SSB periodicity >20 ms)

- Issue 1-5: Paging

- Issue 1-6: SMTC configuration

Cell Reselection

In the previous meeting, RAN4 has discussed the cell reselection issue and whether any update is needed for the case where SSB periodicity is larger than 20 ms [3].

Issue 1-1: Cell Reselection Further discussion: - In cell reselection requirements, discuss whether any update is needed for case where SSB periodicity is larger than 20 ms: - Option 1 (Samsung): FFS - Option 2 (vivo): No update is needed

From a general point of view, the cell reselection requirements for terrestrial networks in NR – from which the NTN requirements were based on – were created considering cells in the standalone and non standalone scenarios. In non-standalone scenarios the SSB periodicity might be extended up to 160 ms.

The SSB periodicity of 160 ms is already an allowed configuration in specification.

Based on this, we do not see a particular justification to adjust the SSB periodicity based solely on a large periodicity of the SMTC configuration.

Do not change requirements for SMTC periodicity equal to 160 ms.

However, RAN2 has also advanced in their discussions about how the different transmit patterns on the “switch on/off” could be informed to the UE via RRC signalling.  During the last RAN2 meeting, it has been discussed different SSB periodicities in the same frequency layer, as follows:

RAN2 assumes it will be possible to have different SSB periodicity among neighbour cells in the same frequency layer. Can come back to this proposal

The periodicity of the SMTC is defined in SSB-MTC periodicityAndOffset. In SMTC4, the SMTC is configured for a group of cells in pci-List with an offset to compensate for the propagation delay difference between the serving and the neighbor cells in the list. The periodicity, tough, is the same as defined in SMTC. If the same periodicity is defined for all cells in the same frequency layer, and if cells in the same frequency layer are allowed different SSB periodicity, one of the following scenarios is true (assuming two different periodicities):

- The SMTC is configured to match the lowest SSB periodicity and there will be cells to be measured with larger SSB period than the current SMTC

- The SMTC is configured to match the largest SSB periodicity and there will be cells to be measured with shorter SSB period than the current SMTC.

In Case B, for every SMTC opportunity there will be SSB available of both cells, and there is no speficiation impact, although, it is an inefficient configuration for measuring on the cells with larger SSB period.

In case A, for some SMTC opportunities there will not be SSB available for the cells with larger SSB periods. This might impact UE implementation to perform meaningful measurements across the cells with larger SSB periodicity. Therefore it might be useful for the UE to be aware of the periodicity of the neighbor cells.

The periodicity and active patterns of the neighbor cells shall be informed to the UE to assist UE measurements.

We assume that the SSBs of DL-CE cells will be available for measurements at least with a default periodicity supported in NR: 20ms or 160 ms. But another aspect to be considered is that the cells might have different “active times” when DL-CE feature is implemented, in order to create a “rotation” of cells to share the satellite transmit power. The limitation on the satellite transmit power is the whole purpose behind the adoption of this feature. Because of this, for one frequency layer, multiple “offsets” might be used to cover for the different active times in the cells belonging to the same satellite.

When  multiple satellites are considered, with multiple different active times (with same periodicity but different offsets), it is easy to see that two SMTCs are not enough to cover for the most relevant mobility scenarios when this feature is implemented.

Based on this we propose:

For UEs supporting DL-CE feature in NR Rel-19, it shall be mandatory the support to 4 SMTCs per frequency layer.

RRC Re-establishment

It was also discussed in the previous meeting whether the UEs supporting DL-CE in Rel-19 should consider a different default value of TSMTC,I  when SMTC information for the inter-frequency layer is not provided..

Further discussion: - In RRC re-establishment requirements, update the value of TSMTC,i to 160ms for case where the SMTC for the inter-frequency carrier is not provided. - FFS: whether the above will apply only for UEs capable of coverage enhancement feature, if such a UE capability is defined. - FFS: whether the above is for only inter-frequency or for both intra-frequency and inter-frequency

If a UE supports DL-CE feature, it supports initial access up to 160 ms, and therefore, in the absence of further information, the UE shall also be capable of supporting RRC Re-establishment up to 160 ms. However, for legacy UEs, which do not support this feature, there might be backwards compatibility issues with Rel-17 and Rel-18 UEs, which support initial access assuming TSMTC,i. equal to 5 ms. So, it seems reasonable that:

For UEs that support DL Coverage Enhancement features, for the inter-frequency carrier, during RRC Re-establishment procedure Tsmtc,i is assumed equal to 160 ms.

For intra-frequency, the situation is different, there is no specification for TSMTC, when there is no SMTC configured in the intra-frequency layer, the reason being that there is always the expectation that SMTC is provided at least in the serving cell. Therefore,

For intra-frequency, the value of TSMTC. depends on the periodicity of the serving cell SMTC.

Another topic discussed was whether to update the requirements for RRC Re-establishment when the SSB Es/IoT for the serving cell is below -8 dB:

Issue 1-2-B: RRC Re-establishment (SSB Ês/Iot < -8dB and SSB periodicity >20 ms) Further discussion: - In RRC re-establishment requirements, discuss whether any update is needed to the TSMTC,i value for case where the SMTC for the inter-frequency carrier is not provided: - Option 1: Yes, an update is needed - Note: With the understanding that the by-default SSB periodicity for initial access is 160ms for Rel-19, from RRM requirement perspective. - Companies are encouraged to provide details on how to update the specification. - Other option is not precluded.

From our point of view, NOTE1 in Tables 6.2C.1.2.1-1 and Table 6.2C.1.2.1-2: was introduced due to the challenges to perform SMTC synchronization when the serving cell information is too poor:

Table 6.2C.1.2.1-1: Time to identify target NR cell for RRC connection re-establishment to NR intra-frequency cell Serving cell; FR of target NR; Tidentify_intra_NR [ms]. SSB Ês/Iot (dB); cell; Known NR cell; Unknown NR cell. ≥ -8; FR1; MAX (200 ms, 5 x TSMTC); Kmulti_SMTC * MAX (800 ms, 10 x TSMTC). < -8; FR1; N/A; k * 800 ms Note1, 3. NOTE 1: The UE is not required to successfully identify a cell on any NR frequency layer when TSMTC > 20 ms and serving cell SSB Ês/Iot < -8 dB. NOTE 2: Kmulti_SMTC is defined in clause 9.2C.5.1. NOTE 3: k = 1 if the cells on the target frequency are served by GEO. k = (N+1) if the cells on the target frequency are served by LEO, where N is the number of different satellites associated to the list of configured neighbour cells in ntn-NeighCellConfigList and ntn-NeighCellConfigListExt.. Table 6.2C.1.2.1-2: Time to identify target NR cell for RRC connection re-establishment to NR inter-frequency cell Serving cell SSB Ês/Iot (dB); FR of target NR cell; Tidentify_inter_NR, i [ms]. Known NR cell; Unknown NR cell. ≥ -8; FR1; MAX (200 ms, 6 x TSMTC, i); K_satellite * MAX (800 ms, 13 x TSMTC, i). < -8; FR1; N/A; k * 800 ms Note1, 3. NOTE 1: The UE is not required to successfully identify a cell on any NR frequency layer when TSMTC,i > 20 ms and serving cell SSB Ês/Iot < -8 dB. NOTE 2: K_satellite is defined in clause 9.3C.4. NOTE 3: k = 1 if the cells on the target frequency are served by GEO. k = (N+1) if the cells on the target frequency are served by LEO, where N is the number of different satellites associated to the list of configured neighbour cells in ntn-NeighCellConfigList and ntn-NeighCellConfigListExt..

The situation is not changed in NTN , regardless of the implementation of the DL-CE features or not. It is still difficult to obtain timing information from a disappearing (non-detectable levels) serving cell.

Do not update the RRC Re-establishment requirements when SSB Es/IoT for the serving cell is below -8 dB.

Paging interruption

Paging interruption requirements state that:

#### 4.2C.2.5 Maximum interruption in paging reception UE shall perform the cell re-selection with minimum interruption in monitoring downlink channels for paging reception. At intra-frequency and inter-frequency cell re-selection, the UE shall monitor the downlink of serving cell for paging reception until the UE is capable to start monitoring downlink channels of the target intra-frequency and inter-frequency cell for paging reception. The interruption time shall not exceed TSI-NR + K*Ttarget_cell_SMTC_period ms. Where, If the target cell belongs to the same satellite as the current one, and if the target cell is known, then K = 2. If the target cell belongs to a different satellite than the current one and the target cell’s satellite is GEO, and if the target cell is known, then K = 2. If the target cell belongs to a different satellite than the current one and the target cell’s satellite is non-GEO, then K = 5 if the target cell is known.

The multiplying factor is added to create to allow the UE to have enough time to acquire DL synchronization to the target cell, in order to be capable to receive the paging messages. However, if a non-GEO target cell is used the current multiplying factor is equal to 5, which was justifiable with short SSB configurations, however, if the same scaling factor is maintained when the SSB periods is equal to 160 ms, the total interruption time will be given by: 5*160=900ms + TSI-NR might be expected, which may exceed 1 second of interruption in a system with already very long natural interruption times due to cell DTX.

Therefore, we propose that:

Adjust the paging interruption requirements when SSB periods equal or larger 160 ms are used in NGSO scenarios.

Conclusion

Upon analyzing the current open issues related to SMTC configurations and extended SSB periodicity, as well as the “on/off” patterns to be introduced in satellite cells, we drew several conclusions and made proposals to deal with the issues that need to be addressed.

Observation 1: The SSB periodicity of 160 ms is already an allowed configuration in specification.

Proposal 1: Do not change requirements for SMTC periodicity equal to 160 ms.

Proposal 2: The periodicity and active patterns of the neighbor cells shall be informed to the UE to assist UE measurements.

Proposal 3: For UEs supporting DL-CE feature in NR Rel-19, it shall be mandatory the support to 4 SMTCs per frequency layer.

Proposal 4: For UEs that support DL Coverage Enhancement features, for the inter-frequency carrier, during RRC Re-establishment procedure Tsmtc,i is assumed equal to 160 ms.

Proposal 5: For intra-frequency, the value of TSMTC. depends on the periodicity of the serving cell SMTC.

Proposal 6: Do not update the RRC Re-establishment requirements when SSB Es/IoT for the serving cell is below -8 dB.

Proposal 7: Adjust the paging interruption requirements when SSB periods equal or larger 160 ms are used in NGSO scenarios.

- References

- RP-234078, New WID: Non-Terrestrial Networks (NTN) for NR Phase 3, Huawei

- RP-241789 Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3, Thales, CATT

- R4-2420105, " WF on RRM requirements for NR_NTN_Ph3_Part2", Moderator (Qualcomm Incorporated), RAN4 #114 Orlando, US, 18th – 22nd November, 2024.